3GPP TSG-RAN WG2 Meeting #115 electronic 	R2-2107048

Online, August 16 - 27, 2021 						     Revision of R2-2104949

Title: 	Mobility and Service continuity for NR Multicast

Source: 	         MediaTek Inc.

Agenda item: 	8.1.2.1

Document for: 	Discussion and Decision

# Introduction

In RAN2 #111-e and RAN2 #112-e Meeting, mobility with service continuity for NR MBS was discussed and following agreements were made:

Agreements in RAN2 #111-e:

Focus on MBS-MBS scenario initially (i.e. shared delivery), including both PTM and PTP (if applicable). Other scenarios later, TBD. Requirements for lossless mobility are TBD. Assume for now that R2 will anyway discuss service continuity functionality for low or no data loss. R2 assumes that for Rel-17 NR multicast Mobility in Connected mode, HO (including variants) is the baseline, TBD exactly which variants.

Agreements in RAN2 #112-e:

R2 aim to support lossless HO for MBS-MBS mobility for service that requires this (TBD which detailed scenario but at least PTP-PTP) In order to support the lossless HO for 5G MBS services, at least DL PDCP SN synchronization and continuity between the source cell and the target cell should be guaranteed by the network side to realize. The design of specific approach to realize this can be involved with WG RAN3. From network side, the source gNB may forward the data to the target gNB and the target gNB will deliver the forwarding data. Meanwhile, the SN STATUS TRANSFER should be extended to cover the PDCP SN for MBS data; Then (TBD after or in parallel) the UE receives the MBS in the target cell by the target cell according to target configuration. From UE side, PDCP status report may be supported as well.

In this paper, we further discuss the HO procedure for NR MBS and clarifies the TBD issues.

# Discussion

### HO variations as baseline

In NR Rel-16, CHO and DAPS were introduced for mobility enhancement. Therefore, current NR supports three types of HO:

- Basic HO

- CHO HO

- DAPS HO

During previous RAN2 Meetings, RAN2 assumed that for Rel-17 NR multicast mobility in Connected mode, HO (including variants) is the baseline, TBD exactly which variants. Since both CHO and DAPS are optional features, which are not necessarily commercialized, multicast mobility with service continuity should support the basic HO mechanism. The next question is whether CHO and DAPS can be supported for multicast mobility.

Assuming source gNB transfers the related information/configuration of the MBS service to the target gNB in HO REQUEST, the source gNB transfers those information to the candidate gNBs in CHO. Only the target gNB that can provide the same MBS service can respond HO REQUEST ACKNOWLEDGE. In other words, the candidate cells provided to the UE in CHO command should be able to provide the same MBS services as the source cell.

Proposal 1: CHO HO can be supported for multicast mobility. The candidate cells prepared for the UE should be able to provide the same MBS services as the source cell.

For NR MBS, HO with different MBS transmission manners, i.e.  PTM, PTP and PTM+PTP should be considered. No matter which protocol stack is selected for reliability enhancement, it is expected the UE needs to receive two DL physical channels addressed by different RNTIs simultaneously for reliability enhancement. One is for PTM and the other one is for PTP. If DAPS is supported for multicast mobility, the UE may need to receive four physical channels (i.e. monitor PDCCH with four RNTIs) simultaneously in the worst case when HO is performed between PTM+PTP transmission, just as illustrated in Figure 1. It requires higher UE process capabilities and is not desirable.

DAPS can be limited to certain HO scenarios, i.e. PTP->PTP, PTP->PTM, PTM->PTP and PTM->PTM. Those four scenarios have the same requirement on UE capability as legacy DAPS HO, where UE only receives DL from two physical channels. Since DAPS can help to reduce the service interruption, it can be supported in multicast mobility for those scenarios.

Figure 1 PTM+PTP<-> PTM+PTP HO

Proposal 2: DAPS HO can be supported for multicast mobility in the scenarios of PTP->PTP, PTM->PTM, PTP->PTM and PTP->PTM HO. UE is not required to receive DL data on more than two DL physical channels simultaneously during HO.

### Scenarios to support lossless HO

Lossless HO requires that there is no PDCP PDU loss for multicast services during HO. According to the agreements, in order to support the lossless HO for 5G MBS services, at least DL PDCP SN synchronization and continuity between the source cell and the target cell should be guaranteed.

Certain 5G MBS services and use cases, e.g. public safety and mission critical services have high reliability requirement. That’s why a unicast channel is associated to PTM transmission to guarantee that the MBS services are delivered without packet loss. The same reliability requirement should be satisfied during HO procedure. However, if the MBS services doesn’t have high reliability requirement and are delivered in PTM transmission without unicast channel, lossless HO doesn’t need to be supported.

Proposal 3: Lossless HO is not supported for the following scenarios:

- Scenario 2.2: PTP->PTM without unicast;

- Scenario 3.2: PTM without unicast->PTP;

- Scenario 4.2: PTM without unicast->PTM without unicast.

How to realize lossless HO for the following 3 scenarios should be considered.

- Scenario 2.1: PTP->PTM with unicast;

- Scenario 3.1: PTM with unicast->PTP;

- Scenario 4.1: PTM with unicast->PTM with unicast.

In the above three scenarios with high reliability requirement, a unicast channel is always available for the UE in both the source cell and the target cell. In general, the data packets loss at the source cell can be retransmitted by the target cell through the unicast channel.

Proposal 4: Lossless HO is supported for the following scenarios:

- Scenario 1: PTP->PTP;

- Scenario 2.1: PTP->PTM with unicast;

- Scenario 3.1: PTM with unicast->PTP;

- Scenario 4.1: PTM with unicast->PTM with unicast.

### Lossless HO Operation

If DL PDCP SN continuity for the same MBS service can be maintained by the network side, current procedure for lossless HO can be reused to avoid packet loss. Same as current HO procedure, the source gNB should forward the data to the target gNB and the target gNB can deliver the forwarding data to the UE(s). Meanwhile, the SN STATUS TRANSFER should be extended to cover the PDCP SN for MBS data; then the UE can receive the MBS in the target cell. It is for further study whether the UE reception of MBS in target cell is after or in parallel with the data forwarding from the source to the target gNB.

With the basic HO procedure, UE starts to receive the data from the target cell after the HO is successfully completed. This means the UE reception of the data from target cell actually starts after SN STATUS TRANSFER message and the data forwarding from the source to the target gNB.  We assume the lossless HO is supported for the scenarios with unicast channel (scenario 1, 2.1, 3.1 and 4.1) by legacy mechanism, then the UE should receive the MBS data in the target cell.

Proposal 5: As a baseline for lossless HO, UE receives the MBS data from the target cell after SN STATUS TRANSFER from the source gNB to the target gNB.

However there may be a difference between MBS transmission and unicast transmission, since the MBS data may be simultaneously transmitted from 5GC to the target gNB, and then for MBS based handover, there may be no need to perform the data forwarding from the source gNB to the target gNB, as long as the same PDCP SN allocation is applied for both source gNB and target gNB and target gNB still buffers the data.

Proposal 5a: FFS if the source gNB need to perform data forwarding to the target gNB during lossless HO for MBS.

Generally, the data loss should be minimized for multicast mobility in all of the HO scenarios.  The scenarios where the MBS service is on-going with PTM transmission at the target cell need to be considered further, just as illustrated in Figure 2. This scenario is different from unicast service that the target delivers the data packets to the UE only after HO is completed.

There is a time duration during which UE can’t perform data transmission/reception during the HO procedure.  If there is on-going MBS service at the target cell when UE performs basic HO, some packets will be lost and can’t be recovered. In the example of Figure 2, UE receives PDCP PDU 1 and 2 at the source side. When UE performs HO towards the target cell, the target cell is transmitting PDU 3~11. When HO is successfully completed, UE receives the PDUs starting from 12. How to minimize the data loss need to be considered.

If DAPS HO is supported for multicast mobility in those scenarios, the data loss can be minimized, since UE can receive data simultaneous from both sides. If DAPS HO is not supported, the target cell should buffer certain amount of DL data packets continuously in case some UEs are moved to its coverage requiring the same MBS service. For example, the network can start to buffer some PDCP PDUs for the UE when it receives HO REQUEST and respond HO REQUEST ACKNOWLEDGE.

Figure 2 PTM<->PTM HO, PTP->PTM HO

Proposal 6: RAN2 consider to minimize the data loss during HO where the MBS service is on-going with PTM transmission at the target cell: utilize DAPS HO if UE capability allowed or buffer DL data at the target side.

# Conclusion

Proposal 1: CHO HO can be supported for multicast mobility. The candidate cells prepared for the UE should be able to provide the same MBS services as the source cell.

Proposal 2: DAPS HO can be supported for multicast mobility in the scenarios of PTP->PTP, PTM->PTM, PTP->PTM and PTP->PTM HO. UE is not required to receive DL data on more than two DL physical channels simultaneously during HO.

Proposal 3: Lossless HO is not supported for the following scenarios:

- Scenario 2.2: PTP->PTM without unicast;

- Scenario 3.2: PTM without unicast->PTP;

- Scenario 4.2: PTM without unicast->PTM without unicast.

Proposal 4: Lossless HO is supported for the following scenarios:

- Scenario 1: PTP->PTP;

- Scenario 2.1: PTP->PTM with unicast;

- Scenario 3.1: PTM with unicast->PTP;

- Scenario 4.1: PTM with unicast->PTM with unicast.

Proposal 5: As a baseline for lossless HO, UE receives MBS in the target cell after SN STATUS TRANSFER from the source gNB to the target gNB.

Proposal 5a: FFS if the source gNB need to perform data forwarding to the target gNB during lossless HO for MBS.

Proposal 6: RAN2 consider to minimize the data loss during HO where the MBS service is on-going with PTM transmission at the target cell: utilize DAPS HO if UE capability allowed or buffer DL data at the target side.

# Annex: Handover scenarios to support service continuity

Scenarios to support service continuity during HO