3GPP TSG-RAN WG2 Meeting #115-electronic	 R2-2107056

Online, August 16th – August 27th, 2021                       Revision of R2-2104763

Source: 	vivo

Title:  	Supporting Small Data Transmission via RA Procedure

Agenda Item:	8.6.4

Document for:	Discussion and Decision

Introduction

In past RAN2 meetings, the following agreements regarding general RA-SDT procedure and PRACH configuration for SDT had been achieved [1][2]:

Agreements: - RSRP threshold for carrier selection is specific to SDT (i.e. separately configured for SDT). This is optional for the network. Confirm that cell selection mechanism is not modified. - RSRP threshold for RA type selection is specific to SDT (i.e. separately configured for SDT). - FFS on the order and missing pieces (e.g. failure, fallback) of the high level procedure. The details of the procedures are left for stage 3. FFS on the procedure below, but copied for information. A. Upon arrival of data only for DRB/SRB(s) for which SDT is enabled, the high level procedure for selection between SDT and non SDT procedure is as follows: If CG-SDT criteria is met: UE selects CG-SDT. UE initiate SDT procedure Else if RA-SDT criteria is met: UE selects RA-SDT. UE initiate SDT procedure Else: UE initiate non SDT procedure. B. CG-SDT criteria is considered met, if all of the following conditions are met, 1) available data volume <= data volume threshold 2) RSRP is greater than or equal to a configured threshold FFS 3) CG-SDT resources are configured on the selected UL carrier and are valid C. RA-SDT criteria is considered met, if all of the following conditions are met, 1) available data volume <= data volume threshold 2) RSRP is greater than or equal to a configured threshold 3) 4 step RA-SDT resources are configured on the selected UL carrier and criteria to select 4 step RA SDT is met; or 2 step RA-SDT resources are configured on the selected UL carrier and criteria to select 2 step RA SDT is met - CFRA is not supported for RA-SDT. - The separate search space is common to the UEs performing RA-SDT. Inform RAN1 of this agreement. - The UE needs to monitor paging after UE initiates SDT for system information change, PWS. FFS for other cases.

In this contribution, we would like to provide our understanding of the RACH based small data transmission procedure in terms of:

- PRACH configuration

- Subsequent transmission

- Fallback and switching

Discussion

PRACH configuration

According to the WID [3], it can be concluded that RA-SDT shall be performed in the RRC_INACTIVE state. Then the first question that comes to us is how does the UE acquire the RA-SDT resource (e.g. PRACH resource in time/frequency/code domain). Generally, we think there are two feasible options:

Option 1: The SDT configuration is sent via the RRC Release message.

Option 2: The SDT configuration is sent via the broadcast channel (e.g. SIB1).

Currently, the CG-SDT procedure with UE-dedicated CG resources has already been studied in the WID. On the other hand, RA-SDT can be performed on a serving cell which is different than the serving cell where the RRC Release message is previously received. Thus, to achieve resource efficiency, we think only contention-based preamble can be used for Rel-17 NR SDT. In this sense, we think the RA-SDT related PRACH resource should be configured within SIB1.

Proposal 1: RA-SDT related PRACH resources are configured within SIB1.

Next, considering it was agreed that RSRP thresholds for carrier selection (i.e. rsrp-ThresholdSSB-SUL) and RA type (i.e. msgA-RSRP-Threshold) selection are specific to SDT. Then it seems a spontaneous logic the other RSRP related to RA resource selection can be specific to SDT. For example, the RSRP thresholds for SSB selection (i.e. rsrp-ThresholdSSB and msgA-RSRP-ThresholdSSB). Thus, we propose,

Proposal 2: RSRP threshold for SSB selection is specific to SDT.

Subsequent transmission

In the previous RAN2 meeting, it was confirmed that a separate common search space (CSS) can be configured for the UEs performing RA-SDT. Then it is worthy to consider how does the UE monitor PDCCH candidates in this new separate CSS during the subsequent transmission (i.e. after contention resolution but before the completeness of RA-SDT procedure).

In legacy NR, according to the following quoted text from TS 38.213 sub-clause 10.1,

If a UE is provided - one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, and - a C-RNTI, an MCS-C-RNTI, a CS-RNTI, a SL-RNTI, a SL-CS-RNTI, or a SL Semi-Persistent Scheduling V-RNTI the UE monitors PDCCH candidates for DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI, the MCS-C-RNTI, or the CS-RNTI in the one or more search space sets in a slot where the UE monitors PDCCH candidates for at least a DCI format 0_0 or a DCI format 1_0 with CRC scrambled by SI-RNTI, RA-RNTI, MsgB-RNTI, or P-RNTI.

We can learn that the UE can monitor consecutively PDCCH address to C-RNTI in the Type0/0A/1/2-PDCCH CSS even in IDLE/INACTIVE state after contention resolution (i.e. the contention resolution id and RRC Setup/Resume message are not multiplexed in the same MAC PDU). Considering that the separate search space for RA-SDT is also common, we think the same design logic can be reused for RA-SDT. That is, the UE monitors PDCCH addressed to C-RNTI in the separate search space for RA-SDT after successful contention resolution if the SDT detection timer is running. Also, we should consult with RAN1 whether this assumption is feasible or not.

Proposal 3: UE monitors PDCCH addressed to C-RNTI in the separate CSS for RA-SDT after successful contention resolution only if the SDT detection timer is running. Send an LS to RAN1 asking whether it is feasible.

During the NR SDT procedure, subsequent transmission can be performed after the first UL transmission with state transition controlled by the network. An example of subsequent transmission within RA-SDT procedure is depicted below. Generally, we think the subsequent transmission is mainly used to provide supplementary UL grants to accommodate the pending that triggers the RA-SDT procedure. Multiple round trips of data transfer are possible during this stage is not expected. Otherwise, PHY measurement and reporting might be needed to require the RRM requirement, which is harmful to UE power saving. If there are a lot of data to be transmitted due to the RAR grant of small TBS, the UE should be able to indicate some UE assistance information for the network. Meanwhile, a reasonable network implementation should be transitting the INACTIVE UE into RRC CONNECTED state, or using a multi-TTI UL grant. Based on this, from RAN2 perspective, we think neither BWP switching operation nor RRC reconfiguration is expected during the RA-SDT procedure. For the multiple-TTI UL grant, RAN2 should send an LS to RAN1 asking whether this is feasible for NR SDT.

Figure 1 Subsequent transmission within RA-SDT procedure

Proposal 4: Only initial BWP is supported for RA-SDT.

Proposal 5: RRC reconfiguration is not supported for RA-SDT.

Fallback and switching

For NR SDT, considering that far more resource blocks need to be allocated to the MsgA PUSCH for UL UP data transmission, the transmission becomes less reliable, compared to the legacy MsgA PUSCH transmission. As a result, the gNB may not successfully decode the MsgA PUSCH with UL UP data due to the time-varying characteristics of the wireless channel and the lack of valid time alignment.

To improve transmission robustness, the legacy fallback and switching mechanism for 2-step RACH should be reused for 2-step RACH based SDT. For example, the network can control the number of times 'N'. If the 2-setp RACH based SDT procedure is not successfully completed even after 'N' times of MsgA transmission, the UE will fallback to use 4-step RACH based SDT resources, if configured, and continue the small data transmission procedure.

Proposal 6: The Rel-16 fallback and switching mechanism for 2-step RACH are reused for 2-step RACH based SDT procedure.

Conclusions

In this contribution, we have provided our understanding of supporting small data transmission via RA procedure. The following proposals are made:

PRACH configuration:

Proposal 1: RA-SDT related PRACH resources are configured within SIB1.

Proposal 2: RSRP threshold for SSB selection is specific to SDT.

Subsequent transmission:

Proposal 3: UE monitors PDCCH addressed to C-RNTI in the separate CSS for RA-SDT after successful contention resolution only if the SDT detection timer is running. Send an LS to RAN1 asking whether it is feasible.

Proposal 4: Only initial BWP is supported for RA-SDT.

Proposal 5: RRC reconfiguration is not supported for RA-SDT.

Fallback and switching:

Proposal 6: The Rel-16 fallback and switching mechanism for 2-step RACH are reused for 2-step RACH based SDT procedure.

- 5. Reference

[1] RAN2#113-e Session Chair Notes (Small data transmission), E-meeting, 25th January - 5th February 2021.

[2] RAN2#114-e Session Chair Notes (Small data transmission), E-meeting, 19th May - 27th May 2021.

[3] 3GPP RP-200954, “New study item: NR smalldata transmissions in INACTIVE state”, ZTE Corporation, RAN#88, E-meeting, June 2020.