3GPP TSG-RAN WG2 Meeting #115-electronic	 R2-2107057

Online, August 16th – August 27th, 2021

Source: 	vivo

Title:  	Supporting small data transmission via CG PUSCH

Agenda Item:	8.6.5

Document for:	Discussion and Decision

Introduction

In past RAN2 meetings, the following agreements regarding the CG-SDT had been achieved [1] - [4]:

Agreements: CG-SDT configuration aspects: - CG-SDT resource configuration is provided to UEs in RRC_Connected only within the RRCRelease message, i.e. no need to also include it in RRCReconfiguration message. - RRCRelease message is used to reconfigure or release the CG-SDT resources while UE is in RRC_INACTIVE. - CG-PUSCH resources can be separately configured for NUL and SUL. FFS if we allow them at the same time. This depends on the alignments CRs for Rel-16. - As a baseline assumption, it’s a network configuration issue whether to support multiple CG-SDT configurations per carrier in RRC_INACTIVE (i.e. we will not restrict network configuration for now). - CG-SDT resources can be configured at the same time on NUL and SUL. - Working assumption: UE-specific search space is configured for UEs performing CG-SDT. RAN2 asks RAN1 whether this working assumption can be confirmed. - CG-SDT resource can be configured on either initial BWP or separate SDT BWP. Ask RAN1 to confirm. - Release of CG-SDT configuration by system information indication is not supported. TAT validation aspects: - A new TA timer for TA maintenance specified for configured grant based small data transfer in RRC_INACTIVE should be introduced. FFS on the procedure, the validity of TA, and how to handle expiration of TA timer. The TA timer is configured together with the CG configuration in the RRCRelease message. - TAT-SDT is started upon receiving the TAT-SDT configuration from gNB, i.e. RRCrelease message, and can be (re)started upon reception of TA command. - From RAN2 point of view, assume similar to PUR, that we introduce a TA validation mechanism for SDT based on RSRP change, i.e. RSRP-based threshold(s) are configured. Ask RAN1 to confirm. FFS on how to handle CG configuration when TA expires or when is invalid due to RSRP threshold. Details of the TA validation procedure can be further discussed. - UE releases CG-SDT resources when TAT expires in RRC_Inactive state. CG-SDT evaluating aspects: - The configuration of configured grant resource for UE small data transmission is valid only in the same serving cell. FFS for other CG validity criteria (e.g. timer, UL/SUL aspect, etc). - The UE can use configured grant based small data transfer if at least the following criteria is fulfilled - user data is smaller than the data volume threshold; - configured grant resource is configured and valid; - UE has valid TA. FFS for the candidate beam criteria. - FFS Discuss further in stage 3 how to specify the agreement that CG-SDT resources are only valid in one cell (i.e. cell in which RRCRelease is received). - CG resource availability delay is not considered as a criterion for CG validation SSB-CG PUSCH association aspects: - From RAN2 point of view: An association between CG resources and SSBs is required for CG-based SDT. FFS up to RAN1 how the association is configured or provided to the UE. Send an LS to RAN1 to start the discussion on how the association can be made. Mention that one option RAN2 considered was explicit configuration with RRC Release message. General procedure aspects: - A SS-RSRP threshold is configured for SSB selection. UE selects one of the SSB with SS-RSRP above the threshold and selects the associated CG resource for UL data transmission. - For CG-SDT, the subsequent data transmission can use the CG resource or DG (i.e dynamic grant addressed to UE’s C-RNTI). Details on C-RNTI, can be the same as the previous C-RNTI or may be configured explicitly by the network can be discussed in stage 3. - Implicit release of CG-SDT resource is not supported. - UE start a window after CG/DG transmission for CG-SDT. FFS whether to design a new timer or to reuse an existing timer. - Support retransmission by dynamic grant for CG-SDT. - Support multiple HARQ processes for uplink CG-SDT. - UL carrier selection is performed before CG-SDT selection. - The UE needs to monitor paging after UE initiates SDT for system information change, PWS. FFS for other cases. - FFS CS-RNTI based dynamic retransmission is reused for CG-SDT. - RAN2 thinks that some feedback may be beneficial in case CG is used for subsequent transmission. RAN2 assumes that existing mechanism can be used. - For initial CG transmission, UE does not select any SSB if none of the SSBs’ RSRP is above the RSRP threshold. FFS if re-evaluation for every CG transmission is necessary. - FFS Switching from CG-SDT to RA-SDT is not allowed.

Based on the above agreements, in this contribution, we would like to further provide our considerations on the CG-SDT procedure.

Discussion

SSB selection and CG initialization

According to the current achieved agreement for CG-SDT, it can be known that CG-SDT can only be selected when there is at least one SSB with SS-RSRP above an SDT-specific threshold. In NR RACH procedure, to determine if there is an SSB with SS-RSRP above the SSB selection threshold (e.g. rsrp-ThresholdSSB), the UE uses the latest unfiltered L1-RSRP measurement. We think the same logic can also apply to CG-SDT. Therefore, we propose,

Proposal 1: To determine if there is an SSB with SS-RSRP above the SSB selection threshold for CG-SDT, the UE uses the latest unfiltered L1-RSRP measurement.

In the email discussion [Post114-e][508][SData], the rapporteur leaves a remaining issue that what should UE do if the UE cannot finish SSB evaluation before the next CG occasion if SSB re-evaluation is supported for every CG transmission [5]. In our understanding, the UE should not store and initialize the CG-SDT resources before finding a suitable SSB. In other words, different from the existing UE behavior that storing and initializing the type-1 CG configuration upon receiving the RRC configuration, the UE should only deliver the CG-SDT UL grant and the associated HARQ information to the MAC entity after selecting CG-SDT. In this sense, the UE anyway can finish the SSB evaluation before the very first initial transmission on the CG transmission occasion. Then if the periodicity of CG is smaller than the minimum timing requirement for SSB evaluation, based on our proposal 1, we think the latest already obtained measurement result can be regarded as the latest measurement result. Subsequently, the UE can still select an SSB and the associated UL grant for the next CG transmission occasion. And then the UE may have sufficient time to update the measurement result for the next and next CG transmission occasion. Based on the above analysis, we think there is no observed technical issue if UE attempts to select an SSB for every CG transmission in CG-SDT. An illustration is given in the flowing figure.

Figure 1: CG-SDT procedure

Proposal 2: UE delivers the CG-SDT UL grant and the associated HARQ information to the MAC entity only after CG-SDT is selected.

Proposal 3: For CG-SDT, RAN2 confirms that SSB selection is performed for every CG transmission.

Based on our proposal 3, after the very initial transmission, the UE has to re-select an SSB for the subsequent transmission on CG transmission occasions. Due to the time-varying characteristic of radio conditions, it is possible that no SSB with SS-RSRP above the configured threshold is available, then should the UE performs fallback to RA-SDT procedure if the corresponding conditions are fulfilled? Or just select any SSB? In our understanding, considering the network can reach the UE within the monitoring window and dynamic DG scheduling (potentially with robust MCS scheme and repetition indication) can be available, we prefer the latter since it helps to simplify the fallback case and relieve the data loss issue.

Proposal 4: After CG-SDT is selected, UE is allowed to select any SSB associated with CG-SDT resource if none of SSB with SS-RSRP above the SSB selection threshold is available.

Response reception

In the email discussion [Post114-e][508][SData], it is proposed that a new window is introduced for CG-SDT. Then it is worthy for us to further consider the detailed operation and UE behavior after its expiry. Basically, in our view, this timer is introduced for PDCCH monitoring and USS is supposed to be used for CG-SDT. Then, it seems spontaneous logic that the new timer should be started at the first PDCCH occasion from the end of the CG-SDT PUSCH transmission. Then if a PDCCH-based response (e.g. DL assignment or dynamic UL grant or L1-level ACK) is successfully received by the UE, the UE can stop this window since there is no need to monitor the network response until the next transmission.

Proposal 5: For CG-SDT, the new timer is started at the first PDCCH occasion from the end of the CG-SDT PUSCH transmission or CG PUSCH transmission or the corresponding transmission carrying the DL HARQ feedback.

Proposal 6: For CG-SDT, as the baseline, the new timer is stopped when a PDCCH transmission is successfully received.

Other aspects

Due to the poor radio condition, a UE triggering CG-SDT may suffer from the expiry of the new timer without successfully receiving any PDCCH-based feedback. In this scenario (e.g. the UE is not touchable from the network side), to improve the link robustness, the UE should perform retransmission in the next available transmission occasion of CG PUSCH. Further, a power ramping operation should be also considered to raise the transmit power. Otherwise, the UE may consistently get stuck in the poor radio quality environment, purely waiting for the expiry of SDT failure detection timer.

Proposal 7: Autonomous retransmission on CG resources is supported for CG-SDT.

Proposal 8: Power ramping is supported for CG-SDT.

To facilitate the autonomous retransmission, we think we should break the rule that the HARQ process ID is bundled with time-domain CG transmission occasions. Instead, we think the UE should be allowed to select a HARQ process for autonomous retransmission or new initial transmission based on UE implementation, which is supposed to be similar to CG transmission in NR-U WI. Consequently, the CG-UCI mechanism should be also supported for HARQ process ID reporting (needs to be confirmed by RAN1).

Proposal 9: For CG-SDT, HARQ process ID is selected by UE implementation.

Proposal 10: For CG-SDT, CU-UCI reporting is supported (check with RAN1).

Conclusions

In this contribution, we have provided our understanding of the CG-SDT procedure. The following proposals are made:

SSB selection and CG initialization:

Proposal 1: To determine if there is an SSB with SS-RSRP above the SSB selection threshold for CG-SDT, the UE uses the latest unfiltered L1-RSRP measurement.

Proposal 2: UE delivers the CG-SDT UL grant and the associated HARQ information to the MAC entity only after CG-SDT is selected.

Proposal 3: For CG-SDT, RAN2 confirms that SSB selection is performed for every CG transmission.

Proposal 4: After CG-SDT is selected, UE is allowed to select any SSB associated with CG-SDT resource if none of SSB with SS-RSRP above the SSB selection threshold is available.

Response reception:

Proposal 5: For CG-SDT, the new timer is started at the first PDCCH occasion from the end of the CG-SDT PUSCH transmission or CG PUSCH transmission or the corresponding transmission carrying the DL HARQ feedback.

Proposal 6: For CG-SDT, as the baseline, the new timer is stopped when a PDCCH transmission is successfully received.

Other aspects:

Proposal 7: Autonomous retransmission on CG resources is supported for CG-SDT.

Proposal 8: Power ramping is supported for CG-SDT.

Proposal 9: For CG-SDT, HARQ process ID is selected by UE implementation.

Proposal 10: For CG-SDT, CU-UCI reporting is supported (check with RAN1).

- 5. Reference

[1] RAN2#112-e Session Chair Notes (Small data transmission).

[2] RAN2#113-e Session Chair Notes (Small data transmission).

[3] RAN2#113bis-e Session Chair Notes (Small data transmission).

[4] RAN2#114-e Session Chair Notes (Small data transmission).

[5] R2-2107930, Report of [Post114-e][508][SData] Open issues for CG-SDT.