3GPP TSG-RAN WG2 Meeting #115-e	R2-2107156

Online, 9th – 27th Aug, 2021                                                          	Revision of R2-2105083

Agenda item:	8.15.2

Source: 	Huawei, HiSilicon

Title: 	Remaining issues on the sidelink DRX for unicast

Document for:	Discussion and Decision

# Introduction

In this contribution, we mainly discuss the remaining issues for the sidelink DRX configuration for unicast, the alignment between SL resource and SL DRX configuration, the handling of HARQ feedback disabled transmission and the handling of PSFCH transmission/reception failure, considering the achieved agreements and the ongoing email discussions.

# Discussion

## How to configure SL DRX configuration

In RAN2#114-e meeting, the question of which party to configure the DRX configuration for a certain unicast link was concluded, and the following agreement was agreed:

For SL unicast, TX-UE centric DRX configuration based on the assistance information from RX-UE is agreed as baseline.

It means that TX UE or its gNB determines the DRX configuration for its peer UE’s reception. As illustrated in Figure 1(a), a TX UE or its gNB determines the DRX configuration for each RX UE it is connected with based on some information e.g. QoS, traffic pattern etc., and signals the DRX configuration to each RX UE via PC5-RRC. The RX UE follows the DRX configuration for reception from the TX UE. When the RX UE performs SL communication with multiple TX UEs, it needs to maintain/coordinate different sets of DRX configurations from these TX UEs simultaneously.

Figure 1:  TX UE centric DRX configuration

In TX UE centric DRX configuration, the resource configuration for SL services is either determined by TX UE or by the TX UE’s serving gNB, so it is convenient to adapt SL DRX configuration to the SL resource configuration by making the SL grants within the active time for the RX UE as much as possible to ensure the transmission performance of data. Nevertheless, due to “M-to-1” nature of SL, one RX UE may perform SL communication with multiple TX UEs simultaneously. Since it is not possible to negotiate among the TX UEs, the ”wake up” time configured by these TX UEs may spread over most of the whole time domain, leading to DRX unrealized at all. To overcome the disadvantages of the TX centric mechanism, some coordination can be considered that, the assistance information is transmitted from RX UE to TX UE and the TX UE determines the DRX configuration while taking the assistance information into account. This TX UE centric DRX configuration based on the assistance information from RX UE is shown in Figure 2.

Figure 2:  TX UE centric DRX configuration based on the assistance information from RX UE

This is a so-called two-way mechanism, something like what we already have in Rel-16. For this mechanism, when determining the DRX configuration, both power consumption and service requirement/resource configuration can be considered at the same time to achieve balance between the data transmission performance and saving the UE power.

Specifically, RX UE may transfer to TX UE some assistance information which may include the Uu DRX configuration of RX UE, the suggested SL DRX configuration, configured SL DRX configurations for other associated SL connections of the RX UE and the power saving requirement of RX UE. e.g., whether power saving enhancement is required by the RX UE. Before RX UE is configured by TX UE with SL DRX, it may has been configured by its gNB with Uu DRX. Considering both Uu DRX and SL DRX will impact the power consumption of RX UE, it is better to make the active time of the SL DRX to overlap with that of the RX UE’s Uu DRX as much as possible. Thus, it is necessary for RX UE to provide its Uu DRX configuration to TX UE for SL DRX configuration determination. Moreover, to reduce the RX UE’s power consumption further, it is reasonable for the TX UE to take the assistance information of the suggested SL DRX configuration and configured SL DRX configurations for other associated SL connections of the RX UE into account to make the active time of the SL DRX configuration to overlap with that of the RX UE’s other SL DRX configurations as much as possible, as well as ensuring the transmission performance of data. From this perspective, compared with TX UE centric mechanism which has no coordination information exchange, this TX UE based mechanism with coordination information exchange can achieve more power saving.  Additionally, the assistance information of the power saving requirement of RX UE can be used by the TX UE to adapt to the real-time situation in RX UE. For instance, when the RX UE’s battery capacity is lower than a threshold, the RX UE can indicate a high power saving demand to the TX UE and the TX UE should try to avoid an aggressive DRX configuration in order to save the RX UE’s power. By contrast, when the RX UE’s battery capacity is sufficient enough, the RX UE can indicate a low power saving demand to the TX UE and in this case the TX UE can wake up the RX UE more frequently for data reception.

Observation 1: Compared with TX UE centric DRX configuration, TX UE centric DRX configuration based on the assistance information from RX UE, where the RX UE transfers to the TX UE some assistance information such as the Uu DRX configuration of RX UE, the suggested SL DRX configuration, configured SL DRX configurations for other associated SL connections of the RX UE and the power saving requirement of the RX UE, can achieve more UE power saving and adapt to the real-time situation in RX UE.

Proposal 1: In TX UE centric DRX configuration based on the assistance information from RX UE, the RX UE transfers to the TX UE some assistance information such as the Uu DRX configuration of RX UE, the suggested SL DRX configuration, configured SL DRX configurations for other associated SL connections of the RX UE and the power saving requirement of the RX UE.

Apart from the content of assistance information from RX UE discussed above, another issue is when to transfer the assistance information. Here the UE assistance information transfer procedure in Uu can be reused, and RX UE can transfer to TX UE the assistance information autonomously based on a timer or/and when the assistance information changes, if the RX UE was configured to do so. In addition, similar to UE capability transfer procedure in Uu where the UE transfers its UE capability information upon receiving a UECapabilityEnquiry from the network, RX UE can transfer to TX UE the assistance information upon receiving the request from TX UE. For instance, when the AS layer or upper layer of TX UE requests to configure SL DRX for the RX UE,  TX UE requests RX UE to provide the assistance information. Therefore, the following two options of assistance information transfer for SL DRX configuration determination can be discussed:

-	Option 1: RX UE transfers to TX UE the assistance information autonomously based on a timer and/or when the assistance information changes.

-	Option 2: RX UE transfers to TX UE the assistance information upon receiving the request from TX UE. When the AS layer or upper layer of TX UE requests to configure SL DRX, TX UE requests RX UE to provide the assistance information.

In the above Option 1, referring to that in Uu, TX UE can configure RX UE to enable the autonomous assistance information transfer by PC5 RRC reconfiguration. In the timer-based method, RX UE start a timer when it transfers the assistance information. Upon expiry of this timer, RX UE may transfer to TX UE the assistance information again, or may transfer to TX UE the assistance information again when this timer is not running and the assistance information changes.

Proposal 2: RAN2 to discuss the following two options of assistance information transfer for SL DRX configuration determination:

- Option 1: RX UE transfers to TX UE the assistance information autonomously based on a timer and/or when the assistance information changes. TX UE can enable RX UE’s the autonomous assistance information transfer by PC5 RRC reconfiguration.

- Option 2: RX UE transfers to TX UE the assistance information upon receiving the request from TX UE. When the AS layer or upper layer of TX UE requests to configure SL DRX, TX UE requests RX UE to provide the assistance information.

## Details of timers

### 2.2.1 The handling of HARQ RTT timer value

It is a working assumption that SL HARQ RTT timer can be derived from the retransmission resource timing when the SCI indicates a retransmission resource, and it is FFS whether explicitly configured SL HARQ RTT timer may be still required. Based on our understanding, for the case when more than one transmission resources are indicated in the SCI, the UE can directly derive SL HARQ RTT timing of up to 2 retransmission resources. As illustrated in Figure 3 (a), for mode 1 transmission, SCI1 indicates three (re)transmission resources i.e. r1, r2 and r3, and SCI2 indicates two retransmission resource i.e., r2 and r3. Thus the UE can derive the value of HARQ RTT timer for the SL retransmission associated to SCI1 and SCI2, i.e., retransmission on r2 and r3 as the UE is able to derive the next retransmission resource r2 and r3. The HARQ RTT timer expires before the start of the next retransmission resource. Retransmission timer is started upon expiry of the HARQ RTT timer so that UE can wake up to receive the retransmission.

However, if only one transmission resource is indicated in SCI, the UE is unable to derive SL HARQ RTT timing for subsequent retransmission only based on resource indication in SCI. In this case, the HARQ RTT timer with explicitly configured value is required as in legacy Uu manner. As illustrated in Figure 3 (a), SCI3 indicates only one retransmission resource, i.e., r3, thus the UE needs to start HARQ RTT timer with configured value for the SL retransmission associated to SCI3, and retransmission timer starts when HARQ RTT timer expires.

For mode 2 transmission, a SCI always indicates next retransmission resources if any. As illustrated in Figure 3 (b), SCI1 indicates three (re)transmission resources i.e., r1, r2 and r3, SCI2 indicates three retransmission resource, i.e., r2, r3 and r4 and SCI3 indicates two retransmission resource, i.e., r3 and r4. As analyzed above, the UE can derive the value of HARQ RTT timer directly for the SL retransmission associated to SCI1, SCI2. Since SCI4 indicates only one retransmission resource, i.e., r4, the UE needs to start HARQ RTT timer with configured value for the retransmission associated to SCI4 and retransmission timers starts when HARQ RTT timer expires.

In addition, for mode 2, a reserved resource by a SCI may be pre-empted when pre-emption is enabled and the UE need to reselect resource for that resource. For example in Figure 3 (b), the reserved resource for SCI3 may be pre-empted by other higher priority SL transmission. In this case, if the value of HARQ RTT timer is derived from next retransmission indicated by SCI2, the UE can only reselect resource after SCI3 when retransmission timer is running, i.e., after the HARQ RTT timer for the retransmission associated to SCI2 expires. This may bring additional latency for sidelink transmission. To avoid this retransmission delay, HARQ RTT timer with configured value needs to start for each SL transmission when the pre-emption is enabled even the retransmission resource is indicated by the SCI.

Figure 3:  HARQ RTT timer with configured value is started if only one transmission resource is present in the SCI

Proposal 3: The configured value for the HARQ RTT timer is required in the following cases:

- If only one transmission resource is indicated in a SCI, i.e., the SCI does not indicate the resource for the subsequent retransmission;

- Pre-emption is enabled for mode 2.

Otherwise, HARQ RTT timer value is derived from the retransmission resource timing.

### 2.2.2 The handling of HARQ feedback disabled transmission

In RAN2#113bis-e meeting, we have the following agreement:

21:	For unicast, sidelink retransmission timer can be supported for at least some cases of HARQ disabled transmissions. FFS whether HARQ RTT is supported or not.

HARQ RTT timer is initially introduced to leave some gap for the scheduler to acquire retransmission resource. For the case of mode 1, even if the HARQ feedback is disabled in SL, the TX UE, based on its own decision, may still request retransmission resources from the gNB after all the transmission opportunities of the current DG/CG period are used up for the corresponding SL process, and thus have to wait for a period of time to get the resources scheduled by the gNB before sending the retransmission to the RX UE. In this sense, it still makes sense for the RX UE to wait for a period of time before monitoring the retransmission resources, that is, for the case of mode 1, even if the HARQ feedback is disabled in SL, HARQ RTT timer should be supported.

Observation 2: for mode 1, even if the HARQ FB is disabled in SL, the TX UE, based on its own decision, may still request retransmission resources from the gNB, and thus have to wait for a period of time to get the resources scheduled by the gNB before sending the retransmission to the RX UE.

In order to simplify UE implementation, it is necessary to pursue a uniform timer handling for HARQ enabled and disabled cases on HARQ RTT timer and retransmission timer. Accordingly, for the case of mode 2, even if the HARQ feedback is disabled in SL, HARQ RTT timer should also be supported.

Proposal 4: Even if the HARQ feedback is disabled in SL, HARQ RTT timer should be supported in SL DRX.

For a HARQ feedback disabled SL process in mode 1, if the TX UE decides to request retransmission resources from the gNB after all the transmission opportunities of the current DG/CG period are used up for the corresponding SL process, in a ‘specific period’ TX UE’s gNB will schedule the grants for SL transmission which may include the retransmission corresponding to the HARQ feedback disabled SL process, the SL retransmission corresponding to other SL process, or new SL transmission corresponding to other SL process. In this case, in order to avoid packet loss, the RX UE should start the HARQ RTT timer in SL DRX for the HARQ disabled SL process and start the corresponding retransmission timer after the HARQ RTT timer expires. However if the TX UE decides not to request retransmission resources from the gNB after all the transmission opportunities of the current DG/CG period are used up for the corresponding SL process, it is not power efficient for the RX UE to start retransmission timer for the HARQ disabled transmission. Therefore, it is necessary for the TX UE to inform RX UE about whether TX UE will request retransmission resource for a HARQ feedback disabled SL process after all the transmission opportunities of the current DG/CG period are used up for the corresponding SL process, so that the RX UE can adopt proper handling of the corresponding HARQ RTT timer.

Proposal 5: If the RX UE receives the indication from TX UE that the TX UE will request retransmission resource for a HARQ feedback disabled SL process after all the transmission opportunities of the current DG/CG period are used up for the corresponding SL process, it should start the corresponding HARQ RTT timer in SL DRX.

### 2.2.3 The handling of PSFCH transmission/reception failure

In RAN2#113bis-e meeting, we have the following agreement:

23:	If the RX UE does not transmit PSFCH for a HARQ enabled transmission (e.g. due to UL/SL prioritization) the RX UE still starts the HARQ RTT timer in the symbol/slot following the end of PSFCH resource.

If the RX UE does not transmit PSFCH for a HARQ enabled transmission (e.g., due to UL/SL prioritization), TX UE will not receive the PSFCH for this transmission and will send NACK to TX UE’s network. And in the following ‘specific period’ TX UE’s network will schedule the grants for SL transmission which may include the retransmission corresponding to the unreceived PSFCH, the SL retransmission corresponding to other SL process, or new SL transmission corresponding to other SL process. No matter whether this RX UE intends to send NACK or ACK but failed, e.g., due to UL/SL prioritization,  it may experience packet loss if TX UE’s network schedule the grants for  the SL retransmission or new SL transmission corresponding to other SL process but the RX UE is not awake in the ‘specific period’. To avoid the packet loss in RX UE, regardless of whether the data corresponding to the unsent PSFCH was decoded successfully or not, RX UE should be awake in the aforementioned ‘specific period’. In other words, if the RX UE does not transmit PSFCH for a HARQ enabled transmission (e.g., due to UL/SL prioritization), and HARQ RTT timer expires, the RX UE should start retransmission timer regardless of whether the data corresponding to the unsent PSFCH was decoded successfully or not.

Proposal 6: If the RX UE does not transmit PSFCH for a HARQ enabled transmission (e.g. due to UL/SL prioritization), and HARQ RTT timer expires, the RX UE starts retransmission timer regardless of whether the data corresponding to the unsent PSFCH was decoded successfully or not.

Moreover, even if RX UE transmits PSFCH for a HARQ enabled transmission, TX UE may not receive PSFCH for a HARQ enabled transmission, e.g. due to intra-UE prioritization in TX UE. In this case, based on R16 V2X, TX UE will send NACK to TX UE’s network. And also, in the following ‘specific period’ TX UE’s network will schedule the grants for SL transmission which may include the retransmission corresponding to the unreceived PSFCH, the SL retransmission corresponding to other SL process, or new SL transmission corresponding to other SL process. If the unreceived SL HARQ feedback is ‘ACK’, the RX UE will not start the corresponding retransmission timer in SL DRX and thus may experience packet loss if TX UE’s network has scheduled the grants for  the SL retransmission or new SL transmission corresponding to other SL process in the ‘specific period’. Accordingly, it is necessary to study how to avoid packet loss in RX UE if the TX UE does not receive PSFCH for a HARQ enabled transmission (e.g. due to intra-UE prioritization in TX UE).

Proposal 7: RAN2 to study how to avoid packet loss in RX UE if the TX UE does not receive PSFCH for a HARQ enabled transmission (e.g. due to intra-UE prioritization in TX UE).

### 2.2.4 The on-duration determination in SL DRX

In RAN2#113bis-e meeting, we have the following agreement:

2:	The RX UE determines the symbol/slot/subframe associated with the start of the DRX cycle using the configured sl-drx-Cycle, sl-drx-StartOffset.  FFS on details.

We should further study how to determine the on-duration in SL DRX using the configured sl-drx-Cycle and sl-drx-StartOffset.

For a SL connection, apart from the slots in the (pre)configured SL resource pool(s), there are several kinds of slots that cannot be used to perform the SL data transmission for this SL connection, such as non SL slots, SL SSB slots, SL reserved slots, and SL slots that belongs to other SL resource pool(s) of other SL connections. Hereafter, we define that for a SL connection, the slots in the (pre)configured SL resource pool(s) of this SL connection are the “logic slot” corresponding to this SL connection. For a SL connection, if we follow the principle of Uu DRX and considering both its corresponding logic slot and other slots, the start of on-duration in SL DRX can be illustrated as follows:

Figure 4. The on-duration in SL DRX when both logic slot and other slots are considered

The above illustration is based on the following assumptions.

Table II: the parameters assumptions for SL DRX

| SCS | 15KHz |
| --- | --- |
| SL DRX cycle | 10ms |
| SL on-duration timer | 1ms |
| SL drx-StartSlotOffset | 3ms |
| SL drx-SlotOffset | 0 |

According to the above figure, the on-duration in both the first and third SL DRX cycle falls into non SL slots, which means that the RX UE will not receive any expected SCI in both the first and third SL DRX cycle. Moreover, if we extend on-duration timer to two or more milliseconds, the problem becomes that the start of on-duration in some SL DRX cycles falls into non SL slots, or the logic slots in some SL DRX cycles may be very limited. If the TX UE cannot acquire the proper SL resource in the limited logic slot of on-duration, the TX UE have to wait for a SL DRX cycle, which impacts the SL service delay performance. Therefore, if we follow the principle of Uu DRX and considering both its corresponding logic slot and other slots, the SL service delay performance may get worse.

Observation 3: If we follow the principle of Uu DRX and determine the SL DRX on-duration of a SL connection considering both the logic slots corresponding to this SL connection and other slots, the SL service delay performance may get worse.

To overcome the above problem, two options can be considered. One option is to only consider the corresponding logic slots when determining the SL DRX on-duration of a SL connection, which can refer to the “level 3 logic slot” based CG equation in [2]. Another option is to follow the principle of Uu DRX to determine the SL DRX on-duration but allow to extend the on-duration when the number of logic slots in the original on-duration is smaller than a threshold.

Proposal 8: RAN2 to discuss the following options to overcome the delay performance degradation if we follow the principle of Uu DRX to determine the SL DRX on-duration:

- Option#1: only consider the corresponding logic slots of the SL connection when determining the SL DRX on-duration of a SL connection.

- Option#2: follow the principle of Uu DRX to determine the SL DRX on-duration but allow to extend the on-duration when the number of logic slots in the original on-duration is smaller than a threshold.

- 2.2.5 The units of the start/restart time and timer value of the DRX timers

In RAN2#113bis meeting, the following agreements are achieved:

For transmissions with HARQ feedback, the RX UE starts the SL HARQ RTT timer in the symbol/slot following the end of PSFCH transmission. If the RX UE does not transmit PSFCH for a HARQ enabled transmission (e.g. due to UL/SL prioritization) the RX UE still starts the HARQ RTT timer in the symbol/slot following the end of PSFCH resource.

Retransmission timer can be started upon expiry of the HARQ RTT timer.

It means that the granularity of the start/restart time of both HARQ RTT timer and retransmission timer needs to be determined between ”symbol” and “slot”. In R16 NR Uu DRX, the granularity of the start/restart time of both HARQ RTT timer and retransmission timer are “symbol” as follows:

1> if a DRX group is in Active Time: 2> monitor the PDCCH on the Serving Cells in this DRX group as specified in TS 38.213 [6]; 2> if the PDCCH indicates a DL transmission: 3> start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback;

1> if a drx-HARQ-RTT-TimerUL expires: 2> start the drx-RetransmissionTimerUL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerUL.

Additionally, in R16 NR Uu DRX, the units of the timer value of HARQ RTT timer and retransmission timer are “symbol” and “slot” respectively as follows:

DRX-Config field descriptions

drx-HARQ-RTT-TimerDL Value in number of symbols of the BWP where the transport block was received.

drx-HARQ-RTT-TimerUL Value in number of symbols of the BWP where the transport block was transmitted.

drx-InactivityTimer Value in multiple integers of 1 ms. ms0 corresponds to 0, ms1 corresponds to 1 ms, ms2 corresponds to 2 ms, and so on.

drx-LongCycleStartOffset drx-LongCycle in ms and drx-StartOffset in multiples of 1 ms. If drx-ShortCycle is configured, the value of drx-LongCycle shall be a multiple of the drx-ShortCycle value.

drx-onDurationTimer Value in multiples of 1/32 ms (subMilliSeconds) or in ms (milliSecond). For the latter, value ms1 corresponds to 1 ms, value ms2 corresponds to 2 ms, and so on.

drx-RetransmissionTimerDL Value in number of slot lengths of the BWP where the transport block was received. value sl0 corresponds to 0 slots, sl1 corresponds to 1 slot, sl2 corresponds to 2 slots, and so on.

drx-RetransmissionTimerUL Value in number of slot lengths of the BWP where the transport block was transmitted. sl0 corresponds to 0 slots, sl1 corresponds to 1 slot, sl2 corresponds to 2 slots, and so on.

drx-ShortCycleTimer Value in multiples of drx-ShortCycle. A value of 1 corresponds to drx-ShortCycle, a value of 2 corresponds to 2 * drx-ShortCycle and so on.

drx-ShortCycle Value in ms. ms1 corresponds to 1 ms, ms2 corresponds to 2 ms, and so on.

drx-SlotOffset Value in 1/32 ms. Value 0 corresponds to 0 ms, value 1 corresponds to 1/32 ms, value 2 corresponds to 2/32 ms, and so on.

The main reason of using “symbol” is that in Uu, a DCI can be configured in different symbol of a slot and thus the scheduling in Uu is on the level of “symbol”. However, in SL a SCI can only be configured in the 2nd ~4th symbol of a slot. Accordingly, even if the unit of the start/restart time of both HARQ RTT timer and retransmission timer is set to “symbol”, it cannot actually achieve the “symbol” level control of the SCI monitoring for the retransmission. Therefore, “slot” is preferred for the granularity of the start/restart time and timer value of both HARQ RTT timer and retransmission timer from the perspective of UE implementation simplicity.

Proposal 9: As to both HARQ RTT timer and retransmission timer in SL DRX, both the start/restart time and timer value are in the unit of “slot".

## The alignment between SL resource and SL DRX configuration

In RAN2#113bis-e meeting, we have the following agreement:

29:	RAN2 assumes LCP enhancements for ensuring a TX UE transmits data in the active time of an RX UE are needed. FFS on the resource (re)selection enhancements (e.g. limiting the resources to the active time for peer UE).

In the following we discuss the details of LCP enhancements. For mode 1, when selecting a destination for a SL grant,  one more condition needs to be added is that the SL grant is within the active time of the selected destination. However, only enhancing the LCP procedure seems not enough to handle this issue since the gNB does not have clear knowledge of the SL active time of RX UE as it is up to the TX UE to choose the destination. As the NW does not know the inactivity timer of which destination is extended, it is possible that a SL grant is not in SL active time of any destination that has data to be sent. In this case, this SL grant is not able to be used. As illustrated in Figure 5, a TX UE uses two SL grants scheduled by gNB for RX UE1 and RX UE2 during respective onduration, which leads to inactivity timer starting for each connection. When there is more data to be sent to RX UE2, gNB schedules TX UE with the third SL grant. However without knowledge of SL active time of each RX UE, the third SL grant can be out of the SL active time of RX UE2. When there is no other RX UE that has data to be sent, the SL grant is not used by the TX UE.

Figure 5. SL grant is not in SL active time of any destination that has data to be sent

To avoid this, the TX UE needs to report some assistance information to enable gNB to maintain each destination’s active time or a joint active time of all the associated destinations, which can assist the SL scheduling by gNB and make sure the allocated SL grant is within at least one destination’s active time.

The other alternative is that the gNB determines the destination of a SL grant. With the knowledge of each destination’s SL DRX configuration, if the gNB is responsible for selecting a destination for a certain SL grant, the gNB is able to maintain each destination’s SL active time based on the scheduling and can avoid the mentioned issue to the most extent. This option introduces less complexity to UE implementation however is not aligned with the Rel-16 principle, i.e., in Rel-16, the selection of the destination is all up to the TX UE.  Besides, this mechanism may have some RAN1 impact if the destination ID needs to be included in the DCI and some further discussion in RAN1 is needed.

Proposal 10: For mode 1, RAN2 to decide how to ensure a TX UE transmits data in the active time of an RX UE by taking into account the following two options:

- Option 1: When TX UE is selecting a destination, one more condition is added that the SL grant is within the SL active time of the selected destination. Some assistance information from the TX UE to the gNB enabling the gNB to maintain each destination’s active time or a joint active time of all the associated destinations may be needed.

- Option 2: The gNB determines the destination for a SL grant and maintains each RX UE’s SL active time based on scheduling.

Similarly for mode 2, the additional condition in LCP procedure discussed for mode 1 is also applicable. Besides LCP, resource (re)selection enhancements are also needed. Without taking the SL DRX state into account in resource selection procedure, it is possible that a selected SL grant is not within the SL active time of any destination that has data to be sent. Resource selection procedure may also need to be enhanced to ensure the TX UE selects a SL grant at least within the active time of a destination. Specifically, when the MAC layer in TX UE receives the candidate resources indicated by the physical layer, it can select among the resources that fall into the SL DRX on-duration/active time of at least one of the candidate destinations.

In addition, resource (re)selection check also needs to be adaptive to SL DRX. For example, if there is no SL grant in the SL active time of the destination that has data to be sent, resource (re)selection is triggered. Another example, if the current reserved resources do not fall into the SL DRX active time of any destination, i.e., the current reserved resources do not match the SL DRX of any destination, resource reselection should be triggered.

Proposal 11: For mode 2, the following options should to be studied to handle the case when the SL grant is out of the active time of the selected destination:

- When TX UE is selecting a destination, one more condition is added that the SL grant is within the SL active time of the selected destination.

- TX UE selects a SL grant that falls into the SL DRX onduration/active time of at least one of the candidate destinations in resource selection procedure.

- If the current reserved resources do not fall into the SL DRX active time of any destination, or if there is no SL grant in the SL active time of the destination that has data to be sent, resource (re)selection is triggered.

## SL-CSI report request impact on SL DRX

There is a working assumption that the slots when the UE is expected CSI report following a CSI request are considered as SL active time. As illustrated in Figure 6, if the UE triggers a CSI report outside the SL DRX of itself, it still needs to wake up to receive at least CSI report. However, the awake time is extended by inactivity timer due to the transmission including CSI report. The active time may be further extended by subsequent data transmissions from peer UE even if the data can be transmitted in the next DRX cycle when CSI report is not triggered. It is not power efficient that the UE wakes up for too long even till next on-duration due to a CSI report trigger. Thus it is proposed that RAN2 considers how to handle the power inefficiency issue due to CSI report request.

Figure 6 Active time extension due to CSI report request

Proposal 12: RAN2 to consider the power inefficiency issue of active time extension due to CSI report request.

# Conclusion

In this contribution, we discuss sidelink DRX configuration for unicast, and derive the following observations:

Observation 1: Compared with TX UE centric DRX configuration, TX UE centric DRX configuration based on the assistance information from RX UE, where the RX UE transfers to the TX UE some assistance information such as the Uu DRX configuration of RX UE, the suggested SL DRX configuration, configured SL DRX configurations for other associated SL connections of the RX UE and the power saving requirement of the RX UE, can achieve more UE power saving and adapt to the real-time situation in RX UE.

Observation 2: For mode 1, even if the HARQ FB is disabled in SL, the TX UE, based on its own decision, may still request retransmission resources from the gNB, and thus have to wait for a period of time to get the resources scheduled by the gNB before sending the retransmission to the RX UE.

Observation 3: If we follow the principle of Uu DRX and determine the SL DRX on-duration of a SL connection considering both the logic slots corresponding to this SL connection and other slots, the SL service delay performance may get worse.

Moreover, we have the following proposals:

Proposal 1: In TX UE centric DRX configuration based on the assistance information from RX UE, the RX UE transfers to the TX UE some assistance information such as the Uu DRX configuration of RX UE, the suggested SL DRX configuration, configured SL DRX configurations for other associated SL connections of the RX UE and the power saving requirement of the RX UE.

Proposal 2: RAN2 to discuss the following two options of assistance information transfer for SL DRX configuration determination:

- Option 1: RX UE transfers to TX UE the assistance information autonomously based on a timer or/and when the assistance information changes. And TX UE can configure RX UE to enable the autonomous assistance information transfer by PC5 RRC reconfiguration.

- Option 2: RX UE transfers to TX UE the assistance information upon receiving the request from TX UE. When the AS layer or upper layer of TX UE requests to configure SL DRX, TX UE requests RX UE to provide the assistance information.

Proposal 3: The configured value for the HARQ RTT timer is required in the following cases:

- If only one transmission resource is indicated in a SCI, i.e., the SCI does not indicate the resource for the subsequent retransmission;

- Pre-emption is enabled for mode 2.

Otherwise, HARQ RTT timer value is derived from the retransmission resource timing.

Proposal 4: Even if the HARQ feedback is disabled in SL, HARQ RTT timer should be supported in SL DRX.

Proposal 5: If the RX UE receives the indication from TX UE that the TX UE will request retransmission resource for a HARQ feedback disabled SL process after all the transmission opportunities of the current DG/CG period are used up for the corresponding SL process, it should start the corresponding HARQ RTT timer in SL DRX.

Proposal 6: If the RX UE does not transmit PSFCH for a HARQ enabled transmission (e.g. due to UL/SL prioritization), and HARQ RTT timer expires, the RX UE starts retransmission timer regardless of whether the data corresponding to the unsent PSFCH was decoded successfully or not.

Proposal 7: RAN2 to study how to avoid packet loss in RX UE if the TX UE does not receive PSFCH for a HARQ enabled transmission (e.g. due to intra-UE prioritization).

Proposal 8: RAN2 to discuss the following options to overcome the delay performance degradation if we follow the principle of Uu DRX to determine the SL DRX on-duration:

- Option#1: only consider the corresponding logic slots of the SL connection when determining the SL DRX on-duration of a SL connection.

- Option#2: follow the principle of Uu DRX to determine the SL DRX on-duration but allow to extend the on-duration when the number of logic slots in the original on-duration is smaller than a threshold.

Proposal 9: As to both HARQ RTT timer and retransmission timer in SL DRX, both the start/restart time and timer value are in the unit of “slot".

Proposal 10: For mode 1, RAN2 to decide how to ensure a TX UE transmits data in the active time of an RX UE by taking into account the following two options:

- Option 1: When TX UE is selecting a destination, one more condition is added that the SL grant is within the SL active time of the selected destination. Some assistance information from the TX UE to the gNB enabling the gNB to maintain each destination’s active time or a joint active time of all the associated destinations may be needed.

- Option 2: The gNB determines the destination for a SL grant and maintains each RX UE’s SL active time based on scheduling.

Proposal 11: For mode 2, the following options should to be studied to handle the case when the SL grant is out of the active time of the selected destination:

- When TX UE is selecting a destination, one more condition is added that the SL grant is within the SL active time of the selected destination.

- TX UE selects a SL grant that falls into the SL DRX onduration/active time of at least one of the candidate destinations in resource selection procedure.

- If the current reserved resources do not fall into the SL DRX active time of any destination, or if there is no SL grant in the SL active time of the destination that has data to be sent, resource (re)selection is triggered.

Proposal 12: RAN2 to consider the power inefficiency issue of active time extension due to CSI report request.

# Reference

- RP-193231, New WID on NR sidelink enhancement.

- R2-2102722, Summary of [POST113-e] [707] [V2X] Spec update to level 3 logical slots, OPPO.