3GPP TSG-RAN WG2 Meeting #115-e	                    R2-2107482

Online, August 16 - August 27, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 2727 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on description of lbt-FailureInstanceMaxCount in LBT-FailureRecoveryConfig |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Core |  |  |  |  |  | Date: |  |  | 03/08/2021 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The field description for lbt-FailureInstanceMaxCount is as below: This field determines after how many consistent uplink LBT failure events the UE triggers uplink LBT failure recovery (see TS 38.321 [3]). Value n4 corresponds to 4, value n8 corresponds to 8, and so on. In 38.321 g50, the usage for lbt-FailureInstanceMaxCount is described as follows. For each activated Serving Cell configured with lbt-FailureRecoveryConfig, the MAC entity shall: 1> if LBT failure indication has been received from lower layers: 2> start or restart the lbt-FailureDetectionTimer; 2> increment LBT_COUNTER by 1; 2> if LBT_COUNTER >= lbt-FailureInstanceMaxCount: 3> trigger consistent LBT failure for the active UL BWP in this Serving Cell; According to the description from 38.321 spec, the LBT failure indications from lower layers are counted. When LBT failure indications reaches lbt-FailureInstanceMaxCount, consistent LBT failure is triggered. Hence, the field description of lbt-FailureInstanceMaxCount is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | “consistent uplink LBT failure events” should be modified as “LBT failure indications received from the physical layer” Impact analysis Impacted functionality: Field descriptions of lbt-FailureInstanceMaxCount in LBT-FailureRecoveryConfig Inter-operability: - If the network is implemented according to the CR and the UE is not, there is no interoperability issue since the MAC spec is correctly implemented anyway. - f the UE is implemented according to the CR and the network is not, there is no inter-operability issue since the MAC spec is correctly implemented anyway. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The field description of lbt-FailureInstanceMaxCount in LBT-FailureRecoveryConfig is not accurate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Related RAN1 LS in R2-2106911 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.3.2	Radio resource control information elements

Start of Changes

#### –	LBT-FailureRecoveryConfig

The IE LBT-FailureRecoveryConfig-r16 is used to configure the parameters used for detection of consistent uplink LBT failures for operation with shared spectrum channel access, as specified in TS 38.321 [3].

LBT-FailureRecoveryConfig information element

-- ASN1START

-- TAG-LBT-FAILURERECOVERYCONFIG-START

LBT-FailureRecoveryConfig-r16 ::=    SEQUENCE {

lbt-FailureInstanceMaxCount-r16      ENUMERATED {n4, n8, n16, n32, n64, n128},

lbt-FailureDetectionTimer-r16        ENUMERATED {ms10, ms20, ms40, ms80, ms160, ms320},

...

}

-- TAG-LBT-FAILURERECOVERYCONFIG-STOP

-- ASN1STOP

LBT-FailureRecoveryConfig field descriptions

lbt-FailureDetectionTimer Timer for consistent uplink LBT failure detection (see TS 38.321 [3]). Value ms10 corresponds to 10 ms, value ms20 corresponds to 20 ms, and so on.

lbt-FailureInstanceMaxCount This field determines after how many LBT failure indications received from the physical layer the UE triggers uplink LBT failure recovery (see TS 38.321 [3]). Value n4 corresponds to 4, value n8 corresponds to 8, and so on.

End of Changes