3GPP TSG RAN WG2#115-e	R2-2107568

Online meeting, August 09-27, 2021

# Title:	[Draft] Reply LS on UE location aspects in NTN

# Response to:	R3-212917/ R2-2106941

# Release:	Release 17

Source:	Qualcomm Inc. [to be RAN2]

To:	RAN3

Cc:	SA2, CT1

Contact Person:

Name:	Bharat Shrestha

E-mail Address:	bshresth@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN2 would like to thank RAN3 for the LS reply and provide answers to the following questions.

Question 1: RAN3 would like RAN2 to confirm whether the gNB will be able to acquire UE location information with an accuracy comparable to TN cell granularity (e.g. GNSS information or otherwise) after AS security, and also to confirm whether it is possible to provide any level of UE location information (i.e. finer than NTN Uu cell accuracy) before AS security.

RAN2 answer: RAN2 confirms that UE can report GNSS coordinates using existing mechanism in the measurement report. In addition, RAN2 has agreed that the UE will report UE location information with a guaranteed accuracy of an area of ~2km radius in Msg5 during initial access if no privacy issue is identified by SA3.

Question 3: RAN3 welcomes any feedback from RAN2 on the described case (i.e. the gNB to trigger inter-AMF handover when crossing country borders).

RAN2 answer: The UE may report UE’s GNSS coordinates in existing measurement report. The measurement report can be transmitted periodically. In addition, location-based measurement trigger is also introduced such that the UE will trigger the measurement report if it moves by a threshold distance since its last report. Therefore, gNB will be able to determine whether UE has moved far away from the last location report. RAN2 understands it is up to the gNB to determine the crossing of country borders based on the location update and take appropriate action.

Question 4: RAN3 requests RAN2, CT1 and SA2 to provide any feedback on above issue (i.e. which TAC should be reported by the gNB in case of multiple broadcast TAC).

RAN2 answer: The gNB will be able to acquire UE location information at initial access and during connected mode periods. Therefore, it would be possible to map such a location into a TAC (since TAC areas are earth-fixed). RAN2 cannot comment however on whether this approach is appropriate for reporting towards the CN.

2. Actions:

To RAN WG3 group.

ACTION:	RAN2 kindly asks RAN WG3 to take the above information into account and provide feedback, if any.

3. Date of Next RAN2 Meetings:

TSG-RAN WG2#116-e	November 1st – November 12th, 2021	Online meeting