3GPP TSG-RAN WG2 Meeting #115-e	R2-2107588

# Electronic, August 16 – August 27, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 2732 | rev | 0 | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of RSSI/CO reporting in MCG/SCGFailureInformation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16, NR_unlic-Core |  |  |  |  |  | Date: |  |  | 2021-07-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RSSI/CO reporting was introduced in NR-U, to reflect the channel loading status and as well help NW to detect hidden node. This measurement result is useful for NW to make mobility decision to UE. However, the RSSI/CO measurement results are missing in MCGFailureInformation and SCGFailureInformation. In order to provide necessary channel status information for unlicensed spectrums so that NW can make proper decision on HO/PSCell change upon MCG/SCG failure, it’s necessary to add RSSI/CO measurement results into MCGFailureInformation and SCGFailureInformation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the measurement results for RSSI/CO on unlicensed NR frequencies into MeasResult2NR which is carried in MCGFailureInformation and SCGFailureInformation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RSSI/CO measuremnt results are not available in MCGFailureInformation and SCGFailureInformation. Impact analysis: Impacted 5G architecture options: SA, MR-DC, NR-DC Impacted functionality: MCG/SCGFailureInformation, NR-U Inter-operability: If NW implements this CR while UE does not, RSSI/CO measurement results for unlicensed NR frequencies are not available to NW upon receiving MCGFailureInformation and SCGFailureInformation. There is no inter-operability issue. If UE implements this CR while NW does not, NW cannot understand the reported RSSI/CO measurement results carried in MCGFailureInformation and SCGFailureInformation. There is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.3.2	Radio resource control information elements

#### <Text omitted>

#### --------------------------------------------------------------------------<Start of 1st change>-------------------------------------------------------------------------

#### –	MeasResult2NR

The IE MeasResult2NR contains measurements on NR frequencies.

MeasResult2NR information element

-- ASN1START

-- TAG-MEASRESULT2NR-START

MeasResult2NR ::=                   SEQUENCE {

ssbFrequency                        ARFCN-ValueNR                           OPTIONAL,

refFreqCSI-RS                       ARFCN-ValueNR                           OPTIONAL,

measResultServingCell               MeasResultNR                            OPTIONAL,

measResultNeighCellListNR           MeasResultListNR                        OPTIONAL,

...,

[[

measResultForRSSI-r16                   MeasResultForRSSI-r16                                                       OPTIONAL

]]

}

-- TAG-MEASRESULT2NR-STOP

-- ASN1STOP