3GPP TSG RAN WG2 #115-e			R2-2107615

e-Meeting, August 16th – August 27th, 2021

Agenda Item:	8.14.2.2

Source:	Apple

Title:	Pause/Resume functionality

Document for:	Discussion/Decision

# Introduction

The topic of QoE measurement reporting was addressed in R2-114e [1], and the following interim agreement was reached.

FFS whether pause resume will affect all configurations or whether pause resume can act selectively per configuration.

In this contribution, we provide our views on this issue.

# Discussion

One of the objectives of the WI is to “Specify QoE measurement handling at RAN overload, including pause and resume of QoE measurement reporting.” Some companies have expressed the view that RAN should be able to selectively pause some QoE measurements during periods of RAN overload. However, it is not clear if such flexibility can really be exploited by RAN.

First, we note that the QoE measurement and configuration are encapsulated in a transparent container, and the gNB is not expected to process this data (with the possible exception of RAN-visible QoE measurement which is still FFS at this point).

Second, in periods of RAN overload, there is no compelling reason to receive QoE measurement reports since these are not delay-sensitive in nature.

Third, we expect that RAN overload is relatively rare, and we don’t really have to optimize this scenario.

For these reasons, we believe there is not much gain in allowing selective suspension of QoE measurements.

Proposal 1: Pause and resume functionality applies to all QoE configurations.

# Conclusions

In this document, we provide our views on whether to allow selective suspension of QoE measurement reporting during RAN overload. Our proposal is summarized below.

Proposal 1: Pause and resume functionality applies to all QoE configurations.

# References

- Draft_R2-114-e_Meeting_Report_v1, Report of 3GPP TSG RAN WG2 meeting #114-e, Online