3GPP TSG-RAN WG2 Meeting #115 electronic	R2- 2107898

E-meeting, August 9th – August 27th, 2021

Agenda item:		8.6.2

Source:		Lenovo, Motorola Mobility

Title:		The UP common issues for small data transmissions

Document for:		Discussion and Decision

- Introduction

In RAN2#112 e-meeting [1], the agreements were achieved for RRC based SDT.

Agreements: 3 The first UL message (i.e. MSG3 for 4-step RACH, MSGA payload for 2-step RACH and the CG transmission for CG) may contain at least the following contents (depending on the size of the message): - CCCH message (needs to be included) LCP can be used to determine to priority of the content below that may be included - DRB data from one or more DRBs which are configured by the network for small data transmission - MAC CEs – (e.g. BSR). FFS other MAC CEs - Padding bits FFS if we need to ensure that SDT data only is included. Depends on whether the UE initiates legacy/normal resume

In RAN2#113bis-e, the agreements were achieved for SDT

Agreements: - RSRP threshold is used to select between SDT and non-SDT procedure, if configured (RSRP refers to the same RSRP measured for carrier selection). - RSRP threshold to select between SDT and non-SDT procedure is used for both CG-SDT and RA-SDT - RSRP threshold to select between SDT and non-SDT procedure is same for both CG-SDT and RA-SDT - RSRP threshold for carrier selection is specific to SDT (i.e. separately configured for SDT). This is optional for the network. - RSRP threshold for RA type selection is specific to SDT (i.e. separately configured for SDT)

Agreements

5	PHR functionality is supported for SDT.   FFS on PHR procedure

Left for RAN1 decision:

Proposal 11: Whether to support BFD and BFR for SDT is up to RAN1 decision.

Considering the details are not very clear, we give our views on the UP common issues for small data transmission in this contribution.

- Discussion

LCP

It is possible that there is subsequent data transmission for both CG type 1 based SDT and RACH based SDT.

The LCP and multiplexing of MAC CEs and MAC SDUs are introduced to guarantee the QoS and fairness among the UL traffics in LTE and NR. Although the UE is in RRC_INACTIVE state when performing SDT, the purpose is to transmit UL data which is the same with RRC_CONNECTED state. And no matter when state the UE is in, the requirements for the traffic performance are the same. Therefore, the existing procedure of LCP and multiplexing of MAC CEs and MAC SDUs when UE transmits UL data in RRC_CONNECTED state should be reused when performing SDT.

Proposal 1: LCP procedure should be reused for SDT in RRC_INACTIVE

NW may configure UEs in RRC_CONNECTED with LCH mapping restrictions, which are used during the LCP procedure, i.e. TB generation procedure. Those mapping restrictions are used to control which LCHs can be mapped to a specific UL grant. When UE is moved to RRC_INACTIVE by NW the UE stores the configured LCH mapping restrictions. However for uplink transmissions in RRC_INACTIVE mode, i.e. small data transmission (SDT) occurring e.g. only on the initial UL BWP, most of the LCH mapping restrictions are actually not suitable and would rather prohibit the UE from using the configured uplink resources for small data transmissions, e.g. mapping restrictions related to allowed SCS, PUSCH duration and allowed serving cells.

The mapping restrictions for UL transmissions in RRC_CONNECTED and transmissions (SDT) in RRC_INACTIVE, e.g. UL transmission on CG-SDT resources or on the uplink resource assigned by the Random Access Response (Msg3) or MsgA, may differ substantially. For example, restrictions related to the allowed numerology (SCS) and PUSCH duration as well as the allowed serving cell(s) are not relevant/applicable for SDT transmission in RRC_INACTIVE since for example UL transmissions in RRC_INACTIVE may happen only on the initial UL BWP. On the other hand – this is still under discussion - gNB may configure explicitly and control which of the configured LCHs are allowed to multiplex data on e.g. CG resources configured for SDT transmissions in RRC_INACTIVE. Such LCH mapping restriction, if agreed, is only applicable for RRC_INACTIVE, i.e. no relevance for RRC_CONNECTED. Therefore, the LCH mapping restriction configuration is different for RRC_CONNECTED and for RRC_INACTIVE, e.g. it may be possible that UE doesn’t apply LCH mapping restrictions in RRC_INACTIVE for SDT.

Proposal 2: LCH mapping restriction configuration/behaviour is different for UL transmissions in RRC_CONNECTED and SDT in RRC_INACTIVE, e.g. UE may not apply LCH mapping restrictions in RRC_INACTIVE for SDT. Details are FFS.

For SDT in RRC_INACTIVE we think that the handling of the token bucket model Bj should be further discussed in RAN2. On one hand there might not be a need for having Bj, since the token bucket mechanism is used in RRC_CONNECTED in order avoid the starvation of low priority LCHs and it can be safely assumed that there is no starvation issue for uplink transmissions in RRC_INACTIVE, e.g. there is no need to apply this mechanism for SDT in RRC_INACTIVE. On the other hand, there could be SDT data to be transmitted on multiple logical channels. It might be also an option to maintain the Bj when moving from RRC_CONNECTED to RRC_INACITVE and not initialize the value. The UE can perform consistently when considering of allocation of resources in RRC_INACTIVE. Therefore, it can be further discussed whether UE needs to maintain the token bucket(s)/PBR for SDT in RRC_INACTIVE.

Proposal 3: RAN2 to discuss the handling of token bucket for SDT in RRC_INACTIVE.

We think that the relative priority order of MAC CEs and MAC SDUs applied in connected state shall be also applied to SDT when multiplexing MAC CEs and MAC SDUs in a MAC PDU.

Proposal 4: The relative priority order of MAC CEs and MAC SDUs applied in connected state shall be applied to SDT when multiplexing MAC CEs and MAC SDUs in a MAC PDU.

According to the discussion by now, at least the following MAC CEs and the corresponding priority should be considered.

Logical channels shall be prioritized in accordance with the following order (highest priority listed first):

-	C-RNTI MAC CE or data from UL-CCCH;

-	MAC CE for BSR, with exception of BSR included for padding;

-	Single Entry PHR MAC CE;

-	data from any Logical Channel, except data from UL-CCCH;

-	MAC CE for BSR included for padding;

While the PHR procedure is FFS.

Proposal 5: The PHR MAC CE is allowed to be contained in the first PUSCH message (i.e. MSG3 for 4-step RACH, MSGA payload for 2-step RACH and the CG transmission for CG).

Data volume calculation

There is a remaining issue from RAN2#111-e [2]. It is FFS how we calculate data volume.

Whether to initiate a SDT is determined by the data volume, and the RBs configured for SDT are resumed after the SDT is initiated. Therefore, the data volume can be calculated in the MAC layer if the MAC can have visibility of data arriving before the RBs configured for SDT are resumed.

Proposal 6: RAN2 to discuss whether the MAC layer can have visibility of data arriving before the RBs configured for SDT are resumed.

Since the small data transmission is configured by the network on a per DRB basis, the data volume can be calculated as the buffered data size of corresponding RLC/PDCP transmission entity which can do SDT configured by the NW. And for another case which is under discussion, the similar calculation mechanism can be applied to the non-SDT DRBs if the NW configures the UE to report BSR also for the non-SDT DRBs.

Proposal 7: The data volume can be calculated as the buffered data size of corresponding RLC/PDCP transmission entity for which the SDT is configured if the MAC layer can have visibility of data arriving before the RBs are resumed.

TAT

A new TA timer for TA maintenance specified for CG-SDT in RRC_INACTIVE was introduced. And it has been agreed to support the subsequent data transmission for RACH based SDT. Then for the subsequent data transmission, it seems better to also maintain a TA from the perspective of traffic performance.

And considering the RACH procedure, it is natural for the UE to receive a TA from the network. In this case, should the UE apply the TAT configured in connected state or reuse the new TAT for CG-SDT if configured? It is simple for UE implementation to reuse the new TAT defined for CG-SDT from the traffic performance perspective.

Observation 1: It is beneficial to maintain a TA for the subsequent data transmission of RACH based SDT.

Proposal 8: The new TAT for CG based SDT is reused for the subsequent data transmission of RACH based SDT.

- Conclusion

In this contribution, the UP common issues for small data transmission including the LCP and data volume calculation are analysed and the following observation and proposals are given:

Observation 1: It is beneficial to maintain a TA for the subsequent data transmission of RACH based SDT.

Proposal 1: LCP procedure should be reused for SDT in RRC_INACTIVE.

Proposal 2: LCH mapping restriction configuration/behaviour is different for UL transmissions in RRC_CONNECTED and SDT in RRC_INACTIVE, e.g. UE may not apply LCH mapping restrictions in RRC_INACTIVE for SDT. Details are FFS.

Proposal 3: RAN2 to discuss the handling of token bucket/PBR for SDT in RRC_INACTIVE.

Proposal 4: The relative priority order of MAC CEs and MAC SDUs applied in connected state shall be applied to SDT when multiplexing MAC CEs and MAC SDUs in a MAC PDU.

Proposal 5: The PHR MAC CE is allowed to be contained in the first PUSCH message (i.e. MSG3 for 4-step RACH, MSGA payload for 2-step RACH and the CG transmission for CG).

Proposal 6: RAN2 to discuss whether the MAC layer can have visibility of data arriving before the RBs configured for SDT are resumed.

Proposal 7: The data volume can be calculated as the buffered data size of corresponding RLC/PDCP transmission entity for which the SDT is configured if the MAC layer can have visibility of data arriving before the RBs are resumed.

Proposal 8: The new TAT for CG based SDT is reused for the subsequent data transmission of RACH based SDT.

References

RAN2#112-e chairman note.

RAN2#111-e chairman note

“Common aspects between CG and RACH”, ZTE Corporation, RAN2#113e, 25 Jan - 05 Feb, 2021.

[POST113-e][502][SDT] General/Other CP issues  (ZTE)