3GPP TSG-RAN WG2 Meeting #115 Electronic	R2-2107953

16 – 27 August 2021

Agenda item:	8.16.1

Source:	Nokia, China Telecom (Rapporteurs)

Title:	RAN2 Work Plan for Enhancement for Private Network Support for NG-RAN

WID/SID:	NG_RAN_PRN_enh - Release 17

Document for:	Discussion and Decision

# 1	Introduction

A work item on Enhancement for Private Network Support for NG-RAN was approved for Rel-17 at RAN#90 in RP-202363. This contribution summarises the agreed objectives and suggests an update to the plan to carry out the work in RAN2.

# 2	Work Item

The agreed work item lists the following objectives for RAN2:

- To support SNPN along with subscription / credentials owned by an entity separate from the SNPN:

-	The broadcasting of information to enable SNPN selection for UEs with subscription/credentials owned by an entity separate from the SNPN

-	The associated cell selection/reselection and connected mode mobility support

- To support UE onboarding and provisioning for NPN:

-	The UE onboarding relevant parameter broadcast from SIB

-	The associated cell selection/reselection, cell access control and the connected mode mobility support

- Support of IMS voice and emergency services for SNPN [RAN2]

-	Broadcasting of relevant parameters [RAN2]

It should be noted that:

-	The work is to support the SA2 agreements done in TR 23.700-07 and TS 23.501; and the above sub-bullets may be subject to further discussion depending on SA2.

-	RAN3 is the primary group responsible for the work.

The time budget for the work item gives RAN2 six meetings to complete the work i.e. until December 2021 [RP-202868].

# 3	Work Plan

Table 3.1 below describes a possible plan to carry out the work in RAN2 based on the time budget allocation.

Table 2.1: NPN Time Budget

| Meeting | Date | TU | Meeting Tasks | Post-meeting email discussions |
| --- | --- | --- | --- | --- |
| RAN2#113 | 2021 Q1 | 0.5 | Clarify the scope of the work and RAN2 impacts Start discussion on the enhancements needed for using 3rd party subscriptions Start discussion on the enhancements needed for onboarding | TBD based on meeting discussion |
| RAN2#113bis | 2021 Q2 | 0.5 | Continue discussion on the enhancements needed for using 3rd party subscriptions Continue discussion on the enhancements needed for onboarding Continue discussion on the enhancements needed for IMS voice and emergency services | 38.300 CR based on meeting outcome |
| RAN2#114 | 2021 Q2 | 0.5 | Continue the discussion on the enhancements needed for using 3rd party subscriptions Continue discussion on the enhancements needed for onboarding Continue discussion on the enhancements needed for IMS voice and emergency services Endorse Stage 2 CR, and stage 3 skeleton CRs | On open issues on enhancements needed for using 3rd party subscriptions |
| RAN2#115 | 2021 Q3 | 0.5 | Conclude the information to be broadcast (stage 3 issues may remain open) Conclude the principles of other (mobility, access control etc) enhancements Continue discussion on the enhancements needed for IMS voice and emergency services and for PWS | Draft 38.300, 38.304, and 38.331 CRs based on meeting discussion |
| RAN2#115bis | 2021 Q4 | 0.5 | Continue discussion on open issues Update stage 2 CR and stage 3 CRs | TBA |
| RAN2#116 | 2021 Q4 | 0.5 | Resolve remaining issues Approve Stage 2 and Stage 3 CRs |  |

# 4	Previous Agreements

## 4.1	RAN2#113 Agreements

Agreements on Support SNPN with subscription or credentials by a separate entity were as follows:

A new indicator that "access using credentials from a separate entity is supported" is broadcasted, and the indicator is broadcasted per SNPN in network sharing scenarios.

RAN2 assumes that the new indicator that "access using credentials from a separate entity is supported" is broadcasted in SIB1.

The supported Group IDs are broadcasted

A new indicator that "whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN" is broadcasted, and the indicator is broadcasted per SNPN in network sharing scenario.

RAN2 assumes that the new indicator that "whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN" is broadcasted in SIB1.

In the UE, AS reports to NAS about the following broadcasted new parameters:

- Indicator that "access using credentials from a separate entity is supported" in the cell per SNPN

- Supported Group IDs

- Indicator that "whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN" per SNPN.

Agreements on Support UE onboarding and provisioning for NPN were as follows:

Broadcast a 1-bit indication for onboarding per O-SNPN.

R2 assumes that the 1-bit indication for onboarding is in SIB1.

The UE sends an indication for onboarding to the gNB at RRC Connection Establishment (intention to support AMF selection).

Focus on the O-SNPN scenario. Wait for SA2 further conclusion on how a PLMN can be used as onboarding network.

Agreements on IMS voice and emergency services for SNPN were as follows:

Extend the ims-EmergencySupport field to SNPN cells (it is FFS whether to reuse the existing IE or add new IEs indicating the support for IMS emergency).

For reserved cells specified in TS 38.304, all acceptable cells of an SNPN supporting emergency services are treated as suitable when the UE has an ongoing emergency call.

R17 UEs in SNPN Access Mode can camp on an acceptable SNPN cell supporting emergency services to obtain emergency services.

The voiceFallbackIndication field in RRCRelease and MobilityFromNRCommand is not applicable to SNPN cells.

## 4.2	RAN2#113bis Agreements

General agreements were as follows:

- support of PWS over SNPN:

It seems feasible to do this in R17 from R2 persepctive. Very small impact foreseen

Agreements on Support SNPN with subscription or credentials by a separate entity were as follows:

Use the term "Credentials Holder (CH)" in future RAN2 discussions for the external entity providing subscription or credential for SNPNs.

Use the term "Group IDs for Network Selection (GINs)" in future RAN2 discussions for the service provider Group IDs.

The following assumptions in last meeting are confirmed as agreements,

- The new indicator that "access using credentials from a separate entity is supported" is broadcasted in SIB1.

- The new indicator that "whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN" is broadcasted in SIB1.

GIDs are broadcasted per SNPN in network sharing scenarios.

RAN2 to revise the previous agreement as following:

- In the UE, AS reports broadcast Group IDs per SNPN to NAS.

To supporting SNPN with subscription or credentials by a separate entity, R2 assumes that there is no impact on cell (re)selection (e.g. no need to change suitable cell criteria).

Agreements on Support UE onboarding and provisioning for NPN were as follows:

UE AS forwards the onboarding indication (and Group IDs if Proposal#1 is agreed) per SNPN to UE NAS for onboarding network selection.

No UE impact on connected mode mobility for onboarding.

A new onboarding indication is included in RRCSetupComplete message.

R2 assumes that no enhancement is needed to support onboarding for provisioning the PNI-NPN credentials to UE.

There is no need to introduce an onboarding request indication in RRC messages for UEs in RRC_INACTIVE.

Group IDs per SNPN for onboarding purpose is broadcast in the SIB. FFS whether the Group IDs for onboarding purpose and for credential by separate entity are different.

R2 assumes that onboarding will not impact cell reselection.

## 4.2	RAN2#114 Agreements

General agreements were as follows:

- Reply for LS on limited service availability of an SNPN (C1-21212601/R2-2104704):

- We reply “YES” (to Q1 of the LS), but need to discuss the details of the additional info and the alternatives.

Agreements on Support SNPN with subscription or credentials by a separate entity were as follows:

- GIN for access using CH is broadcast only if Indication of accessing using CH is broadcast.

- RAN2 assumes that NAS does not send selected GINs and two indications related to external credentials to AS.

- There is no impact on cell (re)selection to support SNPN with subscription or credentials by a separate entity.

- RAN2 assume there is no RAN2 UE impact of connected mode mobility for separate credential.

- RAN2 assumes the selected SNPN ID is enough for AMF selection for separate credential.

- GIN is broadcasted by new SIB

Agreements on Support UE onboarding and provisioning for NPN were as follows:

- No additional information except for the already agreed broadcast parameters is needed, unless requested by other WG.

- There is no need to introduce the 1-bit onboarding indication in SIB1 and optional GINs for PLMNs acting as onboarding networks.

- Toggling the 1-bit onboarding indication in SIB1 allows to control congestion due to onboarding request.

- RAN2 confirms that onboarding does not impact the cell reselection procedure.

- For AMF routing, no extra information is needed in addition to the already agreed onboarding request indication in RRCSetupComplete, unless explicitly requested by other WGs.

- Any limitation to a selected set of UEs using uSIM tags is out of RAN2 scope.

- Send an LS to SA2 to ask about separate or joint GIN list for onboarding and separate credentials and GIN encoding.