3GPP TSG-RAN WG2 Meeting #115 Electronic	R2-2107981

Elbonia, 16 – 27 August 2021

Agenda item:	8.1.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	MCCH considerations

WID/SID:	NR_MBS-Core - Release 16

Document for:	Discussion and Decision

# 1	Introduction

Support for MBMS was introduced to E-UTRAN in Release 9 via single frequency network mode of operation (MBSFN), which could be provided on a same frequency layer as regular unicast. In NR multicast services are to be supported as well as broadcast services, but these are not provided via MBSFN

in RAN2#114-e we agreed:

One-to-one mapping between G-RNTI and MBS session is supported in NR MBS. Other mappings FFS

One-to-one mapping between G-CS-RNTI and MBS session is supported in NR MBS. Other mappings FFS.

A UE can support multiple G-RNTIs/G-CS-RNTIs, It is FFS whether this depends on UE capability. Inform RAN1 of this agreement.

Multiple MBS QoS flows corresponding to the same MBS session can be mapped to one or more than one MBS radio bearers.

MCCH is mapped to the DL-SCH for NR MBS delivery mode 2.

MTCH is specified for PTM transmission of NR MBS.

MTCH is mapped to the DL-SCH.

DTCH is reused for PTP transmission of NR MBS.

FFS if there is a need to have specific LCID spaces for the used channels.

Multiplexing/de-multiplexing of different logical channels associated with the same G-RNTI is supported for NR MBS.

FFS if Multiplexing/de-multiplexing of different logical channels associated with the same G-CS-RNTI is supported for NR MBS.

Multiplexing/de-multiplexing of different logical channels associated with the C-RNTI is supported for NR MBS.

For NR MBS delivery mode 2, LTE SC-PTM DRX scheme is used as baseline.

FFS whether For PTM transmission of NR MBS, DRX scheme is independent of DRX for unicast transmission, e.g. supported on a per G-RNTI basis

FFS whether For PTP transmission, DRX operation for unicast transmission is reused

MBS specific SIB is defined to carry MCCH configuration.

MCCH contents should include information about broadcast sessions such as G-RNTI, MBS session ID as well as scheduling information for MTCH (e.g. search space, DRX). L1 parameters that need to be included in MCCH are pending further RAN1 progress and input.

Postpone the discussion on whether dedicated MCCH configuration is required until RAN1 makes progress on BWP/CFR for MCCH.

Indication of an MCCH change due to modification of an ongoing session’s configuration (including session stop) is provided with an explicit notification from the network  (provided that RAN1 confirms a separate bit for this purpose can be accommodated in the MCCH change notification DCI, in addition to a bit for session start notification). FFS on whether this notification can be reused for modification of other information carried by MCCH, if any.

FFS whether the possibility of UE missing an MCCH change notification needs to be addressed or can be left to UE implementation.

At least in case RAN1 decides to utilize RNTI other than MCCH-RNTI for MCCH change notification, MCCH change notification is sent in the first MCCH monitoring occasion of each MCCH repetition period.

We support single MCCH (in this release)

# 2	Discussion on MCCH/MTCH for broadcast

In LTE MBMS solution one has single MCCH and this is logical as a UE camping on a carrier will receive wide carrier bandwidth. But in NR UE’s camping on a cell will only currently receive initial BWP where the UE can receive BCCH and PCCH. When considering providing MBMS service in NR it will be difficult or basically almost impossible to provide appropriate service level for UEs only receiving initial BWP due to capacity limitations.

Similarly when considering MCCH bandwidth one would have similar problem of scheduling MCCH only on CORESET#0 – Already now the capacity is issue on CORESET#0 and limiting the MCCH scheduling could be disastrous. But as this issue is more in RAN1 domain we would consider that decision on bandwidth (e.g. same CFR as MTCH) should be decided in RAN1.

If the MTCH could reside on BWP not containing initial BWP then we have bit of challenge how the UE will be able to perform regular idle activities e.g. paging, BCCH updates and emergency service reception as well as possible updates to broadcast services available (e.g. indicated on MCCH). Basically, we would have few options possible to ensure that these actions are possible while receiving MTCH:

- BWP where MTCH is sent fully contains initial BWP

- UE would be indicated updates on services required in IDLE while receiving MTCH on same BWP where MTCH is sent. The BWP may not overlap with initial BWP.

- We would allow UE to retune receiver to get initial BWP in order to perform required idle activities

Let’s discuss these options in more detail:

| BWP where MTCH is sent fully contains initial BWP | UE would be indicated updates on services required in IDLE while receiving MTCH on same BWP where MTCH is sent | We would allow UE to retune receiver to get initial BWP in order to perform required idle activities |  |
| --- | --- | --- | --- |
| Pros: - Simple - no RF retuning - no overhead Cons: - Require unnecessary wide BWP to be used to receive MTCH thus causing extra power consumption for the UE. - Also if initial BWP MCCH is used to indicates changes then UE would need to regularly check MCCH to check updates to broadcast services. - Still only limited capacity of a cell available to broadcast | Pros: - No need for RF retuning - BWP could be optimized for each service Cons: - Extra overhead to provide indication to UE | Pros: - No overhead Cons: - RF retuning required causing added latency to broadcast and possibly interruption in broadcast reception due to UE specific timing |

Similarly to MCCH bandwidth discussion also MTCH bandwidth (CFR) is being discussed in RAN1 and we should wait for progress in RAN1 before continuing on details of RAN2 signaling:

Proposal 1: Let RAN1 discuss and decide on how to resolve sufficient bandwidth for MCCH and MTCH

Once UE enters RRC_CONNECTED state UE will apply initial BWP until configured with wider BWP with RRCSetup and RRCResume

Observation 1: NW may change BWP of UE at RRCSetup/Resume

If NW does not have information about MBS interest prior reconfiguring BWP one could reconfigure BWP to such that MBS reception is not possible and causing service interrupt. One should avoid this kind of interrupt if possible:

Observation 2: If NW does not have information about MBS interest prior reconfiguring BWP there will be service interruption

Simple way would be not to reconfigure any UE BWP prior NW gets knowledge whether UE is interested in broadcast reception. UE which will not send interest indication would not indicate NW about its lack of interest for MBS services thus NW needs to have its own implementation for handling such UEs e.g. timer running in the NW after which it would definitely know whether UE is not interested in MBS service. This approach would impact both MBS supporting UEs not indicating interest and UEs not supporting MBS.

Another simple alternative is to never configure BWP to be not overlapping with CFR(s) used for MBS services. This would restrict possible BWP usefulness drastically as it would limit BWP configuration for UEs not interested in MBS services, also UEs not supporting MBS.

Observation 3: To avoid MBS service interruption NW cannot configure any UE BWP prior NW knows UE is not interested in MBS service(s)

From system performance point of view better approach would be to have network information about MBS interest prior possibility to reconfigure BWP.

Proposal 2: UE should provide interest to receive broadcast service(s) prior it is possible to reconfigure BWP for the UE

On time domain from UE power consumption point of view optimal would be to provide LTE like MCCH information same time as MTCH is received so that RF retuning would not be required.

Observation 4: UE power consumption point of view optimal would be to provide broadcast service announcements same time as MTCH is received so that RF retuning would not be required

But one needs to consider time domain aspects in more detail i.e. In IDLE (without MBMS) UE basically is only needed to regularly receive paging message while camping on a cell.

But if we would have MCCH then UE would need to receive that in addition. Optimally MCCH would be designed in such a way that whenever UE needs to receive paging also MCCH information is provided but as UEs are distributed and also there are different periodicities of DRX in IDLE/INACTIVE for different UEs then it would be basically impossible to provide common MCCH so that it is optimally located for each UE. In addition to that, MCCH modification period can be very low (e.g. LTE may use SC-MCCH modification as low as 20 ms) to meet low latency requirement of some services.

Observation 5: It is impossible to provide common MCCH so that UE power consumption is minimized for each UE

One would need to consider situations when UE is interested to receive both multicast (and unicast) and broadcast services at a same time. Easiest from network perspective would be that there would be different receivers for unicast/multicast and broadcast services as then there would not be any scheduling restriction for the network i.e. no need to provide UE with such a configuration for all the UEs that they are able to receive broadcast/unicast same time.

Observation 6: UEs interested in both multicast and broadcast services would be simplest to realize if UEs would have different receivers for multicast and broadcast services

But this might be too cumbersome for some implementations and it is worthwhile to consider how we could achieve solutions that do not require this limitation:

In order to allow this basically bit similar solutions would be applicable to IDLE/INACTIVE UEs but now in addition to PCCH and broadcast service announcement reception there would be needed to provide unicast/multicast services. Either NW can try to always configure very wide BWP so that reception all of these are possible same time or then NW tries to provide unicast/multicast services on a BWP overlapping with broadcast BWP. But of course, there could be situation that UE is interested in broadcast service which is on different band completely than unicast/multicast services and then such a optimization would not be possible.

In RRC_CONNECTED state need for monitoring MCCH would restrict NW BWP configuration possibilities a lot as BWP would need to always overlap with MCCH. Thus it would be better that UE is not required to do MCCH reception in connected and instead the UE shall provide NW supporting MBS with its interest in broadcast services and NW could provide configuration in unicast BWP.

Proposal 3: UE is not required to receive MCCH while in RRC_CONNECTED if the active BWP does not overlap with MCCH

Simple approach to realize above proposal is to provide MTCH configuration in dedicated signaling in connected state thus not requiring reception of MCCH in connected state making it simpler for NW to schedule multicast/unicast services as the BWP configuration is not that limited.

Proposal 4: Dedicated signalling of MTCH configuration is allowed for NR MBS delivery mode 2 (in connected state).

# 4	MCCH notification

RAN2 agreed in meeting 113bis-e:

The modification period is defined for NR MCCH and NR MCCH contents are only allowed to be modified at each modification period boundary.

The updated MCCH message should be sent in the same MCCH modification period where the change notification is sent.

UE in RRC IDLE/INACTIVE should be able to monitor/read both MCCH channel and SI/Paging without BWP switch. It is up to RAN1 to decide how this is ensured.

and in 114-e:

Indication of an MCCH change due to modification of an ongoing session’s configuration (including session stop) is provided with an explicit notification from the network  (provided that RAN1 confirms a separate bit for this purpose can be accommodated in the MCCH change notification DCI, in addition to a bit for session start notification). FFS on whether this notification can be reused for modification of other information carried by MCCH, if any.

FFS whether the possibility of UE missing an MCCH change notification needs to be addressed or can be left to UE implementation.

As the updated MCCH message is sent in same modification period where change notification is sent seems to imply that UE is required to monitor change notification on full MCCH bandwidth.

Observation 7: Current agreements seem to imply that UE is required to monitor MCCH change notification on full MCCH bandwidth

In our view it would be better for UE power consumption that change notifications could be sent on narrower bandwidth than MCCH e.g. on same CORESET as the regular unicast paging.

Proposal 5: One should be able to send MCCH change notification on same BWP as UE is required to do for regular IDLE/INACTIVE unicast operation

Then regarding possible issue of missed change notification – This seems to be analogous with regular unicast paging and for that UE is required to monitor multiple occasions with change period in order to ensure reception of change notification. This seems to be good starting point also for MCCH change notification.

Proposal 6: in order to ensure reliable MCCH change notification reception UE should try to receive change notification multiple times per change modification period

# 6	Conclusion

In this paper we discussed IDLE/INACTIVE operation for multicast and broadcast services and came to following observations and proposals:

Proposal 1: Let RAN1 discuss and decide on how to resolve sufficient bandwidth for MCCH and MTCH

Proposal 2: UE should provide interest to receive broadcast service(s) prior it is possible to reconfigure BWP for the UE

Proposal 3: UE is not required to receive MCCH while in RRC_CONNECTED if the active BWP does not overlap with MCCH

Proposal 4: Dedicated signalling of MTCH configuration is allowed for NR MBS delivery mode 2 (in connected state).

Proposal 5: One should be able to send MCCH change notification on same BWP as UE is required to do for regular IDLE/INACTIVE unicast operation

Proposal 6: in order to ensure reliable MCCH change notification reception UE should try to receive change notification multiple times per change modification period