3GPP TSG-RAN WG2 Meeting #115	   R2-2108076

Online, Aug 16th - Aug 27th, 2021

Source: 			ZTE Corporation, Sanechips

Title: 	Interaction between AS-based solution and NAS-based solution for network switching

Agenda item:		8.3.3

Document for: 	Discussion and Decision

- Introduction

In the last meeting, the following agreements have been achieved based on [1] [2] and an LS has been sent to the CT1/SA2 [3].

=>We support at least AS-based solution (with AS-based response) for network switching while leaving RRC_Connected state in NW A. FFS if this may include NAS information Agreements: => AS-based solution for network switching includes two steps: 1-) If configured, UE can send an RRC message to leave RRC_CONNECTED for MUSIM purpose 2-) gNB may release the UE to Idle/Inactive. => Include the following RAN2#113bis-e agreement in the LS: During switching procedure for leaving RRC_CONNECTED state, UE is allowed to enter RRC_IDLE state if it does not receive response message from network within a certain configured time period. FFS for RRC_INACTIVE state The “configured time” for AS-based solution for the UE to leave RRC_CONNECTED without a response is configured by the gNB. Indicate RAN2 is still discussing this for AS-based solution in the LS. Indicate that RAN2 has not discussed the interaction between AS-based solution and any SA2 agreement on NAS messages or NAS-based solution for network switching. 5. Approved LS R2-2106504 LS on network switching for leaving RRC_CONNECTED RAN2 LS out

Meanwhile, CT1/SA2 have implemented the CR for the NAS-Based solution for leaving connected state [4][5][6]. In this paper, we further discuss the interaction between the AS-based solution and NAS-based solution for network switching.

- Discussion

2.1 NAS-Based solution for leaving connected state

According to the [4] [5] [6], the NAS based solution can be pictured as below:

Fig 1: NAS signaling based leaving connected state procedure

For the AS based leaving connected state, it can be divided into two types:

- Leaving without NAS piggyback

Fig 2a: AS signaling based leaving connected state procedure without NAS piggyback

- Leaving with NAS piggyback

Fig 2b: AS signaling based leaving connected state procedure with NAS piggyback

From the above figures, we can see that there is no problem for AS based Leaving without NAS piggyback. However for the case with NAS piggyback, there would be some problems, e.g.

- According to the current CT1 implemented CR [5] [6], NAS will adopt Service Request/Registration Request for to request leaving, if the same message was adopted as NAS PDU in the step 1, the AMF will release the UE context upon receiving such kind of NAS message, which may cause state mismatch between the NG-RAN and CN when the NG-RAN determine to release the UE to the Inactive state at the Step 3.

- In the step 3, the NG-RAN may change to the UE to the Inactive state, but in the step 3a, if the NAS PDU can’t pass the CN layer integrity check, the CN would still release the UE context.

Observation 1: For the AS-Based leaving procedure with NAS piggyback, the NG-RAN and the AMF may make different decisions upon receiving the respective “leaving” message, e.g. the AMF may release the UE to Idle state while the NG-RAN may have released the UE to the Inactive state.

To avoid these problems, the simple solution is to limit the AS-Based solution to the case without NAS piggyback. Otherwise, at least the CN node need get more information when receive the NAS PDU with leaving indication which can be carried in the NAS PDU or N2 message. However, this would have some additional spec impact to the RAN3/SA2/CT1.

Proposal 1: The AS-Based solution can only be adopted for the leaving case without NAS piggyback.

Proposal 2: Send and LS to SA2/CT1 about RAN2’s decision as in the proposal 1.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Observation 1: For the AS-Based leaving procedure with NAS piggyback, the NG-RAN and the AMF may make different decisions upon receiving the respective “leaving” message, e.g. the AMF may release the UE to Idle state while the NG-RAN may have released the UE to the Inactive state.

Proposal 1: The AS-Based solution can only be adopted for the leaving case without NAS piggyback.

Proposal 2: Send and LS to SA2/CT1 about RAN2’s decision as in the proposal 1.

- Reference

- R2-2105257	Network switching procedures for Multi-SIM	Qualcomm Incorporated	discussion

- R2-2106695	Summary of [241][MUSIM] LS to SA2/CT1 on network switching for leaving RRC_CONNECTED (Qualcomm)

- R2-2106504 LS on network switching for leaving RRC_CONNECTED To:	SA2,CT1  Cc:	RAN3

- S2-2105148	Function Description for Multi-USIM devices	Ericsson,

- C1-213809	Updates to Service Request for MUSIM Leaving and Reject Paging in 5GS Apple

- C1-213806	Updates to Registration procedure for MUSIM Leaving in 5GS Apple