3GPP TSG-RAN WG2 Meeting #115-e	R2-2108096

# Electronic, 09 – 27 August, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1137 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to pdsch-HARQ-ACK-CodeBookList |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_L1enh_URLLC-Core |  |  |  |  |  | Date: |  |  | 2021-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the TS 38.213 section 9, it is specified that If a UE is provided pdsch-HARQ-ACK-CodebookList-r16, pdsch-HARQ-ACK-Codebook is replaced by the relevant entry in pdsch-HARQ-ACK-CodebookList-r16.. In the TS 38.213 section 9.1, it is specified that If a UE is provided pdsch-HARQ-ACK-CodebookList, the UE can be indicated by pdsch-HARQ-ACK-CodebookList to generate one or two HARQ-ACK codebooks. If the UE is indicated to generate one HARQ-ACK codebook, the HARQ-ACK codebook is associated with a PUCCH of priority index 0. If a UE is provided pdsch-HARQ-ACK-CodebookList, the UE multiplexes in a same HARQ-ACK codebook only HARQ-ACK information associated with a same priority index.. However, in the field description of pdsch-HARQ-ACK-CodebookList-r16, it states that - pdsch-HARQ-ACK-CodebookList-r16 is a list of configurations for at least two simultaneously constructed HARQ-ACK codebooks, even though the ASN.1 code allows a list of size one. - if the field pdsch-HARQ-ACK-CodebookList is present, the field pdsch-HARQ-ACK-Codebook is ignored for the case at least two HARQ ACK codebooks are simultaneously constructed. The pdsch-HARQ-ACK-CodebookList-r16 can be a list of size one. The TS 38.213 specifies that even if only one codebook is provided with pdsch-HARQ-ACK-CodebookList-r16, the field pdsch-HARQ-ACK-Codebook is ignored/replaced. There is discrepancy between the RRC field description and the TS 38.213. |
| Summary of change: |  | Align the RRC field description with the TS 38.213: - Remove the restriction in the field description that pdsch-HARQ-ACK-CodebookList is a configuration with a size of at least two, i.e., it can also be a size of one. - Change to “if the field pdsch-HARQ-ACK-CodebookList is present, the field pdsch-HARQ-ACK-Codebook is ignored”, i.e., removing the restriction that it applies only to the case that at least two HARQ ACK codebooks are simultaneously constructed. Impact Analysis Impacted 5G architecture options: NR SA, (NG)EN-DC, NE-DC,NR-DC Impacted functionality: Configuration of PDSCH HARQ-ACK Codebook Inter-operability: 1. If the network is implemented according to the CR and the UE is not, then the network can configure pdsch-HARQ-ACK-CodebookList-r16 with the size of the list equal one (expecting the UE to use this configuration), but the UE may not apply the configuration therein. 2. If the UE is implemented according to the CR and the network is not, then the network can configure pdsch-HARQ-ACK-CodebookList-r16 with the size of the list equal one (not expecting UE to use this configuration), but the UE apply the configuration therein. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Configuration of PDSCH HARQ-ACK Codebook is not correctly captured. There is a misunderstanding on which configuration to be used, since there is discrepancy between the RRC field description and the TS 38.213. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the change

### 6.3.2	Radio resource control information elements

#### –	PhysicalCellGroupConfig

The IE PhysicalCellGroupConfig is used to configure cell-group specific L1 parameters.

PhysicalCellGroupConfig information element

-- ASN1START

-- TAG-PHYSICALCELLGROUPCONFIG-START

PhysicalCellGroupConfig ::=         SEQUENCE {

harq-ACK-SpatialBundlingPUCCH       ENUMERATED {true}                                               OPTIONAL,   -- Need S

harq-ACK-SpatialBundlingPUSCH       ENUMERATED {true}                                               OPTIONAL,   -- Need S

p-NR-FR1                            P-Max                                                           OPTIONAL,   -- Need R

pdsch-HARQ-ACK-Codebook             ENUMERATED {semiStatic, dynamic},

tpc-SRS-RNTI                        RNTI-Value                                                      OPTIONAL,   -- Need R

tpc-PUCCH-RNTI                      RNTI-Value                                                      OPTIONAL,   -- Need R

tpc-PUSCH-RNTI                      RNTI-Value                                                      OPTIONAL,   -- Need R

sp-CSI-RNTI                         RNTI-Value                                                      OPTIONAL,   -- Need R

cs-RNTI                             SetupRelease { RNTI-Value }                                     OPTIONAL,   -- Need M

...,

[[

mcs-C-RNTI                          RNTI-Value                                                      OPTIONAL,   -- Need R

p-UE-FR1                            P-Max                                                           OPTIONAL    -- Cond MCG-Only

]],

[[

xScale                              ENUMERATED {dB0, dB6, spare2, spare1}                           OPTIONAL    -- Cond SCG-Only

]],

[[

pdcch-BlindDetection                SetupRelease { PDCCH-BlindDetection }                           OPTIONAL    -- Need M

]],

[[

dcp-Config-r16                      SetupRelease { DCP-Config-r16 }                                 OPTIONAL,   -- Need M

harq-ACK-SpatialBundlingPUCCH-secondaryPUCCHgroup-r16    ENUMERATED {enabled, disabled}             OPTIONAL,   -- Cond twoPUCCHgroup

harq-ACK-SpatialBundlingPUSCH-secondaryPUCCHgroup-r16    ENUMERATED {enabled, disabled}             OPTIONAL,   -- Cond twoPUCCHgroup

pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup-r16          ENUMERATED {semiStatic, dynamic}           OPTIONAL,   -- Cond twoPUCCHgroup

p-NR-FR2-r16                                              P-Max                                     OPTIONAL,   -- Need R

p-UE-FR2-r16                                              P-Max                                     OPTIONAL,   -- Cond MCG-Only

nrdc-PCmode-FR1-r16                ENUMERATED {semi-static-mode1, semi-static-mode2, dynamic}       OPTIONAL,   -- Cond MCG-Only

nrdc-PCmode-FR2-r16                ENUMERATED {semi-static-mode1, semi-static-mode2, dynamic}       OPTIONAL,   -- Cond MCG-Only

pdsch-HARQ-ACK-Codebook-r16            ENUMERATED {enhancedDynamic}                                 OPTIONAL,   -- Need R

nfi-TotalDAI-Included-r16              ENUMERATED {true}                                            OPTIONAL,   -- Need R

ul-TotalDAI-Included-r16               ENUMERATED {true}                                            OPTIONAL,   -- Need R

pdsch-HARQ-ACK-OneShotFeedback-r16     ENUMERATED {true}                                            OPTIONAL,   -- Need R

pdsch-HARQ-ACK-OneShotFeedbackNDI-r16  ENUMERATED {true}                                            OPTIONAL,   -- Need R

pdsch-HARQ-ACK-OneShotFeedbackCBG-r16  ENUMERATED {true}                                            OPTIONAL,   -- Need R

downlinkAssignmentIndexDCI-0-2-r16     ENUMERATED { enabled }                                       OPTIONAL,   -- Need S

downlinkAssignmentIndexDCI-1-2-r16     ENUMERATED {n1, n2, n4}                                      OPTIONAL,   -- Need S

pdsch-HARQ-ACK-CodebookList-r16        SetupRelease {PDSCH-HARQ-ACK-CodebookList-r16}               OPTIONAL,   -- Need M

ackNackFeedbackMode-r16                ENUMERATED {joint, separate}                                 OPTIONAL,   -- Need R

pdcch-BlindDetectionCA-CombIndicator-r16 SetupRelease { PDCCH-BlindDetectionCA-CombIndicator-r16 }  OPTIONAL,   -- Need M

pdcch-BlindDetection2-r16                SetupRelease { PDCCH-BlindDetection2-r16 }                 OPTIONAL,   -- Need M

pdcch-BlindDetection3-r16                SetupRelease { PDCCH-BlindDetection3-r16 }                 OPTIONAL,   -- Need M

bdFactorR-r16                          ENUMERATED {n1}                                              OPTIONAL    -- Need R

]]

}

PDCCH-BlindDetection ::=                INTEGER (1..15)

DCP-Config-r16 ::=                  SEQUENCE {

ps-RNTI-r16                         RNTI-Value,

ps-Offset-r16                       INTEGER (1..120),

sizeDCI-2-6-r16                     INTEGER (1..maxDCI-2-6-Size-r16),

ps-PositionDCI-2-6-r16              INTEGER (0..maxDCI-2-6-Size-1-r16),

ps-WakeUp-r16                       ENUMERATED {true}                                               OPTIONAL,   -- Need S

ps-TransmitPeriodicL1-RSRP-r16      ENUMERATED {true}                                               OPTIONAL,   -- Need S

ps-TransmitOtherPeriodicCSI-r16     ENUMERATED {true}                                               OPTIONAL    -- Need S

}

PDSCH-HARQ-ACK-CodebookList-r16 ::=     SEQUENCE (SIZE (1..2)) OF ENUMERATED {semiStatic, dynamic}

PDCCH-BlindDetectionCA-CombIndicator-r16 ::= SEQUENCE {

pdcch-BlindDetectionCA1-r16                  INTEGER (1..15),

pdcch-BlindDetectionCA2-r16                  INTEGER (1..15)

}

PDCCH-BlindDetection2-r16 ::=                INTEGER (1..15)

PDCCH-BlindDetection3-r16 ::=                INTEGER (1..15)

-- TAG-PHYSICALCELLGROUPCONFIG-STOP

-- ASN1STOP

PhysicalCellGroupConfig field descriptions

ackNackFeedbackMode Indicates which among the joint and separate ACK/NACK feedback modes to use within a slot as specified in TS 38.213 [13] (clause 9).

bdFactorR Parameter for determining and distributing the maximum numbers of BD/CCE for mPDCCH based mPDSCH transmission as specified in TS 38.213 [13] Clause 10.1.

cs-RNTI RNTI value for downlink SPS (see SPS-Config) and uplink configured grant (see ConfiguredGrantConfig).

downlinkAssignmentIndexDCI-0-2 Indicates if "Downlink assignment index" is present or absent in DCI format 0_2. If the field "downlinkAssignmentIndexDCI-0-2" is absent, then 0 bit for "Downlink assignment index" in DCI format 0_2. If the field "downlinkAssignmentIndexDCI-0-2" is present, then the bitwidth of "Downlink assignment index" in DCI format 0_2 is defined in the same was as that in DCI format 0_1 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 9.1).

downlinkAssignmentIndexDCI-1-2 Configures the number of bits for "Downlink assignment index" in DCI format 1_2. If the field is absent, then 0 bit for "Downlink assignment index" in DCI format 1_2. Note that 1 bit and 2 bits are applied if only one serving cell is configured in the DL and the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic. 4 bits is applied if more than one serving cell are configured in the DL and the higher layer parameter pdsch-HARQ-ACK-Codebook is set to dynamic (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 9.1).

harq-ACK-SpatialBundlingPUCCH Enables spatial bundling of HARQ ACKs. It is configured per cell group (i.e. for all the cells within the cell group) for PUCCH reporting of HARQ-ACK. It is only applicable when more than 4 layers are possible to schedule. When the field is absent, the spatial bundling of PUCCH HARQ ACKs for the primary PUCCH group is disabled (see TS 38.213 [13], clause 9.1.2.1). If the field harq-ACK SpatialBundlingPUCCH-secondaryPUCCHgroup is present, harq-ACK-SpatialBundlingPUCCH is only applied to primary PUCCH group. Network does not configure for a UE both spatial bundling of HARQ ACKs and codeBlockGroupTransmission within the same cell group.

harq-ACK-SpatialBundlingPUCCH-secondaryPUCCHgroup Indicates whether spatial bundling of PUCCH HARQ ACKs for the secondary PUCCH group is enabled or disabled. The field is only applicable when more than 4 layers are possible to schedule (see TS 38.213 [13], clause 9.1.2.1). When the field is absent, the use of spatial bundling of PUCCH HARQ ACKs for the secondary PUCCH group is indicated by harq-ACK-SpatialBundlingPUCCH. See TS 38.213 [13], clause 9.1.2.1. Network does not configure for a UE both spatial bundling of HARQ ACKs and codeBlockGroupTransmission within the same cell group.

harq-ACK-SpatialBundlingPUSCH Enables spatial bundling of HARQ ACKs. It is configured per cell group (i.e. for all the cells within the cell group) for PUSCH reporting of HARQ-ACK. It is only applicable when more than 4 layers are possible to schedule. When the field is absent, the spatial bundling of PUSCH HARQ ACKs for the primary PUCCH group is disabled (see TS 38.213 [13], clauses 9.1.2.2 and 9.1.3.2). If the field harq-ACK SpatialBundlingPUSCH-secondaryPUCCHgroup is present, harq-ACK-SpatialBundlingPUSCH is only applied to primary PUCCH group. Network does not configure for a UE both spatial bundling of HARQ ACKs and codeBlockGroupTransmission within the same cell group.

harq-ACK-SpatialBundlingPUSCH-secondaryPUCCHgroup Indicates whether spatial bundling of PUSCH HARQ ACKs for the secondary PUCCH group is enabled or disabled. The field is only applicable when more than 4 layers are possible to schedule (see TS 38.213 [13], clauses 9.1.2.2 and 9.1.3.2). When the field is absent, the use of spatial bundling of PUSCH HARQ ACKs for the secondary PUCCH group is indicated by harq-ACK-SpatialBundlingPUSCH. See TS 38.213 [13], clauses 9.1.2.2 and 9.1.3.2. Network does not configure for a UE both spatial bundling of HARQ ACKs and codeBlockGroupTransmission within the same cell group.

mcs-C-RNTI RNTI to indicate use of qam64LowSE for grant-based transmissions. When the mcs-C-RNTI is configured, RNTI scrambling of DCI CRC is used to choose the corresponding MCS table.

nfi-TotalDAI-Included Indicates whether the NFI and total DAI fields of the non-scheduled PDSCH group is included in the non-fallback DL grant DCI (see TS 38.212 [17], clause 7.3.1). The network configures this only when enhanced dynamic codebook is configured (pdsch-HARQ-ACK-Codebook is set to enhancedDynamic).

nrdc-PCmode-FR1 Indicates the uplink power sharing mode that the UE uses in NR-DC in frequency range 1 (FR1) (see TS 38.213 [13], clause 7.6).

nrdc-PCmode-FR2 Indicates the uplink power sharing mode that the UE uses in NR-DC in frequency range 2 (FR2) (see TS 38.213 [13], clause 7.6).

pdcch-BlindDetection, pdcch-BlindDetection2, pdcch-BlindDetection3 Indicates the reference number of cells for PDCCH blind detection for the CG. Network configures the field for each CG when the UE is in NR DC and sets the value in accordance with the constraints specified in TS 38.213 [13]. The network configures pdcch-BlindDetection only if the UE is in NR-DC. The network configures pdcch-BlindDetection2 only if the UE is in NR-DC with at least one downlink cell using Rel-16 PDCCH monitoring capability. The network configures pdcch-BlindDetection3 only if the UE is in NR-DC with at least one downlink cell using Rel-15 PDCCH monitoring capability.

pdcch-BlindDetectionCA-CombIndicator Configure one combination of pdcch-BlindDetectionCA1 (for R15) and pdcch-BlindDetectionCA2 (for R16) for UE to use for scaling PDCCH monitoring capability if the number of serving cells configured to a UE is larger than the reported capability, and if UE reports more than one combination of pdcch-BlindDetectionCA1 and pdcch-BlindDetectionCA2 as UE capability. The combination of pdcch-BlindDetectionCA1 and pdcch-BlindDetectionCA2) configured by pdcch-BlindDetectionCACombIndicator is from the more than one combination of pdcch-BlindDetectionCA1 and pdcch-BlindDetectionCA2 reported by UE (see TS 38.213 [13], clause 10).

p-NR-FR1 The maximum total transmit power to be used by the UE in this NR cell group across all serving cells in frequency range 1 (FR1). The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-UE-FR1 (configured total for all serving cells operating on FR1).

p-NR-FR2 The maximum total transmit power to be used by the UE in this NR cell group across all serving cells in frequency range 2 (FR2). The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-UE-FR2 (configured total for all serving cells operating on FR2). This field is only used in NR-DC.

ps-RNTI RNTI value for scrambling CRC of DCI format 2-6 used for power saving (see TS 38.213 [13], clause 10.1).

ps-Offset The start of the search-time of DCI format 2-6 with CRC scrambled by PS-RNTI relative to the start of the drx-onDurationTimer of Long DRX (see TS 38.213 [13], clause 10.3). Value in multiples of 0.125ms (milliseconds). 1 corresponds to 0.125 ms, 2 corresponds to 0.25 ms, 3 corresponds to 0.375 ms and so on.

ps-WakeUp Indicates the UE to wake-up if DCI format 2-6 is not detected outside active time (see TS 38.321 [3], clause 5.7). If the field is absent, the UE does not wake-up if DCI format 2-6 is not detected outside active time.

ps-PositionDCI-2-6 Starting position of UE wakeup and SCell dormancy indication in DCI format 2-6 (see TS 38.213 [13], clause 10.3).

ps-TransmitPeriodicL1-RSRP Indicates the UE to transmit periodic L1-RSRP report(s) when the drx-onDurationTimer does not start (see TS 38.321 [3], clause 5.7). If the field is absent, the UE does not transmit periodic L1-RSRP report(s) when the drx-onDurationTimer does not start.

ps-TransmitOtherPeriodicCSI Indicates the UE to transmit periodic CSI report(s) other than L1-RSRP reports when the drx-onDurationTimer does not start (see TS 38.321 [3], clause 5.7). If the field is absent, the UE does not transmit periodic CSI report(s) other than L1-RSRP reports when the drx-onDurationTimer does not start.

p-UE-FR1 The maximum total transmit power to be used by the UE across all serving cells in frequency range 1 (FR1) across all cell groups. The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-NR-FR1 (configured for the cell group).

p-UE-FR2 The maximum total transmit power to be used by the UE across all serving cells in frequency range 2 (FR2) across all cell groups. The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-NR-FR2 (configured for the cell group).

pdsch-HARQ-ACK-Codebook The PDSCH HARQ-ACK codebook is either semi-static or dynamic. This is applicable to both CA and none CA operation (see TS 38.213 [13], clauses 9.1.2 and 9.1.3). If pdsch-HARQ-ACK-Codebook-r16 is signalled, UE shall ignore the pdsch-HARQ-ACK-Codebook (without suffix). If the field pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup is present, pdsch-HARQ-ACK-Codebook is applied to primary PUCCH group. Otherwise, this field is applied to the cell group (i.e. for all the cells within the cell group).

pdsch-HARQ-ACK-CodebookList A list of configuration for one or two HARQ-ACK codebooks. Each configuration in the list is defined in the same way as pdsch-HARQ-ACK-Codebook (see TS 38.212 [17], clause 7.3.1.2.2 and TS 38.213 [13], clauses 7.2.1, 9.1.2, 9.1.3 and 9.2.1). If this field is present, the field pdsch-HARQ-ACK-Codebook is ignored. If this field is present, the value of this field is applied for primary PUCCH group and for secondary PUCCH group (if configured).

pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup The PDSCH HARQ-ACK codebook is either semi-static or dynamic. This is applicable to both CA and none CA operation (see TS 38.213 [13], clauses 9.1.2 and 9.1.3). It is configured for secondary PUCCH group.

pdsch-HARQ-ACK-OneShotFeedback When configured, the DCI_format 1_1 can request the UE to report A/N for all HARQ processes and all CCs configured in the PUCCH group (see TS 38.212 [17], clause 7.3.1).

pdsch-HARQ-ACK-OneShotFeedbackCBG When configured, the DCI_format 1_1 can request the UE to include CBG level A/N for each CC with CBG level transmission configured. When not configured, the UE will report TB level A/N even if CBG level transmission is configured for a CC. The network configures this only when pdsch-HARQ-ACK-OneShotFeedback is configured.

pdsch-HARQ-ACK-OneShotFeedbackNDI When configured, the DCI_format 1_1 can request the UE to include NDI for each A/N reported. The network configures this only when pdsch-HARQ-ACK-OneShotFeedback is configured.

sizeDCI-2-6 Size of DCI format 2-6 (see TS 38.213 [13], clause 11.5).

sp-CSI-RNTI RNTI for Semi-Persistent CSI reporting on PUSCH (see CSI-ReportConfig) (see TS 38.214 [19], clause 5.2.1.5.2). Network always configures the UE with a value for this field when at least one CSI-ReportConfig with reportConfigType set to semiPersistentOnPUSCH is configured.

tpc-PUCCH-RNTI RNTI used for PUCCH TPC commands on DCI (see TS 38.213 [13], clause 10.1).

tpc-PUSCH-RNTI RNTI used for PUSCH TPC commands on DCI (see TS 38.213 [13], clause 10.1).

tpc-SRS-RNTI RNTI used for SRS TPC commands on DCI (see TS 38.213 [13], clause 10.1).

ul-TotalDAI-Included Indicates whether the total DAI fields of the additonal PDSCH group is included in the non-fallback UL grant DCI (see TS 38.212 [17], clause 7.3.1). The network configures this only when enhanced dynamic codebook is configured (pdsch-HARQ-ACK-Codebook is set to enhancedDynamic).

xScale The UE is allowed to drop NR only if the power scaling applied to NR results in a difference between scaled and unscaled NR UL of more than xScale dB (see TS 38.213 [13]). If the value is not configured for dynamic power sharing, the UE assumes default value of 6 dB.

| Conditional Presence | Explanation |
| --- | --- |
| MCG-Only | This field is optionally present, Need R, in the PhysicalCellGroupConfig of the MCG. It is absent otherwise. |
| SCG-Only | This field is optionally present, Need S, in the PhysicalCellGroupConfig of the SCG in (NG)EN-DC as defined in TS 38.213 [13]. It is absent otherwise. |
| twoPUCCHgroup | This field is optionally present, Need R, if secondary PUCCH group is configured. It is absent otherwise. |