3GPP TSG-RAN WG2 #115e	Tdoc R2-2108098

Electronic meeting, 2021-08-09 - 2021-08-27

Agenda Item:	8.5.3

Source:	Ericsson

Title:	Harmonizing UL CG enhancements in NR-U and URLLC

Document for:	Discussion, Decision

# Introduction

The work item on Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR [1] specifies the below as one objective:

Uplink enhancements for URLLC in unlicensed controlled environments [RAN1, RAN2]:

  - Specify support for UE-initiated COT for FBE with minimum specification effort

  - Harmonizing UL configured-grant enhancements in NR-U and URLLC introduced in Rel-16 to be applicable for unlicensed spectrum

In this contribution, we address remaining FFS resulting from the discussions and agreements from previous meetings, see below.

Agreements (RAN2#113) From RAN2 perspective 1 LCH based prioritization and cg-RetransmissionTimer can be configured together in Rel-17 (consensus) 2 Option 1: AutoTx and CGRT are responsible for deprioritized MAC PDU and LBT-failed MAC PDU, respectively. If CGRT is not configured, LBT-failed MAC PDU is not retransmitted. If AutoTx is not configured, deprioritized MAC PDU is not retransmitted. 3 The MAC entity stops cg-RetransmissionTimer when the CG resource associated with the timer is deprioritized due to LCH-based prioritization. 4 FFS With cg-RetransmissionTimer and LCH-based prioritization configured, the MAC entity can prioritize between initial transmissions and retransmissions on a CG based on priority of multiplexed LCH(s) -or to be multiplexed. 5 LBT failure is not considered when determining a grant priority for intra-UE prioritization (17/22) 6 Configuring a subset of HARQ processes as “restricted processes” for transmission of data from higher priority LCHs is not supported (18/22) 7 Enhancements for handling conflicting DG-CG transmissions of the same HARQ process are not supported (18/22)

Agreements (RAN2#114): From RAN2 perspective 1 When both of lch-based Prioritization and cg-RetransmissionTimer are configured, HARQ processes sharing between multiple CG configurations are allowed. No specification change is required. 2 RAN2 confirm that neither autonomous transmission nor autonomous retransmission is triggered if UL grant is prioritized and LBT fails while AutonomousTx is configured and cg-RetransmissionTimer is not configured. No specification change is required. 3 RAN2 confirm that autonomous retransmission is triggered if UL grant is prioritized and LBT fails while AutonomousTx is not configured and cg-RetransmissionTimer is configured. No specification change is required. 4 RAN2 confirm that autonomous retransmission is triggered if UL grant is prioritized and LBT fails while AutonomousTx and cg-RetransmissionTimer are configured. No specification change is required. 5 RAN2 confirm that autonomous transmission is triggered if UL grant is deprioritized while AutonomousTx is configured and cg-RetransmissionTimer is not configured. No specification change is required. 6 RAN2 confirm that autonomous transmission is triggered if the transmission of the obtained MAC PDU has not been completely performed and if UL grant is deprioritized while AutonomousTx and cg-RetransmissionTimer are configured. No specification change is required. 7 The HARQ process is kept as pending even if a CG is de-prioritized while the HARQ state of the associated HARQ process is pending (i.e. MAC PDU hasn’t been transmitted). No specification change is required. 8 When cg-RetransmissionTimer and lch-basedPrioritization are configured, for overlapping CGs, the MAC entity prioritizes the initial transmission of higher priority data over autonomous retransmission of lower priority data. FFS how to implement this in Rel-17 after some of the Rel-16 discussion takes place.

The structure of the paper is:

- Section 2.1: elaboration of our views for inconclusive issues in the email discussion [5].

- Section 2.2: one aspect that was mentioned but not included in the email discussion [5].

# Discussion

2.1 	When both cg-RetransmissionTimer and lch-basedPrioritization are configured

Here we discuss the behaviour when both cg-RetransmissionTimer and lch-basedPrioritization are configured together, and HARQ process sharing between multiple CG configurations is allowed (i.e. formula for HARQ process ID is not used).

In Rel-16 IIoT, lch-basedPrioritization had been introduced to handle overlapping grants, i.e. to provide a decision basis on which of the overlapping grants is to be used. It was made dependent on which LCH has data for transmission on the grant (due to data availability and LCP-restrictions), and of course on the LCH priority. When discussing the CG-CG overlapping case, let’s first recall the purpose of multiple CG configurations, i.e. to handle traffic flows of different periodicity, size and QoS requirements. These differences motivate configuration of different CGs with different period, TBS, MCS etc. With LCP-restrictions, LCHs of different QoS could be mapped to only use suitable CG configurations. In summary, LCH-based prioritization handles the overlapping CG case.

On the other hand, Rel-16 IIoT didn’t foresee LCH-based prioritization for different LCHs on the same CG. In fact, a single CG configuration (same TBS, MCS, period) is unable to ideally serve different LCHs such as MBB and URLLC, for which also different TBS, MCS, period are expected. We thus don’t see the need to optimize colliding transmission/retransmissions of different LCHs according to their priority within a single CG configuration. Furthermore, since this behaviour was not part of Rel-16 IIoT, we don’t consider it as part of the harmonization with NR-U targeted in this WI either.

The prioritization between initial transmission of higher priority data and retransmission of lower priority data has been discussed in both two discussions [4] and [5]. The clarification in Rel-16 for NR-U behaviour explicated that UE shall prioritize the retransmission before initial transmission for HARQ Process ID selection. Based the clarification in Rel-16 and the following issues, we do not see a need for change in NR-U behaviour in Rel-17 as  the solutions to these issues would require new complex standard behaviours that are beyond the scope of the harmonization WI objective.

- New transmissions for higher-priority LCHs may be prioritized over the same HARQ retransmission of a lower-priority LCH repeatedly. This raises the question of whether LCH-based prioritization should always be applied, or only up to a certain limit of HARQ retransmission de-prioritizations, since subsequent HARQ retransmission de-prioritizations (without possibility to transmit) would lead to data loss eventually on the MAC layer due to CG Timer expiry (i.e., can only be recovered by high layer retransmissions, such as RLC). Note that this wouldn’t be the case for CG-CG overlaps with the LCH data mapped to different CGs, since the CGs would be typically configured to have also non-overlapping resources, where de-prioritized data would be transmittable independent of the higher-priority data.

- In the case that all HARQ processes are occupied for retransmissions (i.e., either the configuredGrantTimer is running or the HARQ process is pending), there may be new arrival data in the higher layer buffer that has a higher priority LCH than any one of the LCH priority of the MAC PDU for retransmissions. The proposal in the email discussion [5] (i.e., the option 2 in the Question 2) is that the new arrival data that has a higher priority LCH can NOT be transmitted due to that there is no available HARQ process for initial transmission. But this is contradictory to the intention that the LCH-based prioritization is used to prioritize between initial transmission and re-transmissions, because there would be high priority URLLC data “blocked” by low priority eMBB data due to the lack of the available HARQ process.

- It is unclear how mechanisms for handling de-prioritization of HARQ retransmissions shall be applied, i.e., whether as regular lch-basedPrioritization with IIoT-based autonomousTx, or as that these HARQ retransmissions shall simply be kept for future potential consideration at the next CG opportunity. The former would require another thorough check and “hack” of the MAC spec upon the already complex specification for autonomousTx, while the latter would introduce deviating behaviours from the current modelling of CG de-prioritizations configured with autonomousTx.

- Network configures the configuredGrantTimer and cg-RetransmissionTimer values assuming a certain number of re-transmissions to fulfil the PER target for all traffic restricted in that CG configuration. For example, cg-RetransmissionTimer = 1/3 configuredGrantTimer allowing about 3 retransmissions to reach the PER target. If operation is impacted, i.e., made dependable by the arrival of the new data so that the actual retransmission times is smaller when de-prioritized, then network cannot configure correct timer values and satisfy the QoS requirement. There is no possibility anymore to optimally configure the system parameters.

- There are comments in the email discussion [5] that the support of the multiple CGs is optional and so it would be more flexible if LCH-based prioritization would be allowed within one CG. However, in Rel-16 IIoT discussion, LCH-based prioritization within one CG is not allowed and LCH-based prioritization is achieved among multiple CGs. Thus, LCH-based prioritization within one CG goes beyond the Work Item scope of harmonizing UL configured-grant enhancements, but a separate enhancement. Even if RAN2 would agree to enhance LCH-based prioritization within one CG, the feature would anyway be optional and so unclear if would be supported by most let alone all UEs and hence the question on the flexibility.

In conclusion, given these concerns that no gain is foreseen (in light of multiple CG configurations) and that the new complex standardization solution is required, we strongly believe:

- LCH-based prioritization does not apply for prioritization between initial transmission and retransmission within one CG.

Another aspect to be discussed relates to HARQ process ID sharing between CG configurations. It is agreed in the previous meeting that current specification allows HARQ process ID sharing between multiple CG configurations when both lch-basedPrioritization and cg-RetransmissionTimer are configured.  But, we don’t see any benefits of HARQ process sharing between CGs that also have the intention to prioritize over one another based on the LCH priority:

- It is technically equivalent to lch-basedPrioritization within one CG for those overlapping HARQ processes (shared process pool), see further analysis leading to the Proposal 1.

- If we allow HARQ process sharing, the same HARQ processes for transmission and autonomous retransmission may overlap between the CGs and this leads to conflicts. With a shared HARQ process pool, according to current specifications (and probably in future given the complexity), some aspects of the prioritization would be left to UE implementation, e.g., if for both overlapping CGs the same HARQ process for retransmission is chosen, then the question is on which CG the retransmission is carried out. Some issues are already brought up in the email discussion [5] (for example section 2.2.3), but there are already diverging views on the existing issues and most likely more issues would be found.

- Allowing HARQ process sharing contradicts with the network’s intention to configure lch-basedPrioritization in which different priority data is assumed to be separated on different CGs.  In other words, this configuration of shared HARQ processes defeats the purpose of network-controlled lch-basedPrioritization among CGs, i.e., the network is not in control anymore to configure the UE to choose a CG according to the LCH priority to be mapped to the CG. In general, HARQ process sharing is only suited for the same priority data, i.e. not for the different priority data and configured with lch-basedPrioritization.

In summary, a network implementation can configure separate HARQ process pools for the different CG configurations for the case that LCH-based prioritization among the CGs is desired/required, which is independent of cg-RetransmissionTimer configuration. For the case when HARQ processing is used among CGs, LCH-based prioritization among the CGs is not needed among those CGs and thus no specification enhancements are required. In this case, the network can configure those CGs so that their resources are not intentionally overlapped. For the occasional overlapping scenarios, it would be acceptable to follow the legacy Rel-16 specification (regardless what they would be), since a strict LCH-based prioritization among those CGs is not needed.

Furthermore, addressing the HARQ processes sharing among CG case would require complex specification changes which cannot be motivated, or eventually due to its complexity, the prioritization is left to UE implementation. Thus, we propose that:

- RAN2 does not introduce any spec enhancements regarding HARQ process sharing between CGs for the case when lch-basedPrioritization is configured.

2.2	Multi-TB scheduling in CG and without cg-retransmissionTimer

One aspect that was not yet discussed in email discussion [5] is whether the HARQ process ID formula is also usable when cg-RetransmissionTimer is not configured, but multiple consecutive slots within an CG period are configured, i.e. based on parameter cg-nrofSlots (or cg-nrofPUSCH-InSlot). These features were introduced in NR-U for allowing to have subsequent CG occasions per CG period, in order for the UE to autonomously transmit pending HARQ processes soon after an LBT failure (when cg-retransmissionTimer is configured and UE chooses HARQ ID, since the features are specified as UE feature (FG 10-28), only supported for unlicensed in Rel-16).  In the case the HARQ formula is used, as agreed for Rel-17 when cg-retransmissionTimer is not configured, as well as when multi-TB scheduling is configured, the HARQ formula, as currently specified, erroneously indicates the same HARQ ID for each of the slots/occasions within the same CG period. This is wrong in case when multiple TBs are used for different data, i.e. when repetitions are not configured.

One option is to allow UE to choose HARQ processes by indicating HARQ process ID in the CG-UCI. But Ran1 rules out the possiblity to use CG-UCI without cg-retransmissionTimer, i.e., CG-UCI is enabled/disabled also by the RRC parameter cg-RetransmissionTimer-r16. Thus, there are the only two following options to solve this issue as part of the NR-U CG harmonization work:

- A) Correct the HARQ formula: considering the multiple occasions per period in the formula for the HARQ ID, leading to different HARQ IDs for the occasions.

- B) Disallow multi-TB scheduling without cg-retransmissionTimer: i.e. only allow it when UE chooses the HARQ ID itself. This would consider that multi-TB scheduling is only considered useful when LBT-failures are assumed typical (in the scenario when cg-retransmissionTimer is configured), and thus subsequent transmission occasions per period are useful to retransmit a pending HARQ processes. It is noted that a similar behavior as multi-TB scheduling could be achieved with an alternative configuration i.e. by allocating CG with short periodicity or multiple parallel CG configurations. Example: instead of cg-nrofSlots = 3 one could configure 3 CG configurations with the same period but offset of 1 after each other.

We believe that the benefits of cg-nrofSlots (and cg-nrofPUSCH-inSlot) can be achieved likewise with configuring multiple CG configurations with high periodicities, for the case when cg-retransmissionTimer is not configured, thus we believe there is no need to do any modification of the HARQ formula.

- If cg-retransmissionTimer is not configured, cg-nrofSlots and cg-nrofPUSCH-InSlot shall not be configured (i.e. no changes to the HARQ ID formula required).

# Conclusion

In the previous sections we made the following proposals:

Proposal 1	LCH-based prioritization does not apply for prioritization between initial transmission and retransmission within one CG.

Proposal 2	RAN2 does not introduce any spec enhancements regarding HARQ process sharing between CGs for the case when lch-basedPrioritization is configured.

Proposal 3	If cg-retransmissionTimer is not configured, cg-nrofSlots and cg-nrofPUSCH-InSlot shall not be configured (i.e. no changes to the HARQ ID formula required).

# References

RP-201310, Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR, 3GPP RAN#88e, June 29 - July 3, 2020.

R2-2100233, Harmonizing UL CG enhancements in NR-U and URLLC, Ericsson, RAN2#113, Electronic meeting, Jan 25th – Feb 5th, 2021

R2-2102087, Summary of URLLC over unlicensed controlled environment, InterDigital, RAN2#113, Electronic meeting, Jan 25th – Feb 5th, 2021

R2-2106556, Summary of [POST113bis-e][505][R17 IIoT] URLLC in UCE, RAN2#114

R2-21xxxxx, [Post114-e][510][URLLC/IIoT] Open issues for UCE (Mediatek), RAN2#115