3GPP TSG-RAN WG2 #115e	R2-2108134

Electronic meeting, 16 – 27 August, 2021

Agenda item:		8.2.2.3	Activation of deactivated SCG

Title:		UE request for SCG activation

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

There had been some agreements in the past meeting, while in the last meeting the Rapporteur input was only discussed due to lack of time, which triggered the post-meeting email discussion [1] for gathering the companies’ view on each open issue. During the email discussion, the UE-triggered SCG activation has been also discussed. In this contribution, we discuss further details of the UE-triggered SCG activation and provide our views.

# 2. Discussion

2.1	Possible solutions for UE-triggered SCG activation

In RAN2#113e, RAN2 had agreed that the SCG activation can be requested by the UE. Then, during the email discussion [Post114-e][231], there are 3 solutions on the table, where the UE-triggered SCG activation is assumed [1]. In the high level, there are two cases, i.e. w/o SCG bearers (solution 1) and w/ SCG bearers (solution 2&3).

# from [Post114-e][231] RAN2 agreed to support UE-triggered SCG activation and the following solutions were suggested: 1) There are no SCG bearers, only split bearers, the primary path is MCG so when the SCG is deactivated, the UE sends BSR/UL data on the MCG leg and the network decides to trigger SCG activation if needed 2) In case of UL data arrival on SCG bearers, the UE sends an indication to the MCG, then the network can trigger SCG activation 3) In case of UL data arrival on SCG bearers, the UE initiates random access towards the SCG then the SN requests SCG activation to the MN using the SN-initiated modification procedure.

Firstly, the solution 1) is anyway necessary regardless whether the UE can directly request the SCG activation to the SN or not in some scenarios. It would be good to confirm this.

Proposal 1: RAN2 to confirm that the solution 1) should be supported as one solution for UE-triggered SCG activation:

1)	There are no SCG bearers, only split bearers, the primary path is MCG so when the SCG is deactivated, the UE sends BSR/UL data on the MCG leg and the network decides to trigger SCG activation if needed

For the solution 2) and 3), the main difference is which CG is used for the UE to send the activation request. On the other hand, the description of solution 3) includes more than that of the solution 2) with respect to the network behaviour, i.e. SN requests the SCG activation to the MN.

However, if the SN needs to confirm with the MN, it would be less attractive to introduce the SCG activation request directly from the UE to the SN, and a potential delay will be almost the same as the solution 2). With this observation, we consider if the UE can initiate the random access to the SN directly, the SN should be also able to decide whether to activate the SCG or not before informing the MN of the UE’s request or before receiving the response (e.g. confirmation) from the MN. Note that it is named solution 3’). Otherwise, the solution 2) might be sufficient.

Proposal 2: RAN2 to select one of two solutions in the case of UL data arrival on SCG bearers:

2)	In case of UL data arrival on SCG bearers, the UE sends an indication to the MCG, then the network can trigger SCG activation

3’)	In case of UL data arrival on SCG bearers, the UE initiates random access towards the SCG, then the SN can trigger SCG activation before receiving MN confirmation.

2.2	Other scenarios for UE-triggered SCG activation

In RAN2#113bis-e, the following proposal 5 was provided based on the companies’ input [2], although it was not confirmed as agreed. There are two more scenarios as FFS to support.

Proposal 5: UE can trigger SCG activation request in following cases. • Arrival of UL data for SCG bearer. • MCG failure while SCG is deactivated. • FFS on arrival of UL data for split bearer with SCG as primary path. • FFS on arrival of UL data for split bearer with total data volume exceeds the threshold.

In the case of UL data arrival for split bearer with SCG as primary path, the same approach can be applied as the SCG bearers, i.e. decision for the proposal 2.

In the case of UL data arrival for split bearer with total data volume exceeds the threshold, there are two sub-cases, i.e. primary path set to MCG or SCG. If the primary path is set to the MCG, the same approach as the solution 1) above would be sufficient and simple. If the primary path is set to the SCG, the same approach as decision for the proposal 2 would be simpler.

Proposal 3: RAN2 to discuss two more scenarios currently marked as FFS, on top of the decisions for proposal 2.

# 3. Conclusion

In this contribution we discussed the UE-triggered SCG activation and made the following proposals.

Proposal 1: RAN2 to confirm that the solution 1) should be supported as one solution for UE-triggered SCG activation:

1)	There are no SCG bearers, only split bearers, the primary path is MCG so when the SCG is deactivated, the UE sends BSR/UL data on the MCG leg and the network decides to trigger SCG activation if needed

Proposal 2: RAN2 to select one of two solutions in the case of UL data arrival on SCG bearers:

2)	In case of UL data arrival on SCG bearers, the UE sends an indication to the MCG, then the network can trigger SCG activation

3’)	In case of UL data arrival on SCG bearers, the UE initiates random access towards the SCG, then the SN can trigger SCG activation before receiving MN confirmation.

Proposal 3: RAN2 to discuss two more scenarios currently marked as FFS, on top of the decisions for proposal 2.

# References

[1] R2-210xxxx, [Post114-e][231][R17 DCCA] SCG activation/deactivation options (Huawei)

[2] R2-2104317, “Summary of AI 8.2.2.3: Activation of deactivated SCG”, ZTE