3GPP TSG-RAN WG2 Meeting #115-e	R2-2108189

# Electronic Meeting, 9 – 27 August, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4709 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ANS.1 misalignment for the SCGFailureInformationNR message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core, NR_unlic-Core, NR_IAB-Core, NR_Mob_enh-Core |  |  |  |  |  | Date: |  |  | 2021-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In current TS 36.331 v16.5.0, in case the UE is in EN-DC or NGEN-DC and the UE experiences an NR SCG RLF, it sends the SCGFailureInformationNR message to the E-UTRA MCG. This message includes different failure causes that are referring to possible RLF use cases that has been identified during Rel-15 and Rel-16. The Rel-16 version of SCGFailureInformationNR message includes the following: FailureReportSCG-NR-r15 ::= SEQUENCE { failureType-r15 ENUMERATED { t310-Expiry, randomAccessProblem, rlc-MaxNumRetx, synchReconfigFailureSCG, scg-reconfigFailure, srb3-IntegrityFailure, other-r16}, measResultFreqListNR-r15 MeasResultFreqListFailNR-r15 OPTIONAL, measResultSCG-r15 OCTET STRING OPTIONAL, ..., [[ locationInfo-r16 LocationInfo-r10 OPTIONAL, logMeasResultListBT-r16 LogMeasResultListBT-r15 OPTIONAL, logMeasResultListWLAN-r16 LogMeasResultListWLAN-r15 OPTIONAL, failureType-v1610 ENUMERATED {t312-Expiry, scg-lbtFailure, beamFailureRecoveryFailure, bh-RLF-r16, spare4, spare3, spare2, spare1} OPTIONAL ]] } The main problem with the Rel-16 version of the SCGFailureInformationNR message is that the field failureType-r15 has a difference number of codepoints with respect to the Rel-15 version of the same field. The Rel-15 version of SCGFailureInformationNR message includes the following: FailureReportSCG-NR-r15 ::= SEQUENCE { failureType-r15 ENUMERATED { t310-Expiry, randomAccessProblem, rlc-MaxNumRetx, synchReconfigFailureSCG, scg-reconfigFailure, srb3-IntegrityFailure}, measResultFreqListNR-r15 MeasResultFreqListFailNR-r15 OPTIONAL, measResultSCG-r15 OCTET STRING OPTIONAL, ... } As shown above, the field failureType-r15 has 7 entries in the Rel-16 version of TS 36.331, whereas in the Rel-15 version of TS 36.331 it has only 6. The reason for this is because in Rel-16 a new failure cause was added (i.e., other-r16) and this was done in a non-backward compatible way with respect the existing Rel-15. The result of this is that a Rel-15 eNB does not have any knowledge on the possibility for the UE to signal the field failureType-r15 with the value “other-r16”. In fact, in the Rel-15 version of the failureType-r15 field there are no spare values that are possible to use in later releases. Due to this ASN.1 misalignment, the consequence is that when a Rel-16 UE includes the field failureType-r15 with the value “other-r16” and sends it to a Rel-15 eNB, this Rel-15 eNB may consider the all SCGFailureInformationNR message as not valid and will trigger a ASN.1 syntax error. While this may be solved by implementation in intra-vendor deployment, for the case of inter-vendor deployment this may be more critical and may degrade the performance of the all MR-DC operations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Section 6.2.2 - A new field, named failureTypeOther-r16, is added in order to let the network to know when it shall ignore the field failureType-r15. Impact Analysis Impacted 5G architecture options: (NG)EN-DC, NE-DC,NR-DC Impacted functionality: SCG failure information Inter-operability: 1. If the network is implemented according to the CR and the UE is not, the network may consider the SCGFailureInformation message as invalid and trigger a RRC re-establishment procedure instead of performing a RRC connection recovery. 2. If the UE is implemented according to the CR and the network is not, there is no inter-operability issue. This CR is considered mandatory to support the impacted functionality. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In the CR is not agreed, the network may consider the SCGFailureInformation message as invalid and trigger a RRC re-establishment procedure instead of performing a RRC connection recovery. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.331 CR 2758 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

### 6.2.2	Message definitions

#### –	SCGFailureInformationNR

The SCGFailureInformationNR message is used to provide information regarding NR SCG failures detected by the UE.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

SCGFailureInformationNR message

-- ASN1START

SCGFailureInformationNR-r15 ::=		SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

scgFailureInformationNR-r15			SCGFailureInformationNR-r15-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

SCGFailureInformationNR-r15-IEs ::=	SEQUENCE {

failureReportSCG-NR-r15				FailureReportSCG-NR-r15				OPTIONAL,

nonCriticalExtension					SCGFailureInformationNR-v1590-IEs	OPTIONAL

}

SCGFailureInformationNR-v1590-IEs ::=	SEQUENCE {

lateNonCriticalExtension					OCTET STRING					OPTIONAL,

nonCriticalExtension						SEQUENCE {}					OPTIONAL

}

FailureReportSCG-NR-r15 ::=		SEQUENCE {

failureType-r15						ENUMERATED {

t310-Expiry, randomAccessProblem,

rlc-MaxNumRetx,

synchReconfigFailureSCG, scg-reconfigFailure,

srb3-IntegrityFailure, dummy-r16},

measResultFreqListNR-r15				MeasResultFreqListFailNR-r15		OPTIONAL,

measResultSCG-r15						OCTET STRING						OPTIONAL,

...,

[[	locationInfo-r16				LocationInfo-r10						OPTIONAL,

logMeasResultListBT-r16			LogMeasResultListBT-r15					OPTIONAL,

logMeasResultListWLAN-r16		LogMeasResultListWLAN-r15				OPTIONAL,

failureType-v1610				ENUMERATED {t312-Expiry, scg-lbtFailure,

beamFailureRecoveryFailure, bh-RLF-r16, spare4,

spare3, spare2, spare1}	OPTIONAL

]],

[[

failureTypeOther-r16				ENUMERATED {other}	                    OPTIONAL,

]]

}

MeasResultFreqListFailNR-r15 ::=	SEQUENCE (SIZE (1..maxFreqNR-r15)) OF MeasResultFreqFailNR-r15

MeasResultFreqFailNR-r15 ::=		SEQUENCE {

carrierFreq-r15						ARFCN-ValueNR-r15,

measResultCellList-r15				MeasResultCellListNR-r15			OPTIONAL,

...

}

-- ASN1STOP

SCGFailureInformationNR field descriptions

failureType Indicates the cause of the SCG failure.

failureTypeOther This field is present when the field failureType-v1610 is also present. When this field is present, the network shall ignore the field failureType-r15.

measResultFreqListNR The field contains available results of measurements on NR frequencies the UE is configured to measure by measConfig.

measResultSCG Includes the NR MeasResultSCG-Failure IE as specified in TS 38.331 [82]. The field contains available results of measurements on NR frequencies the UE is configured to measure by the NR RRCConfiguration message.

END OF CHANGES