3GPP TSG-RAN WG2 #115-e	R2-2108199

E-meeting, 9th – 27th August, 2021

Agenda item:	8.6.4

Source:	Huawei, HiSilicon

Title:	Small data transmission with RA-based schemes

Document for:	Discussion

# Introduction

In this paper, we further discuss the issues related to small data transmission (SDT) with random access (RA) based scheme based on the agreements made by RAN2 so far.

# RA-based schemes

## 2.1 RA-SDT configuration

RAN2 agreed the following in RAN2#112-e and RAN2#113-e.

RAN2#112-e Agreement: 10. As a baseline, the RACH resource i.e. (RO+preamble combination) is different between SDT and non-SDT a) If ROs for SDT and non SDT are different, preamble partitioning between SDT and non SDT is not needed. b) If ROs for SDT and non SDT are same, preamble partitioning is needed FFS if common configuration should be allowed

RAN2#113-e Agreement: - RAN2 continues to progress the work based the separate RACH resources for SDT (i.e. explicit mechanisms to support common resources won’t be pursued unless there is sufficient support for this. However, use of common RACH resources will not be precluded if possible via implementation

In this section, we will share our understanding on the RA-SDT configuration aspects for the case where separated RACH resources for SDT and non-SDT are used and for the case where common RACH resources for SDT and non-SDT are configured.

### 2.1.1 Separated RACH resources for SDT

As agreed previously, separation of RACH resources for SDT and non-SDT can be achieved by using a different RO+preamble combination, which can be further divided into two cases:

a) shared ROs with separate preambles

b) separate ROs with either separate or shared preambles

For configuring shared ROs with separate preambles, the preamble space should be divided into non-SDT preambles and SDT preambles. However, as shown in the following table (1), there are several RACH schemes which may share the whole preamble space.

Table (1): different cases for shared ROs with separated preambles between SDT and non-SDT

| ROs sharing between SDT and non-SDT | 4-step RA-SDT | 2-step RA-SDT |
| --- | --- | --- |
| 4-step RA | 1) Shared or not shared | 2) Shared or not shared |
| 2-step RA | 3) Shared or not shared | 4) Shared or not shared |

As indicated in the above table, the ROs for 4-step RA-SDT can be either shared with 4-step RA (case 1) or with 2-step RA (case 3), and the ROs for 2-step RA-SDT can be either shared with 4-step RA (case 2) or with 2-step RA (case 4), where all the specific configurations are all under the control of the network, for example, the network can configure how the ROs are shared by considering the current resource load. Therefore, in our opinion, all the RO sharing cases as outlined in the table above should be considered.

In addition, if the preambles of 2-step RA-SDT and 4-step RA-SDT are separated, then the ROs between 4-step RA-SDT and 2-step RA-SDT can also be shared as shown in the following table (2)

Table (2): different cases for shared ROs with separated preambles between 2-step RA-SDT and 4-step RA-SDT

| ROs sharing between 2-step RA-SDT and 4-step RA-SDT | 4-step RA-SDT |
| --- | --- |
| 2-step RA-SDT | Shared or not shared |

Proposal 1: To achieve the RACH configuration of shared ROs with separated preambles, the following configurations should be supported:

- 4-step RA-SDT shares ROs with 4-step RA and/or 2-step RA

- 2-step RA-SDT shares ROs with 4-step RA and/or 2-step RA

- 2-step RA-SDT shares ROs with 4-step RA-SDT and/or 4-step RA and/or 2-step RA.

### 2.1.2 Common RACH resources for SDT

In RAN2#113-e meeting, RAN2 agreed the following:

RAN2#113-e Agreement: - For RA-SDT, up to two preamble groups (corresponding to two different payload sizes for MSGA/MSG3) may be configured by the network

According to the above agreement, RAN2 agreed that different preamble groups can be utilized to indicate different MSGA/MSG3 payload sizes, which is similar to the preamble partitioning scheme in legacy RA procedure. For 4-step SDT, if the payload sizes can be indicated by different preambles, UE can assemble packets on the basis of UL grant in RAR. Hence, there is no need for the gNB to blindly detect MSG3 packet as had to be done when flexible TBS was applied, e.g. in LTE EDT. Without blind detection, the gNB behavior for reception of RA-SDT MSG3 and legacy MSG3 are exactly the same, meaning that it is not necessary for UEs to indicate the SDT intention through MSG1 from the perspective of gNB. If common RACH resources for SDT are applied, UE would use current preamble Group B when initiating RA-SDT procedure. UE could transmit the SDT data together with MSG3 if the UL grant is sufficient, or UE could include BSR if the UL grant is not sufficient and small data could be further sent over subsequent SDT transmissions. There may be cases that gNB sends an RRCResume message to move the UE into RRC_ CONNECTED upon receiving MSG3 not including any data nor BSR. However, we believe such scenario can be avoided by network implementation, e.g. by providing the UE with a grant sufficient for at least some UP data.

Proposal 2: For the case of common RACH resources for SDT, the network can distinguish SDT and non-SDT access attempts via LCID of UL data included in MSG3/MSGA, i.e. no additional indication is required.

### 2.1.3 Preamble Group Selection for RA-SDT

In legacy RA procedure, after the initialization of random access procedure, UE needs to perform the preamble group selection, where the preamble group A and preamble group B correspond to different payload sizes as well as different pathloss of the UE. Similarly, RAN2 has agreed that up to two preamble groups are supported for RA-SDT, which means that UE also needs to perform the preamble group selection for both types of RA-SDT, i.e. 4-step RA-SDT and 2-step RA-SDT if RA-SDT is initiated.

As shown below, according to TS 38.321, UE selects the preamble group based on the payload size and the pathloss in current spec for RA procedure. For the RA procedure triggered by CCCH logical channel, i.e. the RRC resume procedure, UE only considers the CCCH SDU size plus MAC subheader for preamble group selection.

else if Msg3 buffer is empty: 3> if Random Access Preambles group B is configured: 4> if the potential Msg3 size (UL data available for transmission plus MAC subheader(s) and, where required, MAC CEs) is greater than ra-Msg3SizeGroupA and the pathloss is less than PCMAX (of the Serving Cell performing the Random Access Procedure) – preambleReceivedTargetPower – msg3-DeltaPreamble – messagePowerOffsetGroupB; or 4> if the Random Access procedure was initiated for the CCCH logical channel and the CCCH SDU size plus MAC subheader is greater than ra-Msg3SizeGroupA: 5> select the Random Access Preambles group B. 4> else: 5> select the Random Access Preambles group A. 3> else: 4> select the Random Access Preambles group A.

Different from the legacy RA procedure, the RA SDT procedure triggered by SDT related RRC resume procedure also includes the transmission of SDT data, which means that in addition to the RRC message UE also includes the SDT data in msg3 or msgA. Therefore, for preamble group selection for SDT, in addition to CCCH SDU size plus MAC subheader, the SDT data size and the pathloss are also factors that need to be considered.

If UE only considers the CCCH SDU without SDT data for preamble selection while UE actually transmits the CCCH SDU together with the SDT data, then the calculated size and the transmission size is different, which is not aligned with the current spec and may lead to unnecessary subsequent transmissions. In addition, the pathloss can also be considered so that the network can be aware of the UE’s radio conditions and schedule suitable grants.

As a result, with respect to data considered for preamble group selection by the UE, there are the following options out of which we think option 3 is most reasonable:

- Option 1: CCCH SDU size plus MAC subheader;

- Option 2: CCCH SDU size plus MAC subheader + SDT data size plus MAC subheader;

- Option 3: CCCH SDU size plus MAC subheader + SDT data size plus MAC subheader + Pathloss;

Proposal 3:  For the RA-SDT preamble group selection, the UE should consider: CCCH SDU size plus MAC subheader + SDT data size plus MAC subheader and Pathloss.

What we need to discuss next is whether the parameters for RA-SDT preamble group selection are the same as in legacy RA or not. If option 1 is adopted, then UE just considers the CCCH SDU and MAC subheader as in legacy, which means the legacy msg3/msgA threshold can be reused. In option 2, UE takes the SDT data size into account, meaning that a new specific msg3/msgA threshold should be introduced. Finally, if UE also considers the pathloss as in option 3, we think SDT specific pathloss should be introduced as the transmission of SDT data needs a better channel condition compared with legacy. Mentioning the pathloss, we notice that UE calculates the pathloss according to several configured parameters as shown above, therefore in our opinion, all of the related parameters could be SDT specific.

Proposal 4: When two preamble groups are configured for RA-SDT, the following parameters should be SDT specific:

- the data volume threshold, i.e.:

- ra-sdt-Msg3SizeGroupA for 4-step RA-SDT

- ra-sdt-MsgA-SizeGroupA for 2-step RA-SDT.

- pathloss related parameters, i.e.:

- preambleReceivedTargetPower, msg3-DeltaPreamble, messagePowerOffsetGroupB for 4-step RA-SDT

- msgA-PreambleReceivedTargetPower, msgA-DeltaPreamble, messagePowerOffsetGroupB for 2-step RA-SDT;

## 2.2 Anchor relocation related aspects

### 2.2.1 Assistance information for anchor relocation

During the last RAN3#111-e meeting, RAN3 has discussed about the anchor relocation issue for RA-SDT and the following progress are described in the LS to RAN2 [1]:

- WA1: The existing Retrieve UE Context procedure can be reused for both with and without anchor relocation scenarios with possible enhancements, which will be discussed later. - WA2: UL data for SDT is buffered at the receiving node in the successful context retrieval procedure. For other cases, the common understanding is that UL data may need to be buffered as well, details are pending. - WA3: The last serving gNB, i.e., anchor gNB, will be the decision maker on whether to relocate anchor or not. Assistance information provided by the receiving gNB may help on the decision. Details of such information are pending to future discussion in RAN3 and/or RAN2 inputs. - RAN3 discussed the assumption in the LS that RLC handling is processed in the receiving gNB and would like to ask if this is confirmed as a firm agreement in RAN2. Several companies in RAN3 proposed to analyse the topic further.

According to the above input from RAN3, the last serving gNB requires some assistance information from the new gNB for making an informed decision on whether to perform anchor relocation. The assistance information can include information about subsequent UL and DL transmission, e.g.:

- Information similar to AS RAI in LTE (see section 6.1.3.19 of TS 36.321): based on this information the last serving gNB can for example determine whether to expect a follow-up DL transmission and decide when to send an RRCRelease message to the UE. For SDT, there are also some use cases (e.g. smart meters, wireless sensors and other IoT applications) where the number of packets to be transmitted can be known at the UE at the time of initiating the SDT. UE can provide the information on the number of packets to be transmitted during a SDT session to the network in MSG3/MSGA. This information can then be sent to the last serving gNB as part of the assistance information to help it make a decision on whether to perform anchor relocation or not.

- Buffer status information: the last serving gNB can determine whether to send the UE to RRC_CONNECTED or to keep the UE in RRC_INACTIVE for subsequent data. If the volume of subsequent data arriving in the buffer is large, it may be more appropriate to terminate the SDT procedure as soon as possible and move the UE context to the new gNB (i.e. perform anchor relocation) for transitioning the UE state to RRC_CONNECTED. If the volume of subsequent data in the buffer is small, the last serving gNB may decide not to perform anchor relocation.

Proposal 5: Assistance information transferred from the receiving gNB to the last serving gNB for allowing the last serving gNB to take an informed decision whether to perform anchor relocation or not   shall at least contain the following:

- UE’s expected traffic pattern, e.g. number of packets to be transmitted for SDT DRB in UL/DL, single-shot/multi-shot transmission etc.

- Buffer status for data from SDT DRBs.

RAN2 should send an LS to RAN3 on assistance information provided to last serving gNB.

### 2.2.2 Issues for SDT with and without anchor relocation

It is RAN3 assumption that the last serving gNB will be the decision maker on whether to relocate anchor or not. Upon the initiation of SDT, the UE context is stored in the last serving gNB meaning that the gNB can be aware of the traffic pattern of the UE. Therefore, based on the assistance information and the UE context, the last serving gNB could decide whether to perform the anchor relocation or not. It may happen that a UE needs to be move to RRC CONNECTED state in the middle of an ongoing SDT without anchor relocation procedure, e.g. in the cases like UL or DL non-SDT data arrival. In this situation, anchor gNB needs to be relocated to the new gNB, even in case it was not relocated earlier. Considering that it is the last serving gNB to control the SDT procedure for without anchor relocation SDT procedure, we think the last serving gNB should be responsible for triggering the anchor relocation.

Proposal 6: In case anchor needs to be relocated to the new gNB in the middle of an ongoing SDT procedure (e.g. due to non-SDT data arrival), anchor relocation is triggered by the last serving gNB (i.e. current/old anchor).

For the cases where the last serving gNB decides to relocate the anchor at the beginning of SDT procedure, the last serving gNB sends RETRIEVE UE CONTEXT RESPONSE with the UE context. Then the new serving gNB performs the path switch procedure with the AMF for SDT data transmission. Question about when the last serving gNB should release the UE context was raised in [2]. In our opinion, the legacy RNAU procedure, as shown below in [3], should be a reference. In this case, the new serving gNB triggers the release of the UE context after the new serving gNB sends RRCRelease to UE. Thanks to this, it is ensured the context is released from the old anchor only once the UE has a new I-RNTI already.

Figure 1: RNA update procedure with UE context relocation as in [3]

Proposal 7: As in legacy RNAU procedure, the last serving gNB releases the UE AS context after the SDT procedure ends, i.e after the new serving gNB sends RRCRelease to UE.

### 2.2.3 Details of without anchor relocation SDT procedure

When the UE initiates the SDT procedure, if the last serving gNB decides to anchor the SDT session (i.e decides not to perform anchor relocation), then the last serving gNB shall have the full control of the SDT session and the receiving gNB will mainly act as the relay node to transfer the signaling and the data between the UE and the last Serving gNB. In our opinion, the legacy RNAU procedure without UE context relocation in [3] can be taken as the basis for extending it to the SDT procedure as shown in the figure below.

Figure 2: Overall SDT procedure without UE context relocation

Looking at the overall SDT procedure without UE context relocation we can make the following observations

Observation 1: Decision to anchor or relocate the UE context will be with the last serving gNB and shall be taken based on the assistance information provided by the UE.

Observation 2: On completion of the SDT procedure, it is the anchor/ last serving gNB that sends the RRC Release message to terminate the SDT procedure via the receiving gNB.

Proposal 8: The overall SDT procedure without UE context relocation shown above is taken as the basis for further discussions in RAN 2.

### 2.2.4 Handling of Non-SDT Data arrival for non-anchor relocation case

During the email discussion [507][SData] Non-SDT data arrival handling the issue of how to provide the security keys separation between the receiving and the last serving gNB and handling the switch from SDT to RRC_CONNECTED during an ongoing SDT was discussed.

After the 2nd phase of the email discussion the view of majority of the companies is that no new solution needs to be defined to address the scenario where the anchor relocation is required in the middle of an SDT session, i.e. network relies on releasing the UE back into RRC_INACTIVE. We believe that this will be a simple workable solution and would reuse the existing procedure as much as possible.

However, one thing worth noting is that when the DL non-SDT data arrives at the anchoring gNB in this scenario, in case RRCRelease is used to terminate the SDT procedure, the gNB would need to subsequently page the UE to bring it to RRC Connected state. This would lead to unnecessary overhead and latency. To avoid this, it would be worth allowing the gNB to include an indication in the RRCRelease message informing the UE it should initiate a new resume procedure right-away.

When the UE imitates a new legacy resume procedure, the UE context can be relocated form the last serving gNB to the receiving gNB and the UE can be transitioned to the RRC_CONNECTED state by re using the legacy procedure. Overall procedure for handling this scenario is shown in the Figure 3

Figure 3: Overall handling of Non-SDT Data arrival while anchoring

Based on the proposed simplified handling of Non-SDT Data arrival while anchoring shown in the figure above following proposals are made.

Proposal 9: When DL non-SDT data arrives during the ongoing SDT procedure, the current anchor gNB shall end the ongoing SDT procedure by sending RRCRelease message to the UE with an indication to initiate a new Resume procedure to avoid having to page the UE.

# Conclusion

Based on the above discussion, we make the following observations and recommend RAN2 to discuss and adopt the following proposals:

Proposal 1: To achieve the RACH configuration of shared ROs with separated preambles, the following configurations should be supported:

- 4-step RA-SDT shares ROs with 4-step RA and/or 2-step RA

- 2-step RA-SDT shares ROs with 4-step RA and/or 2-step RA

- 2-step RA-SDT shares ROs with 4-step RA-SDT and/or 4-step RA and/or 2-step RA.

Proposal 2: For the case of common RACH resources for SDT, the network can distinguish SDT and non-SDT access attempts via LCID of UL data included in MSG3/MSGA, i.e. no additional indication is required.

Proposal 3:  For the RA-SDT preamble group selection, the UE should consider: CCCH SDU size plus MAC subheader + SDT data size plus MAC subheader and Pathloss.

Proposal 4: When two preamble groups are configured for RA-SDT, the following parameters should be SDT specific:

- the data volume threshold, i.e.:

- ra-sdt-Msg3SizeGroupA for 4-step RA-SDT

- ra-sdt-MsgA-SizeGroupA for 2-step RA-SDT.

- pathloss related parameters, i.e.:

- preambleReceivedTargetPower, msg3-DeltaPreamble, messagePowerOffsetGroupB for 4-step RA-SDT

- msgA-PreambleReceivedTargetPower, msgA-DeltaPreamble, messagePowerOffsetGroupB for 2-step RA-SDT;

Proposal 5: Assistance information transferred from the receiving gNB to the last serving gNB for allowing the last serving gNB to take an informed decision whether to perform anchor relocation or not   shall at least contain the following:

- UE’s expected traffic pattern, e.g. number of packets to be transmitted for SDT DRB in UL/DL, single-shot/multi-shot transmission etc.

- Buffer status for data from SDT DRBs.

RAN2 should send an LS to RAN3 on assistance information provided to last serving gNB.

Proposal 6: In case anchor needs to be relocated to the new gNB in the middle of an ongoing SDT procedure (e.g. due to non-SDT data arrival), anchor relocation is triggered by the last serving gNB (i.e. current/old anchor).

Proposal 7: As in legacy RNAU procedure, the last serving gNB releases the UE AS context after the SDT procedure ends, i.e after the new serving gNB sends RRCRelease to UE.

Observation 1: Decision to anchor or relocate the UE context will be with the last serving gNB and shall be taken based on the assistance information provided by the UE.

Observation 2: On completion of the SDT procedure, it is the anchor/ last serving gNB that sends the RRC Release message to terminate the SDT procedure via the receiving gNB.

Proposal 8: The overall SDT procedure without UE context relocation shown above is taken as the basis for further discussions in RAN 2.

Proposal 9: When DL non-SDT data arrives during the ongoing SDT procedure, the current anchor gNB shall end the ongoing SDT procedure by sending RRCRelease message to the UE with an indication to initiate a new Resume procedure to avoid having to page the UE.

# Reference

- R3-211280: Reply LS on small data transmission, Source: RAN3

- R2-2104883: RA-SDT remaining open issues, Intel Corporation, 3GPP TSG RAN WG2 Meeting #114

- 38300: RNA update, Figure 9.2.2.5-1: RNA update procedure with UE context relocation