3GPP TSG-RAN WG2 Meeting #115 electronic	R2-2108234

Online, August 16 – 27, 2021

Agenda item:	8.10.3.2	Idle/Inactive mode

Source:		NEC

Title:		NTN to TN mobility in Idle/Inactive mode

Document for:		Discussion

# 1. Introduction

During RAN2 #114-e [1], it was agreed that UEs can prioritise TN cells over NTN cells:

2.	For idle mode reselection, based on configuration NTN UE can prioritise TN over NTN. Configuration details FFS

In the study Item [2], it was agreed that

The NTN-TN service continuity and mobility mechanisms targeted to minimizing UE power consumption

In this contribution, we highlight issues specific to NTN to TN mobility and discuss enhancements to improve UE power consumption.

# 2. Discussion

2.1. Number of TN neighbours in an NTN cell and its impact on NTN to TN mobility

In NTN cells, beam Footprint can only be as low as 100km [2] for LEO, greater than most TN cells (1km is considered a macro-cell, TR 36.942), and it can be as big as 3500km for GEO cells. NTN can be used to have coverage in deserted places, i.e. places with no TN cell coverage, however most deserted places will not span across thousands of km (with the notable exception of the oceans) with no TN cells.

Observation 1: The minimum beam footprint is much larger than legacy TN cell sizes and will also cover large areas with legacy TN cells

Furthermore, due to their smaller size, there will be many more TN neighbouring cells for an NTN cell, which includes traditional neighbouring TN cells at the NTN cell edge as well as TN cells inside the NTN cell coverage.

Observation 2: Due to the difference in cell radii, NTN cells will have many more TN neighbours compared to legacy TN cells

In Figure 1, we describe the example of a large NTN beam footprint that covers an area comprising of deserts (here, mountainous regions and a large body of water) over two lands with many neighbouring TN cells.

In all scenarios from Figure 1, due to the size of the satellite beam footprint, most neighbouring cells are irrelevant to a single UE, more specifically:

- UE 1 is travelling from a desert zone (e.g. at sea) and would only be interested in the few cells on the shore of Land B

- For UE 2, it is in a geographically limited area with low coverage (e.g. mountains) and may only be in proximity of a few towns in Land A when it gets in line-of-sight

- For UE 3, there is no TN cells in proximity, even in line-of-sight. This case of was highlighted in [3] ‘s proposal to “discuss the enhancement to avoid UE to measure TN neighbour cells when it is in NTN only area.”

Observation 3: For any given NTN UE, the vast majority of TN neighbours are not within range and are not acceptable cells

Figure 1 - Mobility scenarios

Since it was agreed that UEs can prioritise TN cells based on configuration, at least in IDLE mode cell selection, it can be assumed that UEs will try to search for suitable TN cells even when in good NTN coverage. Neighbouring cell information can be used to indicate available TN cells. Indeed, the goal of neighbour information is to provide UEs with information to facilitate UE cell (re)selection and minimise power consumption by indicating which cells may be suitable to a UE, typically frequencies where to scan for available cells (for inter-frequency).

However, we observed that the number of neighbouring cells will be much greater for NTN cells and this NTN cell will need to broadcast every frequency where there is at least one neighbouring cell, which may not be relevant to most UEs (e.g. UE 1 and UE 2 in Figure 1) and other UEs will not have any suitable cell within the broadcasted frequencies (here, UE 3).

This issue is already present in TN cells to some extent for terrestrial borders, where UEs measure frequencies of the cells of all the neighbouring countries (e.g. France, Italy and Switzerland) for which there exists a roaming contract. This represents an additional measuring load for the UE but only concerns a limited number of countries, all of which are relevant to the UE. In NTN cells however, if these are allowed to be operated across many regions/countries, this load could be greatly increased and would not be relevant to the UEs, e.g. frequencies for TN cells in both India and Iran (and all other countries in the area) for a satellite flying above Pakistan.

Observation 4: NTN to TN mobility may cause excessive and unnecessary inter-frequency measurements for UEs, leading to poor energy efficiency

Proposal 1: RAN2 to discuss enhancements to signalling of TN neighbouring frequencies/cells in an NTN cell

2.2. Solutions to improve signalling for Cell Reselection

One solution would be to reduce the number of frequencies that are broadcasted by the gNB in SIB4. For instance, the gNB could only provide a small list of frequencies where all UEs will be able to find at least one TN cell, typically a low frequency. Once the UE finds a TN cell, it can later reselect or be handed over to a more relevant TN cell, which frequency was not broadcasted by the original NTN cell.

However, even with such improvement, because the NTN cell will cover a very large geographical area, it is likely to include many countries, which may use different frequencies. Each country would require at least one or two higher priority frequency broadcasted in SIB4 to make sure that every UE that is within range of one or more neighbouring TN cells can successfully find at least one such cell.

Therefore, even if the gNB can reduce the number of broadcasted frequencies, it would still be too high and inefficient for UEs and further enhancements are necessary to solve this issue.

Observation 5: For an NTN cell covering many countries/regions, the cell can reduce the number of frequencies that it broadcasts but the need to broadcast at least one frequency per country/region may still lead to excessive and unnecessary inter-frequency measurements for UEs

Another solution consists in taking advantage of this heterogeneity of frequencies depending on location. For instance, NTN UEs can know their GNSS location so enhancements such as signalling geographical zones can be understood and used by the UE. Such zones could define e.g. the limits of a country (or zone where specific frequencies are being used) or be used to signal NTN-only zones where UEs should not do inter-frequency measurements to look for TN cells.

Proposal 2: The gNB can indicate NTN-only zones to UEs

Proposal 3: The gNB can indicate groups of frequencies specific to restricted parts of the NTN cell coverage

# 3. Conclusion

In this contribution, we observed and proposed the following:

Observation 1: The minimum beam footprint is much larger than legacy TN cell sizes and will also cover large areas with legacy TN cells

Observation 2: Due to the difference in cell radii, NTN cells will have many more TN neighbours compared to legacy TN cells

Observation 3: For any given NTN UE, the vast majority of TN neighbours are not within range and are not acceptable cells

Observation 4: NTN to TN mobility may cause excessive and unnecessary inter-frequency measurements for UEs, leading to poor energy efficiency

Proposal 1: RAN2 to discuss enhancements to TN neighbour cell lists for NTN UEs

Observation 5: For an NTN cell covering many countries/regions, the cell can reduce the number of frequencies that it broadcasts but the need to broadcast at least one frequency per country/region may still lead to excessive and unnecessary inter-frequency measurements for UEs

Proposal 2: The gNB can indicate NTN-only zones to UEs

Proposal 3: The gNB can indicate groups of frequencies specific to restricted parts of the NTN cell coverage

# References

[1] RAN2 #114-e Chairman notes

[2] 3GPP TR 38.821

[3] R2-2105614 Discussion on service continuity between NTN and TN Huawei, HiSilicon

[4] 3GPP TS 38.304