3GPP TSG-RAN WG2 Meeting #115 Electronic	R2-2108247

# Elbonia, 16 – 27 August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.322 | CR | 0147 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Retransmission conditions upon expiry of t-PollRetransmit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2021-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. Currently, upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC data PDU awaiting for acknowledgements); or - if no new RLC data PDU can be transmitted (e.g. due to window stalling): - consider the AMD PDU with SN = VT(S) – 1 for retransmission; or - consider any AMD PDU which has not been positively acknowledged for retransmission; The intention is that when there is no RLC Data PDU queued for transmission that can naturally carry the re-sent poll, one is artificially created by considering a non-ACKed AMD PDU for retransmission. The current text implements this intention incorrectly: because the second condition (no new RLC data PDU can be transmitted) is now sufficient on its own, an AMD PDU will artifically be considered for retransmission based on the second condition even if the retransmission buffer is non-empty. 2. “await for acknowledgements” is not correct language. “For” is not used with “await”. In 38.322 this is already corrected. 3. Rudundant space in a Note: “E mpty”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In the actions upon expiry of t-PollRetransmit, re-word the conditions for considering for retransmission a non-ACKed AMD PDU to “if the retransmission buffer is empty (excluding transmitted RLC data PDU awaiting acknowledgements) and no new RLC data PDU can be transmitted (e.g. due to empty transmission buffer or window stalling)”. - Occurrences of “awaiting for acknowledgements” changed to “awaiting acknowledgements”. - “E mpty” corrected to “Empty”. Implementation of this CR by a previous release UE will not cause compatibility issues Impact analysis Impacted functionality: retransmissions upon expiry of t-PollRetransmit. Inter-operability: - If the network is implemented according to the CR and the UE is not, the UE may waste uplink resources with unnecessary RLC retransmissions. - If the UE is implemented according to the CR and the network is not, there are no inter-operability problems. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification unnecessarily mandates considering for retransmission a non-ACKed AMD PDU. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.1; 5.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

#### 5.2.2.1	Transmission of a AMD PDU or AMD PDU segment

Upon assembly of a new AMD PDU, the transmitting side of an AM RLC entity except for NB-IoT shall:

-	increment PDU_WITHOUT_POLL by one;

-	increment BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the RLC data PDU;

-	if PDU_WITHOUT_POLL >= pollPDU; or

-	if BYTE_WITHOUT_POLL >= pollByte;

-	include a poll in the RLC data PDU as described below.

Upon assembly of an AMD PDU or AMD PDU segment, the transmitting side of an AM RLC entity shall:

-	if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC data PDU awaiting acknowledgements) after the transmission of the RLC data PDU; or

-	if no new RLC data PDU can be transmitted after the transmission of the RLC data PDU (e.g. due to window stalling);

-	include a poll in the RLC data PDU as described below.

NOTE:	Empty RLC buffer (excluding transmitted RLC data PDU awaiting acknowledgements) should not lead to unnecessary polling when data awaits in the upper layer. Details are left up to UE implementation.

To include a poll in a RLC data PDU, the transmitting side of an AM RLC entity shall:

-	set the P field of the RLC data PDU to "1";

-	set PDU_WITHOUT_POLL to 0, except for NB-IoT;

-	set BYTE_WITHOUT_POLL to 0, except for NB-IoT;

After delivering a RLC data PDU including a poll to lower layer and after incrementing of VT(S) if necessary, the transmitting side of an AM RLC entity shall:

-	set POLL_SN to VT(S) – 1;

-	if t-PollRetransmit is not running:

-	start t-PollRetransmit;

-	else:

-	restart t-PollRetransmit;

Next Modified Subclause

#### 5.2.2.3	Expiry of t-PollRetransmit

Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall:

-	if the retransmission buffer is empty (excluding transmitted RLC data PDU awaiting acknowledgements)

and no new RLC data PDU can be transmitted (e.g. due to empty transmission buffer or window stalling):

-	consider the AMD PDU with SN = VT(S) – 1 for retransmission; or

-	consider any AMD PDU which has not been positively acknowledged for retransmission;

-	include a poll in a RLC data PDU as described in clause 5.2.2.1.

End of Changes