3GPP TSG-RAN WG2 Meeting #115 Electronic	R2-2108249

# Elbonia, 16 – 27 August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.322 | CR | 0044 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Retransmission conditions upon expiry of t-PollRetransmit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI16 |  |  |  |  |  | Date: |  |  | 2021-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Currently, upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements); or - if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling): - consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission; or - consider any RLC SDU which has not been positively acknowledged for retransmission. The intention is that when there is no AMD PDU queued for transmission that can naturally carry the re-sent poll, one is artificially created by considering a non-ACKed SDU for retransmission. The current text implements this intention incorrectly: because the second condition (no new RLC SDU or RLC SDU segment can be transmitted) is now sufficient on its own, an SDU will artifically be considered for retransmission based on the second condition even if the retransmission buffer is non-empty. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the actions upon expiry of t-PollRetransmit, re-word the conditions for considering for retransmission a non-ACKed SDU to “if the retransmission buffer is empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements) and no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to empty transmission buffer or window stalling)”. Implementation of this CR by a previous release UE will not cause compatibility issues Impact analysis Impacted functionality: retransmissions upon expiry of t-PollRetransmit. Inter-operability: - If the network is implemented according to the CR and the UE is not, the UE may waste uplink resources with unnecessary RLC retransmissions. - If the UE is implemented according to the CR and the network is not, there are no inter-operability problems |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification unnecessarily mandates considering for retransmission a non-ACKed SDU. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

#### 5.3.3.4	Expiry of t-PollRetransmit

Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall:

-	if the retransmission buffer is empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements)

and no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to empty transmission buffer or window stalling):

-	consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission; or

-	consider any RLC SDU which has not been positively acknowledged for retransmission.

-	include a poll in an AMD PDU as described in clause 5.3.3.2.

End of Changes