3GPP TSG-RAN WG2 Meeting #115-e		R2-2108376

Electronic Meeting, August 16 – 27, 2021

Title:	[draft] Response LS on Scheduling Location in Advance to reduce Latency

Response to:	R2-2106968 (S2-2105122) Response LS on Scheduling Location in Advance to reduce Latency

Release:	Rel-17

Work Item:	NR_pos_enh

Source:	Qualcomm Incorporated (to be RAN2)

To:	SA2

Cc:	RAN1, RAN3

Contact Person:

#### Name:                   Sven Fischer

E-mail Address:   sfischer@qti.qualcomm.com

1. Overall Description:

RAN2 thanks SA2 for their response to the RAN2 questions on the Scheduling Location in Advance to reduce latency.

With regard to Question A from SA2:

"Some companies in SA2 think the Scheduled Location Time should not be sent to NG-RAN and UE. Other companies believe RAN WGs should decide whether it may be useful to send the Scheduled Location Time to NG-RAN and the UE in order to trigger measurements at or close to the scheduled location time.

Question A: in order to get a clear view from RAN WG, SA2 sincerely ask RAN2 to investigate whether Scheduled Location Time could help the reduction of the LCS latency."

RAN2 would like to provide the following response:

A scheduled location time T allows the latency for obtaining and reporting the location of a target device to be reduced by the duration of the location preparation phase which allows substantial reduction of latency. With the RAN2 assumptions summarized in TR 38.857, the latency reduction can be:

DL+UL NR positioning methods:		Latency reduction of more than 62 %

UL-only NR positioning methods:	Latency reduction of more than 78 %

DL-only NR positioning methods:	Latency reduction of more than 50 %

RAN2 also concluded that it is beneficial to send the Scheduled Location Time T to the UE and TRPs in order to more accurately and reliably trigger measurements at or close to the scheduled location time and will add the corresponding support to RAN2 specifications.

2. Actions:

To SA2 group.

ACTION: 	RAN2 kindly asks SA2 to take the above information into account.

3. Date of Next TSG-RAN2 Meetings:

TSG-RAN2 Meeting #116-e		1-12 November 2021			Electronic Meeting

TSG-RAN2 Meeting #117-e		17-26 January 2022			Electronic Meeting