3GPP TSG-RAN WG2 Meeting #115 Electronic	R2-2108439

# Online, 9th – 27th August, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | 0283 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for fast MCG link recovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  | Date: |  |  | 2021-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to 37.340 clause 7.7: - Upon reception of the MCG Failure Indication, the MN can send RRC reconfiguration message - Upon receiving an RRC reconfiguration message, MobilityFromNRCommand message or MobilityFromEUTRACommand message, the UE resumes MCG transmissions - The UE initiates the RRC connection re-establishment procedure if it does not receive an RRC reconfiguration message, MobilityFromNRCommand message, MobilityFromEUTRACommand message or RRC release message within a certain time after fast MCG link recovery was initiated. This seems to mean that any RRC reconfiguration can be used for fast MCG link recovery. However, according to TS 36.331/38.331: - the UE does not stop the related timer upon reception of an RRC reconfiguration without reconfigurationWithSync / mobilityControlInfo - the UE only resumes MCG transmissions if the RRC reconfiguration includes reconfigurationWithSync / mobilityControlInfo Moreover, processing of an RRC reconfiguration message normally requires sending a complete message but if MCG tranmission is not resumed, it is unclear what the UE will do. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In subclause 7.7, clarify that when receiving the MCG failure indication, the MN responses RRC reconfiguration message for handover to the UE, the UE, and this is the message that can trigger resuming MCG transmisions. Impact analysis Impacted 5G architecture options: MR-DC Impacted functionality: Fast MCG link recovery Inter-operability: If the UE is implemented according to this CR while the network is not, when the MN receives MCG failure indication from UE, it could send RRC reconfiguration message without handover, which which may result in unpredictable failure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Intra-RAT fast MCG link recovery may fail due to description contradicting with stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

## 7.7	SCG/MCG failure handling

RLF is declared separately for the MCG and for the SCG.

If radio link failure is detected for MCG, and fast MCG link recovery is configured, the UE triggers fast MCG link recovery. Otherwise, the UE initiates the RRC connection re-establishment procedure. During the execution of CPC, if radio link failure is detected for MCG, the UE initiates the RRC connection re-establishment procedure.

During fast MCG link recovery, the UE suspends MCG transmissions for all radio bearers and reports the failure with MCG Failure Information message to the MN via the SCG, using the SCG leg of split SRB1 or SRB3.

The UE includes in the MCG Failure Information message the measurement results available according to current measurement configuration of both the MN and the SN. Once the fast MCG link recovery is triggered, the UE maintains the current measurement configurations from both the MN and the SN, and continues measurements based on configuration from the MN and the SN, if possible. The UE initiates the RRC connection re-establishment procedure if it does not receive an RRC reconfiguration message for handover, MobilityFromNRCommand message, MobilityFromEUTRACommand message or RRC release message within a certain time after fast MCG link recovery was initiated.

Upon reception of the MCG Failure Indication, the MN can send RRC reconfiguration message for handover, MobilityFromNRCommand message, MobilityFromEUTRACommand message or RRC release message to the UE, using the SCG leg of split SRB1 or SRB3. Upon receiving an RRC reconfiguration message for handover, MobilityFromNRCommand message or MobilityFromEUTRACommand message, the UE resumes MCG transmissions for all radio bearers. Upon receiving an RRC release message, the UE releases all the radio bearers and configurations.

NOTE 0:	In this clause the term "handover" refers to an E-UTRA handover or to an NR synchronous reconfiguration.

NOTE 1:	It is up to network implementation to guarantee that the RRC-related messages are delivered to the UE by the SN before the release of its control plane resources.

The following SCG failure cases are supported:

-	SCG RLF;

-	SN change failure;

-	For EN-DC, NGEN-DC and NR-DC, SCG configuration failure or CPC configuration failure (only for messages on SRB3);

-	For EN-DC, NGEN-DC and NR-DC, SCG RRC integrity check failure (on SRB3);

-	For EN-DC, NGEN-DC and NR-DC, consistent UL LBT failure on PSCell;

-	For IAB-MT, reception of a BH RLF indication from SCG;

-	CPC execution failure.

Upon SCG failure, if MCG transmissions of radio bearers are not suspended, the UE suspends SCG transmissions for all radio bearers and reports the SCG Failure Information to the MN, instead of triggering re-establishment. If SCG failure is detected while MCG transmissions for all radio bearers are suspended, the UE initiates the RRC connection re-establishment procedure.

SCG/MCG failure handling by UE also applies to IAB MT.

In all SCG failure cases, the UE maintains the current measurement configurations from both the MN and the SN and the UE continues measurements based on configuration from the MN and the SN if possible. The SN measurements configured to be routed via the MN will continue to be reported after the SCG failure.

NOTE 2:	UE may not continue measurements based on configuration from the SN after SCG failure in certain cases (e.g. UE cannot maintain the timing of PSCell).

The UE includes in the SCG Failure Information message the measurement results available according to current measurement configuration of both the MN and the SN.	The MN handles the SCG Failure Information message and may decide to keep, change, or release the SN/SCG. In all the cases, the measurement results according to the SN configuration and the SCG failure type may be forwarded to the old SN and/or to the new SN.

In case of CPC, upon transmission of the SCG Failure Information message to the MN, the UE stops evaluating the CPC execution condition. The UE is not required to continue measurements for candidate PSCell(s) for execution condition upon transmission of the SCG Failure Information message to the MN.

END OF CHANGE