3GPP TSG-RAN WG2 Meeting #115 Electronic	 R2-2108496

# Online Meeting, August, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR |  | rev |  | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Stage 2 Running CR of TS 38.300 for eSL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2021-08-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR introduces the support of Rel17 features for SL DRX and resource allocation enhancements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include consideration of SL-DRX into section 3.1 and 5.5, 16.9.2.2, and 16.9.3.2 New section of 16.9.5 for SL DRX Extend section 11 for SL-DRX |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel17 features for sidelink are not supported in NR |  |  |  |  |  |  |  |  |
| Clauses affected: |  | TBD |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

# 3	Abbreviations and Definitions

## 3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

5GC	5G Core Network

5GS	5G System

5QI	5G QoS Identifier

A-CSI	Aperiodic CSI

AKA	Authentication and Key Agreement

AMBR	Aggregate Maximum Bit Rate

AMC	Adaptive Modulation and Coding

AMF	Access and Mobility Management Function

ARP	Allocation and Retention Priority

BA	Bandwidth Adaptation

BCH	Broadcast Channel

BH	Backhaul

BL	Bandwidth reduced Low complexity

BPSK	Binary Phase Shift Keying

C-RNTI	Cell RNTI

CAG	Closed Access Group

CAPC	Channel Access Priority Class

CBRA	Contention Based Random Access

CCE	Control Channel Element

CD-SSB	Cell Defining SSB

CFRA	Contention Free Random Access

CHO	Conditional Handover

CIoT	Cellular Internet of Things

CLI	Cross Link interference

CMAS	Commercial Mobile Alert Service

CORESET	Control Resource Set

CP	Cyclic Prefix

CPC	Conditional PSCell Change

DAG	Directed Acyclic Graph

DAPS	Dual Active Protocol Stack

DFT	Discrete Fourier Transform

DCI	Downlink Control Information

DCP	DCI with CRC scrambled by PS-RNTI

DL-AoD	Downlink Angle-of-Departure

DL-SCH	Downlink Shared Channel

DL-TDOA	Downlink Time Difference Of Arrival

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

E-CID	Enhanced Cell-ID (positioning method)

EHC	Ethernet Header Compression

ETWS	Earthquake and Tsunami Warning System

FS	Feature Set

GFBR	Guaranteed Flow Bit Rate

HRNN	Human-Readable Network Name

IAB	Integrated Access and Backhaul

I-RNTI	Inactive RNTI

INT-RNTI	Interruption RNTI

KPAS	Korean Public Alarm System

LDPC	Low Density Parity Check

MDBV	Maximum Data Burst Volume

MIB	Master Information Block

MICO	Mobile Initiated Connection Only

MFBR	Maximum Flow Bit Rate

MMTEL	Multimedia telephony

MNO	Mobile Network Operator

MPE	Maximum Permissible Exposure

MT	Mobile Termination

MU-MIMO	Multi User MIMO

Multi-RTT	Multi-Round Trip Time

NB-IoT	Narrow Band Internet of Things

NCGI	NR Cell Global Identifier

NCR	Neighbour Cell Relation

NCRT	Neighbour Cell Relation Table

NGAP	NG Application Protocol

NID	Network Identifier

NPN	Non-Public Network

NR	NR Radio Access

P-MPR	Power Management Maximum Power Reduction

P-RNTI	Paging RNTI

PCH	Paging Channel

PCI	Physical Cell Identifier

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PLMN	Public Land Mobile Network

PNI-NPN	Public Network Integrated NPN

PO	Paging Occasion

PRACH	Physical Random Access Channel

PRB	Physical Resource Block

PRG	Precoding Resource block Group

PS-RNTI	Power Saving RNTI

PSS	Primary Synchronisation Signal

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

PWS	Public Warning System

QAM	Quadrature Amplitude Modulation

QFI	QoS Flow ID

QPSK	Quadrature Phase Shift Keying

RA	Random Access

RA-RNTI	Random Access RNTI

RACH	Random Access Channel

RANAC	RAN-based Notification Area Code

REG	Resource Element Group

RIM	Remote Interference Management

RMSI	Remaining Minimum SI

RNA	RAN-based Notification Area

RNAU	RAN-based Notification Area Update

RNTI	Radio Network Temporary Identifier

RQA	Reflective QoS Attribute

RQoS	Reflective Quality of Service

RS	Reference Signal

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

SCS	SubCarrier Spacing

SD	Slice Differentiator

SDAP	Service Data Adaptation Protocol

SFI-RNTI	Slot Format Indication RNTI

SIB	System Information Block

SI-RNTI	System Information RNTI

SLA	Service Level Agreement

SL-DRX	Sidelink discontinuous reception

SMC	Security Mode Command

SMF	Session Management Function

S-NSSAI	Single Network Slice Selection Assistance Information

SNPN	Stand-alone Non-Public Network

SNPN ID	Stand-alone Non-Public Network Identity

SPS	Semi-Persistent Scheduling

SR	Scheduling Request

SRS	Sounding Reference Signal

SRVCC	Single Radio Voice Call Continuity

SS	Synchronization Signal

SSB	SS/PBCH block

SSS	Secondary Synchronisation Signal

SST	Slice/Service Type

SU-MIMO	Single User MIMO

SUL	Supplementary Uplink

TA	Timing Advance

TPC	Transmit Power Control

TRP	Transmit/Receive Point

UCI	Uplink Control Information

UL-AoA	Uplink Angles of Arrival

UL-RTOA	Uplink Relative Time of Arrival

UL-SCH	Uplink Shared Channel

UPF	User Plane Function

URLLC	Ultra-Reliable and Low Latency Communications

V2X	Vehicle-to-Everything

Xn-C	Xn-Control plane

Xn-U	Xn-User plane

XnAP	Xn Application Protocol

Next Modified Subclause

## 5.5	Transport Channels

The physical layer offers information transfer services to MAC and higher layers. The physical layer transport services are described by how and with what characteristics data are transferred over the radio interface. An adequate term for this is "Transport Channel". This should be clearly separated from the classification of what is transported, which relates to the concept of logical channels at MAC sublayer.

Downlink transport channel types are:

1.	Broadcast Channel (BCH) characterised by:

-	fixed, pre-defined transport format;

-	requirement to be broadcast in the entire coverage area of the cell, either as a single message or by beamforming different BCH instances.

2.	Downlink Shared Channel (DL-SCH) characterised by:

-	support for HARQ;

-	support for dynamic link adaptation by varying the modulation, coding and transmit power;

-	possibility to be broadcast in the entire cell;

-	possibility to use beamforming;

-	support for both dynamic and semi-static resource allocation;

-	support for UE discontinuous reception (DRX) to enable UE power saving.

3.	Paging Channel (PCH) characterised by:

-	support for UE discontinuous reception (DRX) to enable UE power saving (DRX cycle is indicated by the network to the UE);

-	requirement to be broadcast in the entire coverage area of the cell, either as a single message or by beamforming different BCH instances;

-	mapped to physical resources which can be used dynamically also for traffic/other control channels.

Uplink transport channel types are:

1.	Uplink Shared Channel (UL-SCH) characterised by:

-	possibility to use beamforming;

-	support for dynamic link adaptation by varying the transmit power and potentially modulation and coding;

-	support for HARQ;

-	support for both dynamic and semi-static resource allocation.

2.	Random Access Channel(s) (RACH) characterised by:

-	limited control information;

-	collision risk.

Sidelink transport channel types are:

1.	Sidelink broadcast channel (SL-BCH) characterised by:

-	pre-defined transport format.

2.	Sidelink shared channel (SL-SCH) characterised by:

-	support for unicast transmission, groupcast transmission and broadcast transmission;

-	support for both UE autonomous resource selection and scheduled resource allocation by NG-RAN;

-	support for both dynamic and semi-static resource allocation when UE is allocated resources by the NG-RAN;

-	support for HARQ;

-	support for dynamic link adaptation by varying the transmit power, modulation and coding.

-	support for UE SL discontinuous reception (SL-DRX) to enable UE power saving.

Association of transport channels to physical channels is described in TS 38.202 [20].

Next Modified Subclause

#### 16.9.2.2	MAC

The MAC sublayer provides the following services and functions over the PC5 interface in addition to the services and functions specified in clause 6.2.1:

-	Radio resource selection;

-	Packet filtering;

-	Priority handling between uplink and sidelink transmissions for a given UE;

-	Sidelink CSI reporting.

With LCP restrictions in MAC, only sidelink logical channels belonging to the same destination can be multiplexed into a MAC PDU for every unicast, groupcast and broadcast transmission which is associated to the destination. NG-RAN can also control whether a sidelink logical channel can utilise the resources allocated to a configured sidelink grant Type 1 (see clause 16.9.3.2).  For transmissions to RX UE(s) in DRX, LCP restrictions ensure that a TX UE transmits data in the active time of the RX UE.

Next Modified Subclause

#### 16.9.3.2	Scheduled Resource Allocation

NG-RAN can dynamically allocate resources to the UE via the SL-RNTI on PDCCH(s) for NR sidelink communication.

In addition, NG-RAN can allocate sidelink resources to a UE with two types of configured sidelink grants:

-	With type 1, RRC directly provides the configured sidelink grant only for NR sidelink communication;

-	With type 2, RRC defines the periodicity of the configured sidelink grant while PDCCH can either signal and activate the configured sidelink grant, or deactivate it. The PDCCH is addressed to SL-CS-RNTI for NR sidelink communication.

Besides, NG-RAN can also semi-persistently allocate sidelink resources to the UE via the SL Semi-Persistent Scheduling V-RNTI on PDCCH(s) for V2X sidelink communication. If DRX is configured, the MAC entity monitirs the PDCCH for the MAC entity’s SL-RNTI, SL-CS-RNTI, and SL Semi-Persistent Scheduling V-RNTI in the Uu DRX active time, and does not need to monitor the PDCCH for the MAC entity’s SL-RNTI, SL-CS-RNTI, and SL Semi-Persistent Scheduling V-RNTI in Uu DRX inactive time.

Next Modified Subclause (new)

### 16.9.5	SL DRX

#### 16.9.5.1	General

Sidelink supports timer-based SL DRX for unicast, groupcast, and broadcast.  Similar parameters as defined in section 11 for Uu (on-duration, inactivity-timer, retransmission-timer, cycle) are defined for SL to determine the SL active time for SL DRX.  During the SL active time for SL DRX, the UE performs SCI monitoring for data reception (i.e., PSCCH and 2nd SCI on PSSCH).  The UE may skip monitoring of PSCCH and 2nd SCI on PSSCH for data reception during inactive time for SL DRX.

SL DRX configuration is specific to cast type (unicast, groupcast, broadcast).  AS layer can determine the DRX parameters for SL DRX configuration with no additional inputs (other than QoS) from the upper layer, and can provide the PC5 DRX information to the upper layer.  The SL active time of the RX UE includes the time in which any of its applicable on-duration timer(s), inactivity-timer(s) or retransmission timer(s) (for any of unicast, groupcast, or broadcast) are running.

(Editor’s note: WA to be confirmed by RAN2):

The active time also includes the time in which a UE is expecting CSI report following a CSI request.

#### 16.9.5.2	Unicast

For unicast, SL DRX is configured per pair of source/destination.

(Editor’s note: For OOC, per direction DRX configuration is a baseline)

The UE maintains a set of SL DRX timers per pair of source/destination and per direction, and starts/restarts the timers with the value configured for that pair of source/destination and direction. The DRX configuration between a pair of source/destination L2 IDs for a direction may be negotiated between the UEs in the AS layer.  For DRX configuration of each direction, where one UE is the TX UE and the other is the RX UE, TX-centric approach is supported whereby:

- RX UE sends assistance information to the TX UE using a PC5-RRC message

- TX UE sends the DRX configuration to be used by the RX UE to the RX UE using RRCReconfigurationSidelink

When the TX UE is in-coverage and in RRC_CONNECTED, the TX UE may report the received assistance information to its serving gNB and may obtain the DRX configuration to send to the RX UE in dedicated RRC signaling from the network.  When the RX UE is in-coverage and in RRC_CONNECTED, the RX UE can report the received DRX configuration to its serving gNB.  The inactivity timer may take into consideration the QoS.

On-duration, inactivity-timer, HARQ RTT and retransmission timers are supported in unicast. SL HARQ RTT timer and SL retransmission timer are maintained per SL HARQ process at the RX UE.  The TX UE maintains a timer corresponding to the SL inactivity timer in the RX UE for each pair of sourc/destination L2 ID, and uses the timer as part of the criterion for determining the allowable transmission time for the RX UE.

(Editor’s note: WA to be confirmed by RAN2):

SL HARQ RTT timer can be derived from the retransmission resource timing when the SCI indicates a retransmission resource

SL DRX MAC CE is introduced for SL DRX operation in unicast.

#### 16.9.5.3	Groupcast/Broadcast

For groupcast/broadcast, SL DRX is configured commonly among multiple UEs based on QoS profile and L2 ID.  Multiple DRX configurations (beyond just cast type) are supported.

DRX cycle is configured per QoS profile.  The starting offset of the DRX cycle is determined based on the destination L2 ID, and does not take QoS into account.  In-coverage TX and RX UEs in RRC_IDLE/RRC_INACTIVE obtain their DRX configuration from SIB.  In this case, the network coordinates the active time between different cells.  UEs (TX or RX) in RRC_CONNECTED can obtain the DRX configuration from SIB.  For the out of converage case, the DRX configuration is obtained from pre-configuration. The inactivity timer may take into consideration the QoS.

On-duration, inactivity-timer, HARQ RTT and retransmission timers are supported for groupcast. On-duration timer is supported for broadcast.  SL HARQ RTT timer and SL retransmission timer are maintained per SL HARQ process at the RX UE. The TX UE maintains a timer corresponding to the SL inactivity timer in the RX UE for each pair of sourc/destination L2 ID, and uses the timer as part of the criterion for determining the allowable transmission time for the RX UE.

#### 16.9.5.4	Alignment between Uu DRX and SL DRX

Alignment of Uu DRX and SL DRX for a UE in RRC_CONNECTED is supported for unicast, groupcast, and broadcast.  Alignment of Uu DRX and SL DRX at the same UE is considered.  In addition, for mode 1 scheduling, the alignment of Uu DRX of the TX UE and SL DRX or the RX UE is considered.

Alignment may comprise either full overlap or partial overlap in time between Uu DRX and SL DRX. For SL RX UEs in RRC_CONNECTED, alignment is achieved by the gNB.

Next Modified Subclause

# 11	UE Power Saving

The PDCCH monitoring activity of the UE in RRC connected mode is governed by DRX, BA, and DCP.

When DRX is configured, the UE does not have to continuously monitor PDCCH. DRX is characterized by the following:

-	on-duration: duration that the UE waits for, after waking up, to receive PDCCHs. If the UE successfully decodes a PDCCH (for Uu or SL transmission), the UE stays awake and starts the inactivity timer;

-	inactivity-timer: duration that the UE waits to successfully decode a PDCCH, from the last successful decoding of a PDCCH, failing which it can go back to sleep. The UE shall restart the inactivity timer following a single successful decoding of a PDCCH for a first transmission (Uu or SL) only (i.e. not for retransmissions);

-	retransmission-timer: duration until a retransmission can be expected;

-	cycle: specifies the periodic repetition of the on-duration followed by a possible period of inactivity (see figure 11-1 below);

-	active-time: total duration that the UE monitors PDCCH. This includes the "on-duration" of the DRX cycle, the time UE is performing continuous reception while the inactivity timer has not expired, and the time when the UE is performing continuous reception while waiting for a retransmission opportunity.

# Annex	- Collection of RAN2 agreements on NR SL Enhancements

Cyan highlight – agreement captured in stage-2 specifications

Green highlight – stage-3 level agreement, not captured in stage-2 specifications

No highlight – agreement with no direct impact on specifications

RAN2#113-e agreements

Agreements on SA2’s questions:

1: 	For Q1, RAN2 reply AS layer can determine DRX parameters and no additional input from V2X layer other than the currently available QoS is needed.

2:	RAN2 confirms that for unicast, the PC5 DRX may be negotiated between the UEs in AS layer. We can also include this RAN2 confirmation into the response LS.

3:	For Q2, RAN2 further reply that for SL unicast, other than DRX parameter negotiation/sharing reason, AS layer can provide the PC5 DRX related information to the V2X layer, and RAN2 is working on the detailed DRX parameter that applies to each cast type. RAN2 would keep SA2 being update on the RAN2 progress.

4:	For Q3, RAN2 reply that RAN2 does not think it is beneficial for broadcast and groupcast to share the PC5 DRX related information amongst UEs in the vicinity in V2X layer.

5:	For Q4, RAN2 reply that RAN2 is working on this aspects following the WID bullet of “Specify mechanism aiming to align sidelink DRX wake-up time with Uu DRX wake-up time in an in-coverage UE”, RAN2 would keep SA2 updated on related working progress.

Agreements on high-level principles for SL DRX

1: 	For SL unicast (after SL unicast link is established), SL DRX configuration can be configured per a pair of source/destination. FFS whether SL DRX operates per direction or for both directions.

2:	For SL groupcast/broadcast, SL DRX configuration can be configured in common. FFS on granularity of SL DRX configuration.

3:	Short DRX cycle is not introduced for SL unicast, groupcast and broadcast in Rel-17.

4:	For data reception, RAN2 defines the behaviour for monitoring the SCI reception (i.e., PSCCH and 2nd SCI on PSSCH) during the SL active time for SL DRX. For data reception, the UE may skip monitoring of PSCCH and 2nd SCI on PSSCH during inactive time for SL DRX. Sensing aspect is not considered in this agreement.

5a:	At least, On-duration timer and Inactivity timer are supported in SL unicast.

5b: 	HARQ RTT is supported in SL unicast. FFS for the detailed condition when it is supported. FFS whether HARQ RTT is explicitly configured or can be based on SCI. FFS on the need of HARQ retransmission timer.

6a: 	At least, on-duration timer is supported for SL groupcast. FFS for the need and detailed condition when inactivity timer is supported.

6b: 	HARQ RTT is supported in SL groupcast. FFS for the detailed condition when it is supported. FFS whether HARQ RTT is explicitly configured or can be based on SCI. FFS on the need of HARQ retransmission timer.

7: 	At least, on-duration timer is supported for SL broadcast.

8: 	SL DRX Command MAC CE is introduced for SL DRX operation in unicast. FFS on the need of groupcast. FFS on the detailed UE behaviour (including relation to inactivity timer).

9: 	In mode 1, when in RRC_CONNECTED, if DRX is configured, the MAC entity monitors the PDCCH for the MAC entity's SL-RNTI, SLCS-RNTI and SL Semi-Persistent Scheduling V-RNTI in Uu DRX Active Time. MAC entity does not need to monitor the PDCCH for the MAC entity's SL-RNTI, SLCS-RNTI and SL Semi-Persistent Scheduling V-RNTI in Uu DRX in-active Time.

Agreements on SL DRX configurations

1: 	For broadcast/groupcast, for out-of-coverage case, TX-UE/RX-UE obtain DRX configuration from pre-configuration.

2:	For broadcast/groupcast, for in-coverage case, RRC_IDLE/INACTIVE TX-UE/RX-UE obtain DRX configuration from SIB. It is up to network implementation how to coordinate active time between different cells.

3:	For broadcast/groupcast, for in-coverage case, for RRC_CONNECTED TX-UE/RX-UE can obtain DRX configuration from SIB. FFS on whether dedicated-RRC is also used.

4:	For unicast, for OOC scenario, the UE who sends out the DRX configuration decides on the DRX configuration. FFS on whether pre-configuration and/or the assistance information from the peer UE is also taken into account when determining the DRX configuration.

5: 	For unicast, for OOC scenario, adopt per-direction DRX configuration is as baseline. FFS on whether it is TX-centric or Rx-centric, i.e. TX UE or RX UE decides it.

Agreements on granularity of SL DRX operation for groupcast/broadcast

1: 	RAN2 kindly agree that for groupcast and broadcast communication further granularity to multiple sets of DRX configurations (beyond just cast type) is required i.e. more than two DRX Cycle configurations should be supported in specification.

2:	RAN2 will study/discuss how PQI and/or L2 destination ID is used to derive groupcast and broadcast DRX configuration.

Agreements on SL DRX on groupcast/broadcast

1: 	Timer-based SL DRX is also applied to SL groupcast/broadcast.

RAN2#113bis-e agreements

Agreements on details of timer

1: 	The following parameters are supported as part of the SL DRX configuration for all cast types: sl-drx-StartOffset, sl-drx-Cycle, sl-drx-onDurationTimer, and sl-drx-SlotOffset.

2:	The RX UE determines the symbol/slot/subframe associated with the start of the DRX cycle using the configured sl-drx-Cycle, sl-drx-StartOffset.  FFS on details.

3:	The RX UE starts the sl-drx-onDurationTimer after sl-drx-slotOffset from the beginning of the subframe.

4:	The RX UE’s active time includes the time in which sl-drx-on-DurationTimer is running.

5:	For unicast, the TX UE behaviors should be specified to keep aligned with the RX UE regarding the DRX Active time. FFS the specific Spec impacts needed at the TX side.

6:	For unicast, the RX UE maintains a separate SL inactivity timer for each pair of src/dest L2 ID.

7:	For unicast, the SL inactivity timer value may take into consideration the QoS.  Whether any specification impacts are needed is FFS.

8:	For unicast, RX UE starts/restarts the inactivity timer with the value configured for that pair of src/dest L2 ID.

9:	For unicast, the RX UE (re)starts the inactivity timer upon reception of a new SL data transmission from the RX UE perspective for that pair of src/dest L2 ID.

10:	For unicast, the RX UE (re)starts the inactivity timer based on information in SCI (SCI1+SCI2).  FFS if the MAC layer can stop the inactivity timer.

11:	For unicast, the RX UE (re)starts the inactivity timer in the first slot after SCI (SCI1+SCI2) reception.

12:	For unicast, the TX UE maintains a timer corresponding to the SL Inactivity timer in the RX UE for each pair of src/dest L2 ID, and uses the timer as part of criterion for determining the allowable transmission time for the RX UE.

13:	For unicast, the TX UE (re)starts its timer corresponding to the SL inactivity timer at the RX UE at the slot following an SCI transmission indicating a new data transmission. FFS the specific spec impacts needed at the TX side.

14:	SL Inactivity timer is supported for groupcast. FFS on the scenarios where it is supported.

15:	SL Inactivity timer is not supported for broadcast transmissions.

16:	The RX UE is active on sidelink (monitors SCI1+SCI2) as long as at least one of the SL inactivity timers associated with unicast or groupcast (if supported) is running.

17:	As a baseline, agreements 7-13 inclusive are applied to SL inactivity timer for groupcast, with the difference that “src/dest L2 ID pair” is replaced with “groupcast L2 destination ID or src/dest L2 id pair” (dependent on the conclusion of proposal 17).  Any specific handling which may be needed for synchronization of inactivity timers for the groupcast case is FFS.

18:	SL HARQ RTT timer and SL HARQ retransmission timer are maintained per SL HARQ process at the RX UE.

19:	Working assumption: SL HARQ RTT timer can be derived from the retransmission resource timing when the SCI indicates a retransmission resource. FFS whether explicitly configured SL HARQ RTT timer may be still required. If big problem is identified next meeting, we can revisit it.

20:	The value(s) of the SL HARQ RTT Timer, when explicitly configured and not determined via SCI (if agreed to do so), is determined by UE or NW implementation.

21:	For unicast, sidelink retransmission timer can be supported for at least some cases of HARQ disabled transmissions. FFS whether HARQ RTT is supported or not.

22:	For transmissions with HARQ feedback, the RX UE starts the SL HARQ RTT timer in the symbol/slot following the end of PSFCH transmission.

23:	If the RX UE does not transmit PSFCH for a HARQ enabled transmission (e.g. due to UL/SL prioritization) the RX UE still starts the HARQ RTT timer in the symbol/slot following the end of PSFCH resource.

24:	For cases where there is some uncertainty in the timing of a retransmission for a HARQ process (e.g. due to no retransmission resource indicated in the SCI, or possible reselection by the TX UE) the RX UE uses a configured retransmission timer.

25:	Retransmission timer can be started upon expiry of the HARQ RTT timer.

26:	The value(s) of the SL retransmission timer can be determined by UE or NW implementation.

27:	The SL active time of the RX UE includes the time in which any of its applicable sl-drx-OnDuration(s), sl-DRXInactivityTimer(s), or sl-drx-RetransmissionTimer(s) are running.

28:	Working assumption: The slots when the UE is expected CSI report following a CSI request is considered as SL active time.

29:	RAN2 assumes LCP enhancements for ensuring a TX UE transmits data in the active time of an RX UE are needed. FFS on the resource (re)selection enhancements (e.g. limiting the resources to the active time for peer UE).

Agreements on alignment between Uu DRX and SL DRX

1: 	Alignment of Uu DRX and SL DRX for unicast is supported. FFS on how alignment is achieved.

2:	Alignment of Uu DRX and SL DRX for groupcast and broadcast is supported. FFS on whether new mechanisms are needed.

3:	Alignment of Uu DRX and SL DRX for UE in RRC CONNECTED shall be a baseline.

4:	The alignment of Uu DRX and SL DRX of the same UE shall be considered.

RAN2#114-e agreements

Agreements on TX-UE centric or RX-UE centric DRX configuration determination

1: 	In SL unicast, for DRX configuration of each direction where one UE as Tx-UE and the other UE as Rx-UE, support signalling exchange including both 1) Signaling-1: signalling from RX-UE to TX-UE, and 2) Signaling-2: signalling from TX-UE to RX-UE.

2:	For SL unicast, TX-UE centric DRX configuration based on the assistance information from RX-UE is agreed as baseline.

2a: 	In SL unicast, for DRX configuration of each direction where one UE as Tx-UE and the other as Rx-UE, signaling-1 (Rx->Tx) is carried via a new PC5-RRC message, from Rx-UE to Tx-UE.

2b:	In SL unicast, for DRX configuration of the direction where one UE as Tx-UE and the other as Rx-UE, signaling-2 (Tx->Rx) is carried via RRCReconfigurationSidelink, to deliver DRX configuration from Tx-UE to Rx-UE.

3:	In SL unicast, for DRX configuration of each direction where one UE as Tx-UE and the other UE as Rx-UE, when Tx-UE is in-coverage and in RRC_CONNECTED state, Tx-UE may report the information received in signaling-1 (Rx->Tx) to the serving network.

4:	In SL unicast, for DRX configuration of each direction where one UE as Tx-UE and the other as Rx-UE, when Tx-UE is in-coverage and in RRC_CONNECTED state, Tx-UE may obtain DRX configuration from dedicated RRC to generate signalling-2 (Tx->Rx).

5:	In SL unicast, for DRX configuration of each direction where one UE as Tx-UE and the other as Rx-UE, when Rx-UE is in-coverage and in RRC_CONNECTED state, Rx-UE report the DRX configuration received in signalling-2 (Tx->Rx) to the serving network.

Agreements on Uu DRX Impact to Support SL

1: 	SL-specific drx-onDurationTimer is not introduced in Uu.

2:	SL-specific drx-InactivityTimer is not introduced in Uu.

3:	For Tx UE configured with sidelink resource allocation mode 1, it should start or restart the Uu drx-InactivityTimer if the UE receives a PDCCH indicating a new SL transmission.

4:	SL-specific drx-HARQ-RTT-Timer and SL-specific drx-RetransmissionTimer should be introduced in Uu, which are maintained based on sidelink process.

5:	When sl-PUCCH-Config is configured, SL-specific drx-HARQ-RTT-Timer and SL-specific drx-RetransmissionTimer should be maintained for UE configured with sidelink resource allocation mode 1.

6:	Adopt the following definitions of SL-specific drx-HARQ-RTT-Timer and drx-RetransmissionTimer (the detailed name of the timers can be further discussed):

- drx-RetransmissionTimerSL (per Sidelink process): the maximum duration until a grant for SL retransmission is received;

- drx-HARQ-RTT-TimerSL (per Sidelink process): the minimum duration before a SL retransmission grant is expected by the MAC entity.

7:	When sl-PUCCH-Config is configured (and the PUCCH is transmitted), the UE should start the SL-specific drx-HARQ-RTT-Timer in Uu for the corresponding SL HARQ process in the first slot after the end of the corresponding transmission carrying the SL HARQ feedback via the PUCCH.

Agreements on DRX for SL GC and BC

1: 	WA: RAN2 assumes that the V2X layer of Rx UE passes the PC5 QoS parameters together with the corresponding destination layer-2 ID(s) for reception to the AS layer, as per TR 23.776 conclusion, and will further discuss SL DRX design based on this working assumption. RAN2 does not need to send LS to SA2 to clarify this issue.

2:	For GC/BC, DRX cycle should take at least QoS requirement into consideration.

3:	For GC/BC, DRX cycle(s) is configured per QoS profile. FFS on the need of down-select one DRX cycle from available DRX cycles for a specific L2 DST ID if UE has multiple QoS profiles for same DST L2 ID.

4:	For GC/BC, DRX cycle is configured per QoS profile.

5a:	For GC/BC, RAN2 understands that sl-drx-startoffset does not take QoS requirement into consideration.

5b:	For GC/BC, For GC/BC, sl-drx-startoffset is set based on DST L2 ID.

Agreements on alignment between Uu DRX and SL DRX

1: 	Alignment of Uu DRX and SL DRX for UE may comprise the full overlapping between Uu DRX and SL DRX in time.

2:	Alignment of Uu DRX and SL DRX for UE may comprise the partial overlapping between Uu DRX and SL DRX in time.

3:	For at least SL RX-UEs in RRC CONNECTED, the alignment of Uu DRX and SL DRX is up to gNB. FFS for SL TX-UE.

4:	RAN2 to down-scope alignment of Uu DRX and SL DRX for UEs in RRC IDLE and RRC INACTIVE from Rel-17.

5:	In case of Mode 1 scheduling, the alignment of Uu DRX of Tx UE and SL DRX of Rx UE shall be considered. FFS on how alignment is achieved.

Agreements on geolocation based SL DRX

1: 	Geolocation based SL DRX is not supported in Rel-17.

*********************************************************************************************