3GPP TSG-RAN WG2 Meeting #115 electronic	R2-2108515

Online, August, 2021

Agenda item:	8.14.2.2

Source:	CMCC

Title:	Further discussion on start and stop

Document for:	Discussion

1	Introduction

Last meeting the start and stop for QMC has been primarily discussed and an LS has been sent to SA WGs to check where to store QoE reports.

This contribution further discusses the start and stop, i.e. activation, deactivation, pause and resume for NR QoE measurement.

2	Discussion

For LTE QMC, the mechanism for activation and deactivation has been specified in TS 28.405 [3] which is quoted as below,

### 4.2.1	Activation of measurement collection job and reporting of collected information in LTE

The parameters for the network request session are sent from the management system to the eNBs that host the cells that are included in the collection job request in the activateAreaQMCJob operation [3]. The eNB starts a network request session, with the Network request session id [3] given in activateAreaQMCJob operation [3]. For the duration of the network request session, the eNB(s) checks for connections where the UE has the QoE-MeasReport capability [8] for collection of streaming services or the QoE-MTSI-MeasReport capability [8] for collection of MTSI services. The UE capability is sent from the UE to eNB via the core network in message UE CAPABILITY INFO INDICATION [9].

…

### 4.2.3	Deactivation of measurement collection job in LTE

#### 4.2.3.1	Forced deactivation

When the operator technician or the management application wants to deactivate a measurement collection job, the management system sends the deactivateQMCJob operation [3] to the eNB. The eNB sets the network request session to ended, but does not delete the UE request session id and the Collection Entity Address parameters [3], as the UE still may send reports which shall be send to the collection centre. For UE request sessions which have reported that a recording session is started, the eNB sends the RRCConnectionReconfiguration message [8] to relevant UEs. The RRCConnectionReconfiguration message is including measConfigAppLayer set to discard application layer measurement report information in otherConfig [8]. The Access stratum sends +CAPPLEVMC AT command [5] to the application with the discard request. The application stops the recording session and stops recording of the requested information. The UE request session id and the Collection Entity Address parameters [3] in the eNB are deleted when the UE request session is ended.

#### 4.2.3.2	Deactivation of recording session

Regardless of whether the pre-set time has elapsed or not, the recording session continues to be active until the session for the application is ended.

According to above quotations, in LTE, the RRC signalling is used for indicating to discard QoE report collected at the UE side upon QoE deactivation procedure. The same principle can also be applied to NR QoE.

Proposal 1: Add measConfigAppLayer in otherConfig, and measConfigAppLayer is set to discard QoE report collected at the UE side upon QMC deactivation.

Regarding the pause and restart for NR QoE, TS 28.405 [3] has captured such mechanism in LTE, and RAN impact has also been given which is quoted as follows,

### 4.2.4	Temporary stop and restart of QoE information reporting during RAN overload in LTE

In case of overload in RAN, the eNB may temporarily stop the reporting from the UE by send the RRCConnectionReconfiguration message [8] to relevant UEs. The RRCConnectionReconfiguration message is including measConfigAppLayer set to temporarily stop application layer measurement reporting in otherConfig [8]. The Access stratum sends +CAPPLEVMC AT command [5] to the application with the temporary stop request. The application stops the reporting and stops recording further information when the data in the reporting container is used. Then the recorded data is kept until it is reported or when the UE request session is ended.

When the overload situation in RAN is ended the eNB restart the reporting from the UE by send the RRCConnectionReconfiguration message [8] to relevant UEs. The RRCConnectionReconfiguration message is including measConfigAppLayer set to restart application layer measurement reporting in otherConfig [8]. The Access stratum sends +CAPPLEVMC AT command [5] to the application with the restart request. The application restarts the reporting and recording if it was stopped.

Similarly, such mechanism can also be reused by NR QoE reporting, which is clear and simple which does not require too much standardization work in RAN2.

Proposal 2: Support measConfigAppLayer to be set to pause the QMC upon temporary stop procedure.

Proposal 3: Support measConfigAppLayer to be set to restart the QMC upon QMC restart procedure.

For QoE report handling at RAN overload, last meeting has sent LS to SA WGs with thorough analysis regarding the following three options,

Option 1: Application layer is responsible for storing QoE reports when the UE receives QoE pause indication.

Option 2: AS layer is responsible for storing QoE reports when the UE receives QoE pause indication.

Option 3: The QoE container received from application layer is discarded during pause.

Although this meeting hints that the discussion on pause and resume may be de-prioritized until the LS is replied, there’s still some other general issues that can be discussed since the LS is mainly focused on where to store the QoE reports. One general principle that can be discussed is no matter where to store the QoE reports, the UE is not expected to discard the already collected reports that have not been transmitted. Our understanding is that the QoE reports before pause are valuable information on how the RAN degradation impacts the end-to-end QoE analysis, and it is not desirable to discard such valuable information upon pause indication.

Proposal 4: Working assumption is that the QoE report that has not been successfully transmitted to RAN should not be discarded during pause, no matter where to store the QoE reports.

At last, it should also be noted that the start/stop of RAN-visible QMC was also discussed during SI, for which RAN may be able to autonomously configure QMC for those QoE metrics that are visible to RAN; however, because the latest WID [5] has identified that RAN3 is the leading group on the discussion on RAN-visible QoE, it may be better for RAN2 to firstly focus on the discussion on legacy QMC, and wait for more inputs from RAN3 to trigger the discussion on RAN-visible QMC afterwards.

Proposal 5: RAN2 is kindly asked to firstly focus on the discussion on legacy QMC, and wait for more inputs from RAN3 to trigger the discussion on RAN-visible QMC afterwards.

3	Conclusion

This contribution discusses NR QoE management, and provides following proposals,

Proposal 1: Add measConfigAppLayer in otherConfig, and measConfigAppLayer is set to discard QoE report collected at the UE side upon QMC deactivation.

Proposal 2: Support measConfigAppLayer to be set to pause the QMC upon temporary stop procedure.

Proposal 3: Support measConfigAppLayer to be set to restart the QMC upon QMC restart procedure.

Proposal 4: Working assumption is that the QoE report that has not been successfully transmitted to RAN should not be discarded during pause, no matter where to store the QoE reports.

Proposal 5: RAN2 is kindly asked to firstly focus on the discussion on legacy QMC, and wait for more inputs from RAN3 to trigger the discussion on RAN-visible QMC afterwards.

References

- RP-193256 Study on NR QoE management and optimizations for diverse services

- TR 38.890 v0.3.0

- TS 28.405 v16.0.0

- RP-202151

- RP-210913

- TS 36.331 v16.1.1