3GPP TSG-RAN WG2 Meeting #115-e 	R2-2108562

# Online, 16 – 27 August 2021

Title:	Draft Reply LS on QoS Monitoring for URLLC

Response to:	R2-2106943/R3-212937

Release:	Rel-16

Work Item:	NR_SON_MDT-Core

Source:	Huawei (to be RAN2)

To:	RAN3, SA5

Cc:	SA2

Contact Person:

Name:	Jun Chen

E-mail:	jun.chen@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

RAN2 thanks RAN3 for their LS in (R2-2106943/R3-212937).

In the originial SA5 LS S5-211350, SA5 mentions the following:

SA5 would like to provide further clarification that the D2, D3 and D4 measurements defined by SA5 and referenced by RAN3 are the average delays, adding them up could get the average delay between NG-RAN and UE, however using these measurements cannot get the distribution of packet delays which are also defined in TS 28.552 and based on RAN part UL packet delay result excluding the UL D1 packet delay:

- Distribution of UL packet delays between NG-RAN and UE, in clause 5.1.1.1.7 of TS 28.552.

- Distribution of UL packet delays between PSA-UPF and UE, in clause 5.4.9.2.2 of TS 28.552.

RAN2 undersands that the above SA5 definitions require RAN to monitor a specific packet between UE and NG-RAN in order to get the delay result (and then to generate the distribution), which is different from Rel-16 RAN packet delay measurement. However, RAN2 thinks that the RAN part of DL/UL packet delay can be approximately equal to the above SA5 definitions, e.g. the delay result of a specific packet between UE and NG-RAN can be got by  the Rel-16 defined RAN part of DL/UL packet delay. If SA5 still prefers to get the delay result for specific packets, it would need some work for RAN2, and thus Rel-16 may not achieve it. Such work may be considered in Rel-17 in RAN2.

2. Actions:

To RAN3 and SA5:

ACTION: 	RAN2 kindly ask RAN3 and SA5 to take the above information into account.

3. Date of Next TSG-RAN WG2 Meetings:

RAN2#116-e	1 Nov – 12 Nov 2021	Online e-meeting

RAN2#117	21 Feb – 25 Feb 2021	Athens, GR