3GPP TSG-RAN WG2 Meeting #115-e 	R2-2108620

Online, August 9 – 27 2021

Agenda Item:		8.22

Source:				Huawei, HiSilicon

Title:		Considerations on cross-carrier scheduling from SCell to P(S)Cell

Document for:	Discussion and Decision

- Introduction

- PDCCH enhancements for cross-carrier scheduling including [RAN1, RAN2]

  - PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell

- Note: The total PDCCH blind decoding budget should not be changed as a result of this work

- Note: These enhancements are not specific to DSS and are generally applicable to cross-carrier scheduling in carrier aggregation

Dynamic spectrum sharing (DSS) is introduced in Rel-17 with the objective of cross-carrier scheduling from SCell to P(S)Cell to ensure the sufficient scheduling capacity for NR UEs on the shared carriers [1].

It is noted that some progress have been reached in RAN1 since from RAN1#102e and the terminology of “sSCell” is used to denote the SCell configured with CCS to P(S)Cell. In this contribution, we intend to provide some initial considerations on RAN2 impacts of this feature.

- Discussion

2.1 Cross-carrier scheduling configurations

Agreements:

  - Following scheduling combinations are allowed/not allowed when cross-carrier scheduling from an SCell to PCell/PSCell is configured

    - self-scheduling on PCell/PSCell is allowed

    - cross-carrier scheduling from PCell/PSCell to another SCell is not allowed

    - self-scheduling on the ‘SCell used for scheduling PCell/PSCell’ is allowed

    - cross-carrier scheduling from the ‘SCell used for scheduling PCell/PSCell’ to another serving cell is allowed

    - cross-carrier scheduling from another serving cell to the ‘SCell used for scheduling PCell/PSCell’ is not allowed

According to the RAN1#102-e agreements on the principle of scheduling combinations, we can observe that it is allowed to configure self-scheduling on P(S)Cell and cross-carrier scheduling from sSCell to P(S)Cell simultaneously to the UE.

However there is legacy restriction that one serving cell can be only configured with either self-scheduling (i.e. “own”) or cross-carrier scheduling where it is scheduled by a PDCCH on another serving cell (i.e. “other”), which has reflected by the “choice” structure in the IE CrossCarrierSchedulingConfig in the following ASN.1. Regarding P(S)Cell configured by both self-secheduling and cross-carrier scheduling from sSCell simultaneously, the current “choice” structure cannot work anymore for the combined scheduling and it is possible to consider another type by allowing configurations of both “own” and “other” for P(S)Cell.  In addition, the limitation of only SCell which is allowed to be scheduled by a PDCCH on another serving cell should be removed correspondingly. Thus, some updates to the cross-carrier scheduling configurations are needed to accommodate the combined scheduling for the P(S)Cell.

#### –	CrossCarrierSchedulingConfig

The IE CrossCarrierSchedulingConfig is used to specify the configuration when the cross-carrier scheduling is used in a cell.

CrossCarrierSchedulingConfig information element

-- ASN1START

-- TAG-CROSSCARRIERSCHEDULINGCONFIG-START

CrossCarrierSchedulingConfig ::=        SEQUENCE {

schedulingCellInfo                      CHOICE {

own                                     SEQUENCE {                  -- Cross carrier scheduling: scheduling cell

cif-Presence                            BOOLEAN

},

other                                   SEQUENCE {                  -- Cross carrier scheduling: scheduled cell

schedulingCellId                        ServCellIndex,

cif-InSchedulingCell                    INTEGER (1..7)

}

},

...,

[[

carrierIndicatorSize-r16            SEQUENCE {

carrierIndicatorSizeDCI-1-2-r16        INTEGER (0..3),

carrierIndicatorSizeDCI-0-2-r16        INTEGER (0..3)

}                                                                                       OPTIONAL,  -- Cond CIF-PRESENCE

enableDefaultBeamForCCS-r16         ENUMERATED {enabled}                                OPTIONAL  -- Need S

]]

}

-- TAG-CROSSCARRIERSCHEDULINGCONFIG-STOP

-- ASN1STOP

CrossCarrierSchedulingConfig field descriptions

carrierIndicatorSizeDCI-0-2, carrierIndicatorSizeDCI-1-2 Configures the number of bits for the field of carrier indicator in PDCCH DCI format 0_2/1_2. The field carrierIndicatorSizeDCI-0-2 refers to DCI format 0_2 and the field carrierIndicatorSizeDCI-1-2 refers to DCI format 1_2, respectively (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 10.1).

cif-Presence The field is used to indicate whether carrier indicator field is present (value true) or not (value false) in PDCCH DCI formats, see TS 38.213 [13]. If cif-Presence is set to true, the CIF value indicating a grant or assignment for this cell is 0.

cif-InSchedulingCell The field indicates the CIF value used in the scheduling cell to indicate a grant or assignment applicable for this cell, see TS 38.213 [13].

enableDefaultBeamForCCS This field indicates whether default beam selection for cross-carrier scheduled PDSCH is enabled, see TS 38.214 [19]. If not present, the default beam selection behaviour is not applied, i.e. Rel-15 behaviour is applied.

other Parameters for cross-carrier scheduling, i.e., a serving cell is scheduled by a PDCCH on another (scheduling) cell. The network configures this field only for SCells.

own Parameters for self-scheduling, i.e., a serving cell is scheduled by its own PDCCH.

schedulingCellId Indicates which cell signals the downlink allocations and uplink grants, if applicable, for the concerned SCell. In case the UE is configured with DC, the scheduling cell is part of the same cell group (i.e. MCG or SCG) as the scheduled cell. If drx-ConfigSecondaryGroup is configured in the MAC-CellGroupConfig associated with this serving cell, the scheduling cell and the scheduled cell belong to the same Frequency Range.

Proposal 1: RAN2 understands that configurations of cross-carrier scheduling should be updated to accommodate sSCell scheduling P(S)Cell.

2.2 RLM and BFR

Currently radio link monitoring (RLM) is only allowed to be configured on the P(S)Cell for the purpose of radio link failure (RLF) detection. The UE is configured to monitor the downlink radio link quality of the P(S)Cell for indicating “out-of-sync/in-sync” status indications to the RRC layer. RLF is then declared for MCG and/or SCG cells based on the indications. With the introduction of sCell, the PDCCH of scheduling P(S)Cell can be transmitted on the sSCell, and thus it is necessary to revisit the RLM functionality for the robustness of sSCell. According to the recent RAN1 progress on the PDCCH monitoring as follows, it can be seen that from the UE capability point, both USS sets can be configured on both P(S)Cell and sSCell for PDCCH monitoring and whether to support simultaneous monitoring depends on the UE Type A or Type B. However, the possibility of USS for (at least) non-fallback DCI configured only on sSCell has not be excluded and it is up to network implementation. In this case, provided that the non-fallback DCI scheduling PCell can be only transmitted on the sSCell, the radio link quality of the sSCell would be critical to the scheduling performance of PCell, and whether any optimization to the RLM on sSCell needs further discussions.

Agreement

Two types of UEs (Type A and Type B) can support CCS from sSCell to P(S)Cell

- For Type A UE

  - At least following search space sets on P(S)Cell and search space sets on sSCell are configured so that the UE does not monitor them in overlapping [slot/symbol] of P(S)Cell and sSCell

    - search space sets on P(S)Cell

      - USS sets for DCI formats 0_1,1_1,0_2,1_2 (if supported for Type A UE)

      - USS sets for DCI formats 0_0,1_0

      - Type3-CSS set(s) for DCI formats 1_0/0_0 with C-RNTI/CS-RNTI/MCS-C-RNTI

    - search space sets on sSCell

      - USS set(s) for scheduling P(S)Cell

  - FFS: BD/CCE handling

- For Type B UE

  - Following search space sets on P(S)Cell and search space sets on sSCell can be configured so that the UE monitors them in overlapping [slot/symbol] of P(S)Cell and sSCell

    - search space sets on P(S)Cell

      - USS sets for DCI formats 0_0,1_0

      - Type3-CSS set(s) for DCI formats 1_0/0_0 with C-RNTI/CS-RNTI/MCS-C-RNTI

    - search space sets on sSCell

      - USS set(s) for scheduling P(S)Cell

  - For handling ‘USS sets for scheduling P(S)Cell’ on P(S)Cell and/or on sSCell for DCI formats 0_1,1_1,0_2,1_2

    - Alt 2-1 is adopted

  - There is no restriction on Type-0/0A/1/2-CSS sets configurations

  - FFS: BD/CCE handling

- For Type A and/or Type B UE

FFS: switching to ‘normal’ PDCCH monitoring on P(S)Cell when sSCell is deactivated

However, one argument would be it can be left to sensible network implementation to always configure PDCCH monitoring at least for some DCIs on PCell and thus it can still count on RLM on PCell as in the legacy. Considering it is still unclear from the RAN1 progress and the very limited TU allocated for DSS, we would think it is an optimization and is deprioritized in RAN2.

Proposal 2: RAN2 considers RLM related improvement for sSCell can be deprioritized.

NR also introduced the beam failure detection and recovery procedure to cope with the fluctuation of FR2 environment, and the UE is allowed to detect the beam failure indication from physical layer and initiate RA for PCell BFR and BFR MAC CE for SCell BFR. For PCell BFR, the UE is configured to monitor recoverySearchSpaceId for receiving the response and consider the BFR procedure completed, while for SCell BFR, the UE is expected to receive an uplink grant for the HARQ process used for transmission of the BFR MAC CE to consider the BFR procedure completed. Regarding the BFR of sSCell, provided the different potential direction of PCell BFR and SCell BFR, it is therefore necessary to consider which procedure is applied to recover the BFR detected on sSCell. From RAN2 point, it can be first discussed in RAN1 and we would like to minimize the MAC impact with respect to sSCell, i.e. SCell BFR can be reused for sSCell.

Proposal 3: RAN2 considers whether to initiate RA or SCell BFR for sSCell can be pending for RAN1 inputs.

2.3 sSCell deactivation/dormancy

Agreement

- When CCS from sSCell to PCell/PSCell is configured, CA activation/deactivation operation for the sSCell is supported

According to RAN1#104 progress, it is agreed to support CA activation/deactivation operation for the sSCell.

However, currently a SCell can be deactivated either by a configured sCellDeactivationTimer or SCell Activation/Deactivation MAC CE. PUCCH SCell is not allowed to be configured with sCellDeactivationTimer. When a SCell is deactivated, the UE shall stop monitoring PDCCH on all the BWPs of this SCell, which is similar to the sSCell dormancy. When it comes to the sSCell, how to handle the deactivated sSCell and the corresponding PCell in MAC needs further discussions in RAN2, based on further RAN1 progress. For instance, in order to avoid the scheduling performance degradation, any fallback mechanism or PDCCH monitoring “activation” on the P(S)Cell is still discussion in RAN1.

Proposal 4: RAN2 considers sSCell deactivation and dormancy can be pending for RAN1 inputs.

- Conclusions

In this contribution, we have provided an initial RAN2 impact analysis of cross-carrier scheduling from SCell to P(S)Cell and the following observations have been made serving as foundation of further RAN2 discussions.

Proposal 1: RAN2 understands that configurations of cross-carrier scheduling should be updated to accommodate sSCell scheduling P(S)Cell.

Proposal 2: RAN2 considers RLM related improvement for sSCell can be deprioritized.

Proposal 3: RAN2 considers whether to initiate RA or SCell BFR for sSCell can be pending for RAN1 inputs.

Proposal 4: RAN2 considers sSCell deactivation and dormancy can be pending for RAN1 inputs.

- Reference

[1] RP-211345 Revised WID on NR Dynamic spectrum sharing (DSS).