3GPP TSG-RAN WG2 Meeting #115-e	R2-2108622

Online, 9th Aug – 27th Aug, 2021

Agenda Item:	8.7.2.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on service continuity for L2 UE to NW Relay

Document for:	Discussion and Decision

- Introduction

As per the latest revised WID for NR Sidelink Relay, RAN2 needs to specify mechanism for service continuity. In this paper, we will provide some discussions on path switch for L2 UE to NW Relay, including leftover issues for basic procedure, measurement configuration and reporting, and RRC status of Relay UE.

- Discussion

According to the objectives of new WID for NR Sidelink Relay, the service continuity will be limited to intra-gNB cases [1] in Release 17. Currently, the general procedure for path switch from indirect to direct and the procedure for path switch from direct to indirect are captured in TR 38.836 [2], which are shown separately in Fig.1 and Fig 2 respectively. However, there are still some remaining issues on path switch, and we will discuss these issues in this contribution.

Figure 1: Procedure for Remote UE switching to direct Uu cell

Figure 2: Procedure for Remote UE switching to indirect Relay UE

Leftover issues for basic procedure

According to current TR 38.836, there are some remaining notes for indirect to direct path switch procedure and direct to indirect path switch procedure as shown in fig.1 and fig.2, which should be further discussed in WI phase:

1. switching from indirect to direct NOTE: The order of step 6/7/8 is not restricted. Following are further discussed in WI phase, including: - Whether Remote UE suspends data transmission via relay link after step 3; - Whether Step 6 can be before or after step 3 and its necessity; - Whether Step 7 can be after step 3 or step 5, and its necessity/replaced by PC5 reconfiguration; - Whether Step 8 can be after step 5. 2. switching from direct to indirect NOTE: Following are further discussed in WI phase, including: - Whether Step 2 should be after Relay UE connects to the gNB (e.g. after step 4), if not yet before; - Whether Step 4 can be before step 2/3.

Most of these issues are discussed in [3] and some of them have been agreed in RAN2#114 e-meeting [4]. We will discuss the leftover issues as below:

Path switch from indirect to direct

One remaining issue for path switch from indirect to direct is how to deal with the PC5 unicast link after step 3 (RRC Reconfiguration message to Remote UE). For now, there is no conclusion on whether Relay service(s) and Non-Relay service(s) can be multiplexed on one PC5 unicast link or not. In our thinking, it is better to separate Relay service(s) and Non-Relay service(s) into different PC5 unicast links. Otherwise, we need to further distinguish them within one PC5 unicast link (e.g. via different SL LCHs), and this may introduce complex spec impacts in RAN2.

Therefore, supposing SL Relay communication and SL normal communication will not be multiplexed on one PC5 unicast link (i.e. the L2 ID are not shared between SL Relay communication and SL normal communication), the PC5 unicast link for SL relay communication will be released after step 3 (RRC Reconfiguration message to Remote UE) as there is no need to maintain it for SL Relay communication, and the timing to release the PC5 unicast link can be up to UE implementation.

Proposal 1: For path switch from indirect to direct, RAN2 assume the PC5 unicast link for SL relay communication will be released after step 3 (RRC Reconfiguration message to Remote UE), i.e. the L2 IDs are not shared between SL Relay communication and SL normal communication.

Path switch from direct to indirect

Next we discuss the remaining issues for path switch from direct to indirect. Regarding whether Step 2 (Decision of switching to a target Relay UE by gNB, and target (re)configuration is sent to Relay UE optionally) should be after Relay UE connects to the gNB (e.g. after step 4 (Remote UE establishes PC5 connection with target Relay UE, if the connection has not been setup yet) ), if not yet before. It is straightforward that gNB can only send RRC Reconfiguration to a RRC_CONNECTED UE. Therefore, there is no ambiguity that Step 2 (Decision of switching to a target Relay UE by gNB, and target (re)configuration is sent to Relay UE optionally) should be after Relay UE connects to the gNB (e.g. after step 4 (Remote UE establishes PC5 connection with target Relay UE , if the connection has not been setup yet) ).

Proposal 2: For path switch from direct to indirect, sending RRC Reconfiguration message to Relay UE should be performed after target Relay UE connects (i.e. transit from RRC_INACTIVE/RRC_IDLE to RRC_CONNECTED state) to the gNB.

It was agreed that the contents in RRC Reconfiguration message for Remote UE can include Relay UE ID, PC5 RLC configuration for relaying and associated E2E RB. In our thinking, the Relay UE ID is C-RNTI for RRC_CONNECTED Relay UE.

Proposal 3: For path switch from direct to indirect, the target Relay UE ID in RRC Reconfiguration message is C-RNTI of the Relay UE in RRC_CONNECTED state.

In order to associate the remote UE’s C-RNTI with its local remote UE ID at the gNB side, the first UL message from remote UE after connected to the target relay UE should include the information of local remote UE ID (in adaptation layer header) and the information of remote UE’s C-RNTI (in the RRC message). Otherwise, at the gNB side, it has no idea on which remote UE’s PDCP should process the received RRC Reconfiguration Complete.

Proposal 4: For path switch from direct to indirect, remote UE’s C-RNTI should be included in the RRC Reconfiguration Complete of step 5.

It is clear that RRC_CONNECTED Relay UE can support L2 UE to NW Relay path switch, whereas the Remote UE find the candidate Relay UE via discovery procedure and then establish unicast link with the target Relay UE according to NW configuration to establish RRC connection with NW subsequently. However, it is not clear whether RRC_INACTIVE/RRC_IDLE Relay UE can support this. In our view, in order to save power consumption, a candidate Relay UE can be in RRC_INACTIVE/RRC_IDLE status, e.g. when no Remote UE is connected with this Relay UE or the Relay UE has no data transmission requirement in Uu. In other words, these RRC_INACTIVE/RRC_IDLE Relay UE may be able to conserve power and be able to serve the Remote UE later on when required as whereas no services are required from other Remote UEs and the Relay UE itself at the moment. However, it may introduce further spec impacts to supported path switch via RRC_INACTIVE/RRC_IDLE Relay UE (e.g. how to establish/resume the RRC connection between Relay UE and gNB, what is Relay UE ID in RRC Reconfiguration message and etc.). Therefore, it is suggested to prioritize the direct to indirect path switch via RRC_CONNECTED Relay UE and pursue the direct to indirect path switch via RRC_INACTIVE/RRC_IDLE Relay UE if time permitted.

Proposal 5: RAN2 to prioritize the direct to indirect path switch via RRC_CONNECTED Relay UE, and pursue the direct to indirect path switch via RRC_INACTIVE/RRC_IDLE Relay UE if time permitted.

As the Relay UE may be in RRC_INACTIVE/RRC_IDLE state, in order to establish/resume RRC connection between the Remote UE and the NW, it is necessary to ensure that the Relay UE transitions to RRC_CONNECTED state first and then the Relay UE can help the Remote UE to forward RRC connection setup request message to the gNB. In our view, two main options can be considered to trigger transition from RRC_INACTIVE/RRC_IDLE state into RRC_CONNECTED state. Option1 is the NW triggering, where a paging like mechanism can be supported by receiving request from Remote UE’s gNB. Option2 is the UE triggering, the Remote UE can request the Relay UE to enter into RRC_CONNECTED. We prefer Option 2 as it is straight forwards and minimises the impacts to the specifications

For the timing to trigger the target Relay in RRC_INACTIVE/RRC_IDLE state into RRC_CONNECTED state, in our thinking, three main options may be considered. The first option is during PC5 unicast link establishment, e.g. Relay UE is triggered into RRC_CONNECTED once receiving DCR message; the second option is Remote UE’s UE to NW data triggering (e.g. handover complete message); the last option is to introduce new PC5 RRC message to trigger it. Considering that the Remote UE may setup PC5 unicast link with Relay UE only for receiving paging message, and introduce new PC5 RRC message may introduce some spec impacts, we prefer to trigger Relay UE into RRC_CONECTRED state once receiving data (i.e. RRC Reconfiguration complete message) from PC5 RLC.

Proposal 6a: For path switch from direct to indirect, RRC_INACTIVE/ RRC_IDLE Relay UE enters RRC_CONNECTED state (triggered by remote UE), upon receiving data (i.e. Remote UE’s RRC Reconfiguration complete message) from PC5 RLC.

As there is no RRC connection between RRC_IDLE/RRC_INACTIVE Relay UE and gNB before receiving data from PC5 RLC for RRC Reconfiguration complete message, default PC5 RLC and Uu RLC configurations are needed at Relay UE for Remote UE’s SRB1 transmission, moreover the PC5 RLC and Uu RLC can be reconfigured by dedicated RRC signalling for the Remote UE’s SRB1 transmission of RRC Reconfiguration complete message.

Proposal 6b: The PC5 RLC and Uu RLC are configured via dedicated signaling with default configuration, for Remote UE’s SRB1 of RRC Reconfiguration complete message.

In case of direct to indirect path switch via RRC_INACTIVE/RRC_IDLE Relay UE, RAN2 needs to study what is the Relay UE ID in RRC Reconfiguration message. It seems not suitable to directly use I-RNTI or 5G-S-TMSI, as at least gNB does not know 5G-S-TMSI for a RRC_IDLE UE. Therefore, we suggest to use some index to represent the RRC_INACTIVE/RRC_IDLE Relay UE (e.g. Remote UE allocate these indexes during discovery), and these indexes are reported to gNB in measurement report and are included in RRC Reconfiguration message again when gNB deciding the target Relay UE.

Proposal 6c: For path switch from direct to indirect via RRC_INACTIVE/RRC_IDLE Relay UE, the Relay UE ID can be an index (e.g. allocated by Remote UE during discovery), in measurement report and RRC Reconfiguration messages of remote UE.

Measurement configuration and reporting

Based on the discussion in SI and the service continuity procedures captured in TR, the measurement configuration and reporting will be performed for path switch between direct link and indirect link. Whether it is  indirect to direct path switch or direct to indirect path switch, SL measurement report is needed. To be more specific, SL measurement report for serving Relay UE is needed for indirect to direct path switch, and SL measurement report for neighbour Relay UE is needed for direct to indirect path switch.

Proposal 7: SL measurement report is needed in both indirect to direct path switch for serving Relay UE and direct to indirect path switch for neighbor Relay UE.

Moreover, as the measurement configuration and reporting can be different for different kind of path switches  (i.e. indirect to direct and direct to indirect), we will analyse them case by case in the following discussions.

Path switch from indirect to direct

For path switch from indirect to direct, the procedure is shown in Fig.1. Regarding SL measurement in this case, it is clear that the SL measurement object can reuse the R16 SL measurement object (i.e. SL carrier), and the measurement quantity should be SL-RSRP associated with serving Relay UE.

Proposal 8a: For serving SL measurement of indirect to direct path switch, the SL measurement object can reuse R16 SL measurement object (i.e. SL carrier).

Proposal 8b: For serving SL measurement of indirect to direct path switch, the SL measurement quantity is SL-RSRP.

For SL measurement event triggered reporting, the similar logic in Uu could be reused, and the detailed configuration needs some adjustments. For instance, SL measurement event related to SL-RSRP, or SL measurement related to SL-RSRP and Uu-RSRP can be considered. That is to say, both event when the SL-RSRP of serving Relay UE is worse than threshold1, and event when the SL-RSRP of serving Relay UE is worse than threshold1 and Uu-RSRP of neighbour cell is better than threshold2 can be considered.

Proposal 8c: For serving SL measurement of indirect to direct path switch, two new trigger events can be introduced:

- Event1: SL-RSRP of serving Relay UE is worse than a threshold

- Event2: SL-RSRP of serving Relay UE is worse than a threshold and Uu-RSRP of neighbour cell is better than a threshold

Furthermore, for indirect to direct path switch, RAN2 should understand that gNB will not configure any serving cell related SL measurement and trigger events.

Proposal 7d: For indirect to direct path switch, RAN2 understands that gNB will not configure any serving cell related measurement and trigger events.

Path switch from direct to indirect

For path switch from direct to indirect, the procedure is shown in Fig.2. The target path is via the Relay UE, thus neighbour SL measurement should be considered. Similarly, the SL measurement object can reuse R16 SL measurement object (i.e. SL carrier) as baseline. Moreover, in order to enable gNB to have stronger control on path switch with more flexibility, “Allow-list/Blocklist” in Uu measurement can be considered in SL measurement. And it is clear that the measurement quantity can use SD-RSRP, as Remote UE may not establish PC5 unicast link with neighbour Relay UE before discovery.

Proposal 9a: For neighbour/target SL measurement of direct to indirect path switch, the SL measurement object can reuse R16 SL measurement object (i.e. SL carrier). FFS on whether “Allow-list/Blocklist” is supported.

Regarding SL measurement quantity of direct to indirect path switch, it is clear that the measurement quantity can use SD-RSRP, as Remote UE may not establish PC5 unicast link with neighbour Relay UE before discovery. On the other side, Remote UE can also establish PC5 unicast link with neighbour Relay UE before discovery based on UE implementation, and in this case, SL-RSRP may be considered as the measurement quantity.

Proposal 9b: For neighbour/target SL measurement of direct to indirect path switch, the SL measurement quantity should use SD-RSRP as baseline. FFS on whether SL-RSRP can be used in addition.

For SL measurement event triggered reporting, similarly, SL measurement event related to SD-RSRP, or SL measurement related to SD-RSRP and Uu-RSRP can be considered. That is to say, both event when the SD-RSRP of neighbour Relay UE is better than threshold3, and event when the SD-RSRP of neighbour Relay UE is better than threshold3 and Uu-RSRP of serving cell is worse than threshold4 can be considered. Regarding whether only Uu-RSRP of serving cell can be supported in the SL measurement event, RAN2 should further discuss this.

Proposal 9c: For neighbour/target SL measurement of direct to indirect path switch, two new trigger events can be introduced:

- Event1: SD-RSRP of neighbour Relay UE is better than a threshold

- Event2: SD-RSRP of neighbour Relay UE is better than a threshold and Uu-RSRP of serving cell is worse than a threshold

Proposal 9d: RAN2 to discuss whether to consider the A2 event (i.e. Uu-RSRP of serving cell is worse than a threshold) as new trigger event for SL measurement for direct to indirect path switch.

It had been agreed that SL relay measurement report can include at least Relay UE ID, serving cell ID, RSRP information in RAN2#114 e-meeting [4]. In our thinking, the serving cell ID should be CGI+PCI, as CGI+PCI can be used to differentiate the inter-gNB case, and also are forward compatible to future release with inter-gNB path switch supporting.

Proposal 9e: The serving cell ID included in the neighbor Relay UE measurement report is CGI+PCI.

Inter-gNB case

Although inter-gNB path switch is not supported in this release, to have forward compatibility with future release, neighbour cell or neighbour Relay UE belonging to inter-gNB may also be included in SL measurement report for the path switch, and gNB can ensure Remote UE to path switching to intra-gNB neighbour cell or neighbour Relay UE by NW implementation. However, one corner case needs to be discussed, whereas all candidate neighbour cell(s) or neighbour Relay UE(s) belong to inter-gNB compared with serving Relay UE or serving cell. In our thinking, legacy behaviour can be used, that is to say, gNB can release Remote UE to RRC_IDLE/RRC_INACTIVE, or Remote UE to perform RRC re-establishment after detecting the PC5/Uu RLF.

Proposal 10: In case there is no neighbor cell/neighbor Relay UE belonging to intra-gNB , legacy behavior can be used, i.e. gNB to release Remote UE to RRC_IDLE/RRC_INACTIVE, or Remote UE to perform RRC re-establishment after detecting the PC5/Uu RLF.

Path switch failure

For Uu handover, the UE gets a timer value t304 in the RRCReconfiguration message with reconfigurationWithSync, where UE starts timer T304 upon receiving the reconfiguration with sync. The timer T304 is used to decide whether the handover is successfully completed. Upon the UE successfully complete the random access to the target cell, the timer T304 will stop, i.e., this indicates the UE has successfully build the connection with network. If T304 is expired, handover failure is claimed and UE needs to revert back to the UE configuration in the source Pcell and initiate the connection re-establishment procedure.

Path switch from indirect to direct

For path switch from indirect to direct, same as the Uu handover, the remote UE needs to directly access a cell, e.g., an intra-gNB cell. Therefore, T304 can be reused to judge the path switch failure. UE can start timer T304 upon receiving the reconfiguration with sync and declares path switch failure when T304 is expired.

Proposal 11: Legacy T304 based failure detection is applicable to path switch from indirect to direct case, i.e. T304 expiry indicates the path switch failure.

Path switch from direct to indirect

For path switch from direct to indirect, the remote UE needs to establish a connection with relay UE, instead of access a cell directly. This is different from Uu Handover. However, for this switch, it is still needed to judge whether the path switch is successful.

For judging whether the path switch is successful, similar to the Uu handover, timer based solution can still be used. However, T304 is configured via reconfiguration with sync and stopped upon successfully completing the access to the target cell, but for path switch from direct to indirect, there is no RA involved  and the timer cannot be stopped. Therefore, directly use T304 will cause  troubles. To solve the problem, introducing a new T304-like timer is preferred for detecting the path switch failure from direct to indirect case.

Proposal 12: In traduce a new time like T304 to support timer based failure detection for path switch from direct to indirect case.

Procedure after detecting path switch failure

For path switch from indirect to direct case, the remote UE follows the legacy behaviour of T304 expiry, i.e., initiate RRC re-establishment procedure.

For path switch from direct to indirect case, after path switch failure is declared, the remote UE should try to re-establish the connection with the network to reduce the service interruption, i.e., triggered the RRC re-establishment procedure.

Proposal 13: Upon path switch failure detected for direct-to-indirect case and indirect-to-direct case, remote UE should trigger RRC re-establishment procedure.

Relay UE handover case

Since group handover is not supported in current release, it was agreed in RAN2 113bis-e meeting that “when relay performs HO to another gNB, relay UE may send a PC5-S message (similar to LTE) to its connected remote UE(s) and this message may trigger relay reselection” for a RRC_IDLE remote UE. Similar actions can be applied to the remote UE(s) in RRC_CONNECTED state. That is, when the relay UE performs HO to another gNB, it can transmit a PC5-S message to indicate the connected remote UE(s), then release the PC5 connection with them. After receiving the indication, a RRC_CONNECTED remote UE selects a suitable relay or a suitable cell, and re-establishes the RRC connection to the selected relay or cell.

Another way to reduce the remote UEs’ service interruption time is allowing the gNB to release/HO remote UE away before its connected relay performs inter-gNB HO. This can always be supported by gNB implementation.

Proposal 14: In case of relay UE HO, relay UE can send a PC5-S message to its connected remote UE(s) to trigger remote UE’s RRC re-establishment.

Lossless issue

Fugire 3. Data loss case during remote UE’s indirect to direct path switch

When remote UE performs indirect to direct path switch, data loss would happen if some data buffered at relay UE are not timely transmitted to the gNB in uplink or to the remote UE in downlink. We take the uplink data transmission as  example to illustrate this issue. As Figure 3 shows, remote UE transmitted packets 1 through 5 and received all the ACKs for them before path switch. Meanwhile, packets 1 and 2 was transmitted to the gNB but only packet 2 was acknowledged at the relay UE. After the path switch procedure, the remote UE starts uplink data transmission from packet 6, as it has received ACKs for packets 1 through 5. Following the legacy manner, the buffered RLC data of remote UE at relay UE will be discarded and will be not be re-transmitted by remote UE. However, the gNB actually only has packet 2 in the receiver which cause data loss. One simple solution to this issue is relay UE continues the buffered data transmission at relay UE by implementation. Namely, relay UE maintains Uu configuration for remote UE and continues data transmission to gNB for UL buffered data. Similarly for the downlink, relay UE maintains PC5 configuration for remote UE and continues the transmission to remote UE for DL buffered data. Until all the buffered date of Remote UE are transmitted, configurations for remote UE at the relay UE are released.

Observation 1: Legacy Uu PDCP behaviour can guarantee lossless path switch for DL and UL based on sensible network and relay UE implementation.

One corner case during remote UE’s indirect to direct path switch is Uu RLF failure happens between the relay UE and the gNB (as shown in Figure 3), the above solution then may not work. Another enhancement is to initiate PDCP re-transmission for the remote UE. For example, in the uplink, remote UE can re-transmit the lost packets to gNB by receiving and checking the PDCP status report from the gNB. But, when and how to transmit the PDCP status report is an issue. Similar problem exists when remote UE performs RRC re-establishment due to the relay UE’s inter-gNB HO. The buffered data of remote UE at relay UE will be lost if the relay UE performs HO to another gNB. The above two data loss issues exists, but may be corner cases. RAN2 can take the legacy PDCP behaviour as the baseline for the lossless of serving continuity, and discuss the need of remote UE PDCP re-transmission based on the PDCP status report as enhancement, if time is available.

Observation 2: There may be data loss based on legacy Uu PDCP behaviour in case of Relay UE’s Uu RLF or inter-gNB HO during Remote UE’s path switch, which could be considered as corner cases.

Proposal 14: The legacy PDCP behaviour is baseline for the lossless of serving continuity. RAN2 to discuss the need of remote UE PDCP re-transmission based on the PDCP status report as enhancement.

- Conclusion

The observation and proposals are as follows:

Observation 1: Legacy Uu PDCP behaviour can guarantee lossless path switch for DL and UL based on sensible network and relay UE implementation.

Observation 2: There may be data loss based on legacy Uu PDCP behaviour in case of Relay UE’s Uu RLF or inter-gNB HO during Remote UE’s path switch, which could be considered as corner cases.

Proposal 1: For path switch from indirect to direct, RAN2 assume the PC5 unicast link will be released after step 3 (RRC Reconfiguration message to Remote UE), i.e. the L2 IDs are not shared between SL Relay communication and SL normal communication.

Proposal 2: For path switch from direct to indirect, sending RRC Reconfiguration message to Relay UE should be performed after target Relay UE connects (i.e. transit from RRC_INACTIVE/RRC_IDLE to RRC_CONNECTED state) to the gNB.

Proposal 3: For path switch from direct to indirect, the target Relay UE ID in RRC Reconfiguration message is C-RNTI of the Relay UE in RRC_CONNECTED state.

Proposal 4: For path switch from direct to indirect, remote UE’s C-RNTI should be included in the RRC Reconfiguration Complete of step 5.

Proposal 5: RAN2 to prioritize the direct to indirect path switch via RRC_CONNECTED Relay UE, and pursue the direct to indirect path switch via RRC_INACTIVE/RRC_IDLE Relay UE if time permitted.

Proposal 6a: For path switch from direct to indirect, RRC_INACTIVE/ RRC_IDLE Relay UE enters RRC_CONNECTED state (triggered by remote UE), upon receiving data (i.e. Remote UE’s RRC Reconfiguration complete message) from PC5 RLC.

Proposal 6b: The PC5 RLC and Uu RLC are configured via dedicated signaling with default configuration, for Remote UE’s SRB1 of RRC Reconfiguration complete message.

Proposal 6c: For path switch from direct to indirect via RRC_INACTIVE/RRC_IDLE Relay UE, the Relay UE ID in RRC Reconfiguration message can be index (e.g. allocated by Remote UE during discovery).

Proposal 7: SL measurement report is needed in both indirect to direct path switch for serving Relay UE and direct to indirect path switch for neighbor Relay UE.

Proposal 8a: For SL measurement of indirect to direct path switch, the SL measurement object can reuse R16 SL measurement object (i.e. SL carrier).

Proposal 8b: For SL measurement of indirect to direct path switch, the SL measurement quantity is SL-RSRP.

Proposal 8c: For SL measurement of indirect to direct path switch, two new trigger events can be introduced:

- Event1: SL-RSRP of serving Relay UE is worse than a threshold

- Event2: SL-RSRP of serving Relay UE is worse than a threshold and Uu-RSRP of neighbour cell is better than a threshold

Proposal 8d: For indirect to direct path switch, RAN2 understands that gNB will not configure any serving cell related measurement and trigger events.

Proposal 9a: For SL measurement of direct to indirect path switch, the SL measurement object can reuse R16 SL measurement object (i.e. SL carrier). FFS on whether “Allow-list/Blocklist” is supported.

Proposal 9b: For SL measurement of direct to indirect path switch, the SL measurement quantity should use SD-RSRP as baseline. FFS on whether SL-RSRP can be used.

Proposal 9c: For SL measurement of direct to indirect path switch, two new trigger events can be introduced:

- Event1: SD-RSRP of neighbour Relay UE is better than a threshold

- Event2: SD-RSRP of neighbour Relay UE is better than a threshold and Uu-RSRP of serving cell is worse than a threshold

Proposal 9d: RAN2 to discuss whether to consider the A2 event (i.e. Uu-RSRP of serving cell is worse than a threshold) as new trigger event for SL measurement for direct to indirect path switch.

Proposal 9e: The serving cell ID included in the neighbor Relay UE measurement report is CGI+PCI.

Proposal 10: In case there is no neighbor cell/neighbor Relay UE belonging to intra-gNB available, legacy behavior can be used, i.e. gNB to release Remote UE to RRC_IDLE/RRC_INACTIVE, or Remote UE to perform RRC re-establishment after detecting the PC5/Uu RLF.

Proposal 11: Legacy T304 based failure detection is applicable to path switch from indirect to direct case, i.e. T304 expiry indicates the path switch failure.

Proposal 12: In traduce a new time like T304 to support timer based failure detection for path switch from direct to indirect case.

Proposal 13: Upon path switch failure detected for direct-to-indirect case and indirect-to-direct case, remote UE should trigger RRC re-establishment procedure.

Proposal 14: In case of relay UE HO, relay UE can send a PC5-S message to its connected remote UE(s) to trigger remote UE’s RRC re-establishment.

Proposal 15: The legacy PDCP behaviour is baseline for the lossless of serving continuity. RAN2 to discuss the need of remote UE PDCP re-transmission based on the PDCP status report as enhancement.

- Reference

- RP-210904, “New WID on NR Sidelink Relay”, Ericsson (Moderator), OPPO (rapporteur), RAN#91e, March 16-26, 2021

- 3GPP TR 38.836 V17.0.0 (2021-03); 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on NR sidelink Relay; (Release 17)

- R2-2106578, Report of [AT114-e][605][Relay] Summary on AI 8.7.4.2 on L2 relay service continuity, Samsung, RAN2#114e, May 19-27,2021

- Report of 3GPP TSG RAN WG2 meeting#114-e Online 19-27 May, 2021