3GPP TSG-RAN WG2 #115-e                                                                                        R2-2108733

eMeeting, 16th – 27th August, 2021                                                          Revision of R2-2106336

Agenda Item:	    8.2.2.2

Source: 	    MediaTek Inc.

Title:		    UE behavior for SCG deactivation

Document for:	    Discussion and decision

# 1 Introduction

In RAN2#112e, we have the following agreement regarding to UE behavior during SCG deactivation and upon SCG activation.

Agreements

- Continue RAN2 work with the assumption that when the SCG is deactivated, the UE does not monitor PDCCH on the PSCell. This assumption can be reconsidered if issues are found.

- As a baseline, MN-configured RRM measurement/reporting procedures do not depend on the SCG activation state (deactivated or activated). Further optimisations are not precluded.

- While the SCG is deactivated, PSCell mobility is supported. MN- and SN-configured measurements are supported for deactivated SCG.

- RAN2 assumes that UE will not perform SRS transmission while the SCG is deactivated. This assumption can be reconsidered if issues are found.

Agreements

5: When the SCG is in deactivated state, the UE sends MeasurementReport messages for measurement results of SN-configured measurements embedded in the E-UTRA (if the MCG is EUTRA) or in the NR (if the MCG is NR) ULInformationTransferMRDC message via SRB1

6a: When the SCG is in deactivated state, the UE can receive an SCG RRCReconfiguration message embedded in an MCG RRC(Connection)Reconfiguration message on SRB1, like when the SCG is activated, and then the UE

- processes the SCG RRCReconfiguration message according to Rel-15/16 procedures (FFS if any restriction/difference)

- sends an SCG RRCReconfigurationComplete message in the MCG RRC(Connection)ReconfigurationComplete message according to Rel-15/16 procedures

6b: The SCG RRCReconfiguration can change the PSCell.  FFS if the UE does RACH towards the target PSCell, in that case.

7b: FFS whether SCell can be added/reconfigured/released while the SCG is deactivated or this can be done only at SCG activation or after SCG activation.

8a: It is FFS whether the network can configure the UE stop certain configured RRM measurements while the SCG is deactivated, or can release certain RRM measurements at SCG deactivation.

8b: Relaxation of RRM measurement requirements (as compared with non-DRX activated cell requirements) while the SCG is deactivated is FFS.

Agreements

- FFS if RACH is needed for SCG reactivation

In RAN2#113e,

Agreements

1 	Confirm that there is no PUSCH transmission on deactivated SCG. FFS if any other UL is allowed towards SCG.

2 	Confirm that there is no PDCCH monitoring on PSCell of the deactivated SCG.

3 	Confirm that there is no support of SCell dormancy for SCG SCells within a deactivated SCG.

Agreements

5	Continue to discuss whether some kind of beam monitoring (similar to RLM/BFD) should be supported when the SCG is deactivated. FFS if this only applies to when TAT is running.

6	Clarify the meaning of "the UE maintains DL sync while the SCG is deactivated" (e.g. whether that is a consequence of doing RRM measurements of the PSCell or something more is needed).

Agreements

2	The UE behaviour when the SCG activation is indicated to the UE via the MCG is one or more of the following options:

option 1)	similar to reconfiguration with sync, i.e. the UE always initiates random access to the PSCell.

option 2)	in certain cases:

-	the UE does not initiate random access and monitors PDCCH on the PSCell (at the latest after the specified processing time).

-	the SCG can schedule data transmission on the PDCCH

The UE decides not to perform random access (one option to be selected):

option 2a) if the TA timer is still running and possibly other conditions (FFS how TAT starts)

option 2b) based on the contents of the SCG activation indication

FFS for option 2a): in the SCG deactivated state, the UE monitors some DL beams (FFS if the same as BFD or RLM) and, if the UE sees that the beams are not good enough (details FFS), the UE either (one of the options to be selected):

-	will perform random access upon reception of the next SCG activation indication from the MCG

-	reports measurement results (details FFS) via the MCG and wait for reconfiguration.

And in RAN2#113bis-e

Agreements

5	Only the MN can generate an RRC message with SCG (de)activation.

1	Indication of SCG deactivation to the UE via the SCG is not supported.

7	During handover preparation, the target MN can indicate the SCG state in the RRCReconfiguration message to be sent to the UE by the source MN.

8	The MN RRC reconfiguration message used to deactivate SCG and the embedded SN RRC reconfiguration message can reconfigure any parameter (any restriction requires an explicit decision).

9	While the SCG is deactivated, the MN RRC reconfiguration message and the embedded SN RRC reconfiguration message can reconfigure any parameter (any restriction requires an explicit decision).

- RRM requirements for deactivated PSCell may be different than for activated PSCell. What they could be are FFS pending RAN4 work.

In this paper, we continues discuss the UE behavior in SCG deactivation.

# 2 Discussion

## 2.0 Purpose of SCG deactivation

First we would like explain our general view on the objective of “efficient activation/de-activation mechanism for one SCG in (NG)EN-DC, and NR-DC”. The main motivation is of course for power saving purpose. However, 3GPP has introduced several power saving mechanism that could apply to PSCell (e.g. wake-up signal, cross slot scheduling, reduced maximum MIMO layer) in Rel-16. In that case, we consider that the SCG deactivation (or PSCell deactivation) as an extreme way to further reduce SCG power. Secondly, the WI clear specify this objective does not involve RAN1 WG. So, RAN2 should aim to have a solution without RAN1 impact.

Putting PSCell in deactivated state (as SCell deactivation) looks like very simple solution in term of specification complexity and there is no RAN1 impact. It could of course reduce the power consumption to a very low level as only RRM measurement is performed for the SCG cells. One concern on PSCell deactivation would be the delay to activate a deactivated PSCell. We believe that this could be also reduced by the temp RS mechanism to be discussed in RAN1 for this Rel-17 MR-DC enhancement WI. Besides, the PSCell activation could also be used in case of overheating, which is a real concern for FR2 SCG. It would be better to deactivate all SCG instead of releasing the SCG.

Observation 1: For power saving purpose and for thermal protection, PSCell deactivation (as SCell deactivation behaviour) is simple and efficient.

## 2.1 UE activities in SCG deactivation

RAN2 has agreed that UE does not monitor PDCCH of PSCell in SCG deactivation state. We would like to further discuss other UE behavior in “deactivated PSCell” state.

Regarding to whether the UE should perform RLM/BFD on that PSCell, we think the benefit to have RLM/BFD supported for deactivated PSCell is unclear. Does the UE really have to spend power to maintain the perfect serving beam while the data is not really transmitted? We believe RRM is enough to maintain the mobility. However, if reduced RLM/BFD is agreed in power saving WI, we could further discuss whether to have it.

For UL TA value maintenance, it will request to receive the MAC CE TA command and also triggering RACH while TA timer expire. Monitoring MAC CE (i.e. request monitoring PDCCH) or triggering RACH is not preferred at the PSCell deactivation state, which is designed for power saving and low data activity. Similarly, sending CSI-RS reporting on the deactivated PSCell request to maintain TA value and spend power on UL data transmission. This will prevent UE from going to deep sleep mode and increase power consumption. Another alternative is to send CSI reporting in MCG and use MCG MAC CE to maintain the TA value of SCG. Considering the inter-node delay and specification complexity, we don’t think this is a good idea.

Proposal 1: When a PSCell is deactivated

- The UE does not perform RLM/BFD on that PSCell

- The UE does not maintain the TA value for the SCG

- The UE does not report CSI on the PSCell or for the PSCell

## 2.2 RACH aspect on SCG activation

While the NW configure the PSCell back to “activated state”, we think the UE behavior is similar to the SCG addition procedure. The UE should trigger RACH to the configured PSCell and resume the SCG transmission. Based on the discussion from previous RAN2 meetings, a clear majority want to skip RACH if TA timer is not expired (meaning that TA value is still valid). We would like to emphasize that the approach itself does NOT work. In NR, RACH is not only used to get TA value but also to perform basic beam management. Therefore, some additional mechanism is needed to do RACH-less activation.

RAN2 already agreed that UE shall perform RRM measurement for deactivated PSCell. Therefore, the network could determine the TCI State/DL Beam based on the L3 measurement results of PSCell (similar to SCell activation). We consider that it is a simple method for network to indicate the TCI state upon activation (if we want to skip RACH). In addition, there is a late request from RAN4 to add TCI state information in RRC for direct SCell activation in Rel-16[1]. RAN2 does not add this configuration at the end since Rel-16 is frozen. It seems that similar issue occurs if the network want to activate PSCell and SCG SCell at the same time. So, we believe that TCI state indication is needed for both RACH-less PSCell activation and direction SCell activation.

Proposal 2: Add TCI State information in RRC configuration for RACH-less PSCell activation and direct SCell activation.

If TA is still running and the TCI state is provided, the RACH procedure could be skipped. Otherwise, the UE shall perform the RACH upon PSCell activation.

Proposal 3: While the PSCell is activated from deactivated state

- The UE start PDCCH monitoring after some processing time (i.e. no RACH) if TA timer is still running and TCI state of the PSCell is provided

- Otherwise, the UE Trigger RACH to the PSCell

## 2.3 RACH aspect during SCG deactivation

RAN2 agreed that network could reconfigure any parameter while SCG is deactivated. We think that it is also possible that the network change the PSCell but still keeping the SCG in deactivated state. In this case, we understand that the NW assumes no or very low data transmission activity in SCG and triggering RACH may just waste of power. The mobility event (SCG change in deactivated state) does imply that the network want to activate the SCG soon. It may trigger several deactivated PSCell change before activating the SCG. Therefore, we see no need to have RACH every time deactivated PSCell is changed.

Proposal 4: When deactivated SCG PSCell is changed, the UE does not initiate RACH until there is need to activate SCG.

# 3 Conclusions

Base on the discussion in section 2, we propose the following observations and proposals:

Observation 1: For power saving purpose and for thermal protection, PSCell deactivation (as SCell deactivation behaviour) is simple and efficient.

Proposal 1: When a PSCell is deactivated

- The UE does not perform RLM/BFD on that PSCell

- The UE does not maintain the TA value for the SCG

- The UE does not report CSI on the PSCell or for the PSCell

Proposal 2: Add TCI State information in RRC configuration for RACH-less PSCell activation and direct SCell activation.

Proposal 3: While the PSCell is activated from deactivated state

- The UE start PDCCH monitoring after some processing time (i.e. no RACH) if TA timer is still running and TCI state of the PSCell is provided

- Otherwise, the UE Trigger RACH to the PSCell

Proposal 4: When deactivated SCG PSCell is changed, the UE does not initiate RACH until there is need to activate SCG.

# 4 References

[1] R2-2100058 / R4-2017329, “LS on TCI state indication at Direct SCell activation”, RAN4