3GPP TSG-RAN WG2 Meeting #115 electronic	                     R2-2108748

Online, 9th – 27th August, 2021

Agenda item:	8.5.3

Source:	III

Title:	Remaining issues of harmonizing UL CG enhancements for IIoT in UCE

Document for:	Discussion

- Introduction

In 3GPP Rel-17 NR IIoT/URLLC, “the uplink enhancements for URLLC in unlicensed controlled environments (UCE)” had been accepted to be the working item. The objects focus on [1]:

- Specify support for UE-initiated COT for FBE with minimum specification effort

- Harmonizing UL configured-grant enhancements in NR-U and URLLC introduced in Rel-16 to be applicable for unlicensed spectrum

## To harmonize UL configured-grant enhancements for NR-U and IIoT/URLLC in unlicensed controlled environments, there are some issues need to be clarified.

## In Post114-e email discussion, many companies have different opinions on the “HARQ PID selection” and “Deprioritised UL grant when autoTx is not configured and CGRT is configured” issues. In this contribution, we share our views on these issues. Also, we bring the new issue “Configured grant mode switching for IIoT/URLLC in UCE”.

- Discussion

  - HARQ PID selection issue

    - Single CG configuration

## In Post114-e email discussion [2], many companies have different opinions for the follow question:

Question 2: What should the HARQ process ID selection behaviour be in the MAC entity for a single configured grant configuration, when lch-basedPrioritization and cg-RetransmissionTimer are both configured? Option 1: No change to the Rel-16 baseline (MAC entity prioritises the selection of HARQ process IDs for retransmissions over the selection of HARQ processes for initial transmissions) Option 2: MAC entity prioritises the selection of a free HARQ process ID (if available) to transmit higher priority data (if present). Option 3: NW configures whether to follow Option 1 or Option 2 Option 4: Other (please explain)

## For this question, we think the Rel-16 introduced “LCP restrictions” mechanism will let the same configured grant configuration have similar LCH priority, so in the single CG configuration, keep to the Rel-16 baseline (MAC entity prioritizes the selection of HARQ PIDs for retransmissions over the selection of HARQ processes for initial transmissions) is more sensible.

## Observation#1: Rel-16 introduced LCP restrictions will let the same configured grant configuration have similar LCH priority.

## Proposal#1: When lch-basedPrioritization and cg-RetransmissionTimer are both configured, in the MAC entity for a single configured grant configuration, the HARQ PID selection behaviour should keep using the R16 baseline.

  - Deprioritised UL grant when autoTx is not configured and CGRT is configured

## From our perspective, if cg-retransmission timer is configured and autonomousTx is not configured, the following issue should be discussed:

- When both cg-retransmission timer and lch-basedPrioritization are configured, the importance of procedure “retransmit the deprioritized MAC PDU”.

## Proposal#2: RAN2 should first discuss when both cg-retransmission timer and lch-basedPrioritization are configured, how important of the procedure “retransmit the deprioritized MAC PDU”.

## According to the importance of procedure “retransmit the deprioritized MAC PDU”, we have the following two proposals:

- If procedure “retransmit the deprioritized MAC PDU” is necessary, when both cg-retransmission timer and lch-basedPrioritization are configured, autonomousTx is always configured,

- If procedure “retransmit the deprioritized MAC PDU” is unnecessary, autonomousTx can configure or not configure, if autonomousTx is not configured, confirm the earlier agreement that a deprioritized MAC PDU is not retransmitted autonomously.

## Proposal#3: If procedure “retransmit the deprioritized MAC PDU” is necessary, when both cg-retransmission timer and lch-basedPrioritization are configured, autonomousTx is always configured.

## Proposal#4: If procedure “retransmit the deprioritized MAC PDU” is unnecessary, autonomousTx can configure or not configure, if autonomousTx is not configured, confirm the earlier agreement that a deprioritized MAC PDU is not retransmitted autonomously.

  - Configured grant mode switching for IIoT/URLLC in UCE

When observe the following agreements in RAN2# 112-113 meeting [3][4]:

RAN2#112-e:

1	cg-RetransmissionTimer can be configured optionally for shared spectrum

2	When cg-RetransmissionTimer is configured, Rel-16 NR-U mechanism is used for HARQ process ID and RV selection.

3	When cg-RetransmissionTimer is not configured, Rel-16 URLLC mechanism may be used for HARQ process ID and RV selection.

4	As a baseline, HARQ processes sharing between multiple CGs are allowed when cg-RetransmissionTimer is configured as in Rel-16 NR-U.

5	HARQ processes sharing between multiple CGs are not allowed when cg-RetransmissionTimer is not configured.

RAN2#113-e:

1	LCH based prioritization and cg-RetransmissionTimer can be configured together in Rel-17 (consensus)

2	Working assumption: cg-RetransmissionTimer and autonomousTx can be configured together in R-17. (13/22)

3	FFS: whether CG retransmissions due to LBT-failure should be handled when cg-RetransmissionTimer is not configured, e.g. using the autonomousTx framework. (split)

We have the following conclusion:

|  | cg-retransmission timer is configured | cg-retransmission timer is not configured |
| --- | --- | --- |
| HARQ PID & RV | Select by UE (NR-U feature) | Select by formula (IIoT feature) |
| HARQ processes sharing | Support (NR-U feature) | Not support (IIoT feature) |
| LBT failure | Support (NR-U feature) | Support (To be determined) |
| LCH based prioritization | Support (IIoT feature) | Support (IIoT feature) |
| Autonomous Tx | Support (IIoT feature) | Support (IIoT feature) |

1.	Through the RAN2# 112-e meeting agreements, we observe when the cg-retransmission timer is configured, NR-U CG enhancements is baseline, when the cg-retransmission timer is not configured, URLLC CG enhancements is baseline.

2	Through the RAN2# 113-e meeting agreements, we understand that whether configure cg-retransmission timer or not, the LBT failure and de-prioritization issue can be solved.

Observation#2: The cg-retransmission timer is configured or not can represent current configured grant feature (mode) is IIoT or NR-U.

Observation#3: Whether configure cg-retransmission timer or not, the LBT failure and de-prioritization issue

both can be solved.

However, in Rel-16, the URLLC CG enhancements (i.e. URLLC CG mode) is designed to licensed spectrum to solve the latency issue, the NR-U CG (i.e. NR-U CG mode) enhancements is designed to unlicensed spectrum to solve the reliability issue. Rel-17 IIoT/URLLC in UCE, the LBT/Tx failure may happened at any moment (e.g. unexpected interference), when the current communication quality become worse, the URLLC CG mode would not be the optimal solution, the transmission fails will let the resource utilization become lower. Moreover, when the current communication quality is good, the NR-U CG mode’s feedback mechanism will cause the resource redundancy and let the resource utilization become lower.

Observation#4: To get better performance, dynamically switching NR-U and IIoT/URLLC CG mode should be considered.

Thus, the uplink configured grant mode switching between NR-U and URLLC in unlicensed controlled environments should be considered.

Proposal#5: RAN2 should consider an uplink configured grant mode switching mechanism between NR-U and URLLC in unlicensed controlled environments.

- Conclusion

For URLLC/IIoT in unlicensed controlled environments, there are some discussed in this contribution with the following observation made and proposal proposed:

## Observation#1: Rel-16 introduced LCP restrictions will let the same configured grant configuration have similar LCH priority.

## Proposal#1: When lch-basedPrioritization and cg-RetransmissionTimer are both configured, in the MAC entity for a single configured grant configuration, the HARQ PID selection behaviour should keep using the R16 baseline.

## Proposal#2: RAN2 should first discuss when both cg-retransmission timer and lch-basedPrioritization are configured, how important of the procedure “retransmit the deprioritized MAC PDU”.

## Proposal#3: If procedure “retransmit the deprioritized MAC PDU” is necessary, when both cg-retransmission timer and lch-basedPrioritization are configured, autonomousTx is always configured.

## Proposal#4: If procedure “retransmit the deprioritized MAC PDU” is unnecessary, autonomousTx can configure or not configure, if autonomousTx is not configured, confirm the earlier agreement that a deprioritized MAC PDU is not retransmitted autonomously.

Observation#2: The cg-retransmission timer is configured or not can represent current configured grant feature (mode) is IIoT or NR-U.

Observation#3: Whether configure cg-retransmission timer or not, the LBT failure and de-prioritization issue

both can be solved.

## Proposal#5: RAN2 should consider an uplink configured grant mode switching mechanism between NR-U and URLLC in unlicensed controlled environments.

References

[1]	RP-201310, Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NRWID, 3GPP TSG RAN Meeting #88e, Electronic meeting, June 29 – July 3, 2020;

[2]	R2-2108231, Summary of [Post114-e][510][URLLC/IIoT] Open issues for UCE (Mediatek), 3GPP TSG RAN WG2 Meeting #115-e, Electronic meeting, Aug 9 – Aug 27, 2021.

[3] 	R2-2010704, Report for Rel-16 (NR-U, Power Savings and 2-step RACH) and IIoT and Small Data, 3GPP TSG-

RAN WG2 Meeting #112 electronic, Online, November, 2020

[4] 	R2-2101954, Report for Rel-17 Small data and URLLC/IIoT and Rel-16 NR-U, Power Savings, and 2step RACH, 3GPP TSG RAN WG2 Meeting #113e, Electronic meeting, Jan 25 – Feb 5, 2021.