3GPP TSG-RAN WG2#115-e Meeting	R2-2108775

# Online, Aug, 2021																(Revision of R2-2106059)

Agenda Item:		8.2.3.1

Souce:	Samsung

Title:	Further consideration on CPAC stage 2 flow, and remaining issues

Document for:		Discussion and decision

Introduction

In this contribution we discuss the remaining aspects resulting from the last RAN2#114-e meeting. In particular, we discuss the overall signalling flow and a.o. we propose to adopt solution 1 as baseline. As part of the evaluation, we address some key aspects which we think concern capability coordination, radio bearer configuration and support of option for T-SN to suggest alternative candidates. Note that we think that cleanup of measurement configuration concerning not accepted candidates is merely a secondary aspect for this decision.

The main proposals concern:

- Adopt solution 1 as baseline

- Accept the limitation that internode messages include of a single value of the capability coordiniation and radio bearer information, i.e. one value common for all candidates

- Do not introduce specification changes to facilitate cleanup of measurement configuration concerning not accepted candidates

- MN sends SN change confirm after receiving ReconfigurationComplete from the UE

- Baseline for RRM inter-node information carried within Xn messages

- Updating CPC configuration due to S-SN configuration update is supported, and corresponding Xn message sequences needs to be defined. (updated part)

Discussion

SN initiated change of SN, remaining stage 2 level issues

Review agreements in RAN2#114-e

During R2#114e, RAN2 made some agreements as below:

Working assumption (to clarify agreements 1-3 above) 1. Upon SN initiated CPC configuration, S-SN indicates the CPC candidates to MN and for each an execution condition 2. S-SN can provide also measurements to MN/T-SN and this may include cells that are not CPC candidates 3. T-SN can either accept or reject the CPC candidates suggested by S-SN (as in 1) i.e. it cannot come up with any alternative candidates 4. S-SN is informed about which candidates were accepted/ rejected by T-SN 5. S-SN can subsequently update the (measurement) configuration. FFS for execution conditions. 6. S-SN can perform this update after the CPC configuration. FFS whether to support updating during the CPC configuration (i.e. solution 2). FFS whether nested procedure is supported

The primary aspect discussed and concluded is to adopt solution 1 as working assumption. And support of solution 2 is remaining FFS. i.e. whether to have an additional procedure towards S-SN before MN forwards the CPAC configuration to the UE. This contribution focusses on resolving that particular aspect.

Deciding between solution 1 and 2

Starting point

We think solution 1 is clearly the baseline procedure as it is most consistent with general principles and previous agreements. Solution 2 was raised to address cleanup of S-SN configuration related to CHO candidates not admitted by T-SN e.g. measurement gap, measurement IDs, reporting configurations.

We think discussion showed that many companies agree that measurement configurations for evaluation of CHO candidates that were not admitted are not causing any real problem and that specification offers means for network to handle any cleanup network may desire to perform. I.e. we understand a large majority agrees this can be left to network implementation. We think it is clear that issue raised not at all a showstopper for solution 1.

Solution 2 clearly introduces additional complexities and issues, possibly re-opening some previous agreements e.g. regarding how/ when to transfer conditions. Hence, we altogether propose:

Proposal 1:	RAN2 agree the solution 1 as the agreement from working assumption.

As indicated in the previous, we think cleanup of measurement configurations is not a key factor deciding which solution to adopt. For deciding the overall signalling procedure, we think the following aspects are far more important:

- Coordination of UE capabilities i.e. whether this can be common for all CPC candidates indicated to T-SN

- Radio bearer configuration i.e. whether the radio bearer configuration can be common for all CPC candidates indicated to T-SN

- Whether T-SN should be able to suggest alternative candidates, outside the set for which S-SN initiated inter-SN CPC

There may be more of such key aspects, and to identify these we suggested RAN2 to develop a common view regarding the main inter-node RRC related information to be transferred, in particular the aspect of which parts need to be provided per candidate. Anyhow, for now we will focus on aspects a) to c).

Coordination of UE capabilities

For the coordination of how MN and SN share UE capabilities, MN sets configuration restrictions. The question is whether these would be common for all candidates included in an SN Addition Request to T-SN. An an example, the allowedBCs that MN sets, may differ depending on the band of the individual candidate. Let’s try to explain this by an example

- S-SN generates execution conditions for 3 CPAC candidates, say A, B and C

- MN triggers SN Addition Request to T-SN and includes information regarding capability coordination

  - For candidate B, there may be a need to modify the MCG configuration e.g. drop an MCG SCell (while this does not  apply for A and B)

- T-SN accepts a subset of the CPAC candidates

  - T-SN provides capability restriction information e.g. selectedBC. This may depend on whether T-SN refused candidate B or another candidate

We acknowledge that during conventional/ non-conditional inter-SN PSCell change, MN may provide measurement results for multiple PSCell candidates but just a single configuration restriction field for UE capability coordination.

- This suggests it may not be essential to support a different value for each potential candidate. We are hower not sure this observation really holds, as for conventional PSCell change the candidates may typically have similar characteristics, as preparation is triggered by a MR concerning one particular frequency

- On the other hand, it seems possible for network to use different SN addition messages towards T-SN in case it is not possible to apply the same value for all the candidates selected by S-SN

From the previous, it seems possible to just signal a common capability coordination value for all candidates. This approach imposes some limitations on network implementation, but these seem acceptable.

Radio bearer configuration

For the radio bearer configuration, the situation is similar to the coordination of UE capabilities. I.e. the amount of SCG resources may not be the same for all candidates, especially when not all candidates are on the same frequency. The available SCG resource may affect the DRBs that MN wishes to offload.

Let’s again try to explain this by the example

- S-SN generates execution conditions for 3 CPAC candidates, say A, B and C

- MN triggers SN Addition Request to T-SN and includes information regarding radio bearers to setup

  - For candidate B some limitations may apply regarding the data transfer that can be employed e.g. a limited data rate. I.e. for this candidate MN may wish to offload less of the traffic/ fewer DRB

- T-SN accepts a subset of the CPAC candidates

  - T-SN provides information regarding the radio bearer configurations. This may depend on whether T-SN refused candidate B or another candidate

We think the situation is similar to that of UE capability coordination. I.e. for the same reasons it seems possible to just signal a common radio bearer value for all candidates. This again imposes some limitations on network implementation, but these seem acceptable

Although some further analysis seems beneficial, we think it would be good to define a baseline/ starting point and hence we suggest:

Proposal 2:	As baseline, assume a single value of the RRC information (as in CG-Config/ CG-ConfigInfo) i.e. one value common for all candidates

  - Accept the limitation this imposes for capability coordination and radio bearer configuration

  - Except for the execution conditions and the T-SN generated RRCReconfiguration messages i.e. these are per CPC candidate

Further aspects remaining from previous discussions

This section addresses some of the proposal from the previous e-mail discussion and last RAN2#114-e meeting that were so far not concluded.

R2-2103109	Summary of [Post113-e][234][eDCCA] CPAC procedures (CATT)	CATT	discussion	Rel-17	LTE_NR_DC_enh2-Core	Late

First of all, we think the following proposals are still valid and they can simply be adopted:

Proposal 3:	Agree the following proposals remaining from R2-2103109:

- P2: Do not introduce specification changes to facilitate or restrict support of blind CPAC

- P5: Do not introduce specification changes to address cleanup of S-SN configuration related to CPC candidates not admitted by T-SN (i.e. can be left to network implementation, no need to specify UE behaviour)

For some other proposals, we prefer some modification:

P6: We agree that MN sends SN change confirm occurs upon receiving ReconfigurationComplete from the UE, confirming CPAC configuration. We are fine to inform RAN3, but think that the data forwarding aspects are best left to RAN3 (as always) i.e. no need for provide any RAN2 recommendations regarding this

P7: This document included some more discussion on the need for per candidate information from perspective of capability coordination and radio bearer configuration. Based on this, we think the proposal can be modified as shown below

Proposal 4:	Agree the following somewhat modified proposals remaining from R2-2103109:

- P6: MN sends SN change confirm after receiving ReconfigurationComplete from the UE. I.e. Further details regarding data forwarding aspects are left to RAN3

- P7: Baseline for RRC inter-node information needed in Xn messages

  - SN Change Required

    - Existing content (i.e. as in single CG-Config RRC INM)

    - Add execution condition per candidate cell,

  - SN Addition Request

    - Existing content (i.e. as in single CG-ConfigInfo RRC INM)

  - SN Addition Request Acknowledge:

    - Existing content (i.e. as in single CG-Config RRC INM)

    - Add target cell configuration per accepted candidate cell

  - SN change confirm

    - List of (not) accepted CPC candidates.

In the annex of this contribution we provide some analysis of the RRC information exchanged in inter-node messages that may be used as starting point for further analysis.

New aspects on S-SN configuration update

Since CPC configuration will be the delta signalled to the UE which is based on the current S-SN configuration, whenever S-SN configuration change happens, this should be reflected to the CPC configuration currently configured to the UE. Otherwise there could be the mismatch between the current S-Sn configuration and current configured CPC configuration (SCG part). And this is not intended by the network, so should be avoided. Threfore, whenever S-SN configuration is to be given to the UE, then CPC configurated updated also needs to be accompanied to the UE. Moreover, S-SN configuration update may also impact the current MCG/MN configuration for example, S-SN wants to change radio bearer configuration. Then, one S-SN configuration modification procedure should support the current MCG configuration update as well as CPC configuration update, i.e., S-SN configuration/MCG configuration, CPC configuration should be given in single MN format RRCReconfiguration to the UE.

Observation 1. S-SN configuration should be given to the UE with MCG configuration and CPC configuration together.

The thing is that, in current MRDC message sequence, there is no appropriate procedure. The related ones are SN modification procedure and SN change procedure. In SN modification procedure, only same SN configuration and its corresponding MN configuration can be signalled to the UE. In SN change procedure, just T-SN configuration information is only signalled to the UE, and no information of S-SN configuration update is transferred to MN i.e., SN Change Required msg doesn’t have related field. Therefore, once we agree the basic S-Sn update procedure, i.e., S-SN configuration update should be accompanied with corresponding MCG and CPC configuration update to the UE, that needs to be informed to RAN3 for making related procedure or modification of existing procedure.

Proposal 5. Discuss and agree on the S-SN configuration update procedure that S-SN configuration update should be accompanied with corresponding MCG and CPC configuration update to the UE. Send LS to RAN3 on this agreement to let them design related inter node procedure.

Conclusion & recommendation

This document discusses the main stage 2 issues regarding SN initiated inter-SN CPC. The document includes the following proposals that RAN2 is requested to discuss and conclude:

Proposal 1:	RAN2 agree the solution 1 as the agreement from working assumption.

Proposal 2:	As baseline, assume a single value of the RRC information (as in CG-Config/ CG-ConfigInfo) i.e. one value common for all candidates

  - Accept the limitation this imposes for capability coordination and radio bearer configuration

  - Except for the execution conditions and the T-SN generated RRCReconfiguration messages i.e. these are per CPC candidate

Proposal 3:	Agree the following proposals remaining from R2-2103109:

- P2: Do not introduce specification changes to facilitate or restrict support of blind CPAC

- P5: Do not introduce specification changes to address cleanup of S-SN configuration related to CPC candidates not admitted by T-SN (i.e. can be left to network implementation, no need to specify UE behaviour)

Proposal 4:	Agree the following somewhat modified proposals remaining from R2-2103109:

- P6: MN sends SN change confirm after receiving ReconfigurationComplete from the UE. I.e. Further details regarding data forwarding aspects are left to RAN3

- P7: Baseline for RRC inter-node information needed in Xn messages

  - SN Change Required

    - Existing content (i.e. as in single CG-Config RRC INM)

    - Add execution condition per candidate cell,

  - SN Addition Request

    - Existing content (i.e. as in single CG-ConfigInfo RRC INM)

  - SN Addition Request Acknowledge:

    - Existing content (i.e. as in single CG-Config RRC INM)

    - Add target cell configuration per accepted candidate cell

  - SN change confirm

    - List of (not) accepted CPC candidates.

Proposal 5. Discuss and agree on the S-SN configuration update procedure that S-SN configuration update should be accompanied with corresponding MCG and CPC configuration update to the UE. Send LS to RAN3 on this agreement to let them design related inter node procedure.

References

[1] TS 36.331 Radio Resource Control

Status following R2-113e/ R3-112e (Annex)

RAN2 reached the following agreements (summarised, possibly some earlier agreements to be merged)

  - For SN initiated CPC, large majority thinks MN builds Uu message by associating conditions from S-SN and response from T-SN (indicating which candidates are admitted) but no agreement yet

  - For all cases, the CPAC configuration concerns an MN generated message (MN format), including MCG and SCG configuration (i.e. MN parameters e.g. sk-Counter, T-SN generated target configuration and possibly S-SN parameters e.g. conditions). For conditions set by S-SN, FFS whether MN needs to comprehend and how to signal

  - Non-conditional configuration can be included in Reconfiguration message also carrying conditional reconfiguration (MN generated CPAC)

  - Conditions: B1/ A4 supported for MN initiated CPC

  - UE may delay compliance checking for RRC message within condRRCReconfiguration (no specification changes)

  - Upon CPAC configuration, UE returns ReconfigurationComplete message to MN not including ReconfigurationComplete message for T-SN. In case of SN initiated CPC, the message may include ReconfigurationComplete message for S-SN (confirming non-conditional reconfiguration e.g. measConfig)

  - Upon execution, UE returns ReconfigurationComplete message to MN now including ReconfigurationComplete message for SN

  - SRB1 is used for configuration and confirm

  - CPAC configurations are released upon conventional PSCell addition/ change and upon successful completion of CPAC

  - Existing SCG failure is baseline. Contents and other enhancements e.g. attemptCondReconfig are FFS

RAN3 furthermore reached the following agreements

General

- WA: Prepare multiple PSCells in one CPAC procedure

  - Do not provide Location Information and Resource Coordination information in CPAC, use same parameters for other IEs in the response message for different PSCells,

  - FFS for single RRC container or multiple RRC containers which is pending to RAN2.

  - Initiating node decides how many PSCells may be configured for UE (WA)

- WA: In case of SN initiated inter-SN CPC, baseline it to support preparation of multiple PSCells in one target SN by one SN Change procedure

- In CPA and MN initiated inter-SN CPC:

  - MN does not send execution condition(s) to the Target SN,

  - Target SN provides the prepared PSCell id(s) and the corresponding RRC container(s) (RRCReconfiguration) to the MN,

  - MN generates and transmits the conditional configuration message to the UE.

- Direct communication between S-SN and T-SN is not supported.

Earlier

- Target SN to make the decision on the prepared PSCell or PSCells (if decided to be allowed).

- WA: target SN to provide the prepared PSCell id (or PSCell ids, if decided to be allowed) to the MN for CPA, MN initiated inter-SN CPC, and SN initiated inter-SN CPC

User plane handling

- Support Early Data Forwarding in CPAC.

Earlier

- WA: in case of MN initiated inter-SN CPC, to support early data forwarding, the MN needs to inform source SN about CPC triggered (i.e. the successful reconfiguration of CPC at UE), details FFS.

- Support Late Data Forwarding in CPAC.

- WA: in case of both MN and SN initiated inter-SN CPC, to support late data forwarding, it is needed to inform the source SN about the successful CPC execution and UE accesses to the target SN, details FFS. RAN3 waits for RAN2 progress before discussing further details.

We understand that companies agree that RAN2 should decide the main remaining issue regarding handle signalling for SN initiated change, and this contribution addresses this aspect. This is done by preparing stage 2 flows, including overview of the relevant signalling.

Overall stage 2 flow (baseline from RAN2, RAN3 status)

In our understanding, RAN3 WA is to support option 2 i.e. one CPAC procedure can support preparation of multiple candidates. We think that candidates need to be visile at XnAP, e.g. to facilitate subsequent modification/ cancel, but think that further details are still FFS e.g. in which XnAP messages we will have such visibility. We furthermore think we can now assume baseline to be that MN builds the Uu message by associating conditions from S-SN and response from T-SN.

To facilitate the discussion, we provide an overview of the message sequence taking into account the previous starting points:

Fig. 1: Inter-node signalling for SN initiated SN change (inter-SN CPC)

Some remarks regarding the existing signalling, as used for non-conditional change of SN (FFS whether exactly same messages are re-used)

Part 1: Configuration

- SN change required can include multiple candidates on one T-SN, in which case it includes XnAP fields that are specific to one candidate PSCell i.e. candidates are visible at XnAP level. A.o. the execution condition is provided per candidate PSCell

- The SN addition request ack can include multiple PSCell candidates and has similar contents as SN change required. If multiple candidate PSCells are included, it is FFS whether for certain fields it should be possible to signal multiple values e.g. a value per candidate. This might be desirable for resources to be setup at SN, configuration restrictions

- The SN addition request ack includes for each candidate PSCell that is admitted/ accepted by T-SN a CG-Config RRC (INM), embedded within a container

- MN prepares the RRC reconfiguration message that can include a non-conditional S-SN generated RRCReconfiguration message and one or more candidate PSCells by condReconfigToAddModList. Each entry includes an S-SN generated execution condition (referring to the S-SN measConfig) and a MN generated of RRCReconfiguration message that furthermore ncludes a T-SN generated RRCReconfiguration message

- The UE returns an RRCReconfigurationComplete message that the message can include and ReconfigurationComplete message for S-SN (confirming a non-conditional reconfiguration, if included in 4)

- The SN change confirm may include the list of candidate PSCells that were admitted/ accepted

Part 2: Execution

- The UE transmits an RRCReconfigurationComplete message that includes a response to the MN generated RRCReconfiguration message, that again includes a response to the T-SN generated RRCReconfiguration message

- MN initiates release of S-SN

- S-SN acknowledges the SN release

- MN forwards to T-SN the RRCReconfigurationComplete by which UE confirms the T-SN generated Reconfiguration

- UE initiates RA towards PSCell

- S-SN may initiate SN status transfer procedure towards MN

- MN forwards the SN status transfer towards T-SN

Review of RRC related information in INM (Annex)

The following table provides some further details regarding the contents of the different messages.

PC: Per Candidate, PF: Per Frequency, PU: Per UE (same acorss all candidates)

| Option 2b | Type | Options | Remarks |
| --- | --- | --- | --- |
| 1. SN change required |  |  | RRC fields concerns subfields of CG-Config as provide upon PSCell change |
| PSCell candidate Id and details | XN | PC |  |
| CPC execution condition | RRC | PC, PF |  |
| Candidate cell info (measurement results) | RRC | PU | FFS for which cells e.g. only cells that S-SN considers to be suitable PSCell candidates? For each cell ARFCN of the RS (SSB and/ or CSI-RS) and measurement results (both cell and beam, SSB and/ or CSI-RS based) |
| Current S-SN assigned configuration (SCG config, RB config, measConfig and otherConfig) | RRC | PU |  |
| Current S-SN selection for capability coordination e.g. selected BC, FSC | RRC | PU |  |
| S-SN generated RRCReconfiguration (non-conditional changes) | RRC | PU (PF) | Setting measConfig for CHO candidates FFS whether measConfig signaled to UE should depend on which candidates are accepted by UE e.g. by partitioning per frequency. Seems enhancement |
| 2. SN addition request |  |  |  |
| PSCell candidates and for each the details (e.g. ARFCN, PCI) | XN | PC |  |
| SN UE AMBR | XN | PU? |  |
| PDU session resources to add | XN | PU? | FFS whether resources to be setup depend on actual candidates i.e. certain frequencies may be more loaded than others. MN would be sufficiently aware?? |
| SRB split to setup | XN | PU? | See previous |
| MR-DC resource coordination Info | XN | PU? | Not sure about the usage and whether it may depend on candidate, as previous |
| Mobility restriction/ RFSP | XN | PU? | Assume restrictinos would depend on UE and not on candidate, but not sure? |
| Candidate cell info (measurement results) | RRC | PU | FFS for which cells e.g. only for candidates? See SN change required. SN suggests candidates, so only SN configured measurements |
| Current SN assigned configuration (SCG config, RB config, measConfig and otherConfig) | RRC | PU |  |
| Configuration restrictions for capability coordination | RRC | PC, PF? | PC baseline/ preferred. Optimisations can be considered |
| Current SN selection for capability coordination e.g. selected BC, FSC | RRC |  |  |
| 3. SN addition request ack |  |  |  |
| PSCell candidates admitted by T-SN (e.g. by list of Id’s) | XN | PC |  |
| PDU session resources admitted/ not admitted | XN | PU? | See corresponding field in requesti |
| SRB split admitted/ not admitted | XN | PU? | Same as previous |
| MR-DC resource coordination Info | XN | PU? |  |
| T-SN generated Reconfiguration | RRC | PC | For each candidate, a container carrying the CG-Config RRC INM |
| Further fields for RRC based inter-node coordination e.g. SN selected BC for capability coordination, measurement coordination, bla | RRC | PC | All covered by same RRN INM as for T-SN generated reconfiguration |
| 4. RRC Reconfiguration |  |  |  |
| 5. RRC Reconfiguration Complete |  |  |  |
| 6. SN change confirm |  |  |  |
| PSCell candidates admitted by T-SN (e.g. by list of Id’s) | XN | PC |  |

Tab. 1: SN initiated change of SN, usage of R17 CPAC related field in each messages