3GPP TSG-RAN WG2 Meeting #115 Electronic	R2-2108818

Online, 16 – 27 August 2021

Agenda item:	8.21.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Draft reply LS to CT1 on UAC extensions for MINT (R2-2106902/C1-213527)

WID/SID:	FS_MINT-CT - Release 17

Document for:	Discussion and Decision

# 1	Introduction

RAN2#115-e received the LS in R2-2106902 from CT1 asking feedback on UAC aspects of Solutions #38 and #40 of 3GPP TR 24.811. In this contribution we investigate the RAN2 aspects of the solutions and propose a reply LS.

# 2	Discussion

Solution #38 is based on the assumption that within UAC-BarringInfoSet, an NG-RAN node can include specific barring factor for Access Identity 3.

The current structure of UAC-BarringInfoSetList information element is the following (6.3.2 of TS. 38.331):

-- ASN1START

-- TAG-UAC-BARRINGINFOSETLIST-START

UAC-BarringInfoSetList ::=          SEQUENCE (SIZE(1..maxBarringInfoSet)) OF UAC-BarringInfoSet

UAC-BarringInfoSet ::=              SEQUENCE {

uac-BarringFactor                   ENUMERATED {p00, p05, p10, p15, p20, p25, p30, p40,

p50, p60, p70, p75, p80, p85, p90, p95},

uac-BarringTime                     ENUMERATED {s4, s8, s16, s32, s64, s128, s256, s512},

uac-BarringForAccessIdentity        BIT STRING (SIZE(7))

}

-- TAG-UAC-BARRINGINFOSETLIST-STOP

-- ASN1STOP

UAC-BarringInfoSetList field descriptions

uac-BarringInfoSetList List of access control parameter sets. Each access category can be configured with access parameters corresponding to a particular set by uac-barringInfoSetIndex. Association of an access category with an index that has no corresponding entry in the uac-BarringInfoSetList is valid configuration and indicates no barring.

uac-BarringForAccessIdentity Indicates whether access attempt is allowed for each Access Identity. The leftmost bit, bit 0 in the bit string corresponds to Access Identity 1, bit 1 in the bit string corresponds to Access Identity 2, bit 2 in the bit string corresponds to Access Identity 11, bit 3 in the bit string corresponds to Access Identity 12, bit 4 in the bit string corresponds to Access Identity 13, bit 5 in the bit string corresponds to Access Identity 14, and bit 6 in the bit string corresponds to Access Identity 15. Value 0 means that access attempt is allowed for the corresponding access identity.

uac-BarringFactor Represents the probability that access attempt would be allowed during access barring check.

uac-BarringTime The average time in seconds before a new access attempt is to be performed after an access attempt was barred at access barring check for the same access category, see 5.3.14.5.

This includes the uac-BarringForAccessIdentity, but the current encoding does no enable to specify specific UAC parameters for Access Identity 3 (see text highlighted by yellow). Therefore, solution#38 requires an extension of SIB1 with UAC parameters for Access Identity 3.

Observation 1: Solution#38 requires an extension SIB1 with UAC parameters for Access Identity 3.

Solution#40 proposes to introduce a new UAC parameter uac-DisasterOffsetToBarringFactor that indicates to the disaster roaming UEs the offset value by which the BarringFactor must be reduced when evaluating the access barring condition for that access category. From NG-RAN perspective this requires to advertise a new UAC parameter. Our understanding that this new parameter should be PLMN specific. Therefore, solution#40 requires an extension of SIB1 with the UAC parameter per PLMN. The solution requires the UE has to calculate:

“uac-BarringFactor = max (p00, (uac-BarringFactor - uac-DisasterOffsetToBarringFactor for the access category))”

Observation 2: Solution#40 requires an extension SIB1 with the new uac-DisasterOffsetToBarringFactor per PLMN and additional UE procedure to calculate uac-BarrignFactor.

An extension of SIB1 for Solution#40 with the new UAC parameters is feasible in a backward compatible manner, though Solution#40 requires the UE calculates “uac-BarringFactor” according to new formula, with the new parameter included.

Regarding impacts towards SIB, Solution#38 and Solution#40 are similar. A small disadvantage of Solution #40 though is extra complexity for the UE calculate “uac-BarringFactor = max (p00, (uac-BarringFactor - uac-DisasterOffsetToBarringFactor for the Access Category))”, whereas Solution #38 directly indicates the barring factor to be applied.

Proposal 1: RAN2 sends a reply LS to CT1. (Draft reply LS can be found in the Annex.)

# 3	Conclusion

This document has made the following observations and proposal:

Observation 1: Solution#38 requires an extension SIB1 with UAC parameters for Access Identity 3.

Observation 2: Solution#40 requires an extension SIB1 with the new uac-DisasterOffsetToBarringFactor per PLMN and additional UE procedure to calculate uac-BarrignFactor..

Proposal 1: RAN2 sends a reply LS to CT1. (Draft reply LS can be found in the Annex.)

# Annex: Draft Reply LS to R2-2106902/C1-213527

Title:	[Draft] Reply LS on UAC enhancements for minimization of service interruption when disaster condition applies

Response to:	R2-2106902/C1-213527

Release:	Rel-17

Work Item:	FS_MINT-CT

Source:	RAN2

To:	CT1

Cc:	SA1

Contact person:	Name

malgorzata.tomala@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN2 thanks CT1 for their LS on "UAC enhancements for minimization of service interruption when disaster condition applies" (R2-2106902/C1-213527).

RAN2 investigated the RAN2 impacts of Solution#38 and Solution#40. It was found that both solutions require extensions of SIB messages to carry the additional new UAC parameters. More complexity is brought by Solution #40, which requires the UE to calculate “uac-BarringFactor” according to the new formula, whereas Solution #38 directly indicates the barring factor to be applied for an eligible access attempt. The extensions are feasible in a backward compatible manner and require further RAN2 work.

2. Actions:

To CT1 group:

ACTION:  RAN2 kindly requests CT1 to take the above into account.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP RAN2#116-e	from 2021-11-01	to 2021-11-12		Electronic Meeting