3GPP TSG-RAN WG2 Meeting #115-e 	R2-2108826

Electronic, 9th – 27th Aug, 2021

Agenda item:	8.11.3

Source:	ZTE, Sanechips

Title:	Summary of AI 8.11.3 for RRC_INACTIVE positioning

Document for:	Discussion and Decision

- Introduction

In this contribution, the following papers submitted to AI 8.11.3 are summarized:

| R2-2107092 | Discussion on positioning in RRC INACTIVE state | ZTE |
| --- | --- | --- |
| R2-2107142 | Discussion on Positioning for UEs in RRC_INACTIVE state | CATT |
| R2-2107149 | Considerations on positioning in RRC_INACTIVE mode | Fraunhofer IIS; Fraunhofer HHI |
| R2-2107358 | Discussion on positioning in RRC_INACTIVE state | Spreadtrum Communications |
| R2-2107643 | Enhancement of DL positioning in RRC_INACTIVE | vivo |
| R2-2107644 | Configuration of UL positioning in RRC_INACTIVE | vivo |
| R2-2107671 | Support of Positioning in RRC_INACTIVE | Intel Corporation |
| R2-2107683 | Discussion on Positioning in RRC INACTIVE state | InterDigital, Inc. |
| R2-2107684 | Discussion on reporting of Positioning Information with SDT | InterDigital, Inc. |
| R2-2107829 | Supporting positioning in RRC_INACTIVE state | OPPO |
| R2-2107830 | Discussion on UL Positioning methods in RRC_INACTIVE state | OPPO |
| R2-2108068 | Considerations on positioning RRC Inactive | Sony |
| R2-2108128 | On Positioning in RRC_INACTIVE state | Lenovo, Motorola Mobility |
| R2-2108173 | Discussion on positioning for UEs in RRC Inactive | Xiaomi |
| R2-2108394 | Inactive mode Positioning | Ericsson |
| R2-2108703 | Considerations on positioning in RRC_INACTIVE | Nokia, Nokia Shanghai Bell |
| R2-2108772 | On message segmentation for transmitting in Inactive state | Samsung Electronics |
| R2-2108764 | Considerations on Positioning in RRC_INACTIVE state | CMCC |
| R2-2108605 | Way-forward for INACTIVE Positioning | Huawei, China Unicom, China Telecom, Futurewei, HiSilicon, Intel Corporation, Interdigital, Spreadtrum Communications, VIVO, Xiaomi, ZTE Corporation |

As the WID[1][2] specifies, the DL positioning is specified as the first priority while the UL/UL+DL positioning is specified as second priority. Therefore, this summary mainly aims at summarizing views and generating proposals for DL positioning(section 2), for which RAN2 is desired to make some progress within the limited meeting time in RAN2#115e. The common aspects of UL and DL(section 3) and way-forward(section 5) are treated as medium priority, UL/UL+DL positioning(section 4) and others(section 6) are treated as low priority for RAN2#115e meeting.

In addition, other stage 2 procedure in RRC_INACTIVE is not included in this summary.

For the sake of meeting progress as well as saving the online meeting time, we suggest to treat the following proposals with high priority during the online session:

DL positioning

Proposal 1

Proposal 2

Proposal 3

Common aspects of UL and DL

Proposal 8

UL/UL+DL positioning

Proposal 9

- DL Positioning

The following company contributions provide their views towards DL positioning in RRC_INACTIVE:

| R2-2107829 | Supporting positioning in RRC_INACTIVE state | OPPO |
| --- | --- | --- |
| R2-2108173 | Discussion on positioning for UEs in RRC Inactive | Xiaomi |
| R2-2107142 | Discussion on Positioning for UEs in RRC_INACTIVE state | CATT |
| R2-2107149 | Considerations on positioning in RRC_INACTIVE mode | Fraunhofer IIS; Fraunhofer HHI |
| R2-2107358 | Discussion on positioning in RRC_INACTIVE state | Spreadtrum Communications |
| R2-2107671 | Support of Positioning in RRC_INACTIVE | Intel Corporation |
| R2-2107683 | Discussion on Positioning in RRC INACTIVE state | InterDigital, Inc. |
| R2-2107684 | Discussion on reporting of Positioning Information with SDT | InterDigital, Inc. |
| R2-2108068 | Considerations on positioning RRC Inactive | Sony |
| R2-2108128 | On Positioning in RRC_INACTIVE state | Lenovo, Motorola Mobility |
| R2-2108703 | Considerations on positioning in RRC_INACTIVE | Nokia, Nokia Shanghai Bell |
| R2-2108772 | On message segmentation for transmitting in Inactive state | Samsung Electronics |
| R2-2107643 | Enhancement of DL positioning in RRC_INACTIVE | vivo |
| R2-2108764 | Considerations on Positioning in RRC_INACTIVE state | CMCC |

Based on the input, the summary of DL positioning in RRC_INACTIVE is divided into several aspects:

- RAT independent method

  - This chapter discusses about whether to support RAT independent method in RRC_INACTIVE.

- DL assistance data transfer

  - This chapter discusses about where to configure DL assistance data and how to transmit DL assistance data.

- Alignment between positioning measurement and SDT

  - This chapter focuses on the alignment enhancement between SDT and positioning measurement, including the alignment of periodicity and data size.

- Assistance information from UE to network

  - This chapter focuses on additional assistance information from UE to network, in order to ensure network can configure appropriate SDT resource and period of PRS transmission.

- DL PRS configuration

  - This chapter focuses on some consideration when configuring DL PRS configuration, including the mobility and the validity conditions (e.g. time validity, area validity).

- Measurement report reporting

  - This chapter focuses on enhancing the UE measurement report, including the measurement report with mobility.

- Segmentation of LPP message

  - This chapter discusses about feasibility of segmenting LPP message in order to satisfy the data volume threshold of SDT.

  - RAT independent method

Intel Proposal 1: All RAT independent positioning methods, including Barometric pressure sensor, WLAN, Bluetooth, TBS, Motion Sensor, A-GNSS, etc can be supported in RRC_INACTIVE. Proposal 2: To confirm, the agreements on using SDT and RRC state transparent are applied for RAT independent positioning methods. Lenovo Proposal 2: Support the following the RAT-independent UE-assisted and UE-based positioning methods in RRC_INACTIVE state: A-GNSS, Motion Sensor, WLAN, TBS and Bluetooth (UE-assisted). ZTE Proposal 1: If RAN2 time budget is allowed, both RAT-independent positioning methods and RAT-dependent positioning method including ECID-positioning, UL/DL+UL positioning can be specified.

Rapporteur’s comments

Intel, Lenovo and ZTE proposed that RAT independent positioning methods should be supported in RRC_INACTIVE state. From rapporteur’s point of view, this is a straightforward proposal. However, since it may need further study that whether the signalling and procedures of supporting RAT independent positioning methods is different from the signalling and procedures of supporting RAT dependent positioning methods, the only concern is the time limit of RAN2.

(High priority)Proposal 1: Support all the RAT independent positioning methods in RRC_INACTIVE state.

  - DL assistance data transfer

OPPO Proposal 1: For positioning in RRC_INACTIVE state, the positioning assistance data can be delivered to UE in the following ways: a) positioning system information, b) pre-configured when UE in RRC_CONNECTED state; c) send to UE during ongoing SDT procedure. Xiaomi Proposal 3: gNB determine how to send DL LCS, LPP message and RRC message to inactive UE, either triggers UE to transition to RRC connected or sends the message to UE directly if there is ongoing SDT. Proposal 4: The positioning assistance data for RRC Inactive UE can be provided when UE in RRC inactive or in RRC Connected, and the following options can be considered for different cases: Option 1: The existing deferred MT-LR procedure Option 2: The existing LPP provide assistance data message with additional indication Option 3: The gNB broadcasts posSIB Option 4: The existing LPP provide assistance data message sent to UE directly when there is ongoing SDT Proposal 5: The following options can be considered for sending the request location information message to RRC Inactive UE: Option 1: The existing deferred MT-LR procedure Option 2: RequestLocationInformation with additional indication sent to UE in RRC Connected Option 3: The network send the location information request message to UE directly when there is ongoing SDT CATT Proposal 2: The UE performs DL location measurements in RRC_INACTIVE based on stored assistance data. If the stored assistance data is not valid, the UE in RRC_INACTIVE can acquire valid assistance data based on existing mechanism. Fraunhofer Proposal 2: Support providing the UE with assistance data during the RRC_CONNECTED state, which is applicable to the RRC_INACTIVE state. FFS: How is the assistance data structured FFS: LPP enhancements needed for providing valid assistance data during the RRC_INACTIVE state FFS: How are the resources downselected from the “redundant” assistance data provided (e.g. via SDT signaling or via measurement / configurations) Ericsson Proposal 5: Support of posSIB retrieval using SDT in Inactive mode is supported along with unicast tag. CMCC Proposal 2: With the request from UE in RRC_INACTIVE state, PRS configuration for specific UE can be delivered by ongoing SDT procedure in addition to the current LPP procedure and positioning SI.

Rapporteur’s comments

This section focuses on how the assistance data is transmitted to UE in RRC_INACTIVE state. It is proposed by OPPO and Xiaomi that there are several options for transmitting assistance data in RRC_INACTIVE state:

- Option 1: The existing deferred MT-LR procedure

- Option 2: positioning system information, i.e. posSIB

- Option 3: pre-configured assistance data when UE in RRC_CONNECTED state

- Option 4: ongoing SDT procedure

In addition, Xiaomi also proposed that the RequestLocationInformation message can be delivered using the same way as the assistance data transfer(except for the posSIB).

For option 2, Ericsson has mentioned to add a unicast tag in posSIB. If a unicast tag is included in posSIB, UE may not try to receive the broadcast AD. For more information please refer to R2-2108394.

So we have the following proposal for discussion:

(High priority)Proposal 2: Support to deliver the positioning assistance data to UE via the following options:

- Option 1: The existing deferred MT-LR procedure

- Option 2: positioning system information, i.e. posSIB

  - FFS: whether to introduce a unicast tag in posSIB

- Option 3: pre-configured assistance data when UE in RRC_CONNECTED state

- Option 4: ongoing SDT procedure

  - Alignment between positioning measurement and SDT configuration

About optimizing the measurement report, company contributions provided their views as follows:

Spreadtrum Proposal 2: The LMF can adjust configuration for positioning data report to adjust the data size considering the SDT data volume threshold. Proposal 3: gNB should inform LMF the SDT data volume threshold. Proposal 4: LMF should indicate gNB the estimated data size of measurement reports or location estimates. vivo Proposal 3: The data size optimization of ProvideLocationInformation should be considered. Proposal 4: Introduce differential report to optimize the data size of report in deferred MT-LR. Lenovo Proposal 4: The LMF reporting intervals in RRC_INACTIVE state are aligned with the CG SDT configuration for low latency delivery of the positioning measurement reports. Sony Proposal 1: Specify configuration of UE locations measurements in RRC Inactive state for DL based measurements, and the possibility to use small data uplink transmission (SDT) and/or PUR for positioning measurement reporting. OPPO Proposal 3: No optimization is introduced for positioning measurement report.

For optimizing the SDT configuration, the company contribution provided their views as follows:

Spreadtrum Proposal 1: Considering the data size of positioning reports in RRC_INACTIVE state, either RAN2 can optimize the data size of positioning reports or RAN2 can modify the SDT data volume threshold. Proposal 5: RAN2 can modify the SDT data volume threshold which can be discussed in SDT session. InterDigital Proposal 6: Support configuration of CG for SDT with parameters (e.g. periodicity) aligned with measurement reporting occasions in INACTIVE. InterDigital Proposal 11: Support configuring of CGs that are valid across different cells for sending periodic measurement report/location estimates when in INACTIVE. Nokia Proposal 1: RAN2 to discuss whether to specify standardized triggering criteria as to when an RRC_INACTIVE UE can initiate UL SDT transmission for signaling to the LMF and also how to control how frequently the UE can trigger signaling to LMF using UL SDT. Xiaomi Proposal 2: Whether use SDT or transition to RRC connected to send uplink LCS and LPP messages is depended on the SDT mechanism. Ericsson Proposal 1: LMF provides the configured measurement periodicity and positioning requirements to gNB. Proposal 2: RAN2 to send an LS to RAN3 to include additional information of positioning requirements in NRPPa. CATT Proposal 1: LMF can provide period of Deferred positioning to assist the gNB to decide whether to release the UE into RRC_INACTIVE, configure SDT for SRB2, or configure CG resource for SDT.

Rapporteur’s comments

Firstly, it reached a consensus that if the measurement report is transmitted using SDT in RRC_INACTIVE state, it requires a transmission data size smaller than the data volume threshold. The purpose is to align the period and data volume threshold of SDT with the period and data size of measurement report. Whether to enhance measurement report or SDT needs further discussion:

- If the measurement report is to be enhanced, some company contributions proposed to optimize the data size UE measurement report in order to align with the SDT data volume threshold. Furthermore, company contributions provided different views on whether the data size of UE measurement report is determined by UE itself or by the network.

  - If it is determined by the network, Spreadtrum pointed out that gNB may need to inform LMF the SDT data volume threshold.

  - If it is determined by UE, vivo proposed to introduce differential report to optimize the data size of report in deferred MT-LR.

  - OPPO proposed that no optimization should be introduced for positioning measurement report.

- If the SDT is to be enhanced, Spreadtrum proposed that RAN2 can modify the data volume threshold of SDT.

Another issue is to ensure appropriate SDT configuration for positioning in RRC_INACTIVE. Considering mobility when UE in RRC_INACTIVE state, InterDigital proposed to configure CGs for SDT that are valid across different cells. It is also proposed that gNB needs to consider the period of measurement report when configuring SDT period in order to align them. Ericsson and CATT proposed to send measurement periodicity and positioning requirements(such as response time) from LMF to gNB, in order to guide the SDT configuration at gNB side. From rapporteur’s point of view, if gNB acquires the measurement periodicity and positioning requirements, it is benefit for gNB to configure SDT resources or PRS/SRS configuration which is suitable for RRC_INACTIVE state.

In addition, it reached a consensus that RAN2 should discuss the alignment. It may be further determined whether to discuss this issue in SDT agenda or positioning agenda.

Based on the summary, we have the following proposal for discussion:

(High priority)Proposal 3: For the alignment between the positioning measurement and the SDT configuration, consider the following alternatives:

- Alt 1: Support LMF to inform gNB the estimate data size of measurement report

- Alt 2: Support LMF to inform gNB the measurement periodicity

- Alt 3: Support LMF to inform gNB the positioning requirements

- Alt 4: Support gNB to inform LMF the SDT data volume threshold

- Alt 5: Support differential measurement report

- Alt 6: No optimization should be introduced for positioning measurement report

FFS: Whether to discuss this issue in SDT agenda or positioning agenda.

  - Assistance information from UE to gNB

InterDigital Proposal 9: Support transmission of assistance information from UE to gNB for aligning DL-PRS reception and DRX on-duration during INACTIVE. InterDigital Proposal 1: Support transmission of assistance information from UE to gNB for ensuring suitable SDT resources (e.g. RA-SDT/CG-SDT) and data volume threshold is configured in UE. Proposal 2: The assistance information sent by UE to gNB when supporting positioning in INACTIVE includes at least type of reporting (e.g. periodic, aperiodic), and payload size of LPP message (e.g. measurement report/location estimates). Proposal 3: Support transmission of an indication (e.g. SDT-BSR) for requesting resource grants for transmitting NAS PDUs (e.g. containing LPP messages) in SDT-SRB2 when the payload size of the NAS PDUs is greater than the configured data volume threshold. Proposal 7: Support transmission of assistance information from UE to gNB for indicating start timing, measurement duration and reporting periodicity such that the CG-SDT parameters can be configured to align with the positioning measurement reporting occasions.

Rapporteur’s comments

It is proposed by InterDigital that an assistance information from UE to the network is needed to ensure network can configure appropriate SDT resource and period of PRS transmission. From Rapporteur’s point of view, since this is only proposed by one company, it may be depriorized due to the limit of discussion time.

(Medium priority)Proposal 4: Further study the assistance information transmitted from UE to gNB. The assistance information may include following aspects:

- type of reporting (e.g. periodic, aperiodic)

- payload size of LPP message (e.g. measurement report/location estimates)

- start timing

- measurement duration

- reporting periodicity

  - DL PRS configuration

Fraunhofer Proposal 1: Support providing UE with assistance data describing the DL-PRS available within the RNA, some of which may be outside the hearability region of the UE at its current location (i.e. redundant assistance data). FFS: Whether the A/D received with a larger area scope (e.g. positioning system area) can be divided into A/D applicable to smaller area scope (e.g. cell-level), so that the AD corresponding to the area where the UE is camped can be triggered via SDT mechanism. InterDigital Proposal 1: Support transmission of network-initiated DL indication to UE for initiating measurement of preconfigured PRS when in INACTIVE. Proposal 2: Support configuring PRS configurations which are intended to be used by UE when operating in RRC INACTIVE state. Proposal 3: The PRS configurations intended for usage during RRC INACTIVE state should ensure that at least a minimum positioning accuracy requirement (i.e. required by positioning service/LCS client) is satisfied. Proposal 4: The PRS configurations intended for usage during RRC INACTIVE state should take into acount of the parameters of SDT configuration (e.g. RA-SDT/CG-SDT resources, data volume threshold) possibly configured in UE. Proposal 5: Support configurating validity conditions (e.g. time validity, area validity) when configuring PRS configurations intended to be used by UE during RRC INACTIVE state. Proposal 8: Support alignment between the DL-PRS reception/measurements and DRX configuration. vivo Proposal 2: Introduce validity criteria for PRS configuration in RRC_INACTIVE received in RRC_CONNECTED. OPPO Proposal 2: No additional information is needed to differentiate the assistance information for RRC_INACTIVE and RRC_CONNECTED.

Rapporteur’s comments

In the aspect of DL PRS configuration, considering the mobility that UE may suffer during the RRC_INACTIVE state, several company contributions suggested to differ DL PRS configuration in RRC_INACTIVE state and DL PRS configuration in RRC_CONNECTED state. For example, Fraunhofer proposed that support providing UE with assistance data describing the DL-PRS available within the RNA. InterDigital proposed to configure PRS which are intended to be used by UE when operating in RRC INACTIVE state, including configuration requirements and validity conditions (e.g. time validity, area validity). vivo also suggested to introduce validity criteria for PRS configuration in RRC_INACTIVE. Oppo suggest that no additional information is needed to differentiate AD for RRC_INACTIVE and RRC_CONNECTED.

Based on the above analysis, we have the following proposal for discussion:

(Medium priority)Proposal 5: For the PRS configuration used for RRC_INACTIVE state, consider the following alternatives:

- Alt 1: Configure RNA information in the PRS configuration

- Alt 2: Configure validity conditions (e.g. time validity, area validity) in the PRS configuration

- Alt 3: No change should be introduced in PRS configuration

Note: This does not necessarily mean to expose RRC state to LMF.

  - Measurement reporting

InterDigital Proposal 10: Support sending of measurement reports/location estimates which are aligned with the transmission of RNA updates. Lenovo Proposal 1: RAN2 to support RRC_INACTIVE reporting of RRM measurements along with other DL-based positioning methods.

Rapporteur’s comments

Another issue worth to be discussed is that, some company contributions mentioned to specify configuration of UE locations measurements in RRC_INACTIVE state, for example, InterDigital proposed that different measurement report may be needed, i.e., support sending of measurement reports/location estimates which are aligned with the transmission of RNA updates. Based on the above summary, we have the following proposal for discussion:

(Medium priority)Proposal 6: Further study on whether to associate UE location measurement report with RNA update.

  - Segmentation of LPP message

InterDigital Proposal 4: Support segmentation and transmission of segmented LPP messages (i.e. with suitable identifiers) over multiple SDT subsequent transmission occasions. Proposal 5: Support LMF providing the UE with segmentation configuration information consisting of indications (e.g. IDs, flag, end-marker) and sequence numbers to use when segmenting and transmitting the segmented LPP messages. Lenovo Proposal 3: LMF can configure UE segmentation of the measurement report for large measurement report size exceeding the data volume threshold size to support SDT transmissions in RRC_INACTIVE state based on a set of criteria. FFS threshold and segmentation criteria for the measurement report and whether to capture these aspects in Stage 2 specifications. This can also be applicable to other UL LCS messages. Samsung Proposal 1: There is no need to specify the BSR operation to accommodate the LPP segmentation for LPP uplink transmission in inactive state.

Rapporteur’s comments

In RRC_INACTIVE state, if UE is in an ongoing SDT, both DL and UL transmission are restricted by data volume threshold of SDT. That is to say, if data size of the LPP message is larger than the threshold, the segmentation may be needed. Some company thinks the segmentation of LPP message should be indicated to perform, rather than implementation. InterDigital and Lenovo proposed that network should set criteria for segmentation, and network should configure the criteria to UE for guidance of segmentation.

Based on the summary, we have the following proposal for discussion:

(Medium priority)Proposal 7: Further study to provide segmentation configuration information from LMF to UE. The segmentation configuration information includes:

- the segmentation criteria

- indications (e.g. IDs, flag, end-marker) and sequence numbers

Note: this does not necessarily mean to expose RRC state to LMF.

- Common aspects of UL and DL positioning

In this section, the following company contributions provided their view about common aspects of UL and DL positioning in RRC_INACTIVE:

| R2-2107092 | Discussion on positioning in RRC INACTIVE state | ZTE |
| --- | --- | --- |
| R2-2107142 | Discussion on Positioning for UEs in RRC_INACTIVE state | CATT |
| R2-2107149 | Considerations on positioning in RRC_INACTIVE mode | Fraunhofer IIS; Fraunhofer HHI |
| R2-2107643 | Enhancement of DL positioning in RRC_INACTIVE | vivo |
| R2-2107683 | Discussion on Positioning in RRC INACTIVE state | InterDigital, Inc. |
| R2-2107684 | Discussion on reporting of Positioning Information with SDT | InterDigital, Inc. |
| R2-2108068 | Considerations on positioning RRC Inactive | Sony |
| R2-2108128 | On Positioning in RRC_INACTIVE state | Lenovo, Motorola Mobility |
| R2-2108173 | Discussion on positioning for UEs in RRC Inactive | Xiaomi |
| R2-2108394 | Inactive mode Positioning | Ericsson |
| R2-2108764 | Considerations on Positioning in RRC_INACTIVE state | CMCC |

Based on the input, the summary of DL positioning in RRC_INACTIVE is divided into several aspects:

- Supported positioning type

  - This chapter discusses about which kind of positioning types should be supported in RRC_INACTIVE, including MO-LR, MT-LR and deferred MT-LR.

- Mobility and validity

  - This chapter focuses on the mobility and validity in RRC_INACTIVE for both DL and UL configurations. There is no proposal for discussion.

- Signalling configuration

  - This chapter discussed about general signalling configuration of DL and UL. There is no proposal for discussion.

  - Supported positioning type

Xiaomi Proposal 1: Both MO-LR, MT-LR and deferred MT-LR can be supported for RRC inactive UE. vivo Proposal 1: LS to SA2 to confirm whether positioning in RRC_INACTIVE is restrict to specific location type (e.g. low power periodic and triggered 5GC-MT-LR). ZTE Proposal 4: Support all location service types from RAN2 perspective including MT-LR, MO-LR, NI-LR and deferred MT-LR.

Rapporteur’s comments

Xiaomi and ZTE proposed to support multiple kinds of positioning type, including MO-LR, MT-LR, deferred MT-LR and NI-LR. vivo also proposed to ask SA2 to determine the supported positioning type. In RAN2#114e, we had reached an agreement that:

Agreement: Any uplink LCS or LPP message can be transported in RRC_INACTIVE from RAN2 perspective. Follow Rel-17 SDT framework for INACTIVE UL and DL positioning: If the UE initiated data transmission using UL SDT, the network can send DL LCS, LPP message and RRC message (e.g. to configure SRS (TBD on what message is used), if UL positioning supported) to the UE. Otherwise, if UE did not initiate UL SDT, rely on legacy operation, i.e. the network shall transition the UE to RRC_CONNECTED, e.g. based on RAN paging.

From rapporteur’s point of view, since MO-LR and deferred MT-LR are both triggered by UE, so they may work at RRC_INACTIVE with UL SDT based on the agreement above. For MT-LR, it may have concern that this is the network triggered type, which should be based on RRC_CONNECTED. Based on the summary, we have the following proposal for discussion:

(High priority)Proposal 8: Support MO-LR, MT-LR and deferred MT-LR for RRC_INACTIVE state.

  - Mobility and validity

InterDigital Proposal 8: For UE-based positioning, support transmission of multiple buffered location estimates determined in RRC INACTIVE after transitioning to RRC CONNECTED. InterDigital Proposal 12: Support mechanism for ensuring usage of PRS/SRS configurations in INACTIVE that are valid across different cells/gNBs for making DL-PRS measurements and/or SRS transmission during mobility. Lenovo Proposal 6: Support new additional positioning-specific cell reselection indication for UEs performing RRC_INACTIVE positioning. CMCC Proposal 1: Introduce validity area or valid timer for the positioning related configuration.

Rapporteur’s comments

Considering the mobility that UE may suffer during the RRC_INACTIVE state, some pre-configured signaling or resources in RRC_CONNECTED state may not be timely appropriate, such as assistance data(mentioned in section 2.1/2.2), CG resources, SRS configuration, etc. Therefore, InterDigital suggested to support mechanism for ensuring usage of PRS/SRS configurations that are valid across different cells/gNBs. InterDigital proposed that support to configure CGs that are valid across multiple cells, and UE reports multiple buffered location estimate results in RRC_INACTIVE state to gNB when the UE is transitioning to RRC_CONNECTED state. Lenovo proposed to add new additional positioning-specific cell re-selection indication. From rapporteur’s point of view, since this section is a general description, no proposal will be generate in this section. Please refer to 2.3 for DL configuration and 4.2 for UL configuration, respectively.

  - Signalling configuration

Sony Proposal 5: The positioning configuration for positioning in RRC inactive is obtained when the UE is still in RRC-Connected mode. Fraunhofer Proposal 2: UE shall receive several configurations during the RRC_CONNECTED state, each with a validity scope and the UE shall be configured to determine the applicable configuration corresponding to its coarse location. FFS: Whether the configurations are full configurations or partial configurations (supplemented e.g. by suspendConfig or SDT). FFS: Delivery mechanism (e.g. broadcast, SDT, unicast) FFS: Trigger mechanism

Rapporteur’s comments

This section is a general description including UL and DL configurations in RRC_INACTIVE state. For specific view please find section 2.3 for DL configuration and section 4.2 for UL configuration, respectively. So this is not going to generate a proposal for discussion.

- UL/UL+DL Positioning

In this section, the following company contributions provided their view about UL/UL+DL positioning in RRC_INACTIVE:

| R2-2107092 | Discussion on positioning in RRC INACTIVE state | ZTE |
| --- | --- | --- |
| R2-2107142 | Discussion on Positioning for UEs in RRC_INACTIVE state | CATT |
| R2-2107149 | Considerations on positioning in RRC_INACTIVE mode | Fraunhofer IIS; Fraunhofer HHI |
| R2-2107644 | Configuration of UL positioning in RRC_INACTIVE | vivo |
| R2-2107671 | Support of Positioning in RRC_INACTIVE | Intel Corporation |
| R2-2107683 | Discussion on Positioning in RRC INACTIVE state | InterDigital, Inc. |
| R2-2107830 | Discussion on UL Positioning methods in RRC_INACTIVE state | OPPO |
| R2-2108068 | Considerations on positioning RRC Inactive | Sony |
| R2-2108128 | On Positioning in RRC_INACTIVE state | Lenovo, Motorola Mobility |
| R2-2108764 | Considerations on Positioning in RRC_INACTIVE state | CMCC |
| R2-2108605 | Way-forward for INACTIVE Positioning | Huawei, China Unicom, China Telecom, Futurewei, HiSilicon, Intel Corporation, Interdigital, Spreadtrum Communications, VIVO, Xiaomi, ZTE Corporation |

Based on the input, the summary of DL positioning in RRC_INACTIVE is divided into several aspects:

- UE capability of SRS transmission

  - This chapter discussed about UE capability of SRS transmission(aperodic, semi-persistent and periodic SRS), both at RRC_INACTIVE with ongoing SDT and RRC_INACTIVE without ongoing SDT.

- SRS configuration

  - This chapter discussed about where to configure SRS used in RRC_INACTIVE, and whether different SRS parameters are needed to be configured.

- UE request SRS configuration

  - This chapter focuses on the request that UE sends to the network, in order to ask for suitable SRS configuration in RRC_INACTIVE.

  - UE capability of SRS transmission

Lenovo Proposal 7: RAN2 to confirm if the capability information among UEs performing RRC_CONNECTED or RRC_INACTIVE positioning will be different and whether special RRC_INACTIVE capability indication is necessary. ZTE Proposal 3: If UL/DL+UL positioning methods are supported in RRC INACTIVE, it should be clarified whether SRS transmission can be allowed outside SDT active period. CATT Proposal 6: When UL positioning in RRC_INACTIVE is triggered while the UE has valid UL pos-RS configuration, the UE in RRC_INACTIVE transmits UL pos-RS signal. Otherwise, the UE in RRC_INACTIVE sends request for UL pos-RS configuration. Intel Proposal 4: The UE can transmit SRS (aperiodic, semi-persistent, periodic SRS) in RRC_INACTIVE if configured by the network. Proposal 5: Follow Rel-17 SDT framework for INACTIVE UL and DL positioning: If the UE initiated data transmission using UL SDT, the network can send SRS activation command (MAC CE, DCI), if UL positioning supported) to the UE. Otherwise, if UE did not initiate UL SDT, rely on legacy operation, i.e. the network shall transition the UE to RRC_CONNECTED, e.g. based on RAN paging. OPPO Proposal 3: Only periodic SRS is supported for positioning in RRC_INACTIVE.

Rapporteur’s comments

For UL-TDOA/UL-AoA/multi-RTT positioning method, UL SRS should be sent from UE to gNB to perform measurements. However in RRC_INACTIVE state, it is not clear whether UE can transmit SRS like in RRC_CONNECTED state. Lenovo and ZTE think this issue should be clarified, while CATT, Intel and OPPO proposed that at least periodic SRS can be transmitted in RRC_INACTIVE state. Since semi-persistent and aperiodic SRS need to be triggered by MAC-CE or DCI, whether and how to support these kind of SRS may need further discussion. ZTE further mentioned that it should be discussed separately for RRC_INACTIVE state without SDT and RRC_INACTIVE state with ongoing SDT.

Based on the above analysis, we have the following proposal for discussion:

(High priority)Proposal 9: Support at least periodic SRS in RRC_INACTIVE state.

FFS: whether it should be discussed separately for RRC_INACTIVE state without SDT and RRC_INACTIVE state with ongoing SDT

FFS: whether and how to support semi-persistent and aperiodic SRS in RRC_INACTIVE state

  - SRS configuration

CATT Proposal 4: Support the gNB sends UL pos-RS configuration within RRC Release message to the UE for UL positioning in RRC_INACTIVE. Proposal 5: The received UL pos-RS configuration in RRC Release message is valid only in the cell which releases the UE into RRC_INACTIVE. InterDigital Proposal 6: Support the use of pre-configured SRS configuration received by UE during RRC CONNECTED for SRS transmission when in INACTIVE. Proposal 7: Support transmission of SRS configuration via RRCRelease message (e.g. with SuspendConfig) or indication to UE for initiating SRS transmission when in INACTIVE. Proposal 13: Support mechanism for ensuring timing alignment maintenance in INACTIVE when transmitting SRS to different TRPs/gNBs during mobility. OPPO Proposal 1: SRS can be configured in RRC_CONNECTED or RRC_INACTIVE state. Proposal 2: Similar as in RRC_CONNECTED mode, SRS is configured by RRC signalling. Proposal 4: To support UL positioning in RRC_INACTIVE, reuse SDT TA timer for TA validation. Proposal 5: To support UL positioning in RRC_INACTIVE, reuse RSRP change based solution for TA validation. Proposal 6: The SRS configuration is considered as invalid if TA is not valid. SONY proposal 3: Specify configuration for UE transmission of SRS for UL positioning during RRC Inactive. vivo Proposal 2: SRS configuration for positioning received in RRC_CONNECTED can be used in RRC_INACTIVE UL positioning. Proposal 3: The UE shall regard the SRS configuration for positioning as valid within a specific area and period. Proposal 4: The SRS configuration and the validity criteria can be delivered in RRCRelease message. ZTE Proposal 2: Support SRS configuration carried by: - SDT DL RRC message - Message B or 4 can be considered in the case when 2 or 4 step RACH based access is chosen for SDT - RRCRelease with SuspendConfig - SRS configuration in RRC_CONNECTED Fraunhofer Proposal 3: RAN2 shall send an LS to RAN1 to consider the following issues regarding transmission of SRS in RRC_INACTIVE state: - Power control - Determining timing advance - Triggering mechanism for activating and deactivating SRS Intel Proposal 6: The issues on how transfer UL reference signalling, e.g. power control, TA, etc, should be resolved by the same solution on UL data transmission via CG-SDT, and therefore should not be discussed in positioning WI. CMCC Proposal 3: For UL positioning, positioning system information is used to deliver the SRS configuration in RRC_INACTIVE. FFS indication for RRC_INACTIVE. Proposal 4: RAN2 is kindly asked to confirm that one or multiple sets of the SRS or DL PRS configuration can be pre-configured to the RRC_INACTIVE UE. Huawei: Proposal 2.1: SRS configuration for UL positioning in RRC_INACTIVE is carried in RRCRelease message with suspendConfig. Proposal 2.2: The SRS configuration is released when the UE sends RRCResumeRequest to an gNB other than the gNB where it is released to RRC_INACTIVE state. Proposal 2.3: TA configuration is included in RRCRelease with suspendConfig for UL positioning in RRC_INACTIVE.

Rapporteur’s comments

This section focuses on how to configure SRS used in RRC_INACTIVE, and whether some SRS parameter configuration need to be reconsidered or not. Several company contributions supported to put SRS configuration into RRCRelease message. some company contributions proposed to reuse SRS configuration in RRC_CONNECTED. Fraunhofer and Intel also pointed out that RAN2 should consider some SRS parameters determination, such as power control and TA. From rapporteur’s point of view, all possible ways of configuring SRS for RRC_INACTIVE can be further discussed. Whether Power control and TA should be determined by RAN2 or other WG, e,g. RAN1, can be further discussed.

Based on the summary, we have the following proposal for discussion:

(Low priority)Proposal 10:  Support SRS configuration carried by:

- SDT DL RRC message

- Message B or 4 can be considered in the case when 2 or 4 step RACH based access is chosen for SDT

- RRCRelease with SuspendConfig

- SRS configuration in RRC_CONNECTED

- positioning system information, i.e. posSIB

- FFS: whether power control and TA should be discussed by RAN2

  - UE request SRS configuration

CATT Proposal 7: RAN2 discuss which option is adopted for the UE in RRC_INACTIVE to request UL pos-RS configuration. - Option 1: The UE in RRC_INACTIVE sends request for UL pos-RS configuration via RRC message. - Option 2: The UE in RRC_INACTIVE sends request for UL pos-RS configuration via LPP message. vivo Proposal 5: UE can initiate update request of SRS configuration for positioning in RRC_INACTIVE.

Rapporteur’s comments

CATT and vivo proposed that UE can request SRS configuration in RRC_INACTIVE state. Based on the summary, we have the following proposal for discussion:

(Low priority)Proposal 11: Further study UE to update request of SRS configuration for positioning in RRC_INACTIVE state.

- Option 1: the request is sent via RRC message

- Option 2: the request is sent via LPP message

- Way-forward of stage 2 modification

In this section, we summarized the way-forward solution for stage 2 modification in RRC_INACTIVE:

| R2-2108605 | Way-forward for INACTIVE Positioning | Huawei, China Unicom, China Telecom, Futurewei, HiSilicon, Intel Corporation, Interdigital, Spreadtrum Communications, VIVO, Xiaomi, ZTE Corporation |
| --- | --- | --- |

Huawei: Figure 1. section 3.1.1-modified stage 2 UL procedure for RRC_INACTIVE positioning Figure 2. section 3.1.2-modified stage 2 DL+UL procedure for RRC_INACTIVE positioning Proposal 1: RAN2 to confirm the consolidated solution in Section 3.1.1 and 3.1.2 for UL and UL+DL positioning as the baseline for positioning in RRC_INACTIVE state. FFS how to capture into the stage2 spec.

Rapporteur’s comments

Huawei has provided a baseline as a way-forward, which is different from the stage 2 modification in post email discussion. Further details can be found in R2-2108605. based on the summary, we have the following proposal for discussion:

(Medium priority)Proposal 12: RAN2 to confirm the consolidated solution in Section 3.1.1 and 3.1.2 for UL and UL+DL positioning as the baseline for positioning in RRC_INACTIVE state. FFS how to capture into the stage2 spec.

- Others

This section collects some single proposals that are proposed by single company, which are not much relevant to the main proposals. Since the meeting time is limited and issues in this section are treated as low priority, for the meeting progress, this section is not going to generate proposals for discussion.

Ericsson Proposal 3: Upper layers can provide information to the lower layer to decide the RRC mode of transmission for delivering positioning data. Proposal 4: Send a LS to SA2 to allow repetition of LCS Periodic Trigger. Lenovo Proposal 5: RAN2 to consider updating the UE of a change in the posSIB via a paging DCI in cases where the posSIB changes too frequently or the low latency notification of a posSIB change. Sony Proposal 4: Support initiation of DL and UL based positioning triggered via paging the UE to either start and/or for configuration of respective Positioning in RRC inactive. Huawei Proposal 2.4: When cell reselection is performed when UE is configured with SRS in RRC_INACTIVE, the TAT for SRS is considered as expired. Proposal 2.5: gNB configures upper and lower threshold for RSRP change for the TA validation of SRS transmission.

6 Conclusion

In this section, we summary all the proposals for discussion:

DL positioning:

(High priority)Proposal 1: Support all the RAT independent positioning methods in RRC_INACTIVE state.

(High priority)Proposal 2: Support to deliver the positioning assistance data to UE via the following options:

- Option 1: The existing deferred MT-LR procedure

- Option 2: positioning system information, i.e. posSIB

  - FFS: whether to introduce a unicast tag in posSIB

- Option 3: pre-configured assistance data when UE in RRC_CONNECTED state

- Option 4: ongoing SDT procedure

(High priority)Proposal 3: For the alignment between the positioning measurement and the SDT configuration, consider the following alternatives:

- Alt 1: Support LMF to inform gNB the estimate data size of measurement report

- Alt 2: Support LMF to inform gNB the measurement periodicity

- Alt 3: Support LMF to inform gNB the positioning requirements

- Alt 4: Support gNB to inform LMF the SDT data volume threshold

- Alt 5: Support differential measurement report

- Alt 6: No optimization should be introduced for positioning measurement report

FFS: Whether to discuss this issue in SDT agenda or positioning agenda.

(Medium priority)Proposal 4: Further study the assistance information transmitted from UE to gNB. The assistance information may include following aspects:

- type of reporting (e.g. periodic, aperiodic)

- payload size of LPP message (e.g. measurement report/location estimates)

- start timing

- measurement duration

- reporting periodicity

(Medium priority)Proposal 5: For the PRS configuration used for RRC_INACTIVE state, consider the following alternatives:

- Alt 1: Configure RNA information in the PRS configuration

- Alt 2: Configure validity conditions (e.g. time validity, area validity) in the PRS configuration

- Alt 3: No change should be introduced in PRS configuration

Note: This does not necessarily mean to expose RRC state to LMF.

(Medium priority)Proposal 6: Further study on whether to associate UE location measurement report with RNA update.

(Medium priority)Proposal 7: Further study to provide segmentation configuration information from LMF to UE. The segmentation configuration information includes:

- the segmentation criteria

- indications (e.g. IDs, flag, end-marker) and sequence numbers

Note: this does not necessarily mean to expose RRC state to LMF.

Common aspects of UL and DL positioning:

(High priority)Proposal 8: Support MO-LR, MT-LR and deferred MT-LR for RRC_INACTIVE state.

UL/DL+UL positioning:

(High priority)Proposal 9: Support at least periodic SRS in RRC_INACTIVE state.

FFS: whether it should be discussed separately for RRC_INACTIVE state without SDT and RRC_INACTIVE state with ongoing SDT

FFS: whether and how to support semi-persistent and aperiodic SRS in RRC_INACTIVE state

(Low priority)Proposal 10:  Support SRS configuration carried by:

- SDT DL RRC message

- Message B or 4 can be considered in the case when 2 or 4 step RACH based access is chosen for SDT

- RRCRelease with SuspendConfig

- SRS configuration in RRC_CONNECTED

- positioning system information, i.e. posSIB

- FFS: whether power control and TA should be discussed by RAN2

(Low priority)Proposal 11: Further study UE to update request of SRS configuration for positioning in RRC_INACTIVE state.

- Option 1: the request is sent via RRC message

- Option 2: the request is sent via LPP message

Way-forward of stage 2 modification:

(Medium priority)Proposal 12: RAN2 to confirm the consolidated solution in Section 3.1.1 and 3.1.2 for UL and UL+DL positioning as the baseline for positioning in RRC_INACTIVE state. FFS how to capture into the stage2 spec.

7 Reference

- RP-210903, Revised WID on NR Positioning Enhancements, Intel Corporation, CATT.

- R2-2104921, Clarification on work scope of Rel-17 positioning enhancement, Intel Corporation.