3GPP TSG-RAN WG2#115-e	R2-2108965

Electronic meeting, 16th August – 27th August 2021

Agenda Item:	8.13.3.2

Source:	Ericsson

Title:	Report of [Offline-872][SONMDT] Logged MDT enhancements (Ericsson)

Document for:	Discussion, Decision

# 1	Introduction

This document provides the outcome of the following offline discussion conducated during RAN2#115 meeting:

[AT115e][872][SON/MDT] Logged MDT enhancements (Ericsson)

Scope: Focus on the set of proposals which are highlighted as such for discussions and potential agreements in this meeting in R2-2109016

Intended outcome: Report with Agreements

Please provide your comments by Tuesday, August 24th 11:00 UTC so that there is enough time to summarize and verify the summary which has the deadline of Wednesday, August 25th 11:00 UTC.

# 2	Contact Information

To make it easier to find the correct contact delegate in each company for potential follow-up questions, the rapporteur encourages the delegates who provide input to provide their contact information in this table:

| Company | Contact: Name (E-mail) |
| --- | --- |
| Ericsson (Rapporteur) | Pradeepa Ramachandra (pradeepa.ramachandra@ericsson.com) |
| Qualcomm | Rajeev Kumar (rkum@qti.qualcomm.com) |
| vivo | Ming Wen (ming.wen@vivo.com) |
| Apple | Sasha Sirotkin <ssirotkin@gmail.com> |
| ZTE | Zhihong Qiu (qiu.zhihong@zte.com.cn) |
| Huawei, HiSilicon | Jun Chen (jun.chen@huawei.com) |
| LGE | HyunJung Choe (stella.choe@lge.com) |
| Nokia, Nokia Shanghai Bell | malgorzata.tomala@nokia.com |

# 3	Discussion

## 3.1	On demand SI related

### 3.1.1	Scenarios

It has already been agreed that the failed on-demand SI request related measurements shall be reported by the UE in RAN2#114 meeting.

UE records intended SIBs for failed on-Demand SI request. FFS the successful case.

It has also been agreed in RAN2#113bis meeting that the UE shall generate the report for both msg-1 based and msg-3 based on demand SI request.

Both Msg1-based and Msg3-based SI request related information are supported.

However, the following scenarios associated to on-demand SI request needs to be discussed. The supporting companies (direct proposals or via indirect proposals) are indicated in the round brackets.

- Successful on-demand SI request ([4], [5], [6], [8], [13])

- On-demand positioning SI/SIB request ([4], [13])

- On-demand SI request in connected mode ([5])

Question-1: Which of the following scenarios should be included as part of the logging of measurements associated to on-demand SI request?

- Upon successful on-demand SI request

- Upon on-demand positioning SI/SIB request

- Upon on-demand SI request in connected mode

| Company name | Agreeable scenarios? None, 1, 2, 3, All | Comments |
| --- | --- | --- |
| Qualcomm | None | 1) SI and SIB mapping cannot be optimized based on each single UE. 2) can be studied when we consider SON/MDT enhancements for the sideline. 3) Network already knows the requested SIB in the connected state. Therefore, we don’t need this |
| vivo | 1/2 | 1) is needed, shares the same logic with failed case; 2) can be further discussed. We are not convinced that 3) is needed. According to TS 38.300 Clause 7.3.2, the intended SIB(s) that UE requested can be explicitly known by NW, therefore no need to send such information via any other report: ‘For UEs in RRC_CONNECTED, a request for Other SI may be sent to the network in a dedicated manner (i.e., via UL-DCCH) and the granularity of the request is one SIB. The gNB may respond with an RRCReconfiguration including the requested SIB(s). It is a network choice to decide which requested SIBs are delivered in a dedicated or broadcasted manner.’ DedicatedSIBRequest-r16-IEs ::= SEQUENCE { onDemandSIB-RequestList-r16 SEQUENCE { requestedSIB-List-r16 SEQUENCE (SIZE (1..maxOnDemandSIB-r16)) OF SIB-ReqInfo-r16 OPTIONAL, requestedPosSIB-List-r16 SEQUENCE (SIZE (1..maxOnDemandPosSIB-r16)) OF PosSIB-ReqInfo-r16 OPTIONAL } OPTIONAL, SIB-ReqInfo-r16 ::= ENUMERATED { sib12, sib13, sib14, spare5, spare4, spare3, spare2, spare1 } |
| Ericsson | 1,2 | We believe 1) is a must as the network needs to collect information from both successful and failed on demand SI requests. The successful on demand SI request also provide information related to the allocation of RA resources were optimal or not. For example, if RA premable-x is allocated to both SIB-A and SIB-B, but UE wants only SIB-A, sending preamble-x in msg1 would result in transmission of both SIB-A and SIB-B. However, the UE might be interested only in SIB-A and thus the network is wasting the transmission of SIB-B and network never realizes the inefficient mapping of msg-1 and SI/SIB. Therefore, we support successful on demand SI related info to be logged by the UE. As positioning SIBs are amongst the most likely SI to be sent on demand, it is fair to include positioning SI as part of the on-demand SI reports to give more info to the operators to understnad its efficiency. We believe, connected mode on-demand SI request can be looked at in Rel-18. |
| Apple | None | We are not convinced this provides a useful information. |
| ZTE | 1, 2 can be considered in Rel-18. | As explained in our paper,similar to failed case, successful cases also needs to logged to allow NW to obtain complete information for optimization. Positioning SIB can be considered in Rel-18. |
| OPPO | None |  |
| Sharp | None | 1) is not needed, NW can get the information as the RA procedure is successful. 2) maybe yes, but prefer not to consider in this release. 3) is not needed, NW can get the information from the DedicatedSIBRequest message. |
| Huawei, HiSilicon | 1 | 1): We see some benefits of logging on-demand SI information for successful case, because it could help optimize SI and SIB mapping. The mapping is set by the network and the UE may only request some specific SIBs, so the network may adjust the mapping based on the necessary on-demand information. |
| CATT | 1,3 | For 1), the main purpose of the on-demand SI request information report is to help the network side to acquire the UE requirement of SIBs, regardless of whether the request is successful or failure. Both successful and failed on-demand SI request should be considered. For 2), at this stage, we prefer to focus on the left issues, the positioning SIBs can be considered later. For 3), we think the network can only know the SIBs information which have been requested by UE not the UE actual intends to request. The network should get full picture of the on-demand SI request information to better optimize the broadcast type and on-demand SI request related system resources. Of cource, the connected on-demand SI request can be discussed in Rel-18 due to the current time limit. |
| LGE | None | Similar view with Qualcomm. It is not clear how much the network can optimize radio resources using these information. |
| Nokia | None | We think successful case would be overwhelming case for optimization with doubtful benefits (vs complexity) |

Rapporteur Summary:

The support of companies for different scenarios associated to on-demand SI request is summarized as follows.

None 6/11 companies

Option-1 (Upon successful on-demand SI request): 5/11 companies

Option-2 (Upon on-demand positioning SI/SIB request): 2/11 companies + 1 company proposing to include it in Rel-18

Option-3 (Upon on-demand SI request in connected mode): 1/11 company

Based on the above, it seems like the scenario of logging of measurements associated to on-demand SI request upon on-demand positioning SI/SIB request and upon on-demand SI request in connected mode are not pursued in Rel-17.

The scenario of logging of measurements associated to on-demand SI request upon on-demand positioning SI/SIB request and upon on-demand SI request in connected mode are not pursued in Rel-17.

Half of the companies (5/11) have shown support for including the of measurements associated to successful on demand SI related procedure. Some companies who are not supporting this scenario have indicated that the network can use the RA report based measurements to derive the information required for successful on-demand SI procedure. Based on this the rapporteur proposes to postpone the discussion associated to successful on demand SI procedure to the next meeting but also encourages the supporting companies to see if they can convince the opposing companies.

The scenario of logging of measurements associated to successful on-demand SI request procedure is postponed to the next RAN2 meeting.

### 3.1.2	Report contents

The following report contents related to on demand SI request has been agreed in previous RAN2 meetings.

- Agreements in RAN2#114:

Include information to differentiate between Msg1-based or Msg3-based on-demand SI request.

UE records intended SIBs for failed on-Demand SI request.

Agreements in RAN2#113bis:

UE reports the SIBs that UE actually intends to request.

Both Msg1-based and Msg3-based SI request related information are supported.

There are further proposals from companies on the following measurements to be included in the on-demand SI related report. The supporting companies (direct proposals or via indirect proposals) are indicated in the round brackets.

- The number of times each SIB was intended to be requested by the UE ([5])

- Failed or successful on-demand SI indicator ([5])

- Failure type - failure at RA procedure or failure at acquiring SI messages ([6])

- The information of the beams used to acquire the requested SI messages ([6])

- The time between consecutive SI requests ([6])

- The location information at the time of performing the SI request ([6])

- an indicator to indicate if SI request was performed over either NUL or SUL ([13])

As the outcome of Qestion-1 impacts 2) in the above list, it is separated into a different question.

Question-2: Which of the following report contents associated to on-demand SI request is agreeable?

- The number of times each SIB was intended to be requested by the UE

- Failure type (failure at RA procedure or failure at acquiring SI messages)

- The beam identities used to acquire the requested SI messages

- The time between consecutive SI requests

- The location information at the time of performing the SI request

- An indicator to indicate if the SI request was performed over NUL or SUL

| Company name | Agreeable report contents? None, 1, 2, 3, 4, 5, 6, All | Comments |
| --- | --- | --- |
| Qualcomm | 3 (can be determined using existing fields in RA-report) | 1, 2, 4 can be determined by the network, UE does not have to report these. 6 can be determined by existing BWP information in the RACH report. We don’t need option 5, cell information should be sufficient. |
| vivo | 3 may be useful | - 1 should be clarified, not clear why there would be ‘the number of times each SIB’ that UE intended to request. Since each entry only includes a one-shot request for SIB, how could the number be counted/ why should this be sent to NW (as it always equals to 1)? - Not sure for 2. What matters in the failed case is to make NW aware of the intended SIBs UE requested, regardless of whether the SIBs are successfully acquired. - 4 is not needed, don’t see how the consecutive SI requests are interrelated with each other. NW should adjust the broadcasting SIBs according to a large scale of UEs instead of a single UE pattern. - 5 seems not needed, cell ID is sufficient, should we include detailed location information in this case? - 6 can be derived by RACH report. |
| Ericsson | 2,3,4,5,6 | In the comments by Qualcomm they indicate that 2,,3, 4, 6 can be determined by the network. We are okay if they can be implicitly derived but we should agree that these information should be either explcitly or implicitly needed in the on-demand SI report. On (2), it is important to know what needs to be optimized, is it the RA coverage (UL) or the SIB coverage (DL). Therefore, it is necessary to report the failure type. On (4), this parameter provides information about whether two different Sis are required by the same UE together or not i.e., if an application in the UE wants to have SIB-A and SIB-B at the same time and the network had assigned SIB-A to one msg-1 and SIB-B to msg-2, then these two requests come back to back in time but the network cannot recognize that it is the same UE that requested these SIBs. Therefore, to make sure that SIB-A and SIB—B can be assigned to a single msg-1, it is necessary to know the time between their requests. On (5), we believe location info is one of the most useful information in SON reports for the operators to identify where a particular service is needed. We agree that (6) can be derived imp[licitly but it is important to agree that this information is needed. Whether to include ir implicitly or explicitly can be FFS. |
| Apple | 3 | 3 is sufficient |
| ZTE | 3 | 3 is useful for configuring the RA resource. |
| OPPO | 3 | 3 is unknown by the network. |
| Sharp | 3 | 3 seems related to the RA parameter setting, which is not known by the NW. For others, the benefits are not clear. |
| Huawei, HiSilicon | 3, 5 | For option 5, similar as location reporting for SON functionalities, we think the location info for on-demand SI logging can let the network accurately identify where the problem happened. |
| CATT | 1,3 | For 1), we consider the UE could request many times for a SIB or SI when the SIB or SI request failed. The UE can record and report the request times for the SIB to network to help optimization. 3) may be useful. |
| LGE | Maybe 3 | But if the beam information can be implicitly derived, this may not be needed in the on-demand SI report. |
| Nokia | None or 2 | It depends on the report design, if it is reusing RAreport or LoggedMDT 2 maybe sufficient, while 3 can be deduced from the legacy fields. Introducing another set of beams (measurements?) may double and impact report size significantly |

Rapporteur Summary:

The support of companies for different on-demand SI procedure related report contents is summarized as follows.

- The number of times each SIB was intended to be requested by the UE (1/11)

- Failure type (failure at RA procedure or failure at acquiring SI messages) (2/11)

- The beam identities used to acquire the requested SI messages (10/11)

- The time between consecutive SI requests (1/11)

- The location information at the time of performing the SI request (2/11)

- An indicator to indicate if the SI request was performed over NUL or SUL (1/11)

Based on the above, it seems like all the companies are fine with including the beam identifiers used to acquire the SI messages.

The UE includes the beam identifiers used to acquire the SI message(s) in the on-demand SI procedure related report.

- FFS: How to capture this information

As there is no significant support for any of the other measurements, the rapporteur proposes not to pursue them.

The following measurements aer not included in the on-demand SI related report.

      - The number of times each SIB was intended to be requested by the UE

      - Failure type (failure at RA procedure or failure at acquiring SI messages)

      - The time between consecutive SI requests

      - The location information at the time of performing the SI request

      - An indicator to indicate if the SI request was performed over NUL or SUL

If the option-1 in question-1 is agreed and if the same report (associated signaling design discussed in 3.1.3) is used to include both successful and failed on-demand SI procedure, then there are proposals from companies to include an indication in the report that indicates whether the on-demand SI request was successful or not.

Question-3: Do you agree to include an indicator in the on-demand SI request related report indicating whether the on-demand SI request was successful or not?

Note: This questions assumes option-1 in question-1 is agreeable and a single report (associated signaling design discussed in 3.1.3) is used to include both successful and failed on-demand SI procedure related measurements.

| Company name | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Depends | It depends upon what signaling method we use. For example, if option 3 is adopted then we don’t need an indicator. IF option1 or 2 is adopted then we may need an indicator. |
| vivo | Depends | Similar view with QC. |
| Ericsson | Agree |  |
| Apple | Depends | Agree with QC and vivo |
| Sharp | Depents | Agree with Qualcomm. |
| Huawei, HiSilicon | Agree |  |
| CATT | Agree |  |
| LGE | Depends | Agree with QC |
| Nokia | No | Agree with QC, depending on the selected form of signalling the indication may be useful (added) or not (e.g. if separate report is defined) |

Rapporteur Summary:

All the companies agree that the information that whether the on-demand SI procedure was successful or not is needed. But the outcome of this question is dependent on the inclusion of the successful on-demand SI procedure scenario and as this is not agreeable yet in the Q1, the rapporteur proposes to postpone this topic until the next meeting.

Decision on inclusion of an indicator in the on-demand SI request related report indicating whether the on-demand SI request was successful or not is postponed to next RAN2 meeting.

### 3.1.3	Signaling design

During RAN2#114 meeting, companies ([4], [5], [6], [8], [11] and [13]) disussed the what signaling design could be used to include the on-demand SI related report. However, no agreement was made as there were divergent views. Companies have submitted contributions to indicate their views on this topic again in this meeting. As this has been discussed in the past, the rapporteur requests companies to indicate their preferred option(s), their acceptable option(s) and any option(s) that are not acceptable at all. This would help to progress this topic during the meeting.

Question-4: Which of the following option is to used to indicate the on-demand SI related report?

Option 1:   Extend Logged MDT

Option 2:   Extend RA report for both successful and failure on-demand SI request

Option 3:	Extend RA report for successful on-demand SI request and extend other report (RA report, CEF report, new report) for failure on-demand SI request

Option 4:   A separate and dedicated report

| Company name | Which is the preferred option? | Which option(s) is (are) acceptable | Which option(s) is (are) NOT acceptable | Comments |
| --- | --- | --- | --- | --- |
| Qualcomm | Option 2 | Option 1 | Option3 and Option 4 | We do not want to introduce a new report for this. We are okay with option 3 if the failed scenario is logged in the CEF report. |
| vivo | Option 2 | Option 3 | Option 1/4 | Prefer to decouple logged MDT with RACH mechanism. |
| Ericsson | Option-4 | Option-3, Option-2 | Option-1 |  |
| Apple | 2 | 3 | 4 |  |
| ZTE | Option 1/Option 4 | Option 2 | Option 3 | We prefer to includes complete information within one report, and both option1/4 support this with fully flexibility. Option 2 is also acceptable as RA resource optimization is alsp part of the RA-report use case. On-demand SI will not result in connection failure, and it is irrelevant to coverage issues, therefore we don’t consider CEF report as a proper report to place this kind of information. |
| OPPO | 3 | 1 | 2 | From LTE, the RACH report is for logging the information related to the successful RACH procedure. Such principle should be followed. |
| Sharp | Option 2 | Option 1 | Option 3/4 | If SI information for successful case is agreed, we prefer to include information for successful case and failure case in a single report. For option 3, share ZTE’s view that on-demand SI is not relevant to CEF report which is for coverage issues. |
| Huawei, HiSilicon | Option 3 | Option 2 | Option 1 Option 4 |  |
| CATT | Option-1 | Option-4 | Option 2/3 | Considering that there are some mandatory fields in logged MDT report which is not necessary for on-demand SI report, we could accept the Option 4, i.e. introduce a new report for on-demand SI optimization to avoid some unnecessary fields report. |
| LGE | Option 2 | Option 3, 4 | Option 1 |  |
| Nokia | Option 2 | Option 1 | Option 3 | RAreport contains most of the useful information for On-Demand SI and would serve the purpose without too much signalling overhead |

Rapporteur Summary:

Answers from companies can be summarized as follows.

|  | Option-1 | Option-2 | Option-3 | Option-4 |
| --- | --- | --- | --- | --- |
| Preferred | 2/11 | 6/11 | 2/11 | 2/11 |
| Acceptable | 4/11 | 3/11 | 4/11 | 2/11 |
| Not acceptable | 4/11 | 2/11 | 5/11 | 5/11 |

Based on the above summary table, it is clear that option-2 is the least controversial and most preferred option amongst the different options. The only limitation expressed by one company for this option is that RA report is always used for only successful RA procedure and this principle should be kept. Based on the replies from companies, it seems like companies are okay to include the failed on-demand SI request procedure related information in the RA report itself. As we have been discussing this topic for few meetings now, the rapporteur proposes to agree on this option.

Extend RA report for both successful and failure on-demand SI request. FFS: Whether successful one-demand SI request related scenario is included or not is postponed to RAN2#116 meeting.

## 3.2	Signaling based logged MDT override protection

### 3.2.1	Scenarios

It has already been agreed in RAN2#114 meeting that the solution involving UE providing assistance has been agreed.

1	In order to avoid overwriting of signalling-based logged MDT, UE-assisted and network-based solution, which relying on network implementation through UE providing assistance, is introduced.

There are some associated scenarios based on the above agreement that has been brought up by companies in their respective contribution.

- Scenario-1: Signaling based Logged MDT is configured, but no results are available e.g. so far nothing stored, or all previously stored results retrieved ([2], [11])

- Scenario-2: Signaling based Logged MDT configuration is stopped (i.e. the expiry of T330), but UE still has un-retrieved results that would be discarded upon accepting a new configuration ([11] and [14])

- Scenario-3: Signaling based logged MDT is configured in LTE (NR), the UE comes to connected in NR (LTE). ([6], [14])

- Scenario-4: Signaling based logged MDT is configured, the UE comes to connected in a PLMN that is not in the plmn-IdentityList. ([6], [14])

Question-5: Should the signaling based logged MDT override protection is applicable in the following scenarios?

Scenario 1: Signaling based Logged MDT is configured, but no results are available e.g. so far nothing stored, or all previously stored results retrieved

Scenario 2: Signaling based Logged MDT configuration is stopped (i.e. the expiry of T330), but UE still has un-retrieved results that would be discarded upon accepting a new configuration.

Scenario 3:	Signaling based logged MDT is configured in LTE (NR), the UE comes to connected in NR (LTE)

Scenario 4: Signaling based logged MDT is configured, the UE comes to connected in a PLMN that is not in the plmn-IdentityList.

| Company name | Agreeable scenarios? None, 1, 2, 3, 4, All | Comments |
| --- | --- | --- |
| Qualcomm | scenario 1 and 2 | Inter-RAT protection should not be needed. During inter-RAT mobility, a different RAT should be allowed to override the S-based configuration. Similarly in scenario 4. |
| vivo | 1/2 |  |
| Ericsson | All | Regarding the comment from Qualcomm, this is not our understanding. A signaling based MDT should have the highest priority independent of the RAT |
| Apple | All | OK to consider all the options |
| ZTE | All |  |
| OPPO | 1 | Scenario 2 is not needed. If UE still has un-retrieved results, when the UE comes back to the RRC_Connected state, the UE will send the flag of the available of the logged measurement results (idleMeasAvailable) towards the network in RRCSetupComplete/RRCResumeComplete msg. Upon reception of such information, the network, if demands, should send RRC msg towards UE for retrieving the logged measurement results. Otherwise, the network will do nothing. |
| Sharp | All |  |
| Huawei, HiSilicon | All | OK to consider all scenarios |
| CATT | 1 and 2 |  |
| LGE | 1/2 |  |
| Nokia | 1 | Agree with OPPO |

Rapporteur Summary:

Support from companies can be summarized as follows.

Scenario-1: 11/11

Scenario-2: 9/11

Scenrario-3: 5/11

Scenario-4: 5/11

As scenario-1 (11/11) and scenario-2 (9/11) are supported by vast majority, they can be agreeable in this meeting. One company has expressed concern for scenario-2 but all other companies seem to agree that the UE should include an indication of signaling based logged MDT configuration so that the network can prioritize the fetching of these results.

Signaling based logged MDT override protection is applicable in the following scenarios:

- Signaling based Logged MDT is configured, but no results are available e.g. so far nothing stored, or all previously stored results retrieved

- Signaling based Logged MDT configuration is stopped (i.e. the expiry of T330), but UE still has un-retrieved results that would be discarded upon accepting a new configuration

Half of the companies (5/10) also support the inter-RAT and inter-PLMN related scenarios. As there is no clear majority in any direction, the rapporteur proposes to discuss this further.

The following scenarios associated to Signaling based logged MDT override protection are postponed to RAN2#116 meeting:

- Signaling based logged MDT is configured in LTE (NR), the UE comes to connected in NR (LTE)

- Signaling based logged MDT is configured, the UE comes to connected in a PLMN that is not in the plmn-IdentityList.

### 3.2.2	Signaling

During RAN2#113bis meeting, it has already been agreed that the LoggedMeasurementConfiguration sent by the network includes a flag indicating whether the logged MDT configuration is a signaling based MDT configuration or not.

5	Introduce the logged MDT type (i.e. the management based MDT or the signalling based MDT) in the logged MDT configuration.

The contributions to this meeting include discussions related to the how the UE indicates to the network about the availability of the signaling based logged MDT configuration or signaling based logged MDT report content.

Question-6: Which of the following UL RRC messages can carry the indicator (flag) indicating the availability of signaling based logged MDT configuration?

- RRCSetupComplete/RRCConnectionSetupComplete

- RRCResumeComplete/RRCConnectionResumeComplete

- RRCReestablishmentComplete/RRCConnectionReestablishmentComplete

- RRCReconfigurationComplete/RRCConnectionReconfigurationComplete

| Company name | Agreeable RRCxxComplete messages? 1, 2, 3, 4, All | Comments |
| --- | --- | --- |
| Qualcomm | We don’t need this indicator. A single indicator for questions 6 and 7 is sufficient. | I believe both indicators in questions 6 and 7 are not needed. A conditional indication using a single indicator is sufficient. For example, if signaling-based MDT is configured, UE can indicate whether T330 is running or not. In general, whether s-based is configured at the UE is known at the UE, as s-based PLMN identity list is shared during inter-PLMN, intra-PLMN, UE context retrieval, and others. |
| vivo | 1/2 | Upon RRC transition from IDLE/INACTIVE to CONNECTED state, UE can report the availability indicator to NW, and NW can be aware it is the signalling-based logged MDT report that exists at UE side. There is no need to notify NW every time after UE comes back to CONNECTED state. |
| Ericsson | 1 and 2 |  |
| Apple | 1 and 2 |  |
| ZTE | 1 and 2 |  |
| OPPO | 1 and 2 |  |
| Sharp | 1&2 | But agree with Qualcomm that this indicator may not be needed if something in Q7 is agreed. |
| Huawei, HiSilicon | All | For 3) and 4), if the UE performs RRC reestablishment or handover, it may select to another cell and T330 is still running. And then it is useful for the UE to send the flag to that cell, otherwise, the override issue may still happen. |
| CATT |  | Agree with Qualcomm, a single indicator for questions 6 and 7 is sufficient. |
| LGE | 1 and 2 |  |
| Nokia | 1,2 and 3 | No ned to update the NW upon each RRC connection change, but Reestablishment may happen due to RLF, upin which the indicator would quickly notify the NW |

Rapporteur Summary:

Based on the comments from the companies, 9/11 companies support the inclusion of an indicator to indicate the signaling based logged MDT configuration related availability in RRCSetupComplete / RRCConnectionSetupComplete and RRCResumeComplete / RRCConnectionResumeComplete. Couple of companies mention that this indication is not required if the T330 related flag is included. Based on this, rapporteur proposes to include the indications in RRCSetupComplete / RRCConnectionSetupComplete and RRCResumeComplete / RRCConnectionResumeComplete but whether this indication is implicit (T330 flag) or explicit can be an FFS.

Include an indicator to indicate the signaling based logged MDT configuration availability in RRCSetupComplete / RRCConnectionSetupComplete and RRCResumeComplete / RRCConnectionResumeComplete.

- FFS: Implicit (flag indicating T330 is running or not) vs explicit indication

### 3.2.3	Further assistance

Several companies have provided proposals regarding including indication regarding the status of the T330 timer to the network node with respect to the signaling based logged MDT configuration. The proposals can be broadly summarized as the following options.

- The UE indicates whether T330 timer is still running or not in the RRCxxComplete messages agreeable in Question-6.

- The UE indicates the remaining T330 timer value in the RRCxxComplete messages agreeable in Question-6.

- The UE indicates whether T330 timer is still running or not in the UEAssistanceInformation message.

- The UE indicates the remaining T330 timer value in the UEAssistanceInformation message.

Question-7: Which of the following information is to be reported by the UE?

- The remaining T330 timer value

- An indication (1-bit flag) as to whether T330 is running or not.

| Company name | Agreeable option? None, 1, 2, Both | Comments |
| --- | --- | --- |
| Qualcomm | Option 2 | The use case can be satisfied by a single indicator. Therefore, we don’t need 1. |
| vivo | Option 2 | Simpler to go for Opt2 so that NW knows when is the right timing to send another configuration. |
| Ericsson | Option-1 | Option-1 gives the most information to the network |
| Apple | Option 2 |  |
| ZTE | Option 1 |  |
| Sharp | Option 1 | Option 1 provides more information than option 2, which is useful for the NW. |
| Huawei, HiSilicon | Option 2 |  |
| CATT | Option-2 | Agree with Qualcomm, we prefer to a simple way. |
| LGE | Option 2 | Slightly prefer Option 2 but Option 1 is also acceptible. |
| Nokia | Option 2 |  |

Rapporteur Summary:

Answers to this question is summarized with question-8 answers.

Question-8: If the answer to previous question is not ‘None’ then which RRC message is used to carry the information in Question-7?

- The RRCxxComplete message(s) that were agreeable in Question-6

- The UEAssistanceInformation message.

| Company name | Agreeable option? None, 1, 2, Both | Comments |
| --- | --- | --- |
| Qualcomm | Option 1 | The indicator can be indicated in RRCComplete messages where we indicate the availability of the logged MDT. |
| vivo | Option 1 |  |
| Ericsson | Option-2 | We do not want to increase the RRCxxComplete message size as it seems like slowly we are adding more and more bits to the mandatory messages |
| Apple | Option 1 |  |
| ZTE | Option 1 |  |
| Sharp | Option 1 |  |
| Huawei, HiSilicon | Option 1 |  |
| CATT | Option 1 | The indicator can be indicated in all the RRCComplete messages along with the available indicator of logged MDT. |
| LGE | Option 1 |  |
| Nokia | Option 1 | However it should be combined condition T330 running and/or data unretrieved if Reestablishment is also considered |

Rapporteur Summary:

For question-7, option-1 is agreeable to 3/10 companies whereas option-2 is preferred by 7/10 companies. Based on this rapporteur proposes to go with the majority i.e., option-2.

For question-8, most companies (9/10) support the inclusion of the flag in the RRCxxComplete message(s) that were agreeable in Question-6.

Based on the above, the rapporteur proposes the following:

UE includes an indication regarding whether the T330 timer is running or not in RRCSetupComplete / RRCConnectionSetupComplete and RRCResumeComplete / RRCConnectionResumeComplete.

## 3.3	RAN3 LS related (R3-212824)

RAN3 has sent an LS to RAN2 with some questions.

RAN3 discussed the configuration of Area Scope of Neighbour Cells for logged MDT and the NR Frequency Band in the Area Scope of Neighbour Cells. RAN3 would like to check if there is alignment between TS 38.413 and TS 38.331.

- Area Scope of Neighbour Cells for logged MDT

The Area Scope of Neighbour Cells was introduced for signalling based logged MDT in Rel-16 as an IE that does not depend on the presence of the Area Scope of MDT, e.g. see TS38.413. However, RAN3 observed that the Area Scope of Neighbour Cells cannot be configured to the UE if the Area Scope of MDT is configured as PLMN wide at NGAP level. The reason is that in TS 38.331, the areaConfiguration-r16 is optional, and the interFreqTargetList-r16 is encoded inside the areaConfiguration-r16.If the Area Scope of MDT is configured as PLMN wide, the IE AreaConfiguration-r16 would not be configured to the UE which leads to the fact that the interFreqTargetList-r16 cannot be configured in this case.

RAN3 would like RAN2 to check whether this is an erroneous implementation in TS 38.331.

- Frequency band info

In TS 38.413, the NR Frequency Info in Area Scope of Neighbour Cells supports NR Frequency Band List configuration. While in TS 38.331, there isn’t any NR Frequency Band configuration in the InterFreqTargetInfo configured to the UE.

RAN3 would like RAN2 to feedback whether NR Frequency Band needs to be supported for the Area Scope of Neighbour Cells.

In the contribution [12], Huawei has provided the following related proposals.

Proposal 1: It is proposed RAN2 to discuss whether the area scope of neighbour cells is dependent on the area scope of serving cells or not:

- If there is a dependency, from Rel-17, one option (for RAN3) is to add a clarificaiton to TS 38.413 that “Area Scope of Neighbour Cells” should be simultaneously configued with “the Area Scope of MDT is configured as PLMN wide”

- If there is no dependency, from Rel-17, one option (for RAN2) is to introduce AreaConfiguration-r17 including areaConfig-r16 and interFreqTargetList-r16 inside, and both fields are optional

- Rel-16 specifications are unchanged (leave it to network implementation)

Proposal 2: It is proposed to reply to RAN3 that NR Frequency Band is not supported for the Area Scope of Neighbour Cells.

Based on the above, the following question is used to collect companies’ views regarding the question of interFreqTargetList within AreaConfiguration:

Question-9: Which of the following option(s) are preferred regarding the RAN3’s question on the presence of interFreqTargetList within AreaConfiguration?

- Add a clarificaiton to TS 38.413 that “Area Scope of Neighbour Cells” should be simultaneously configued with “the Area Scope of MDT is configured as PLMN wide” i.e., no change to RAN2 specification.

- Introduce AreaConfiguration-r17 including areaConfig-r16 and interFreqTargetList-r16 inside, and both fields are optional.

- Rel-16 specifications are unchanged

| Company name | Agreeable option(s)? 1, 2, 3, All | Comments |
| --- | --- | --- |
| Qualcomm | For rel-16, we should adopt option 3 and RAN3 can add clarification as in option 1. We can consider option-2 in rel-17. |  |
| vivo | 2) | This issue can be fixed in Rel-17. |
| Ericsson | 2 and 3 |  |
| Apple | 3 |  |
| ZTE | (3) |  |
| OPPO | 2） |  |
| Sharp | 3 |  |
| Huawei, HiSilicon | 3 | From RAN2 point of view, we think Rel-16 specification can be unchanged. RAN3 may discuss it based on RAN2 agreement, e.g. 1) may be considered in RAN3. We are open for solutions in Rel-17, e.g. 2). |
| CATT |  | Agree with Qualcomm. |
| LGE | 3 |  |
| Nokia | 3 or 2 | Agree with Qualcomm suggestion |

Rapporteur Summary:

9/11 companies agree that rel-16 specifications should remain unchanged and 7/11 companies indicate that in Rel-17 RAN2 can study the introduction of AreaConfiguration-r17 including areaConfig-r16 and interFreqTargetList-r16 inside it with both fields being optional.

Rel-16 RAN2 specifications are unchanged with respect to RAN3’s question on the presence of interFreqTargetList within AreaConfiguration.

RAN2 works on the introduction of AreaConfiguration-r17 (including areaConfig-r16 and interFreqTargetList-r16 inside it with both fields being optional) in Rel-17.

Based on the past discussions in RAN2 quoted in [REF], it is clear that the frequency band list configuration is not supported in interFreqTargetList configuration.

Question-10: RAN2 confirms that frequency band list configuration is not supported in interFreqTargetList configuration?

| Company name | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Agree. |  |
| vivo | Agree |  |
| Ericsson | Yes |  |
| Apple | Agree |  |
| ZTE | Yes |  |
| OPPO | Agree |  |
| Sharp | Agree |  |
| Huawei, HiSilicon | Agree |  |
| CATT | Agree |  |
| LGE | Yes |  |

Rapporteur Summary:

All companies agree that frequency band list configuration is not supported in interFreqTargetList configuration.

RAN2 confirms that frequency band list configuration is not supported in interFreqTargetList configuration.

## 3.3	Other logged MDT topics

There are several proposals on the following topics but the topic is brought up by few companies only.

- Clarifications related to early measurements logging in logged MDT report

- Frequency-specific and RAT-specific coverage hole indication in logged MDT report and its associated configuration

- Enhancements associated to CEF report and RLF report for UL/DL coverage imbalance issues

- MDT for logging slice availability

If any of the above proposals need to be brought up for discussion in this meeting itself then the rapporteur requests companies to bring it up in the answer for the following question.

Question-10: Are any of the following topics need to be discussed in this meeting (if not, they are postponed to the next meeting)?

- Clarifications related to early measurements logging in logged MDT report

- Frequency-specific and RAT-specific coverage hole indication in logged MDT report and its associated configuration

- Enhancements associated to CEF report and RLF report for UL/DL coverage imbalance issues

- MDT for logging slice availability

| Company name | Comments |
| --- | --- |
| Qualcomm | For 1), We had earlier agreements on logging early measurement frequency in the logged MDT, therefore, clarification related to early measurements logging in logged MDT may be needed. 2) may be required considering heterogeneous network deployment and it may be required to know the RAT-specific and frequency-specific coverage holes. As the logged MDT size is limited, an event triggered measurement logging upon detection of RAT or frequency-specific coverage is required. 3) we had earlier discussions on this, however, we need to consider UE memory for addressing this issue. A simpler solution that does not require high UE memory consumption at UE can be discussed. 4) invloves NAS at the UE. In the current SON reports, we don’t involve NAS, therefore, we can deprioritize this in rel-17. |
| Ericsson | We can discuss all four of them in the next meeting i.e., they are relevant to Rel-17 but can be postponed in this meeting. We are also open for discussing them over an email discussion between the RAN2#115 and RAN2#116 meeting. |
| OPPO | We agree with Qualcomm that 2) is required. At least, a post RAN2#115e email discussion is needed. |
| Huawei, HiSilicon | We think there may not be enough time to progress on these above 4 enhancements, and a post-meeting email may be helpful, e.g. collect possible supports, identify possible impacts, and etc. |
| Nokia | 2 and 3 are required 4 is too premature considering RAN slicing WI related concept is under development |

Rapporteur Summary:

Based on the above it seems like there is some support for working on each of the four topics in Rel-17 itself. Some companies have indicated that this can be progressed over email discussion between RAN2#115 and RAN2#116 meeting. Rapporteur proposes to postpone the discussion to next meeting.

RAN2 postpones the discussions on the following to RAN2#116 meeting:

- Clarifications related to early measurements logging in logged MDT report

- Frequency-specific and RAT-specific coverage hole indication in logged MDT report and its associated configuration

- Enhancements associated to CEF report and RLF report for UL/DL coverage imbalance issues

RAN2 to further discuss whether MDT for logging slice availability is considered in Rel-17.

# 4	Conclusion

The following proposals are captured in section 3.

Proposal 1	The scenario of logging of measurements associated to on-demand SI request upon on-demand positioning SI/SIB request and upon on-demand SI request in connected mode are not pursued in Rel-17.

Proposal 2	The scenario of logging of measurements associated to successful on-demand SI request procedure is postponed to the next RAN2 meeting.

Proposal 3	The UE includes the beam identifiers used to acquire the SI message(s) in the on-demand SI procedure related report.

FFS: How to capture this information

Proposal 4	The following measurements aer not included in the on-demand SI related report.

1.	The number of times each SIB was intended to be requested by the UE

2.	Failure type (failure at RA procedure or failure at acquiring SI messages)

3.	The time between consecutive SI requests

4.	The location information at the time of performing the SI request

5.	An indicator to indicate if the SI request was performed over NUL or SUL

Proposal 5	Decision on inclusion of an indicator in the on-demand SI request related report indicating whether the on-demand SI request was successful or not is postponed to next RAN2 meeting.

Proposal 6	Extend RA report for both successful and failure on-demand SI request. FFS: Whether successful one-demand SI request related scenario is included or not is postponed to RAN2#116 meeting.

Proposal 7	Signaling based logged MDT override protection is applicable in the following scenarios:

1)	Signaling based Logged MDT is configured, but no results are available e.g. so far nothing stored, or all previously stored results retrieved

2)	Signaling based Logged MDT configuration is stopped (i.e. the expiry of T330), but UE still has un-retrieved results that would be discarded upon accepting a new configuration

Proposal 8	The following scenarios associated to Signaling based logged MDT override protection are postponed to RAN2#116 meeting:

1)	Signaling based logged MDT is configured in LTE (NR), the UE comes to connected in NR (LTE)

2)	Signaling based logged MDT is configured, the UE comes to connected in a PLMN that is not in the plmn-IdentityList.

Proposal 9	Include an indicator to indicate the signaling based logged MDT configuration availability in RRCSetupComplete / RRCConnectionSetupComplete and RRCResumeComplete / RRCConnectionResumeComplete.

FFS: Implicit (flag indicating T330 is running or not) vs explicit indication

Proposal 10	UE includes an indication regarding whether the T330 timer is running or not in RRCSetupComplete / RRCConnectionSetupComplete and RRCResumeComplete / RRCConnectionResumeComplete.

Proposal 11	Rel-16 RAN2 specifications are unchanged with respect to RAN3’s question on the presence of interFreqTargetList within AreaConfiguration.

Proposal 12	RAN2 works on the introduction of AreaConfiguration-r17 (including areaConfig-r16 and interFreqTargetList-r16 inside it with both fields being optional) in Rel-17.

Proposal 13	RAN2 confirms that frequency band list configuration is not supported in interFreqTargetList configuration.

Proposal 14	RAN2 postpones the discussions on the following to RAN2#116 meeting:

1)	Clarifications related to early measurements logging in logged MDT report

2)	Frequency-specific and RAT-specific coverage hole indication in logged MDT report and its associated configuration

3)	Enhancements associated to CEF report and RLF report for UL/DL coverage imbalance issues

Proposal 15	RAN2 to further discuss whether MDT for logging slice availability is considered in Rel-17.

# 5	References

- R2-2107394 logged MDT enhancement regarding RAT-specific coverage hole OPPO

- R2-2107395 Futher consideration of MDT configuration priority OPPO

- R2-2107508	 Logged MDT in EN-DC and other enhancements Nokia, Nokia Shanghai Bell

- R2-2107720	 On-demand SI request enhancements vivo

- R2-2107827	 Considerations on MDT Enhancements for On-demand SI CATT

- R2-2108306	 On logged MDT related enhancements Ericsson

- R2-2108331	 Logged measurement Enhancements QUALCOMM INCORPORATED

- R2-2108357	 Consideration on on-demand SI request information report ZTE Corporation, Sanechips

- R2-2108505 MDT for Slice unavailability CMCC, Ericsson, Huawei	discussion

- R2-2108543	 Further consideration on UL-DL coverage mismatch	CMCC

- R2-2108566	 Discussion on logged MDT enhancements Huawei, HiSilicon

- R2-2108568 Discussion on Area scope configuration and Frequency band info in MDT configuration based on RAN3 LS R3-212824 Huawei, HiSilicon

- R2-2108650 SON Enhancements for SI Request Optimization Samsung

- R2-2108739 Discussion on Logged MDT issues  Samsung Electronics Co., Ltd