3GPP TSG RAN2 #115-e								R2-2109021

Online, 16 – 27 August 2021

Agenda Item:		8.13.3.1 Immediate MDT enhancements

Source:			Huawei, HiSilicon

Title:			Summary on agenda item 8.13.3.1 Immediate MDT enhancements

Document for:		Discussion and Decision

1	Introduction

This contribution is to summarize proposals from RAN2 contributions under AI 8.13.3.1 ([1]~[7]), and the email title is as below:

[Pre115-e][810][SON/MDT] Summary of 8.13.3.1 Immediate MDT enhancements (Huawei)

2	Discussion

2.1	Proposals related to RAN3 LS [8]

[1], Vivo

RAN2 to confirm the understanding that the reportAmount is a mandatory field in RRC, so the restriction on the number of reports also applies to the reporting of ul-DelayValueConfig-r16 (namely D1).

RAN2 to reply that there is no drawbacks incurs even though Report Amount is absent for M6.

[5], ZTE

Proposal 2: Update RAN3’s specs to include Report Amount IE for M6 measurement.

[Rapp] Suggest to discuss above proposals because the RAN3 LS [8] has actions for RAN2.

2.2	Proposals related to RAN3 LS [9]

[1], Vivo

To solve the erroneous implementation, LoggedMeasurementConfiguration-r16-IEs is extended to include interFreqTargetList-r16 again out side of areaConfiguration-r16.

From the UE perspective, NR Frequency Band is not necessarily required.

[Rapp] Suggest to move it to logged MDT session.

2.3	M5 measurements

[2], CATT

Proposal 3: For split bearer, the M5 measurement results of MN and SN can be calculated in the DU respectively.

Proposal 4: For the accuracy of the result, a data marker (duplication indicator) can be sent with M5 measurement results to the OAM.

[Rapp] More related to RAN3?

[3], Ericsson

Proposal 4	For the throughput measurements (M5) in split bearer configurations, the throughput is computed at individual DUs and sent to TCE.

Proposal 5	For throughput measurements (M5) in split bearer configurations, CU-UP indicates to the OAM the status of packet duplication during the measurement period.

[Rapp] More related to RAN3?

[4], Qualcomm

Proposal 1: Considering both M5 and M7 measurements, we argue MN and SN report the following measurements to TCE at the end of the measurement period for M5 and M7 measurements:

- Burst Size of data transmitted over SN (taking multiple transmission slots)

- The point in time when the transmission is started for the first data in the data burst over SN

- The point in time when the data until the second last piece of data burst TX over SN has been successfully received at the UE

- The RLC SDU sequence number of packets lost over the Uu interface

- The RLC SDU sequence number of the packet discard at the RLC or MAC for traffic management for which part is transmitted over the air.

[Rapp] In RAN2#113b-e, based on the Imm MDT summary R2-2104441, a similar proposal was discussed and the RAN action is “Proposal 1: Considering both M5 and M7 measurements, we argue to introduce an X2/Xn message to obtain the following measurement at the end of the measurement period:”, and that propsals did not have enough supports.

The above proposal 1 (in [4]) has a different RAN action.

2.4	M6 measurements (including D1 measurements)

[2], CATT

Proposal 1: For split bearer, D1 values (UL PDCP Packet Average Delay) over MN and SN are different.

Proposal 2: For split bearer, UE reports M6 UL D1 results to the MN or SN which configures the D1 measurement.

[3], Ericsson

Proposal 1	In MN terminated split bearer and SN terminated split scenarios, both the MN CU-CP and the SN CU-CP can configure the D1 measurement to the UE.

Proposal 2	In MN terminated split bearer and/or SN terminated split scenarios, if the UE receives the D1 measurement configuration from the MN CU-CP then the UE reports D1 measurement values associated to packets sent over MCG to MN CU-CP.

Proposal 3	In MN terminated split bearer and/or SN terminated split scenarios, if the UE receives the D1 measurement configuration from the SN CU-CP then the UE reports D1 measurement values associated to packets sent over SCG to SN CU-CP.

[7], Huawei, HiSilicon

Proposal 1: For split bearer, only one node can configure D1 measurements.

Proposal 2: For D1 measurements for split bearer (i.e. MN terminated split bearer, SN terminated split bearer),

RAN2 to adopt option 2 for more accurate measurement results.

[Rapp] In RAN2#113b-e, based on the Imm MDT summary R2-2104441, it seemed to be no consensuses on any solutions, so a proposal was suggested (but not treaded due to lack of time). From Rapporteur’s point of view, it may be good to discuss the proposal (with some modifications).

Proposal 1: For D1 measurements for split bearer (i.e. MN terminated split bearer, SN terminated split bearer), try to find a compromise solution, and if there are no consensuses on any solutions, RAN2 is to decide on one understanding from the following two:

- Understanding 1: D1 measurements are not used for this scenario, i.e. the network will not configure D1 measurements for the UE for this scenario

- Understanding 2: D1 measurements are allowed for this scenario and how it works can be clarified

2.5	M7 measurements

[2], CATT

DL Packet Uu  Loss Rate in the DL per DRB per UE

Proposal 5: For split bearer, the MN DU and the SN DU send the results of M7 measurement to OAM respectively.

Proposal 6: For the accuracy of the result, a data marker (duplication indicator) can be sent with M7 measurement results to the OAM.

UL PDCP SDU Packet Loss Rate

Proposal 7: For the accuracy of the result, measurement of M7 packet loss rate of UL PDCP SDU with data marker (duplication indicator) can be sent to the OAM by CU.

DL/UL F1-U Packet Loss Rate

Proposal 8: Measurement of M7 packet loss rate of DL/UL F1-U is performed by CU and it will not be impacted by DC scenario, no matter for split bearer or for MN/SN terminated SCG/MCG bearer.

[Rapp] More related to RAN3?

[3], Ericsson

Proposal 6	Packet Loss rate measurements (M7) are performed at DUs and sent to TCE.

Proposal 7	For the packet loss rate measurements in split bearer configurations, CU-UP indicates to the OAM the status of packet duplication during the measurement period.

[Rapp] More related to RAN3?

2.6	Immediate MDT and IDC

[5], ZTE

Proposal 1: No enhancement in RAN2 is needed to support IDC tagging in immediate MDT.

[7], Huawei, HiSilicon

Proposal 3: For immediate MDT, the reporting of MDT measurements are not affected by IDC, i.e. follow LTE design.

[Rapp] There were some contributions on this topic in previous meetings, but they were not treated.

3	Conclusion

Based on the analysis in section 2, the summary is as below:

Related to RAN3 LS [8]: (from [1], Vivo; [5], ZTE)

RAN2 to confirm the understanding that the reportAmount is a mandatory field in RRC, so the restriction on the number of reports also applies to the reporting of ul-DelayValueConfig-r16 (namely D1).

RAN2 to reply that there is no drawbacks incurs even though Report Amount is absent for M6.

Proposal 2: Update RAN3’s specs to include Report Amount IE for M6 measurement.

For M6 measurements: (related to [2], CATT, [3], Ericsson and [7], Huawei, HiSilicon)

Proposal 1: For D1 measurements for split bearer (i.e. MN terminated split bearer, SN terminated split bearer), try to find a compromise solution, and if there are no consensuses on any solutions, RAN2 is to decide on one understanding from the following two:

- Understanding 1: D1 measurements are not used for this scenario, i.e. the network will not configure D1 measurements for the UE for this scenario

- Understanding 2: D1 measurements are allowed for this scenario and how it works can be clarified

For M5 and M7 measurements: (the following proposals seem more related to RAN3)

[2], CATT

Proposal 3: For split bearer, the M5 measurement results of MN and SN can be calculated in the DU respectively.

Proposal 4: For the accuracy of the result, a data marker (duplication indicator) can be sent with M5 measurement results to the OAM.

[3], Ericsson

Proposal 4	For the throughput measurements (M5) in split bearer configurations, the throughput is computed at individual DUs and sent to TCE.

Proposal 5	For throughput measurements (M5) in split bearer configurations, CU-UP indicates to the OAM the status of packet duplication during the measurement period.

[2], CATT

DL Packet Uu  Loss Rate in the DL per DRB per UE

Proposal 5: For split bearer, the MN DU and the SN DU send the results of M7 measurement to OAM respectively.

Proposal 6: For the accuracy of the result, a data marker (duplication indicator) can be sent with M7 measurement results to the OAM.

UL PDCP SDU Packet Loss Rate

Proposal 7: For the accuracy of the result, measurement of M7 packet loss rate of UL PDCP SDU with data marker (duplication indicator) can be sent to the OAM by CU.

DL/UL F1-U Packet Loss Rate

Proposal 8: Measurement of M7 packet loss rate of DL/UL F1-U is performed by CU and it will not be impacted by DC scenario, no matter for split bearer or for MN/SN terminated SCG/MCG bearer.

[3], Ericsson

Proposal 6	Packet Loss rate measurements (M7) are performed at DUs and sent to TCE.

Proposal 7	For the packet loss rate measurements in split bearer configurations, CU-UP indicates to the OAM the status of packet duplication during the measurement period.

Others: ([5], ZTE; [7], Huawei, HiSilicon)

Proposal 1: No enhancement in RAN2 is needed to support IDC tagging in immediate MDT.

Proposal 3: For immediate MDT, the reporting of MDT measurements are not affected by IDC, i.e. follow LTE design.

4	Tdocs under 8.13.3.1 Immediate MDT enhancements

[1]	R2-2107719	On RAN3 LS on MDT issues	vivo	discussion

[2]	R2-2107826	Further Considerations on Immediate MDT Enhancements	CATT	discussion

[3]	R2-2108302	On Immediate MDT Enhancements	Ericsson	discussion

[4]	R2-2108349	On accurate M5 and M7 measurements	QUALCOMM INCORPORATED	discussion

[5]	R2-2108356	Consideration on immediate MDT aspects	ZTE Corporation, Sanechips	discussion

[6]	R2-2108564	Report of [Post114-e][851][SONMDT] Procedures and Modeling of successful HO report (Huawei)	Huawei	discussion	(this contribution should be under 8.13.2.1, so it is not summarized here)

[7]	R2-2108565	Discussion on immediate MDT enhancements	Huawei, HiSilicon	discussion

[8]	R3-212961	LS on Report Amount for M4, M5, M6, M7 measurements

[9]	R3-212824,	LS on Area scope configuration and Frequency band info in MDT configuration