3GPP TSG-RAN WG2 Meeting #115 electronic	R2-2109043

Online, August 16th – August 27th, 2021

Agenda Item:	9.2.3

Source: 	OPPO

Title:  	Summary of [AT115-e][037][IoT-NTN] User Plane Impact (OPPO)

Document for:	Discussion and Decision

- Introduction

This document aims to summarize the following offline discussion.

[AT115-e][037][IoT-NTN] User Plane Impact (OPPO)

Scope: Treat documents under 9.2.3. Identify potential agreements (e.g. confirm SI agreements), Open points, potential alternatives.

Intended outcome: Report

Deadline: CB Monday W2

- Discussion

Proposals from companies [1-10] will be discussed in the following sections.

2.1 MAC

2.1.1 RACH

In NTN, due to the large propagation delay, during random access procedure, after sending Msg1, UE could not receive Msg2 until a time interval of RTT has passed. In Rel-17 NR NTN WI, RAN1 has made the following agreements in RAN1#105 e-meeting:

Agreement: The starts of ra-ResponseWindow and msgB-ResponseWindow are delayed by an estimate of UE-gNB RTT. - The estimate of UE-gNB RTT is equal to the sum of UE’s TA and K_mac. Note 1: The UE’s TA is based on the RAN1#104bis-e agreement on Timing Advance applied by an NR NTN UE given by TTA=NTA+NTA, UE-specific+NTA,common+NTA,offset×Tc. The estimate of gNB-satellite RTT is equal to the sum of and K_mac. How to treat NTA and NTA,offset can be further discussed. Note 2: According to the RAN1#104bis-e agreement: When UE is not provided by network with a K_mac value, UE assumes K_mac = 0. Note 3: The accuracy of the estimated UE-gNB RTT with respect to the true UE-gNB RTT can be further discussed. Note 4: Other options of determining the estimate of UE-gNB RTT can be further discussed.

Based on above RAN1 agreements, in NR NTN, an offset is used to delay to start of ra-ResponseWindow, and the offset is the estimated UE-gNB RTT.

In [1], [2], [3], [7], [9] and [10], it is proposed to introduce an offset to delay the start of the ra-ResponseWindow for IoT-NTN. Regarding the offset value, there are different options:

- Option 1: Derive the offset based on UE-eNB RTT

  - Option 1-1: The offset is defined as max (current offset, UE-eNB RTT), where the current offset is fixed to 3 subframes for eMTC, and can be either 4 subframes or 41 subframes for NB-IoT as defined in TS36.321. [1]

  - Option 1-2: The offset is an estimate of UE-eNB RTT, which aligns with NR NTN. [7] [9]

- Option 2: Postpone the discussion on offset to starting of RA response window until further agreements regarding RACH are made in RAN1. [3][5]

It seems that all the companies have a common understanding that the start of the ra-ResponseWindow should be delayed by an offset. Rapporteur would like to ask the following question:

Question 1: If the start of the ra-ResponseWindow is delayed by an offset, which is your preferred option regarding the offset value?

- Option 1: Derive the offset based on UE-eNB RTT

  - Option 1-1: The offset is defined as max (current offset, UE-eNB RTT), where the current offset is fixed to 3 subframes for eMTC, and can be either 4 subframes or 41 subframes for NB-IoT as defined in TS36.321.

  - Option 1-2: The offset is an estimate of UE-eNB RTT.

- Option 2: Postpone the discussion on offset to starting of RA response window until further agreements regarding RACH are made in RAN1.

| Company | Option | Additional comments |
| --- | --- | --- |
| OPPO | Option 1-1 | For NB-IoT and eMTC, there is already an offset for the start of ra-ResponseWindow. For eMTC, the offset is fixed to 3 subframes, while for NB-IoT, the offset can be either 4 subframes or 41 subframes, which depends on the NPRACH transmission duration. The UE-eNB RTT for different IoT NTN scenarios are as below. Scenarios; Maximum UE-eNB RTT; Minimum UE-eNB RTT. GEO (Scenario A); 541.46ms; 520.86. LEO with satellite altitude of 600 km (Scenario B&C); 19.53ms; 25.77 ms. LEO with satellite altitude of 1200 km (Scenario B&C); 35.41ms; 41.77ms. MEO (Scenario D); 160.1 ms; 186.9 ms. It can be observed that for eMTC the current offset value is shorter than UE-eNB RTT for all the IoT NTN scenarios. For NB-IoT, the current offset value may be shorter or longer than UE-eNB RTT. Therefore, a straight way is to adjust the offset for the start of ra-ResponseWindow based on UE-eNB RTT, e.g., the offset for the start of ra-ResponseWindow can defined as max (current offset, UE-eNB RTT). |
| Xiaomi | Option 1-1 | In case X=41, the delay is not negligible, thus it deserves to optimize for this case, i.e. using maximum {X, UE-eNB RTT}. |
| CATT | See comments | The current offset mentioned in option 1-1 is applied to PRACH repetition mechanism. The offset is introduced based on the decoding capacity of network. However, the UE-eNB RTT is propagation delay in NTN. Therefore, the offset is defined as max (current offset, UE-eNB RTT) is not correct. Thus, we agree with the following option: Option 3: An offset is defined as sum (current offset, UE-eNB RTT). is introduced at the start of RAR Window, where the current offset is fixed to 3 subframes for eMTC, and can be either 4 subframes or 41 subframes for NB-IoT as defined in TS36.321. |
| Huawei, HiSilicon | Option 2 |  |
| MediaTek | Option 1-1 | Agree with Oppo and Xiaomi. |
| Qualcomm | Option 2 | Let the RAN1 first confirm calculation of the UE-eNB RTT as in NR NTN. |
| Lenovo | Option 2 | We would like to have RAN1’s conclusion first. |
| Nokia | Option 1-2 or Option2 | In WID, RAN1 agreed to take the time and frequency synchronization agreements in NR NTN as baseline for IoT NTN. Option1-2 follow NR NTN agreement on the start of RAR window. Furthermore, what we understand about Option1-2 is an additional offset on top of current offset defined in TS36.321. For Option 1-1, the question is NW may not know the exact UE-gNB RTT before RACH thus don’t know when UE will monitor RAR. We are also fine to wait for RAN1 conclusion first if it is the majority view. |
| ZTE | Option 2 | We would like to wait for RAN1 decision, although RAN1 may follow the NR NTN agreements. |
| Apple | Option 2 | We also believe that the UE-eNB RTT should be a considered as an additional offset beyond the current offsets defined in 36.321. We are not sure why RAN1 decision on how UE-eNB RTT is calculated should have a bearing on how we specify the offset in the MAC spec. |
| LG | Option 2 |  |
| Sequans | Option 2 |  |
| CMCC | Option 2 | The description of option1-1 “The offset is defined as max (current offset, UE-eNB RTT), …” may be not right. The UE-eNB RTT should be enhanced on the existing mechanism, rather than a maximum selection solution. |
| NEC | Option2 | Since it was RAN1 who made the final decision on offset value in NR NTN case, we can leave it to RAN1 for IOT NTN case as well. |
| Rakuten Mobile Inc. | Option 2 | We think that value of UE-eNB RTT will deciding factor to consider whether current offset + UE-eNB RTT or max (x,UE-eNB RTT). So better to wait for RAN1 decision. |
| Ericsson | Option 1-1 | Agree with OPPO, Xiaomi and MediaTek. |

Rapporteur summary:

All companies have a common understanding that the start of the ra-ResponseWindow should be delayed by an offset. Regarding the offset value, 4 (OPPO, Xiaomi, MediaTek, Ericsson) of 16 support option 1-1. 1 (Nokia) supports option 1-2. 11 companies support option 2 (i.e. to postpone discussion). Rapporteur proposes to postpone the discussion after RAN1 made some progress.

Proposal 1: Start of ra-ResponseWindow is delayed by an offset. Postpone discussion on the offset value until further agreements regarding RACH are made in RAN1.

Regarding ra-ResponseWindowSize length, the following agreement was made in RAN2#112e for NR NTN:

Agreements:

- If the start of the ra-ResponseWindow and msgB-ResponseWindow is accurately compensated by UE-gNB RTT, ra-ResponseWindow and msgB-ResponseWindow are not extended in LEO/GEO.

In [3], [7], [8] and [9], it is proposed that if the start of the RA Response window is accurately compensated by UE-eNB RTT and no extension of repetition is required, there is no need to extend the ra-ResponseWindowSize for IoT NTN.

Question 2: Do companies agree that if the start of the RA Response window is accurately compensated by UE-eNB RTT and no extension of repetition is required, there is no need to extend the ra-ResponseWindowSize for IoT NTN?

| Company | Agree / Disagree | Additional comments |
| --- | --- | --- |
| OPPO | Agree |  |
| Xiaomi | yes |  |
| CATT | Agree |  |
| Huawei, HiSilicon | Agree |  |
| MediaTek | Agree |  |
| Qualcomm | Agree |  |
| Lenovo | Agree |  |
| Nokia | Agree |  |
| ZTE | Agree |  |
| Apple | Agree |  |
| LG | Agree |  |
| Sequans | Agree |  |
| CMCC | Agree |  |
| NEC | Agree |  |
| Rakuten Mobile Inc | Agree |  |
| Ericsson | Agree |  |

Rapporteur summary:

All companies agree to Q2.

Proposal 2: If the start of the RA Response window is accurately compensated by UE-eNB RTT and no extension of repetition is required, there is no need to extend the ra-ResponseWindowSize for IoT NTN.

In RAN2#115e, the following agreement has been made in NR NTN WI:

Agreements:

- In the MAC specification section 5.1.5, delay the start of ra-ContentionResolutionTimer by the UE-gNB RTT (i.e. sum of UE's TA and K_mac)

In [1], [2], [3], [7], [9] and [10], it is proposed to introduce an offset to delay the start of the mac-ContentionResolutionTimer for IoT-NTN. Regarding the offset value, in [1] and [9], it is further proposed that the offset value should be set to UE-eNB RTT. In [7], it is suggested that the exact meaning of the offset should follow NR NTN agreement. On the other hand, in [3] and [5], it is proposed to postpone the discussion on offset to starting of mac-ContentionResolutionTimer until further agreements regarding RACH are made in RAN1. In rapporteur’s understanding, since the start of mac-ContentionResolutionTimer is captured in TS36.321, this issue should be addressed in RAN2. However, it would be good to collect companies’ views.

Question 3: If the start of the mac-ContentionResolutionTimer is delayed by an offset, which is your preferred option regarding the offset value?

- Option 1: use UE-eNB RTT as the offset value

- Option 2: Postpone the discussion on offset to starting of mac-ContentionResolutionTimer until further agreements regarding RACH are made in RAN1.

| Company | option | Additional comments |
| --- | --- | --- |
| OPPO | Option 1 | It could align with conclusion in NR NTN. |
| Xiaomi | Option 1 |  |
| CATT | Option 1 |  |
| Huawei, HiSilicon | Option 2 |  |
| MediaTek | Option 1 |  |
| Qualcomm | Option 2 |  |
| Lenovo | Option 2 | We would like to have RAN1’s conclusion first. |
| Nokia | Option 1 | As RAN2-115 agreed that, for NR NTN, the offset to start ra-ContentionResolutionTimer is UE-gNB RTT, we think Option1 is agreeable to follow NR NTN agreements. |
| ZTE | Option 2 |  |
| Apple | Option 1 |  |
| LG | Option 1 |  |
| Sequans | Option 2 |  |
| CMCC | Option 1 or option 2 | We are also fine to wait for RAN1. |
| NEC | Option1 | Follow the agreement made for NR NTN, we can go with option1. It can always be revisited if any RAN1 agreement bring up any issue |
| Rakuten Mobile Inc | Option 2 |  |
| Ericsson | Option 1 |  |

Rapporteur summary:

9 out 16 support option 1. 6 companies suggest to postpone. 1 company is fine with ether option 1 or 2. For the sake of progress, rapporteur suggests to go for majority views, meanwhile adding a condition that if RAN1 decides something that requires to change this RAN2 can revisit it.

Proposal 3: Start of mac-ContentionResolutionTimer is delayed by an offset, which is equal to UE-eNB RTT. This can be revisited if RAN1 decides something that requires to change this.

Regarding the mac-ContentionResolutionTimer length, considering that the offset for the start of mac-ContentionResolutionTimer can be accurately compensated by UE-eNB RTT, it is proposed in [9] that mac-ContentionResolutionTimer is not extended.

Question 4: Do companies agree that there is no need to extend the mac-ContentionResolutionTimer for IoT NTN?

| Company | Agree / Disagree | Additional comments |
| --- | --- | --- |
| OPPO | Agree with comment | If the start of mac-ContentionResolutionTimer is accurately compensated by UE-eNB RTT and no extension of repetition is required, there is no need to extend the mac-ContentionResolutionTimer for IoT NTN |
| Xiaomi | yes |  |
| CATT | Agree |  |
| Huawei, HiSilicon | Agree |  |
| MediaTek | Agree |  |
| Qualcomm | Agree |  |
| Lenovo | Agree |  |
| Nokia | Agree with modification | Same view as OPPO. |
| ZTE | Agree |  |
| Apple | Agree |  |
| LG | Agree |  |
| Sequans | Agree |  |
| CMCC | Agree |  |
| NEC | Agree | And fine with the clarification on additional condition from OPPO |
| Rakuten Mobile Inc | Agree | Same as OPPOs view |
| Ericsson | Agree with modification | Same view as OPPO |

Rapporteur summary:

All companies agree not to extend the mac-ContentionResolutionTimer for IoT NTN. And it is reasonable that this should be based on the condition if the start of mac-ContentionResolutionTimer is accurately compensated by UE-eNB RTT and no extension of repetition is required.

Proposal 4: If the start of mac-ContentionResolutionTimer is accurately compensated by UE-eNB RTT and no extension of repetition is required, there is no need to extend the mac-ContentionResolutionTimer for IoT NTN.

If we agree to use UE-eNB RTT as the start of some UP timers (e.g. ra-ResponseWindow, mac-ContentionResolutionTimer), the next issue is how to determine UE-eNB RTT.

In NR NTN WI, based on RAN1 agreement in RAN1#105e, the estimate of UE-gNB RTT is equal to the sum of UE’s TA and K_mac, while K_mac is needed when downlink timing and uplink timing are not aligned at gNB, in which case this parameter donotes the TA value pre-compensated by gNB and can be provided by gNB.

It is proposed in [1] that UE estimates UE-eNB RTT in a similar way as in NR NTN, i.e., the estimated UE-eNB RTT is the sum of UE’s TA and K_mac. In [3], [7] and [9], it is suggested to wait for RAN1 on how UE obtains UE-NB RTT.

Question 5: Please provide your views on how UE obtains UE-eNB RTT in IoT NTN?

- Option 1: UE determines UE-eNB RTT in a similar way as in NR NTN, i.e., the UE-eNB RTT is the sum of UE’s TA and K_mac

- Option 2: wait for RAN1 on how UE obtains UE-NB RTT.

| Company | Option | Additional comments |
| --- | --- | --- |
| OPPO | Option 1 | In our understanding, the method for UE-eNB RTT estimation in NR NTN could also apply to IoT NTN. However, if most companies prefer option 2, we are also ok. |
| Xiaomi | Option 1 |  |
| CATT | Option 1 |  |
| Huawei, HiSilicon | Option 2 |  |
| MediaTek | Option 1 |  |
| Qualcomm | Option 2 | In NR NTN, it was RAN1 who decided to use UE’s TA and K_mac for UE-gNB RTT. At least we can wait if RAN1 confirms same for IoT NTN. |
| Lenovo | Option 2 | We would like to have RAN1’s conclusion first. |
| Nokia | Option 2 | In WID, RAN1 may have minimum change on how to obtain UE-gNB RTT for IoT NTN based on NR NTN agreements. |
| ZTE | Option 2 |  |
| Apple | Option 2 | Agree with Qualcomm’s view |
| LG | Option 1 |  |
| Sequans | Option 2 |  |
| CMCC | Option 2 |  |
| NEC | Option 2 | Same as NR NTN, this should be decided by RAN1 |
| Rakuten Mobile Inc | Option 2 |  |
| Ericsson | Option 1 with comment | We can add “Can be revisited if RAN1 decides on a different UE-eNB RTT estimation method.” |

Rapporteur summary:

6 out of 16 support option 1 and 10 companies suggest to postpone. For the sake of progress, rapporteur proposes to take option 1 as starting point and capture in chairman’s notes that RAN2 can revisit this if RAN1 decides something different which requires change to RAN2.

Proposal 5: RAN2 assume that UE determines UE-eNB RTT in a similar way as in NR NTN, i.e., the UE-eNB RTT is the sum of UE’s TA and K_mac. This can be revisited if RAN1 decides something that requires to change this.

For UE with TA pre-compensation capability, when scheduling Msg3’s transmission for the UE, network may not know the exact value of UE-specific TA, due to UE’s autonomous pre-compensation for Msg1 transmission. This can be solved by using the maximum propagation delay of the cell to schedule Msg3’s transmission.

In RAN2#112e meeting, the following agreement was made in NR NTN WI.

Agreements:

- From RAN2 perspective, for UE with UE-specific pre-compensation as a baseline it is up to gNB implementation to ensure sufficient time on UE side for the Msg3 transmission.

In [3], it is proposed that the above agreement also applies to IoT NTN.

Question 6: Do companies agree that from RAN2 perspective, for UE with UE-specific pre-compensation as a baseline it is up to eNB implementation to ensure sufficient time on UE side for the Msg3 transmission for IoT NTN?

| Company | Agree / Disagree | Additional comments |
| --- | --- | --- |
| OPPO | Agree | It could align with conclusion in NR NTN. |
| Xiaomi | yes |  |
| CATT | Agree |  |
| Huawei, HiSilicon | Agree |  |
| MediaTek | Agree |  |
| Qualcomm | Agree |  |
| Lenovo | Agree |  |
| Nokia | Agree |  |
| ZTE | Agree |  |
| Apple | Agree |  |
| LG | Agree |  |
| Sequans | Agree |  |
| CMCC | Agree |  |
| NEC | Agree |  |
| Rakuten Mobile Inc | Agree |  |
| Ericsson | Agree | This is true for all scheduling. In the case of HALF DUPLEX mode, eNB shall also ensure the UE is not scheduled to receive and transmit at the same time and not during guard periods. For this purpose, the eNB needs to know the UEs TA. |

Rapporteur summary:

All companies agree to Q6.

Proposal 6: From RAN2 perspective, for UE with UE-specific pre-compensation as a baseline it is up to eNB implementation to ensure sufficient time on UE side for the Msg3 transmission for IoT NTN.

TA report during RACH has been discussed in NR NTN WI. The motivation of introducing UE-calculated TA report during RACH is to let gNB know the absolute TA value of the UE, so that gNB could utilize UE’s absolute TA to schedule UL transmission for the UE, which could reduce unnecessary latency. So far, RAN2 has reached the following agreement on TA report.

Agreement in RAN2#104e:

- If enabled by the network, the UE reports information about UE specific TA pre-compensation at the random access procedure (MSGA/MSG3 or MSG5) using a MAC CE. Actual content is FFS and also depends on further RAN1 input (we can revise this whole agreement if RAN1 come to a different conclusion in terms of what needs to be conveyed to the NW)

Agreement in RAN2#105e:

- UE specific TA reporting during RACH procedure is enabled/disabled by SI (FFS for RACH in connected mode)

TA reporting was not discussed during the SI for IoT NTN. In [3], it is proposed to wait for RAN1 to decide whether reporting of the UE specific TA pre-compensation is supported in IoT NTN. However, rapporteur understands that RAN2 could address this TA reporting issue similar as for NR NTN. Anyway, it would be good to collect companies’ views.

Question 7: Do companies agree to support TA report during RACH in IoT NTN?

- Option 1: Yes

- Option 2: No

- Option 3: wait for RAN1 progress.

| Company | Option | Additional comments |
| --- | --- | --- |
| OPPO | Option 1 | We could reuse conclusion in NR NTN as baseline. |
| Xiaomi | Option 2 | For IOT, delay is not an issue, no need to report TA in RACH procedure. |
| CATT | Option 1 | In NR NTN, reporting UE-specific TA is to assist network for the following scheduling. Thus, this method maybe beneficial for scheduling in IOT NTN. |
| Huawei, HiSilicon | Option 3 |  |
| MediaTek | Option 1 | We could reuse conclusion in NR NTN as baseline. |
| Qualcomm | Option 1 | In NR NTN also, RAN2 made agreement on this. |
| Lenovo | Opyion 1 | Align with NR NTN. |
| Nokia | Option 3 | The topic is listed as RAN1 objective in IoT NTN WID (as below). No need to duplicate the discussion in RAN1 and RAN2. "Signalling aspects in UE-specific TA maintenance and reporting, techniques to reduce the signalling load and determination of the UE-specific TA." |
| ZTE | Option 1 | Align with NR NTN. |
| Apple | Option 2 | We agree with Xiaomi that UE specific TA is not so useful for IoT NTN |
| LG | Option 1 |  |
| Sequans | Option 3 |  |
| CMCC | Option 3 | RAN1 discussion goes first. |
| NEC | No strong opinion | We understand TA report in RACH mainly to enable network configure UE special Koffset or enable UE special scheduling timing, so it would be interesting to know whether RAN1 decide to support UE special Koffset in IoT NTN case or not. Without supporting TA report during RACH will simplify network implementation in our understanding. Following NR NTN design is also fine to us. |
| Rakuten Mobile Inc | Option 1 | We think this will enable eNB to schedule more accurately thereby reducing latency |
| Ericsson | Option 1, with comment | This is not only a RAN1 issue. Even if there will be no UE specific Koffset (RAN1 may add it or not) in IoT NTN there is HALF DUPLEX constraints (for NB-IoT and some eMTC cases). To respect a HALF DUPLEX constraint, the eNB must know the UEs TA. Therefore, the TA shall be reported. We propose From a RAN2 perspective, the UE needs to report the TA to enable the eNB to respect HALF DUPLEX constraints including guard periods. |

Rapporteur summary:

9 out of 16 support TA report during RACH in IoT NTN. 2 companies don’t support. 4 companies want to wait for RAN1 progress. 1 company has no strong opinion. Note that in NR NTN, TA report during RACH is discussed and agreed in RAN2. As more companies support option 1, rapporteur suggests to go with it and add the statement that revisiting is pending to RAN1’s different decision.

Proposal 7: TA report during RACH is supported in IoT NTN. This can be revisited if RAN1 decides something that requires to change this.

2.1.2 (UL) HARQ RTT Timer

SI agreements for HARQ-RTT-Timer and UL-HARQ-RTT-Timer enhancement in IoT NTN has been captured in TR36.763, as below.

As the challenges associated with the expiry of MAC timers in NR NTN [3] remain the same in IoT NTN, it is assumed that the same solutions as NR NTN for the start of DL HARQ RTT Timer and UL HARQ RTT Timer can be reused as a baseline to support IoT NTN.

In NR NTN WI, RAN2 has made the following agreement on DL/UL HARQ RTT Timer:

Agreement in RAN2#112-e:

- For UE with pre-compensation capability (at least for the HARQ-feedback enabled case. FFS for HARQ-feedback disabled, if supported), drx-HARQ-RTT-TimerDL is offset by UE-specific RTT (UE-gNB delay) in LEO/GEO. FFS if offset is applied to: 1) the start of the timers or 2) the timer value range (i.e. existing values within value range increased by offset)

Agreement in RAN2#113-e:

- For HARQ processes with DL HARQ feedback enabled, drx-HARQ-RTT-TimerDL length is increased by offset (i.e. existing values within value range increased by offset). RAN2 working assumption: offset is equal to UE-gNB RTT (if RAN1 decides something that requires to change this we can revisit it) (RAN2#113-e)

Agreement in RAN2#114-e:

- The following options are supported for drx-HARQ-RTT-TimerUL in NTN per HARQ process: 1) Timer length is extended by offset; 2) Timer set to zero and/or 3) Timer disabled (i.e. not started). FFS if this is based on explicit configuration or not. We can also come back to see whether both 2 and 3 are needed. - RAN2 working assumption: Offset for drx-HARQ-RTT-TimerUL is equal to UE-gNB RTT (if RAN1 decides something that requires to change this we can revisit it).

In TR36.763, it is described that “It was concluded that from a physical layer perspective, there is no consensus on disabling HARQ feedback for NTN IoT in Rel-17”. Since disabling HARQ feedback is not supported for IoT NTN, adaptation of HARQ RTT Timer and/or UL HARQ RTT timer are needed.

Unlike NR, for eMTC and NB-IoT, both HARQ RTT timer and UL HARQ RTT Timer are derived by a formula in TS36.321. In general, the formula can be expressed as below.

HARQ RTT Timer = TPDSCH to UCI + TUCI + Tprocessing + delta PDCCH

UL HARQ RTT Timer = Tprocessing + delta PDCCH

Where,

- TPDSCH to UCI is the time interval from last PDSCH repetion reception to HARQ-feedback transmission;

- TUCI is HARQ-feedback transmission duration;

- Tprocessing is UE processing delay after PUSCH or HARQ-feedback, which is defined as one or a few milliseconds;

- delta PDCCH is not a fixed value but an offset to align with the next NPDCCH occasion, which applies to NB-IoT only.

In [1], [2], [3], [7], [9] and [10], it is proposed to extend the HARQ RTT Timer length by an offset. Regarding the offset value, there are some different views. In [1], company thinks that the offset value should be max(UE-eNB RTT - Tprocessing, 0), since the UE processing delay after PUSCH or HARQ-feedback transmission and the waiting time of UE-eNB RTT can happen in parallel. In [2], [3], [7], [9] and [10], it is suggested to use UE-eNB RTT as the offset. On the other hand, in [5], it is proposed to delay the start of HARQ RTT timer and/or UL HARQ RTT timer with an offset (e.g. UE-eNB RTT) for IoT NTN.

Based on above, rapporteur would like to ask the following question:

Question 8: For (UL) HARQ RTT Timer enhancement, which is your preferred option?

- Option 1: Add an offset, equal to max(UE-eNB RTT - Tprocessing, 0), to the formula calculating the (UL) HARQ RTT timer

- Option 2: Add an offset, equal to UE-eNB RTT, to the formula calculating the (UL) HARQ RTT timer.

- Option 3: delay the start of (UL) HARQ RTT timer with an offset of UE-eNB RTT.

| Company | Option | Additional comments |
| --- | --- | --- |
| OPPO | Option 1 | Since the UE processing delay after PUSCH or HARQ-feedback transmission and the waiting time of UE-eNB RTT happen in parallel, we think it is reasonable to take the maximum value of the two. |
| Xiaomi | Option 2 | Processing time can be ignored since it is quite small. |
| CATT | Option 2 | We are confusion with option 1, maybe it is max (UE-eNB RTT -Tprocessing, Tprocessing)? In TS36.321, Tprocessing is constant in the formula, meanwhile, it cannot larger than UE-eNB RTT. As the formula to calculate RTT timer in the background is used for TDD where the timing between the PDSCH and the transmission of the associated HARQ feedback is not fixed. Therefore, the simpler solution (option 2) should be used in NTN. |
| Huawei, HiSilicon | Option 2 |  |
| MediaTek | Option 2 |  |
| Qualcomm | Option 2 | It is simple. |
| Lenovo | Option 2 |  |
| Nokia | Option 2 | Follow NR NTN solution is the simple way forward. |
| ZTE | Option 3 | The difference between option 3 and option 1/2 is whether or not invalid PDCCH subframes are counted in the offset. In legacy IoT, the time length of the (UL) HARQ RTT timer are defined with valid subframes. If we go for option 1/2, e.g., to directly add an offset into the time length, we assume such offset should also be counted with valid subframes in order to align with the existing time length definition. We think it’s a bit complicated. For option3, this offset is added before start of (UL) HARQ RTT timer and no transmission and/or reception is performed during this offset. We think it does not matter whether the counted subframes for this offset are valid or invalid. Then option3 may be simpler. |
| Apple | Option 2 |  |
| LG | Option 2 |  |
| Sequans | Option 2 |  |
| CMCC | Option 2 |  |
| NEC | Option2 | Simple |
| Rakuten Mobile Inc | Option 2 | We should follow NR NTN . |
| Ericsson | Postpone | We would like to investigate the issue further. |

Rapporteur summary:

1 out 16 supports option 1. 1 company favors option 3. 1 company suggests to postpone discussion. The other 13 companies are in favor of option 2. Based on majority views, rapporteur suggests to go with option 2.

Proposal 8: An offset, equal to UE-eNB RTT, is added to the formula calculating the (UL) HARQ RTT timer.

2.1.2 SR-Prohibit timer

SI agreements for sr-ProhibitTimer enhancement in IoT NTN have been captured in TR36.763, as below.

The sr-ProhibitTimer will be modified for including larger values to support IoT NTN. Alignment to NR NTN can be considered.

In Rel-17 NR NTN WI, RAN2 has agreed to extend the timer length of sr-ProhibitTimer, and the details are FFS.

In [1], [2], [3], [5], [7], [9] and [10], it is proposed to extend sr-ProhibitTimer to support IoT NTN, and the following options are raised for the details.

- Option 1: increase the sr-ProhibitTimer length by UE-eNB RTT, where the unit of this UE-eNB RTT should be aligned with the configured sr-ProhibitTimer [1][3]

- Option 2: sr-ProhibitTimer value range for eMTC over NTN is extended with INTEGER (8...4096) and INTEGER (8...128) for eMTC and NB-IoT, respectively. [5]

- Option 3: Postpone treatment of sr-ProhibitTimer values until the NR NTN details have been decided. [7][9][10]

All companies seem to consider that the extension of sr-ProhibitTimer is needed. Based on this, rapporteur would like to ask the following question:

Question 9: If sr-ProhibitTimer is extended, which is your preferred option?

- Option 1: increase the sr-ProhibitTimer length by UE-eNB RTT, where the unit of this UE-eNB RTT should be aligned with the configured sr-ProhibitTimer

- Option 2: sr-ProhibitTimer value range for eMTC over NTN is extended with INTEGER (8...4096) and INTEGER (8...128) for eMTC and NB-IoT, respectively.

- Option 3: Postpone treatment of sr-ProhibitTimer values until the NR NTN details have been decided.

| Company | Option | Additional comments |
| --- | --- | --- |
| OPPO | Option 1/3 | We think sr-ProhibitTimer should be increased by UE-eNB RTT. If majority companies prefer option 3, we are ok to postpone the discussion. |
| Xiaomi | Option 3 |  |
| CATT | Option 3 |  |
| Huawei, HiSilicon | Option 1 |  |
| MediaTek | Option 1 |  |
| Qualcomm | Option 1 | This timer just needs extension. |
| Lenovo | Option 1 |  |
| Nokia | Option 3. |  |
| ZTE | Option 2 or Option 3 | In IoT, sr-ProhibitTimer is used to prohibit frequent SR and the value range of sr-ProhibitTimer is defined as multiple times of SR period. For evaluating the maximum value for the sr-ProhibitTimer in IoT over NTN, we think the case that the UL grant scheduling for BSR to UE may be lost or the BSR to eNB may be lost need to be considered. With reference to that in TN network, e.g., at most 7 SR period are set for the sr-ProhibitTimer, we also suggest that at most 7 BSR retransmission (e.g., 7 times of the UE-eNB RTT) need to be considered. Based on the following calculation, we suggest Option2: - For eMTC over LEO, the maximal sr-ProhibitTimer value can be 32 (4ms*7=28, and ceiling to a value with 2^n=32). For eMTC over GEO, the maximal sr-ProhibitTimer can be 4096 (541*7=3787, and ceiling to a value with 2^n=4096). So the final maximum value can be 4096. Here the unit is still the SR period, 1ms, as legacy. - Similarly, for NB-IoT over NTN, the maximal sr-ProhibitTimer value can be 128 (541ms*7/40ms=94.675, and ceiling a value with 2^n =128). Here 40ms is length of nprach-Periodicity. We are also ok to postpone the discussion until more NR NTN agreements are achieved. |
| Apple | Option 1 |  |
| LG | Option 3 |  |
| Sequans | Option 3 |  |
| CMCC | Option 1 or option 3 |  |
| NEC | Option 3 |  |
| Rakuten Mobile Inc | Option 3 | There no opposition for option 1or 2 but it’s better to wait for NR NTN decision. |
| Ericsson | Option 3 | This is not finalized in NR NTN, there is no need to discuss the same thing here until NR NTN have concluded. |

Rapporteur summary:

7 out 16 support option 1. 1 company favors option 2. 11 companies are ok to postpone discussion until the NR NTN details have been decided. Since there is still no agreement in NR NTN, rapporteur suggests to postpone discussion.

Proposal 9: Postpone treatment of sr-ProhibitTimer values until the NR NTN details have been decided.

2.1.4 PUR

In order to improve latency and reduce UE power consumption, PUR has been introduced in NB-IoT and eMTC.

In [1], [6] and [9], it is proposed that the offset for the start of pur-ResponseWindowTimer should be adjusted based on UE-eNB RTT for IoT NTN.

Question 10: Do companies agree that the start of pur-ResponseWindowTimer should be delayed by the UE-eNB RTT for IoT NTN?

| Company | Agree / Disagree | Additional comments |
| --- | --- | --- |
| OPPO | Agree | This is similar as ra-ResponseWindow. |
| Xiaomi | yes |  |
| CATT | Agree |  |
| Huawei, HiSilicon | Agree with comments | In our understanding, enhancements to PUR were considered as not essential and excluded from the WID. If enhancements are supported, then we agree with the proposal |
| MediaTek | Agree |  |
| Qualcomm | Agree | But it is to note that PUR can be supported only in GEO and RAN1 work may be needed. |
| Lenovo | Agree |  |
| Nokia | Agree |  |
| ZTE | Agree |  |
| Apple | Agree |  |
| LG | Agree |  |
| CMCC | Agree |  |
| NEC | Agree |  |
| Rakuten Mobile Inc | Agree |  |
| Ericsson | Agree with comment. | This is a small enhancement acceptable for a non-essential item. If RAN1 discovers issues, we can revisit. We propose From a RAN2 perspective, delayed start of pur-ResponseWindowTimer with UE-eNB RTT can be supported. This can be revised if RAN1 finds issues to support PUR that are not small. |

Rapporteur summary:

All companies agree to this. Rapporteur considers Ericsson’s comments are valid.

Proposal 10: From RAN2’s perspective, delayed start of pur-ResponseWindowTimer with UE-eNB RTT can be supported. This can be revised if RAN1 finds issues to support PUR that are not small.

Regarding pur-ResponseWindowSize, it is proposed in [9] that it is not extended.

Question 11: Do companies agree that pur-ResponseWindowSize is not extended for IoT NTN?

| Company | Agree / Disagree | Additional comments |
| --- | --- | --- |
| OPPO | Agree with comments | If no extension of repetition is required, there is no need to extend the pur-ResponseWindowSize for IoT NTN |
| Xiaomi | yes |  |
| CATT | Agree |  |
| Huawei, HiSilicon | Agree with comments | In our understanding, enhancements to PUR were considered as not essential and excluded from the WID. |
| MediaTe | Agree |  |
| Qualcomm | Agree |  |
| Lenovo | Agree |  |
| Nokia | Agree with comments | Same view as Huawei. Enhancements to PUR is not in the scope of WID. Furthermore, if the start of pur-ResponseWindowSize can be accurately compensated by UE-eNB RTT, there is no need to extend the mac- pur-ResponseWindowSize for IoT NTN |
| ZTE | Agree |  |
| Apple | Agree |  |
| LG | Agree |  |
| CMCC | Agree with comments | PUR enhancements discussion depends on the progress of the IoT-NTN. |
| NEC | Agree |  |
| Rakuten Mobile Inc | Agree | Same view as Nokia. If start of “pur-ResponseWindowSize” compensated accurately then no need to extend. |
| Ericsson | Agree |  |

Rapporteur summary:

All companies agree to this, while some companies consider PUR as not essential or not in the scope of the WID. Anyway, the proposal does not introduce any changes.

Proposal 11: pur-ResponseWindowSize is not extended for IoT NTN.

2.1.5 SPS

UL SPS can be supported in both eMTC and NB-IoT. In NB-IoT, UL SPS is only supported for BSR.

In [3], it is proposed that SPS for eMTC and SPS-BSR for NB-IoT are supported without modification for IoT NTN.

Question 12: Do companies agree that SPS is supported without modification for IoT NTN?

| Company | Agree / Disagree | Additional comments |
| --- | --- | --- |
| OPPO | Agree |  |
| Xiaomi | yes |  |
| CATT | Agree |  |
| Huawei, HiSilicon | Agree |  |
| MediaTek | Agree |  |
| Qualcomm | Agree |  |
| Lenovo | Agree |  |
| Nokia | Agree with comment | Since short and sporadic transmissions is assumed for Rel-17 IoT NTN, we are not sure if SPS could be configured. |
| ZTE | Agree |  |
| Apple | Agree |  |
| LG | Agree |  |
| CMCC | Agree |  |
| NEC | Agree |  |
| Rakuten Mobile Inc | Agree | Same view as Nokia. Short & sporadic transmission should be enough. |
| Ericsson | Agree | May not be useful except in GEO scenarios. |

Rapporteur summary:

All companies agree to this.

Proposal 12: SPS is supported without modification for IoT NTN.

  - RLC

Both AM and UM modes use the t-Reordering timer to control the RLC waiting interval for out-of-order MAC data before considering the missing data as lost and handing any received data off to the PDCP layer. The t-Reordering timer can be configured with fixed values between 0 and 1600ms. Large propagation delay might have impacts on t-Reordering timer. In the SI phase, it is agreed that the value range of the RLC t-Reordering timer will be extended to support IoT NTN.

In [2], [3], [5], [7], [9] and [10] it is proposed to extend the value range of t-Reordering. For the exact value, in [5], it is further suggested that the RLC t-Reordering timer value is extended with ENUMERATED (ms3200, ms6400) for IoT NTN, and in [3] and [9], it is proposed to FFS on the new values. On the other hand, it is stated in [8] that as the target data rates in NB-IoT are much lower than NR, and data transmission consists of a pretty small number of packets over a relatively long period of time, there is no need to extend RLC t-Reordering timer.

Question 13: Do companies confirm the SI agreement that the value range of the RLC t-Reordering timer will be extended to support IoT NTN?

| Company | Yes / No | Additional comments |
| --- | --- | --- |
| OPPO | Yes | In order to accommodate large propagation delay in NTNT, RLC t-Reordering timer need to be extended. the exact value can be FFS. |
| Xiaomi | yes | For NR NTN, It was agreed that: “The value range of t-Reassembly shall be extended. The following set of values are possibly added for t-Reassembly timer: {ms210, ms220, ms340, ms350, ms550, ms1100, ms1650, ms2200}” Similar to NR RLC t-Reassembly, LTE RLC t-Reordering should also be extended. |
| CATT | Agree |  |
| Huawei, HiSilicon | Yes |  |
| MediaTek | Preferably No |  |
| Qualcomm | Yes |  |
| Lenovo | Yes |  |
| Nokia | Yes |  |
| ZTE | Agree | Considering the large RTT of GEO (e.g. about 541ms), the number of DL data PDU will be less than that in TN in a certain period. To avoid unnecessary PDU re-ordering and re-transmission, the value range of RLC t-Reordering timer should be extended. Comparing with the largest PDU transmission interval of eMTC (e.g. r256 is set to mpdcch-NumRepetition, and n128 is set to pucch-NumRepetitionCE), the large RTT of GEO (e.g. about 541ms) corresponds to 2 times of the largest PDU transmission interval. So the maximal value of RLC t-Reordering timer can be only enlarged by 2 times, e.g. the extended RLC t-Reordering timer value can be ENUMERATED (ms3200, ms6400). Considering that the largest PDU transmission interval of NB-IoT (e.g. the largest value of npdcch-NumRepetitions is r2048) is larger than that of eMTC, the value range of RLC t-Reordering timer extended for eMTC is enough for NB-IoT. |
| Apple | Yes |  |
| LG | Yes |  |
| Sequans | Yes |  |
| CMCC | Yes | To adapt to the large RTT of NTN scenarios. |
| NEC | Yes | Follow the SI agreement |
| Rakuten Mobile Inc | Yes | We think only incase GEO need RLC t-Reordering timer need to be extended. |
| Ericsson | Yes |  |

Rapporteur summary:

Majority (15 out of 16) companies agree to this.

Proposal 13: RAN2 confirm the SI agreement that the value range of the RLC t-Reordering timer will be extended to support IoT NTN.

  - PDCP

The transmitting PDCP entity shall discard the PDCP SDU when the discardTimer expires for a PDCP SDU or when a status report confirms the successful delivery. The discardTimer can be configured up to 1500ms for eMTC and up to 81920ms for NB-IoT, or can be switched off by choosing infinity. The discardTimer mainly reflects the QoS requirements of the packets belonging to a service.

The following note is given in TR36.763

NOTE: PDCP discardTimer enhancements can be considered during the Work Item phase provided the impact to the Technical Specifications is minimal.

In [5], it is stated that the PDCP discardTimer for eMTC should be enlarged by 2 times, e.g. the extended PDCP discardTimer for eMTC can be ENUMERATED (ms3000, ms6000), and considering that the PDCP discardTimer for NB-IoT is already large enough, it is not necessary to be extended any more.

Question 14: Do companies agree that the PDCP discardTimer should be extended to support IoT over NTN?

| Company | Agree / Disagree | Additional comments |
| --- | --- | --- |
| OPPO | Disagree with comments | Since discardTimer mainly reflects the QoS requirements of the packets belonging to a service, we see no need to extend this timer unless new QoS requirement are defined in SA2 for IoT NTN. |
| Xiaomi | yes | If t-Reordering is agreed to be extended to 2200ms similar to t-Reassembly, considering that RAN2 has agreed that the values of PDCP discardTimer should be greater than the RLC t-Reassembly timer, the current maximum 1500ms PDCP discardTimer value needs to be extended. Although PDCP discardTimer extension is not essential, but since the change is small, RAN2 has decided it can be considered. Besides, the new defined 5QI for NTN is applicable to LTE-M connected to 5GS case. |
| CATT | Agree |  |
| Huawei, HiSilicon | Disagree with comments | In our understanding, enhancements to PDCP discardTimer were considered as not essential and excluded from the WID. |
| MediaTek | See Comment | It should be possible to configure PDCP Discard timer larger than RLC t-Reordering timer. |
| Qualcomm | Agree for eMTC | For NB-IoT, we agree this timer does not need to be extended. |
| Lenovo | Postpone | Depends on whether SA2 define new QoS requirement. |
| Nokia | Disagree | Since there is no new QoS requirement for IoT NTN service, it is not necessary to extend the PDCP discardTimer. |
| ZTE | Agree | With similar reason for extending RLC t-Reordering timer, the PDCP discardTimer for eMTC should also be enlarged by 2 times, e.g. the extended PDCP discardTimer for eMTC over NTN can be ENUMERATED (ms3000, ms6000). Considering that the value range of PDCP discardTimer for NB-IoT is already large enough, it is not necessary to be extended any more. |
| Apple | Disagree | PDCP discardTimer is used for QoS purposes and should not be updated based on lower layer considerations. |
| LG | Disagree | Same view as OPPO |
| Sequans | Agree |  |
| CMCC | Agree for met | While for NB-IoT, whether new QoS requirement will be introduced should be studied first. |
| NEC |  | Should we wait and follow NR NTN agreement |
| Rakuten Mobile Inc | Agree | We need to extend for eMTC for sure ,but also need to extend a bit for NB IoT to support GEO scenarios. |
| Ericsson | Disagree | Agree with Nokia. |

Rapporteur summary:

6 out of 16 disagree to extend the PDCP discardTimer. 7 companies think extension is needed and 5 companies out to these 7 clearly mention that the extension is needed for eMTC, but not needed for NB-IoT. 2 suggest to wait. For NB-IoT, rapporteur suggests to go for majority views not to extend the timer. For eMTC, given the split views, rapporteur suggest to further discuss this.

Proposal 14a: Do not extend the PDCP discardTimer for NB-IoT over NTN.

Proposal 14b: RAN2 to further discuss whether to extend the PDCP discardTimer for eMTC over NTN.

In [8], it is mentioned that in IoT NTN the data rates are much lower, and data transmission will consist of a pretty small number of packets over a relatively long period of time. Hence, there is no need to extend PDCP t-Reordering timer and the existing range of the PDCP t-Reordering timer is sufficient.

Question 15: Do companies agree that there is no need to extend PDCP t-Reordering for IoT NTN?

| Company | Agree / Disagree | Additional comments |
| --- | --- | --- |
| OPPO | Disagree | In LTE, PDCP t-Reordering can only be used for LWA bearers and PDCP duplication. If these two features are not supported by eMTC and NB-IoT, there would be no impact on PDCP t-Reordering for IoT NTN. |
| Xiaomi | See comment | PDCP t-Reordering is only applicable to LWA, which is not valid for IOT NTN. |
| CATT | Agree |  |
| Huawei. HiSilicon |  | PDCP t-Reordering does not apply to IOT. At least it does not apply to NB-IoT. |
| MediaTek | Agree |  |
| Qualcomm | Disagree | Not applicable |
| Lenovo | Disagree | Does not apply. |
| Nokia | Agree |  |
| ZTE | Agree |  |
| Apple | Agree | We agree that there is no need to extend PDCP t-Reordering. Companies disagreeing above seem to be actually agreeing? |
| LG | Disagree | PDCP t-Reordering is used only when the PDCP entity is associated with at least two RLC entities. Thus, in the current specification, PDCP t-Reordering is not used for IOT NTN. |
| Sequans |  | Not applicable |
| CMCC | Disagree | The feature may be not applicable. |
| NEC |  | Not applicable |
| Rakuten Mobile Inc | Agree | Agree means actually no need to extend timer since it’s not applicable to NB IoT. |
| Ericsson | Agree that there is no need to extend PDCP t-Reordering |  |

Rapporteur summary:

Rapporteur understands that companies replying with “disagree” all mean the timer is not applicable for IoT NTN.

Proposal 15: Do not extend PDCP t-Reordering for IoT NTN.

3. Summary and Proposals

This section summarizes the discussion and reports the following proposals:

Proposal 1: Start of ra-ResponseWindow is delayed by an offset. Postpone discussion on the offset value until further agreements regarding RACH are made in RAN1.

Proposal 2: If the start of the RA Response window is accurately compensated by UE-eNB RTT and no extension of repetition is required, there is no need to extend the ra-ResponseWindowSize for IoT NTN.

Proposal 3: Start of mac-ContentionResolutionTimer is delayed by an offset, which is equal to UE-eNB RTT. This can be revisited if RAN1 decides something that requires to change this.

Proposal 4: If the start of mac-ContentionResolutionTimer is accurately compensated by UE-eNB RTT and no extension of repetition is required, there is no need to extend the mac-ContentionResolutionTimer for IoT NTN.

Proposal 5: RAN2 assume that UE determines UE-eNB RTT in a similar way as in NR NTN, i.e., the UE-eNB RTT is the sum of UE’s TA and K_mac. This can be revisited if RAN1 decides something that requires to change this.

Proposal 6: From RAN2 perspective, for UE with UE-specific pre-compensation as a baseline it is up to eNB implementation to ensure sufficient time on UE side for the Msg3 transmission for IoT NTN.

Proposal 7: TA report during RACH is supported in IoT NTN. This can be revisited if RAN1 decides something that requires to change this.

Proposal 8: An offset, equal to UE-eNB RTT, is added to the formula calculating the (UL) HARQ RTT timer.

Proposal 9: Postpone treatment of sr-ProhibitTimer values until the NR NTN details have been decided.

Proposal 10: From RAN2’s perspective, delayed start of pur-ResponseWindowTimer with UE-eNB RTT can be supported. This can be revised if RAN1 finds issues to support PUR that are not small.

Proposal 11: pur-ResponseWindowSize is not extended for IoT NTN.

Proposal 12: SPS is supported without modification for IoT NTN.

Proposal 13: RAN2 confirm the SI agreement that the value range of the RLC t-Reordering timer will be extended to support IoT NTN.

Proposal 14a: Do not extend the PDCP discardTimer for NB-IoT over NTN.

Proposal 14b: RAN2 to further discuss whether to extend the PDCP discardTimer for eMTC over NTN.

Proposal 15: Do not extend PDCP t-Reordering for IoT NTN.

4. References

- R2-2107082	Discussion on UP impact for IoT over NTN	OPPO	discussion	Rel-17	LTE_NBIOT_eMTC_NTN

- R2-2107320	User Plane Impact for IOT NTN	CATT	discussion	Rel-17	LTE_NBIOT_eMTC_NTN

- R2-2107425	User plane for IOT NTN	Huawei, HiSilicon	discussion	Rel-17	LTE_NBIOT_eMTC_NTN

- R2-2107614	Provision of ephemeris	Apple	discussion	Rel-17	LTE_NBIOT_eMTC_NTN

- R2-2107766	User plane aspects of IoT NTN	ZTE Corporation, Sanechips	discussion	Rel-17	FS_LTE_NBIOT_eMTC_NTN

- R2-2107915	Further enhancement for PUR in IoT NTN	Lenovo, Motorola Mobility	discussion	Rel-17

- R2-2108117	Discussion on User Plane impact for IoT NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-17	LTE_NBIOT_eMTC_NTN

- R2-2108335	On User-Plane Timers in NB-IoT based NTN	MediaTek Inc.	discussion

- R2-2108454	User plane aspects of NB-IoT and LTE-M in NTNs	Ericsson	discussion	Rel-17	LTE_NBIOT_eMTC_NTN

- R2-2108529	User plane for IoT-NTN	CMCC	discussion	Rel-17	LTE_NBIOT_eMTC_NTN

Contact information

| Company | Delegate contact |
| --- | --- |
| COMPANY_NAME | NAME (email@address.com) |
| OPPO | Haitao Li(lihaitao@oppo.com) |
| Huawei, HiSilicon | odile.rollinger@huawei.com |
| MediaTek | Abhishek.Roy@mediatek.com |
| Qualcomm | bshrestha@qti.qualcomm.com |
| Lenovo | Min Xu (xumin13@lenovo.com) |
| Nokia | Ping.1.Yuan@Nokia-sbell.com |
| ZTE | Ting Lu (lu.ting@zte.com.cn) |
| LG | Geumsan Jo (geumsan.jo@lge.com) |
| Sequans | Olivier Marco (omarco at sequans.com) |
| NEC | Yuhua.chen@emea.nec.com |
| Rakuten Mobile Inc | Pankaj Shete ( pankaj.shete@rakuten.com) |