3GPP TSG-RAN WG2 Meeting #115e	R2- 2109076

# e-Meeting,  16th - 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.323 | CR | 0296 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for LTE PDCP operation after DAPS release |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh-Core |  |  |  |  |  | Date: |  |  | 2021-08-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In LTE, the PDCP operation is explicitly clarified in both cases that split bearer/LWA bearer is configured and it is re-configured to normal DRB in Section 5.1.2.1.4. However, for DAPS handover, the LTE PDCP operation is specified only for the case that PDCP entity is reconfigured to configure DAPS and thus it is not clear which PDCP operation would be performed in the case that PDCP entity is reconfigured to release DAPS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To clarify the LTE PDCP operation in the case that PDCP entity is reconfigured to release DAPS. Impact analysis Impacted functionality: PDCP procedures Inter-operability: No interoperability issue would be foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The LTE PDCP operation is not clear in case that PDCP entity is reconfigured to release DAPS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.2.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modification>

5.1.2.1.4	Procedures for DRBs mapped on RLC AM or RLC UM, for LWA bearers and SLRB when the reordering function is used

For DRBs mapped on RLC AM and RLC UM, for LWA bearers and when PDCP duplication is used, the PDCP entity shall use the reordering function as specified in this clause when:

-	the PDCP entity is associated with two RLC entities; or

-	the PDCP entity is configured for a LWA bearer; or

-	the PDCP entity is associated with one AM RLC entity after it was, according to the most recent reconfiguration, associated with two AM RLC entities or configured for a LWA bearer without performing PDCP re-establishment; or

-	the PDCP entity is configured with PDCP duplication; or

-	the PDCP entity is configured for a DAPS bearer; or

-	the PDCP entity is not configured for a DAPS bearer after it was, according to the most recent reconfiguration, configured for a DAPS bearer; or

-	the PDCP entity is associated with at least one RLC entity configured with rlc-OutOfOrderDelivery.

For SLRBs mapped on RLC UM, the PDCP entity shall use the reordering function as specified in this clause when:

-	the PDCP entity receives a PDCP SN which is not "0".

The PDCP entity shall not use the reordering function in other cases.

<End of modification>