3GPP TSG-RAN WG2 #115e	Tdoc R2-2109101

Electronic meeting, 16th – 27th August, 2021

Agenda Item:	8.22 (NR R17 Other)

Source:	Ericsson

Title:	Band n77 issues in the US – Comparison of solutions

Document for:	Discussion and Agreement

# 1	Introduction

In the following table, we compare the signalling solutions that are under discussion in RAN2 and RAN4 to solve the n77 band issues in the US.

|  | Initial Access | Mobility and SCell addition | Mobility from EUTRA & EN-DC | Notes |
| --- | --- | --- | --- | --- |
| Capability bit in UE-NR-Capabilities | Does not prevent access by non-certified UEs. | Possible but additional implementation required | eNB unaware (gNB rejects incoming request) | ASN.1 change required. |
| Capability bit in UE-NR-Capabilities and in UE-EUTRA-Capabilities | Does not prevent access by non-certified UEs. | Possible but additional implementation required | Possible but additional implementation and configuration required also in eNB. | ASN.1 change required. |
| Modified MPR behaviour | Does not prevent access by non-certified UEs. | Possible but additional implementation required | eNB unaware (gNB rejects incoming request) | No ASN.1 change required. Misuse of MPR and NS value. |
| New NS-Value | Possible DoD cell broadcasts new NS-Value which prevents legacy UEs from accessing it. | Cannot be used (gNB unaware of supported NS values) | Cannot be used (eNB unaware of supported NS values) | No ASN.1 change required. Misuse of MPR and NS value. Bad precedence of optional NS-Value. |
| New NS-Value + New Capability | Possible | Possible but additional implementation required | Possible but additional implementation required | ASN.1 change required. Bad precedence of optional NS-Value. |
| New NS-Value + Modified MPR behaviour | Possible | Possible but additional implementation required | eNB unaware (gNB rejects incoming request) | No ASN.1 change required. Bad precedence of optional NS-Value. Misuse of MPR and NS value. |
| New band number (same definition, range, requirements and BC as n77) | Possible. DoD cell broadcasts only the new band number. Legacy n77 cells in CBRS spectrum broadcast n77 and nXX. | Possible. Existing procedures. | Possible. Existing procedures. | No ASN.1 change. No procedural change. If NW requests BCs for n77 and nXX, UEs supporting both bands send BCs for both Increased size of UE capabilities. |
| New band number + Rule to omit BCs for nXX in UE capabilities (but support the same as for n77). | Possible. (same as above) | Possible but additional implementation required (gNB must treat reported n77 BC as nXX BC in capability evaluation) | Possible. (same as above since eNB does not care about NR band parameters in BC) | No ASN.1 change. UE lists nXX in band list but not in BC list. |