3GPP TSG RAN WG2#115-e	R2-2109098

e-Meeting, 9th - 27th August, 2021

Title:	LS on initial state of elements controlled by MAC CEs

Response to:

Release:	Release 15, Release 16

Work Item:	NR_newRAT-Core

Source:	RAN2

To:	RAN1

Cc:	RAN4

Contact Person:

#### Name:	Chong Lou

#### E-mail Address:	louchong@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: -

1. Overall Description:

RAN2 has discussed the handling of the radio resources/states which are de-/activated by MAC CEs. RAN2 has identified some unclarities in 38.321. Since the corresponding text was based on the MAC CEs from RAN1, RAN2 would like to check the following UE behaviours with RAN1.

--------------------------------------------------------------From 38.321 ----------------------------------------------------------------

..

5.18.2 The configured Semi-persistent CSI-RS/CSI-IM resource sets are initially deactivated upon configuration and after a handover.

..

5.18.4 The configured TCI states for PDSCH are initially deactivated upon configuration and after a handover.

..

5.18.6 The configured Semi-persistent CSI reporting on PUCCH is initially deactivated upon configuration and after a handover.

..

5.18.7 The configured Semi-persistent SRS resource sets are initially deactivated upon configuration and after a handover.

..

5.18.9 The configured Semi-persistent ZP CSI-RS resource sets are initially deactivated upon configuration and after a handover.

5.18.17 The configured resource sets Semi-persistent Positioning SRS are initially deactivated upon configuration and after a handover.

--------------------------------------------------------------From 38.321 ----------------------------------------------------------------

Firstly, since a handover only refers to PCell change, RAN2 respectfully asks RAN1 to clarify whether the initial deactivation when using handover should be applied for both PCell change and PSCell change/addition in case of DC.

Secondly, RAN2 respectfully asks RAN1 to clarify whether the initial deactivation when using configuration should be applied for both “initial configuration by RRC” and “reconfiguration by RRC”.

Thirdly, RAN2 respectfully asks RAN1 to clarify whether the UE behavior relevant to (Enhanced) PUCCH spatial relation Activation/Deactivation MAC CE should be aligned with the other MAC CEs.

2. Actions:

To RAN1:

RAN2 respectfully asks RAN1 to answer the three questions above.

3. Date of Next TSG-RAN2 Meetings:

TSG-RAN2 Meeting #116-e	11 October – 19 October 2021