3GPP TSG-RAN2 Meeting #116bis Electronic                R2-2200207

# 17th January – 25th January 2022

Agenda item:	8.19.2

Source:	Samsung

Title:	RA Procedure Aspects

Document for:	Discussion & Decision

# Introduction

In this contribution we discuss RA procedure aspects for feature/feature combination specific RACH.

# Discussion

RAN2 has discussed the RA procedure aspects for feature/feature combination specific RACH in RAN2 #115e and made the following agreements.

RAN2 #115 Agreements [1]

1.	As a baseline, the RA procedure design for Rel-17 should adhere to the following general principles:

a:  Carrier selection (between NUL/SUL) should happen ahead of the initial RACH resource selection (i.e. feature combination is not considered in carrier selection).

b:  Initial RACH resource should be selected based on the selected carrier for the selected feature combination (i.e., selected slice, SDT or not, REDCAP or not etc). Only the RACH resource matching the feature and/or feature combination of current RACH procedure will be considered as available in the RACH resource selection.

c:  As a general rule, all RACH retransmissions (if any are needed, until RACH failure happens) shall be performed over the same RACH resources (and same carrier – NUL/SUL) as the one selected for initial RACH resource.  However, we can discuss fallback on a case by case basis if there is a strong motivation and discuss them together in this AI.

In R17, there can be several feature/feature combinations. Network can configure several RACH configuration (s) where each RACH configuration corresponds to a feature/feature combination. A cell may support one or more features. Will the network provide RACH configurations for each possible combinations of the supported features or is the network allowed to configure RACH configurations for a subset of combination of the supported features? For example, let’s say the cell supports SDT and both redcap UEs and non-redcap UEs. Cell does not support coverage extension for Msg3 and slice based RACH. In this case does cell has to support all the below configurations? Is the cell allowed to skip RACH configuration for some combination, e.g. it may not provide RACH configuration for combination 2 i.e. redcap + SDT? If RACH configuration for combination 2 is not configured, will the redcap UE not perform SDT i.e. uses RACH configuration for combination 0 or can it still perform SDT using RACH configuration for combination 1?

| Feature/Feature Combination | Redcap | SDT |
| --- | --- | --- |
| 0 | Yes | No |
| 1 | No | Yes |
| 2 | Yes | Yes |
| 3 (legacy) | No | No |

One can argue that in the above example if Msg3/msgA based redcap indication using LCID is supported, redcap UE still use the RACH configuration for combination 1.

In RAN2#116e [2], it was agreed that:

- RAN2 assumes that the network may not provide all possible permutation.

- FFS whether the selection in case of missing combination is specified or left to UE implementation

If the network is allowed to configure RACH configurations for a subset of all possible combinations of the features supported in the network:

- If RACH configuration for a combination of the features (supported by network) for which UE needs to initiate RACH is not available, the order in which the UE prioritize the features to select the feature combination from the feature combinations for which RACH configurations are available, can be left to UE implementation. We need to only mandate that UE can select a RACH configuration corresponding to a combination of features only if all those features are applicable for the initiated random access procedure.

Proposal 1: If RACH configuration for a combination of the features (supported by network) for which UE needs to initiate RACH is not available, the order in which the UE prioritize the features to select the feature combination from the feature combinations for which RACH configurations are available, can be left to UE implementation.

- UE can select a RACH configuration corresponding to a combination of features only if all those features are applicable for the initiated random access procedure.

Fallback during RA procedure

For RAN slicing, as per the discussion in RAN2 #115, following is captured in chairman notes:

- For RACH type selection, UE first selects between slice-specific and common RACH, then selects between 2-step and 4-step.

- The following fallback case is supported:

  - Fallback case 2: Fallback from 2-step slice specific RACH to 4-step common RACH, if 4-step slice specific RACH is not configured.

According to this during random access procedure, UE can fallback from slice specific RACH configuration to non-slice specific RACH configuration. This is not aligned with the following agreement in common RACH session:

- As a general rule, all RACH retransmissions (if any are needed, until RACH failure happens) shall be performed over the same RACH resources (and same carrier – NUL/SUL) as the one selected for initial RACH resource.

Network may configure only slice specific 2 step RA for fast access users belonging to certain slices. However, network may not configure slice specific 4 step RA configuration to be used during switching (after transmitting MsgA configurable number of times) from 2 step RA to 4 step RA, as switching does not happen often. However, switching is still beneficial for random access to be successful and non-slice specific common configuration can be used for this purpose.

Proposal 2: RAN2 to discuss whether the Fallback from 2-step slice specific RACH to 4-step common

RACH, if 4-step slice specific RACH is not configured.

# Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

Proposal 1: If RACH configuration for a combination of the features (supported by network) for which UE needs to initiate RACH is not available, the order in which the UE prioritize the features to select the feature combination from the feature combinations for which RACH configurations are available, can be left to UE implementation.

- UE can select a RACH configuration corresponding to a combination of features only if all those features are applicable for the initiated random access procedure.

Proposal 2: RAN2 to discuss whether the Fallback from 2-step slice specific RACH to 4-step common    RACH, if 4-step slice specific RACH is not configured.

# References

[1] RAN2-115-e_ Session Notes URLLC_Small Data_RACH config

[2] RAN2-116-e_ Session Notes URLLC_Small Data_RACH config