3GPP TSG-RAN WG2 Meeting #116-bis-e	R2-2200299

Electronic meeting, January 17th – 25th, 2022

Agenda item:		8.11.7

Source:				CATT

Title:					Discussion on mitigating UE and gNB RxTx timing delays

Document for:	Discussion and Agreement

1	Introduction

In [1], RAN1 sent LS to RAN2 to indicate the UE Tx TEG reporting as an outcome of the discussion of “Accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays for NR positioning enhancements” at RAN1#107e. In [2], RAN1 sent all parameters including UE Rx/Tx and gNB Rx/Tx timing delays for NR positioning enhancements to RAN2 together with the corresponding RAN1’s agreements, but leave it to RAN2 on how and where to capture them.

Agreement: Confirm and modify the working assumption with the following modifications: - For mitigating UE Tx timing errors for UL TDOA, subject to UE’s capability, support the serving gNB to request a UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB if the UE supports multiple UE Tx TEGs for UL TDOA. - The serving gNB should forward the association information provided by the UE to the LMF. - FFS: whether to support the serving gNB to forward the association information to the neighboring gNBs - UE should report its capability of supporting multiple UE Tx TEGs for UL TDOA to serving gNB. - For mitigating UE Tx timing errors for Multi-RTT, subject to UE’s capability, support the LMF to request a UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF if the UE supports multiple Tx TEGs for Multi-RTT. - FFS: whether to support the LMF to forward the association information to the serving and neighboring gNBs - UE should report its capability of supporting multiple UE Tx TEGs for Multi-RTT directly to the LMF. - Note: For mitigating UE Tx timing errors when both UL-TDOA and Multi-RTT, or UL-TDOA and DL-TDOA are used, the UE should provide the association information of UL SRS resources for positioning with Tx TEGs, subject to UE capability (in the bullets above): - to the serving gNB if a request to provide the association information is received from the gNB - to the LMF if a request to provide the association information is received from the LMF. - FFS: Mitigation of UE Tx timing errors when Multi-RTT, UL-TDOA and/or DL-TDOA are used.

Agreement - For UL-TDOA, supporting the following for the serving gNB to request a UE to report the Tx TEG association information between UE Tx TEG IDs and SRS resources for positioning, subject to UE capability of supporting UE Tx TEG: - Based on a configured periodicity, a UE may report the UE Tx TEG association for the SRS resources for positioning that have already been transmitted during the configured period - It is up to RAN2 to decide how to indicate the change of the Tx TEG association during the configured period (e.g., using the timestamps) - It is up to RAN4 to decide when the Tx TEG association is changed - The values of the configurable periodicities are up to RAN2 - Note: Tx TEG association information reporting by single request/response mode is assumed already supported with the previous agreement. - Send an LS to RAN2/RAN4 (cc: RAN3) - to RAN2, including the following RAN1’s agreement related to the reporting of the UE Tx TEG, for RAN2 to work on the signaling - to RAN4 for checking the agreement and work on how to decide when the Tx TEG association is changed

This discussion paper will discuss how to enable the signalling support according to RAN1’s agreements related to the mitigation of UE Rx/Tx and gNB Rx/Tx timing delays.

2	Summary of parameters UE Rx/Tx and/or gNB Rx/Tx timing delays in RAN1

Based on the consolidated higher layers parameter list for Rel-17 NR in [2] and the LS in [1], the UE measurement report for mitigation of UE Rx/Tx timing delays can be grouped in below table:

Table 1. UE Measurements report for mitigation of UE Rx/Tx and gNB Rx/Tx timing delays

| Positioning Methods | UE (Measurements) Report | Signalling between UE and NW(LMF/NG-RAN node) |
| --- | --- | --- |
| UL-TDOA | UE Tx TEG - ueTxTEG-ID - [srs-PosResourceSetId] - srs-PosResourceId | UE serving gNB LMF |
| Multi-RTT | ueRxTxTEG-ID-group with UE Rx-Tx time difference measurements - ueRxTxTEG-ID - ueTxTEG-ID - ueRxTEG-ID Note: Multiple UE Rx-Tx time difference measurements can be obtained from: - different DL PRS resources with the same UE Rx/RxTx TEGs - the same DL PRS resources with different UE Rx/RxTx TEGs | UE LMF |
|  | UE Tx TEG - ueTxTEG-ID - [srs-PosResourceSetId] - srs-PosResourceId | UE LMF |
| DL-TDOA | UE RxTEG-ID with RSTD measurements Note: Multiple RSTD measurements can be obtained from: - the same DL PRS resources with different UE Rx TEGs - different DL PRS resources with the same UE Rx TEG | UE LMF |

Based on the consolidated higher layers parameter list for Rel-17 NR in [2], the gNB measurement report for mitigation of gNB Rx/Tx timing delays can be grouped in below table:

Table 2. gNB Measurements report for mitigation of UE Rx/Tx and gNB Rx/Tx timing delays

| Positioning Methods | gNB Measurements Report | Signalling between gNB and LMF |
| --- | --- | --- |
| DL-TDOA | trpTxTEG - trp TxTEG-ID - NR-DL-PRS-ResourceSetID - NR-DL-PRS-ResourceID | gNB LMF |
| Multi-RTT | trpRxTxTEG-ID-group with gNB Rx-Tx time difference measurements - trpRxTxTEG-ID - trpTxTEG-ID - trpRxTEG-ID Note: Multiple gNB Rx-Tx time difference measurements can be obtained from: • the same UL SRS resources with different trp Rx/RxTx TEGs • different UL SRS resources with the same trp Rx/RxTx TEGs | gNB LMF |
|  | trpTxTEG - trp TxTEG-ID - NR-DL-PRS-ResourceSetID - NR-DL-PRS-ResourceID | gNB LMF |
| UL-TDOA | trp RxTEG-ID with RTOA measurements Note: Multiple RTOA measurements can be obtained from: • the same UL SRS resources with different trp Rx TEGs • different UL SRS resources with the same trp Rx TEG | gNB LMF |

For stage 2, the three RAT-dependent techniques including UL-TDOA, Multi-RTT and DL-TDOA will be impacted for accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays for NR positioning enhancements according to RAN1 LS [1] [2].

The high-level LPP procedures(Request/Provide Location Information, Capabilities) and NRPPa procedures needs to be extended for mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays for NR positioning enhancements according to RAN1 LS [1] [2].

For stage 2, UE Tx TEG reporting needs to be captured in RRC procedure and other reportings, including UE RxTxTEG-ID-group and UE RxTEG-ID for Multi-RTT and DL-TDOA need to be captured in LPP and  NRPPa procedure according to RAN1 LS [1] [2] as shown in Table 1 and Table 2.

3	Stage-2 impact

3.1	Procedure impact

3.1.1 RRC Procedure

The agreement in RAN1 shows that UE Tx TEG association information reporting can be a single request/response between UE and serving gNB and periodic reporting. For periodic reporting, RAN2 needs to specify the configurable periodicities:

- For UL-TDOA, supporting the following for the serving gNB to request a UE to report the Tx TEG association information between UE Tx TEG IDs and SRS resources for positioning, subject to UE capability of supporting UE Tx TEG:

  - Based on a configured periodicity, a UE may report the UE Tx TEG association for the SRS resources for positioning that have already been transmitted during the configured period

    - It is up to RAN2 to decide how to indicate the change of the Tx TEG association during the configured period (e.g., using the timestamps)

    - It is up to RAN4 to decide when the Tx TEG association is changed

  - The values of the configurable periodicities are up to RAN2

  - Note: Tx TEG association information reporting by single request/response mode is assumed already supported with the previous agreement.

For stage 2, the single request/response mode reporting and periodic reporting of the UE Tx TEG association information for UL-TDOA positioning method need to be added in RRC procedure. RAN2 needs to specify the configurable periodicities for periodic reporting.

3.1.2 LPP and NRPPa Procedure

Based on the summary table of parameters UE Rx/Tx and/or gNB Rx/Tx timing delays included the consolidated higher layers parameter list for Rel-17 NR in [2], both LPP procedures(Request/Provide Location Information, Capabilities) and NRPPa procedures can be extended.

3.2	Positioning methods Impact

3.2.1 DL-TDOA

Information that may be transferred from the LMF to UE

| Information | UE-assisted | UE-based |
| --- | --- | --- |
| Request UE to report UE RxTEG-ID associated with RSTD measurements | Yes | No |
| Request UE to measure the same DL PRS with different UE Rx TEGs and report the corresponding multiple RSTD measurements | Yes | No |
| Provide TRP TxTEG to UE | No | Yes |

Information that may be transferred from the UE to LMF

| Information | UE-assisted | UE-based |
| --- | --- | --- |
| UE Rx TEG-IDs associated with RSTD measurements | Yes | No |
| Timestamp of the measurements | Yes | No |

Information that may be transferred from the LMF to gNB

Request gNB to provide TRP TxTEG information

Information that may be transferred from the gNB to LMF

TRP TxTEG information (TRP TxTEG-ID, DL-PRS-ResourceSetID, and DL-PRS-ResourceID)

3.2.2 UL-TDOA

Information that may be transferred from the serving gNB to UE via RRC

Measurement request

Request UE to provide UE TxTEG information

Information that may be transferred from UE to the serving gNB via RRC

Measurement results

UE Tx TEGs (TxTEG-ID, srs-PosResourceSetId, srs-PosResourceId)

Timestamp of the measurement

Information that may be transferred from the LMF to gNB

Request gNB to provide TRP RxTEG IDs with RTOA measurements

Information that may be transferred from the gNB to LMF

TRP Rx TEG-IDs associated with RTOA measurements

3.2.3 Multi-RTT

Information that may be transferred from LMF to the UE

Request UE to report UE RxTxTEG-ID-group information

Request UE to report UE Tx TEG association information

Request UE to measure the same DL-PRS with different UE Rx TEGs for UE Rx-Tx measurements

Request UE to measure the same DL-PRS with different UE RxTx TEGs for UE Rx-Tx measurements

Information that may be transferred from UE to the LMF

UE RxTxTEG-ID-group (UE RxTxTEG-ID, UE TxTEG-ID, UE RxTEG-ID) with UE Rx-Tx time difference measurements

UE Tx TEG association information (UE TxTEG-ID, posSRS-ResourceSetID, posSRS-ResourceID)

Measurement results that may be transferred from gNBs to the LMF

TRP TxTEG association information (TRP TxTEG-ID, DL-PRS-ResourceSetID, DL-PRS-ResourceID)

TRP RxTxTEG-ID-group (TRP RxTxTEG-ID, TRP TxTEG-ID and TRP RxTEG-ID)

TRP Measurement request information that may be transferred from LMF to gNBs

Request gNB to provide TRP RxTxTEG-ID-group information

Request gNB to measure the same UL positioning SRS resource with different TRP RxTx TEGs for gNB Rx-Tx measurements

Request gNB to measure the same UL positioning SRS resource with different TRP Rx TEGs for gNB Rx-Tx measurements

For stage 2, capture the TP of stage-2 on TEG for accuracy enhancement as baseline in Annex.

5	Conclusion

- For stage 2, the three RAT-dependent techniques including UL-TDOA, Multi-RTT and DL-TDOA will be impacted for accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays for NR positioning enhancements according to RAN1 LS [1] [2].

The high-level LPP procedures(Request/Provide Location Information, Capabilities) and NRPPa procedures needs to be extended for mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays for NR positioning enhancements according to RAN1 LS [1] [2].

For stage 2, UE Tx TEG reporting needs to be captured in RRC procedure and other reportings, including UE RxTxTEG-ID-group and UE RxTEG-ID for Multi-RTT and DL-TDOA need to be captured in LPP and  NRPPa procedure according to RAN1 LS [1] [2] as shown in Table 1 and Table 2.

For stage 2, the single request/response mode reporting and periodic reporting of the UE Tx TEG association information for UL-TDOA positioning method need to be added in RRC procedure. RAN2 needs to specify the configurable periodicities for periodic reporting.

For stage 2, capture the TP of stage-2 on TEG for accuracy enhancement as baseline in Annex.

6	Reference

- R2-2200092	LS on the reporting of the Tx TEG association information (R1-2112968; contact: CATT) RAN1	LS in	Rel-17	NR_pos_enh-Core 	To:RAN2, RAN4	Cc:RAN3

- R2-2200095	LS on updated Rel-17 LTE and NR higher-layers parameter list (R1-2112977; contact: Ericsson)	RAN1  LS in	Rel-17    NR_pos_enh, To:RAN2, RAN3	Cc:RAN4

- 3GPP TS38.305 : “Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN” V16.7.0

- 3GPP TS38.331: “Radio Resource Control (RRC) protocol specification(Release 16)”. V16.7.0

- 3GPP TS37.355: “LTE Positioning Protocol (LPP)”. V16.7.0

7	Text Proposals of Stage-2

-------------------------Start of stage-2 Text Proposal1--------------------------------------------------------------

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

As used in this document, the suffixes "-based" and "-assisted" refer respectively to the node that is responsible for making the positioning calculation (and which may also provide measurements) and a node that provides measurements (but which does not make the positioning calculation). Thus, an operation in which measurements are provided by the UE to the LMF to be used in the computation of a position estimate is described as "UE-assisted" (and could also be called "LMF-based"), while one in which the UE computes its own position is described as "UE-based".

Transmission Point (TP): A set of geographically co-located transmit antennas (e.g. antenna array (with one or more antenna elements)) for one cell, part of one cell or one DL-PRS-only TP. Transmission Points can include base station (ng-eNB or gNB) antennas, remote radio heads, a remote antenna of a base station, an antenna of a DL-PRS-only TP, etc. One cell can include one or multiple transmission points. For a homogeneous deployment, each transmission point may correspond to one cell.

Reception Point (RP): A set of geographically co-located receive antennas (e.g. antenna array (with one or more antenna elements)) for one cell, part of one cell or one UL-SRS-only RP. Reception Points can include base station (ng-eNB or gNB) antennas, remote radio heads, a remote antenna of a base station, an antenna of a UL-SRS-only RP, etc. One cell can include one or multiple reception points. For a homogeneous deployment, each reception point may correspond to one cell.

PRS-only TP: A TP which only transmits PRS, DL-PRS signals and is not associated with a cell.

SRS-only RP: An RP which only receives UL-SRS signals and is not associated with a cell.

Transmission-Reception Point (TRP): A set of geographically co-located antennas (e.g. antenna array (with one or more antenna elements)) supporting TP and/or RP functionality.

UE Timing Error Group(s) (TEG(s)):

-	ueRxTEG is associated with one or more DL timing measurements, which have the Rx timing error difference within a certain margin.

- 	ueRxTxTEG is associated with one or more UE Rx-Tx time difference measurements, which have the ‘Rx timing errors+Tx timing errors’ difference within a certain margin.

-    ueTxTEG is associated with the transmissions of one or more UL SRS resources for the positioning purpose, which have the Tx timing error difference within a certain margin.

TRP Timing Error Group(s) (TEG(s)):

-	trpRxTEG is associated with one or more UL timing measurements, which have the Rx timing error difference within a certain margin.

- 	trpRxTxTEG is associated with one or more gNB Rx-Tx time difference measurements, which have the ‘Rx timing errors+Tx timing errors’ difference within a certain margin.

-    trpTxTEG is associated with the transmissions of one or more DL PRS resources, which have the Tx timing error difference within a certain margin.

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

/***Skip unrelated parts***/

TEG	Timing Error Group

/***Skip unrelated parts***/

4	Main concepts and requirements

/***Skip unrelated parts***/

6.2.2	Radio Resource Control (RRC) for NR

The RRC protocol for NR is terminated between the gNB and the UE. It provides transport for LPP messages over the NR-Uu interface.

In addition to providing transport for LPP messages over the NR-Uu interface, it supports transfer of measurements that may be used for positioning purposes through the existing measurement systems specified in TS 38.331 [14].

The RRC protocol for NR also supports broadcasting of assistance data via positioning System Information messages.

The RRC protocol for NR is also used to configure UEs with a sounding reference signal (SRS) to support NG-RAN measurements for NR positioning and report the UE TxTEG (Tx Timing Error Group) for UL-TDOA.

/***Skip unrelated parts***/

7.4	General RRC procedures for UE Positioning

7.4.1	NR RRC Procedures

NR RRC supports the following positioning related procedures:

-	Location Measurement Indication.

-	UEAssistance Inforamtion for UE TxTEG

7.4.1.2	UEAssistance Inforamtion for UE TxTEG

RRC reconfiguration and RRC UEAssistancecInformation procedures are used to request UE and report Tx ‘timing error group’ (UE TxTEG) for UL-TDOA. A UE TxTEG is associated with the transmissions of one or more UL SRS resources for the positioning purpose, which has the Tx timing error difference within a certain margin.

Figure 7.4.1.2-1: UEAssistance Information procedure for UE TxTEG

Precondition:	The serving gNB of a UE has received a NRPPa message from an LMF requesting the TxTEG of the UE for NR UL-TDOA positioning.

- If the serving gNB receives a NRPPa message from an LMF requesting the TxTEG of a UE, subject to the UE’s capability, the serving gNB may send a RRC reconfiguration message to the UE, requesting the UE to provide the association information of UL SRS resources for positioning with Tx TEGs for UL-TDOA to the serving gNB if the UE supports multiple UE Tx TEGs for UL TDOA. Based on the request from the LMF, the RRC Reconfiguration message from the serving gNB to the UE indicates the UE should provide either a single report or a periodic report of UE TxTEG association for UL-TDOA to the serving gNB.

- When the UE receives the request via RRCReconfiguration, UE will report the UE TxTEG information, including all the changes of the UE TxTEG during the report period if the UE is required to report UE Tx TEG periodically. UE will report all the UE TxTEG at the time when the RRC Reconfiguration is received immediately if the UE is only required to report the one-shot UE TxTEG information.

/***Skip unrelated parts***/

8.10	Multi-RTT positioning

8.10.1	General

In the Multi-RTT positioning method, the UE position is estimated based on measurements performed at both, UE and TRPs. The measurements performed at the UE and TRPs are UE/gNB Rx-Tx time difference measurements (and optionally DL-PRS-RSRP and UL-SRS-RSRP) of DL-PRS and UL-SRS, which are used by an LMF to determine the RTTs.

The UE may require measurement gaps to perform the Multi-RTT measurements from NR TRPs. The UE may request measurement gaps from a gNB using the procedure described in clause 7.4.1.1.

The UE may be requested to report UE RxTxTEG-ID-group IE with UE Rx-Tx measurements to LMF. The TRP may be requested to report TRP RxTxTEG-ID-group IE with gNB Rx-Tx measurements to LMF.

8.10.2	Information to be transferred between NG-RAN/5GC Elements

This clause defines the information that may be transferred between LMF and UE/gNB.

8.10.2.1	Information that may be transferred from the LMF to UE

The information that may be transferred from the LMF to the UE are listed in Table 8.10.2.1-1.

Table 8.10.2.1-1: Assistance data that may be transferred from LMF to the UE

Information

Physical cell IDs (PCIs), global cell IDs (GCIs), and PRS IDs, ARFCNs of candidate NR TRPs for measurement

Timing relative to the serving (reference) TRP of candidate NR TRPs

DL-PRS configuration of candidate NR TRPs

SSB information of the TRPs (the time/frequency occupancy of SSBs)

PRS-only TP indication

Table 8.10.2.1-2: Location information request that may be transferred from LMF to the UE

Information

Requesting the UE to provide UE RxTxTEG-ID-group information (UE RxTxTEG-ID, UE TxTEG-ID, and/or UE RxTEG-ID) for UE Rx-Tx measurements

Requesting the UE to provide UE TxTEG information

Requesting the UE to measure the same DL-PRS with different UE Rx TEGs for UE Rx-Tx measurements

Requesting the UE to measure the same DL-PRS with different UE RxTx TEGs for UE Rx-Tx measurements

Requesting UE TxTEG

8.10.2.2	Information that may be transferred from the UE to LMF

The information that may be signalled from UE to the LMF is listed in Table 8.10.2.2-1. The individual UE measurements are defined in TS 38.215 [37].

Table 8.10.2.2-1: Measurement results that may be transferred from UE to the LMF

Information

PCI, GCI, and PRS ID, ARFCN, PRS resource ID, PRS resource set ID for each measurement

DL-PRS-RSRP measurement

UE Rx-Tx time difference measurement

Time stamp of the measurement

Quality for each measurement

TA offset used by UE

UE RxTxTEG-ID-group (UE RxTxTEG-ID, UE TxTEG-ID, and/or UE RxTEG-ID) for UE Rx-Tx measurements

UE TxTEG information (TxTEG-ID, srs-PosResourceSetId, srs-PosResourceId)

8.10.2.3	Information that may be transferred from the gNB to LMF

The assistance data that may be transferred from gNB to the LMF is listed in Table 8.10.2.3-1.

Table 8.10.2.3-1: Assistance data that may be transferred from gNB to the LMF

Information

PCI, GCI, ARFCN and TRP IDs of the TRPs served by the gNB

Timing information of TRPs served by the gNB

DL-PRS configuration of the TRPs served by the gNB

SSB information of the TRPs (the time/frequency occupancy of SSBs)

Spatial direction information of the DL-PRS Resources of the TRPs served by the gNB

Geographical coordinates information of the DL-PRS Resources of the TRPs served by the gNB

The configuration data for a target UE that may be transferred from the serving gNB to the LMF is listed in Table 8.10.2.3-2.

Table 8.10.2.3-2: UL information/UE configuration data that may be transferred from serving gNB to the LMF

UE configuration data

UE SRS configuration

SFN initialization time for the SRS configuration

The measurement results that may be signalled from gNBs to the LMF is listed in Table 8.10.2.3-3.

Table 8.10.2.3-3: Measurement results that may be transferred from gNBs to the LMF

Measurement results

NCGI and TRP ID of the measurement

gNB Rx-Tx time difference measurement

UL-SRS-RSRP

UL Angle of Arrival (azimuth and elevation)

Time stamp of the measurement

Quality for each measurement

Beam Information of the measurement

TRP RxTxTEG-ID-group(TRP RxTxTEG-ID, TRP TxTEG-ID and TRP RxTEG-ID)

TRP TxTEG information(TRP TxTEG-ID, DL-PRS-ResourceSetID, DL-PRS-ResourceID)

8.10.2.4	Information that may be transferred from the LMF to gNBs

The requested UL-SRS transmission characteristics information that may be signalled from the LMF to the gNB is listed in Table 8.10.2.4-1.

Table 8.10.2.4-1: Requested UL-SRS transmission characteristics information that may be transferred from LMF to gNB.

Information

Number Of Transmissions/duration for which the UL-SRS is requested

Bandwidth

Resource type (periodic, semi-persistent, aperiodic)

Number of requested SRS resource sets and SRS resources per set

Pathloss reference: - PCI, SSB Index - DL-PRS ID, DL-PRS Resource Set ID, DL-PRS Resource ID

Spatial relation info - PCI, SSB Index - DL-PRS ID, DL-PRS Resource Set ID, DL-PRS Resource ID - NZP CSI-RS Resource ID - SRS Resource ID - Positioning SRS Resource ID

Periodicity of the SRS for each SRS resource set

SSB Information

Carrier frequency of SRS transmission bandwidth

The TRP measurement request information that may be signalled from the LMF to the gNBs is listed in Table 8.10.2.4-2.

Table 8.10.2.4-2: TRP Measurement request information that may be transferred from LMF to gNBs.

Information

TRP ID, and NCGI of the TRP to receive UL-SRS

UE-SRS configuration

UL timing information together with timing uncertainty, for reception of SRS by candidate TRPs

Report characteristics for the measurements

Measurement Quantities

Measurement periodicity

Measurement beam information request

Request the gNB to provide TRP RxTxTEG-ID-group information (TRP Rx TEG, Tx TEG and/or RxTx TEG association information) for TRP Rx-Tx measurements

Request gNB to measure the same UL positioning SRS resource with different TRP Rx TEGs

Request gNB to measure the same UL positioning SRS resource with different TRP RxTx TEGs that have the same TRP Tx TEG

Requesting the gNB to provide TRP TxTEG information

The Positioning Activation/Deactivation request information that may be signalled from the LMF to the gNB is listed in Table 8.10.2.4-3.

Table 8.10.2.4-3: Requested positioning activation/deactivation information that may be transferred from LMF to gNB.

Information

SP UL-SRS: - Activation or Deactivation request - Positioning SRS Resource Set ID which is to be activated/deactivated - Spatial relation for Resource IDi - Activation Time

Aperiodic UL-SRS - Aperiodic SRS Resource Trigger List - Activation Time

UL-SRS: - Release all

/***Skip unrelated parts***/

8.12	DL-TDOA positioning

8.12.1	General

In the DL-TDOA positioning method, the UE position is estimated based on DL RSTD (and optionally DL-PRS-RSRP) measurements taken at the UE of downlink radio signals from multiple NR TRPs, along with knowledge of the geographical coordinates of the TRPs and their relative downlink timing.

The UE while connected to a gNB may require measurement gaps to perform the DL-TDOA measurements from NR TRPs. The UE may request measurement gaps from a gNB using the procedure described in clause 7.4.1.1.

The UE may be requested to report UE RxTEG-ID with RSTD measurements (including additional DL RSTD measurements) to LMF.

The specific positioning techniques used to estimate the UE's location from this information are beyond the scope of this specification.

8.12.2	Information to be transferred between NG-RAN/5GC Elements

This clause defines the information that may be transferred between LMF and UE/gNB.

8.12.2.1	Information that may be transferred from the LMF to UE

The information that may be transferred from the LMF to the UE are listed in table 8.12.2.1-1.

Table 8.12.2.1-1: Assistance data that may be transferred from LMF to the UE

| Information | UE-assisted | UE-based |
| --- | --- | --- |
| Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement | Yes | Yes |
| Timing relative to the serving (reference) TRP of candidate NR TRPs | Yes | Yes |
| DL-PRS configuration of candidate NR TRPs | Yes | Yes |
| SSB information of the TRPs (the time/frequency occupancy of SSBs) | Yes | Yes |
| Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB | No | Yes |
| Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) | No | Yes |
| Fine Timing relative to the serving (reference) TRP of candidate NR TRPs | No | Yes |
| PRS-only TP indication | Yes | Yes |
| TRP TxTEG (TRP TxTEG-ID, DL-PRS-ResourceSetID, and DL-PRS-ResourceID) | No | Yes |

Table 8.12.2.1-2: Location information request that may be transferred from LMF to the UE

| Information | UE-assisted | UE-based |
| --- | --- | --- |
| Request to the UE to report UE RxTEG-ID with RSTD measurements | Yes | No |
| Request UE to measure the same DL PRS with different UE Rx TEGs and report the corresponding RSTD measurements with UE RxTEG-IDs | Yes | No |

8.12.2.2	Information that may be transferred from the UE to LMF

The information that may be signalled from UE to the LMF is listed in Table 8.12.2.2-1. The individual UE measurements are defined in TS 38.215 [37].

Table 8.12.2.2-1: Measurement results that may be transferred from UE to the LMF

| Information | UE-assisted | UE-based |
| --- | --- | --- |
| Latitude/Longitude/Altitude, together with uncertainty shape | No | Yes |
| PCI, GCI, ARFCN, PRS resource ID, PRS resource set ID and PRS ID for each measurement | Yes | No |
| DL RSTD measurement, including DL RSTD measurement measured from different UE Rx TEGs | Yes | No |
| DL-PRS-RSRP measurement | Yes | No |
| Time stamp of the measurements | Yes | No |
| Time stamp of location estimate | No | Yes |
| Quality for each measurement | Yes | No |
| UE Rx TEG-IDs for DL RSTD measurements | Yes | No |

8.12.2.3	Information that may be transferred from the gNB to LMF

The assistance data that may be transferred from gNB to the LMF is listed in Table 8.12.2.3-1.

Table 8.12.2.3-1: Assistance data that may be transferred from gNB to the LMF

Information

PCI, GCI, ARFCN, and TRP IDs of the TRPs served by the gNB

Timing information of TRPs served by the gNB

DL-PRS configuration of the TRPs served by the gNB

SSB information of the TRPs (the time/frequency occupancy of SSBs)

Spatial direction information of the DL-PRS Resources of the TRPs served by the gNB

Geographical coordinates information of the DL-PRS Resources of the TRPs served by the gNB

TRP TxTEG (TRP TxTEG-ID, DL-PRS-ResourceSetID, and DL-PRS-ResourceID)

8.12.3	DL-TDOA Positioning Procedures

The procedures described in this clause support UE-assisted and UE-based DL-TDOA.

8.12.3.1	Procedures between LMF and UE

8.12.3.1.1	Capability Transfer Procedure

The Capability Transfer procedure for DL-TDOA positioning is described in clause 7.1.2.1.

8.12.3.1.2	Assistance Data Transfer Procedure

The purpose of this procedure is to enable the LMF to provide assistance data to the UE (e.g., as part of a positioning procedure) and the UE to request assistance data from the LMF (e.g., as part of a positioning procedure).

8.12.3.1.2.1	LMF initiated Assistance Data Delivery

Figure 8.12.3.1.2.1-1 shows the Assistance Data Delivery operations for the DL-TDOA positioning method when the procedure is initiated by the LMF.

Figure 8.12.3.1.2.1-1: LMF-initiated Assistance Data Delivery Procedure

(1)	The LMF determines that assistance data needs to be provided to the UE (e.g., as part of a positioning procedure) and sends an LPP Provide Assistance Data message to the UE. This message may include any of the DL-TDOA positioning assistance data defined in Table 8.12.2.1-1.

8.12.3.1.2.2	UE initiated Assistance Data Transfer

Figure 8.12.3.1.2.2-1 shows the Assistance Data Transfer operations for the DL-TDOA positioning method when the procedure is initiated by the UE.

Figure 8.12.3.1.2.2-1: UE-initiated Assistance Data Transfer Procedure

(1)	The UE determines that certain DL-TDOA positioning assistance data are desired (e.g., as part of a positioning procedure when the LMF provided assistance data are not sufficient for the UE to fulfil the request) and sends an LPP Request Assistance Data message to the LMF. This request includes an indication of which specific DL-TDOA assistance data are requested. Additional information concerning the UE's approximate location and serving and neighbour cells may also be provided in the Request Assistance Data message and/or in an accompanying Provide Location Information message to help the LMF provide appropriate assistance data. This additional data may include the UE's last known location if available, the cell IDs of the UE serving NG-RAN node and possibly neighbour NG-RAN nodes, as well as NR E-CID measurements.

(2)	The LMF provides the requested assistance in an LPP Provide Assistance Data message, if available at the LMF. If any of the UE requested assistance data in step (1) are not provided in step 2, the UE shall assume that the requested assistance data are not supported, or currently not available at the LMF. If none of the UE requested assistance data in step (1) can be provided by the LMF, return any information that can be provided in an LPP message of type Provide Assistance Data which includes a cause indication for the not provided assistance data.

8.12.3.1.3	Location Information Transfer Procedure

The purpose of this procedure is to enable the LMF to request location estimate from the UE, or to enable the UE to provide location measurements to the LMF for position calculation.

8.12.3.1.3.1	LMF-initiated Location Information Transfer Procedure

Figure 8.12.3.1.3.1-1 shows the Location Information Transfer operations for the DL-TDOA positioning method when the procedure is initiated by the LMF.

Figure 8.12.3.1.3.1-1: LMF-initiated Location Information Transfer Procedure

(1)	The LMF sends an LPP Request Location Information message to the UE. This request includes indication of DL-TDOA measurements requested, including any needed measurement configuration information, and required response time.

(2)	The UE obtains DL-TDOA measurements as requested in step 1. The UE then sends an LPP Provide Location Information message to the LMF, before the Response Time provided in step (1) elapsed, and includes the obtained DL RSTD measurements and, optionally, the DL-PRS-RSRP measurements, and optionally UE RxTEG-ID for DL RSTD measurements. If the UE is unable to perform the requested measurements, or the Response Time elapsed before any of the requested measurements were obtained, the UE returns any information that can be provided in an LPP message of type Provide Location Information which includes a cause indication for the not provided location information.

8.12.3.1.3.2	UE-initiated Location Information Delivery procedure

Figure 8.12.3.1.3.2-1 shows the Location Information Delivery procedure operations for the DL-TDOA positioning method when the procedure is initiated by the UE.

Figure 8.12.3.1.3.2-1: UE-initiated Location Information Delivery Procedure.

(1)	The UE sends an LPP Provide Location Information message to the LMF. The Provide Location Information message may include any UE DL-TDOA measurements and, optionally, the DL-PRS-RSRP measurements already available at the UE and UE RxTEG-ID with DL RSTD measurement.

8.12.3.2	Procedures between LMF and gNB

8.12.3.2.1	Assistance Data Delivery procedure

The purpose of this procedure is to enable the gNB to provide assistance data to the LMF, for subsequent delivery to the UE using the procedures of clause 8.12.3.1.2 or for use in the calculation of positioning estimates at the LMF.

8.12.3.2.1.1	LMF-initiated assistance data delivery to the LMF

Figure 8.12.3.2.1.1-1 shows the TRP Information Exchange operation from the gNB to the LMF for the DL-TDOA positioning method.

Figure 8.12.3.2.1.1-1: LMF-initiated TRP Information Exchange Procedure

(1)	The LMF determines that certain TRP configuration information is desired (e.g., as part of a periodic update or as triggered by OAM) and sends an NRPPa TRP INFORMATION REQUEST message to the gNB. This request includes an indication of which specific TRP configuration information including the TRP TxTEG is requested.

(2)	The gNB provides the requested TRP information in an NRPPa TRP INFORMATION RESPONSE message, if available at the gNB. If the gNB is not able to provide any information, it returns an TRP INFORMATION FAILURE message indicating the cause of the failure.

8.13	UL-TDOA positioning

8.13.1	General

In the UL-TDOA positioning method, the UE position is estimated based on UL-RTOA (and optionally UL-SRS-RSRP) measurements taken at different TRPs of uplink radio signals from UE, along with other configuration information.

The specifics of any UL-TDOA positioning methods or techniques used to estimate the UE's location from these measurements are beyond the scope of this specification.

In order to obtain uplink measurements, the TRPs need to know the characteristics of the SRS signal transmitted by the UE for the time period required to perform uplink measurement. These characteristics should be static over the periodic transmission of SRS during the uplink measurements. Hence, the LMF will indicate to the serving gNB the need to direct the UE to transmit SRS signals for uplink positioning. It is up to the serving gNB to make the final decision on resources to be assigned and to communicate this SRS configuration information back to the LMF so that LMF can forward the SRS configuration to the TRPs. The gNB may decide (e.g., in case no resources are available) to configure no resources for the UE and report the empty resource configuration to the LMF.

The UE may be requested by the serving gNB to report UE Tx TEG to the serving gNB using the procedure described in clause 7.4.1.2.

8.13.2	Information to be transferred between NG-RAN/5GC Elements

This clause defines the information that may be transferred between LMF and gNB/TRPs.

8.13.2.0	Assistance Data that may be transferred from the gNB to the LMF

The assistance data that may be transferred from the gNB to the LMF is listed in Table 8.13.2.0-1.

Table 8.13.2.0-1: Assistance data that may be transferred from gNB to the LMF

Information

PCI, GCI, and TRP IDs of the TRPs served by the gNB

SSB information of the TRPs (the time/frequency occupancy of SSBs)

Geographical coordinates information of the DL-PRS Resources of the TRPs served by the gNB

8.13.2.1	Configuration Data that may be transferred from the gNB to the LMF

The configuration data for a target UE that may be transferred from the serving gNB to the LMF is listed in Table 8.13.2.1-1.

Table 8.13.2.1-1: UE configuration data that may be transferred from serving gNB to the LMF

UE configuration data

UE SRS configuration

Timing information of the TRP, which configured the UE SRS transmission

UE TxTEG information (UE TxTEG-ID, [srs-PosResourceSetId], srs-PosResourceId)

8.13.2.2	Location Information that may be transferred from the gNBs to LMF

The information that may be transferred from gNBs to the LMF include measurement results listed in Table 8.13.2.2-1. The individual measurements are defined in TS 38.215 [37].

Table 8.13.2.2-1: Measurement results that may be transferred from gNBs to the LMF

Measurement results

NCGI and TRP ID of the measurement

UL-RTOA

UL-SRS-RSRP

Time stamp of the measurement

Quality for each measurement

Beam Information for each measurement

TRP Rx TEG-ID with RSTD measurements

8.13.2.3	Information that may be transferred from the LMF to gNBs

The requested UL-SRS transmission characteristics information that may be signalled from the LMF to the gNB is listed in Table 8.13.2.3-1.

Table 8.13.2.3-1: Requested UL-SRS transmission characteristics information that may be transferred from LMF to gNB.

Information

Number Of Transmissions/duration for which the UL-SRS is requested

Bandwidth

Resource type (periodic, semi-persistent, aperiodic)

Pathloss reference: - PCI, SSB Index, SSB configuration (time/frequency occupancy of SSBs) - DL-PRS ID, DL-PRS Resource Set ID, DL-PRS Resource ID

Spatial relation info - PCI, SSB Index, SSB configuration (time/frequency occupancy of SSBs) - DL-PRS ID, DL-PRS Resource Set ID, DL-PRS Resource ID - NZP CSI-RS Resource ID - SRS Resource ID - Positioning SRS Resource ID

SSB Information

Periodicity of the SRS for each SRS resource set

Carrier frequency of SRS transmission bandwidth

Request the serving gNB to ask the UE to provide UE TxTEG

The TRP measurement request information that may be signalled from the LMF to the gNB is listed in table 8.13.2.3-2.

Table 8.13.2.3-2: TRP Measurement request information that may be transferred from LMF to gNB.

Information

TRP ID, cell ID of the TRP to receive UL-SRS

UE-SRS configuration

UL timing information together with timing uncertainty, for reception of SRS by candidate TRPs

Report characteristics for the measurements

Measurement Quantities

Measurement periodicity

Measurement beam information request

Request the gNB to measure RTOA from the same SRS resource for positioning with different TRP Rx TEGs

Request to report TRP RxTEG ID with RTOA measurements

The Positioning Activation/Deactivation request information that may be signalled from the LMF to the gNB is listed in Table 8.13.2.3-3.

Table 8.13.2.3-3: Requested positioning activation/deactivation information that may be transferred from LMF to gNB.

Information

SP UL-SRS: - Activation or Deactivation request - Positioning SRS Resource Set ID which is to be activated/deactivated - Spatial relation for Resource IDi - Activation Time

Aperiodic UL-SRS: - Aperiodic SRS Resource Trigger List - Activation time

UL-SRS: - Release all

8.13.2.4	Information that may be transferred from the serving gNB to UE

The information that may be transferred from serving gNB to the UE includes request listed in Table 8.13.2.4-1.

Table 8.13.2.4-1: Measurement request that may be transferred from the serving gNB to UE

Measurement request

Report UE TxTEG

8.13.2.5	Information that may be transferred from UE to the serving gNB

The information that may be transferred from UE to the serving gNB includes parts of measurement results listed in Table 8.13.2.2-1. The individual measurements are defined in TS 38.215 [37].

Table 8.13.2.5-1: Measurement results that may be transferred from UE to the serving gNB

Measurement results

UE Tx TEG (TxTEG-ID, [srs-PosResourceSetId], srs-PosResourceId) with time stamp

8.13.3	UL-TDOA Positioning Procedures

/***Skip unrelated parts***/

8.13.3.4	Sequence of Procedure for UL-TDOA positioning

Figure 8.13.3.4-1 shows the messaging between the LMF, the gNBs and the UE to perform UL-TDOA procedure.

Figure 8.13.3.4-1: UL-TDOA positioning procedure

0.	The LMF may use the procedure in Figure 8.13.3.2.1-2 to obtain the TRP information required for UL-TDOA positioning.

1.	The LMF may request the positioning capabilities of the target device using the LPP Capability Transfer procedure as described in clause 8.13.3.1.

2.	The LMF sends a NRPPa POSITIONING INFORMATION REQUEST message to the serving gNB to request UL-SRS configuration information for the target device as described in Figure 8.13.3.2.1-1.

3.	The serving gNB determines the resources available for UL-SRS and configures the target device with the UL-SRS resource sets at step 3a.

4.	The serving gNB provides the UL information to the LMF in a NRPPa POSITIONING INFORMATION RESPONSE message.

5.	In the case of semi-persistent or aperiodic SRS, the LMF may request activation of UE SRS transmission by sending the NRPPa Positioning Activation Request message to the serving gNB of the target device as described in clause 8.13.3.3a. The gNB then activates the UL-SRS transmission and sends the NRPPa Positioning Activation Response message. The target device begins the UL-SRS transmission according to the time domain behavior of UL-SRS resource configuration. Serving gNB will send the RRCReconfiguration to UE to request the UE to provide UE TxTEG if the serving gNB receives the request from LMF in the clause 8.13.2.4. Accordingly, UE will report the UE TxTEG to gNB via RRC message which is associated with the transmissions of one or more UL positioning SRS resources to mitigate UE Tx timing errors in the clause 8.13.2.5.

6.	The LMF provides the UL-SRS configuration to the selected gNBs in a NRPPa MEASUREMENT REQUEST message as described in clause 8.13.3.3. The message includes all information required to enable the gNBs/TRPs to perform the UL measurements.

7.	Each gNB configured at step 6 measures the UL-SRS transmissions from the target device.

8.	Each gNB reports the UL-SRS measurements to the LMF in a NRPPa Measurement Response message as described in clause 8.13.3.3.

9.	The LMF sends a NRPPa POSITIONING DEACTIVATION message to the serving gNB as described in clause 8.13.3.3a.

-------------------------End of Text Proposal1---------------------------------------------------------------------------