3GPP TSG-RAN WG2 #116-bis-e	R2-2200344

Electronic meeting, 17th –25th January, 2022

Agenda Item:	8.15.2

Source:	NEC

Title:	Further discussions on leftover issues of sidelink DRX configuration

Document for:	Discussion, Decision

Introduction

In the last meeting (RAN2#116-e), the following issues were skipped due to shortage of time.

R2-2109478	[POST115-e][716][V2X/SL] Identified FFS and open issues (CATT)	CATT discussion	Rel-17	NR_SL_enh-Core:

[Low priority for online session]:

Proposal 14: RAN2 to further discuss whether the drx-inactivity timer should be included in the RX UE’s desired SL DRX configuration.

Proposal 15: RAN2 to further discuss whether the HARQ RTT timer should be included in the RX UE’s desired SL DRX configuration.

Proposal 16: RAN2 to further discuss whether the HARQ retransmission timer should be included in the RX UE’s desired SL DRX configuration.

Proposal 19: RAN2 to further discuss when the Rx UE rejects the SL DRX configuration included in the RRCReconfigurationSidelink, which PC5-RRC signaling should be sent from Rx UE to Tx.

- Proposal 14, 15, 16 and 19 are skipped.

Proposal 25: RAN2 further discuss that whether SL DRX should be applied for the PC5-S messages which are sent after the DCR message and before SL unicast DRX configuration is applied.

- skipped

This contribution discusses the above skipped issues.

Discussion

Issue 1: Whether the drx-inactivity timer/ HARQ RTT timer/ HARQ retransmission timer should be included in the RX UE’s desired SL DRX configuration?

During [POST115-e][716][V2X/SL] Identified FFS and open issues e-mail discussion, all companies agreed that the desired SL DRX configuration in the assistance information is defined by the conventional SL DRX parameters. Therefore, the straight forward way for signaling design is to adopt the same framework as the conventional SL DRX parameters. If the Rx UE has no preference or requirement on some parameters (e.g., drx-inactivity timer/HARQ RTT timer/the HARQ retransmission timer), then the Rx UE does not include them in the assistance information. It is also aligned with the following agreements achieved at RAN2#116-e meeting.

- It is up to Rx UE’s implementation to determine its desired SL DRX configuration.

- When TX UE doesn’t receive any assistance information from RX UE, TX UE considers that RX UE is ok with any DRX configuration (including no DRX configuration).

From signaling design point of view, include drx-inactivity timer / HARQ RTT timer/ HARQ retransmission timer to the assistance information signaling.

Whether to indicate RX UE’s desired drx-inactivity timer / HARQ RTT timer/ HARQ retransmission timer is up to RX UE’s implementation.

When TX UE doesn’t receive drx-inactivity timer / HARQ RTT timer/ HARQ retransmission timer from assistance information, TX UE considers that RX UE is ok with any drx-inactivity timer / HARQ RTT timer/ HARQ retransmission timer.

Issue 2: When the Rx UE rejects the SL DRX configuration included in the RRCReconfigurationSidelink, which PC5-RRC signalling should be sent from Rx UE to Tx UE?

During [POST115-e][716][V2X/SL] Identified FFS and open issues e-mail discussion, 10 companies preferred option 1, RRCReconfigurationFailureSidelink whereas 11 companies preferred option 2, RRCReconfigurationCompleteSidelink.

First of all, as many companies analyzed, both RRCReconfigurationFailureSidelink and RRCReconfigurationCompleteSidelink could work. For unicast sidelink communication, it is desirable to apply DRX operation from sidelink DRB establishment, therefore a more likely scenario is to include DRX parameters in the initial RRCReconfigurationSidelink to establish sidelink DRBs. In such a case, if RRCReconfigurationFailureSidelink is used to send Rx UE rejection of SL DRX configuration, then the sidelink DRB establishment might be failure due to there is no successful configuration prior to corresponding RRCReconfigurationSidelink message. Therefore, we prefer to use RRCReconfigurationCompleteSidelink with a new DRX rejection indication.

When the Rx UE rejects the SL DRX configuration included in the RRCReconfigurationSidelink, RRCReconfigurationCompleteSidelink with a new DRX rejection indication should be sent from Rx UE to Tx UE.

Summary

This contribution discussed leftover issues of sidelink DRX configuration.

- From signaling design point of view, include drx-inactivity timer / HARQ RTT timer/ HARQ retransmission timer to the assistance information signaling.

Whether to indicate RX UE’s desired drx-inactivity timer / HARQ RTT timer/ HARQ retransmission timer is up to RX UE’s implementation.

When TX UE doesn’t receive drx-inactivity timer / HARQ RTT timer/ HARQ retransmission timer from assistance information, TX UE considers that RX UE is ok with any drx-inactivity timer / HARQ RTT timer/ HARQ retransmission timer.

When the Rx UE rejects the SL DRX configuration included in the RRCReconfigurationSidelink, RRCReconfigurationCompleteSidelink with a new DRX rejection indication should be sent from Rx UE to Tx UE.

- References

[1] R2-2109478