3GPP TSG-RAN WG2 Meeting #116bis electronic	R2-2200414

Online, Jan 17-25, 2022

Agenda item:		8.3.2

Source:		Lenovo, Motorola Mobility

Title:		EPS Fallback

Document for:		Discussion and Decision

# Introduction

It needs to be discussed how ETWS or CMAS capable UEs, or UEs in RRC_IDLE or in RRC_INACTIVE shall monitor SI change.

# Change of system information

UEs in RRC_IDLE or in RRC_INACTIVE shall monitor for SI change indication in its own paging occasion every DRX cycle. If the UE’s own paging occasion collides with the PO of other USIM, UE shall rather monitor for SI change indication in any paging occasion at least once per modification period or in every DRX cycle. The same solution is applied for ETWS or CMAS capable UEs in RRC_IDLE or in RRC_INACTIVE i.e. such a UE shall rather monitor for SI change indication in any paging occasion at least once per modification period or in every DRX cycle.

Proposal: To monitor SI change, if the UE’s own paging occasion collides with the PO of other USIM, UE shall monitor for SI change indication in any paging occasion at least once per modification period or in every DRX cycle. Similarly, for ETWS or CMAS capable UEs in RRC_IDLE or in RRC_INACTIVE state.

# Conclusion

It was be discussed how ETWS or CMAS capable UEs, or UEs in RRC_IDLE or in RRC_INACTIVE shall monitor SI change Following proposal is made:

Proposal: To monitor SI change, if the UE’s own paging occasion collides with the PO of other USIM, UE shall monitor for SI change indication in any paging occasion at least once per modification period or in every DRX cycle. Similarly, for ETWS or CMAS capable UEs in RRC_IDLE or in RRC_INACTIVE state.