3GPP TSG-RAN WG2 Meeting #116bis electronic	R2-2200420

Online, January 17 – January 25, 2022

Source:	CATT

Title:	Discussion on MAC procedure for feature combinations ‎

Agenda Item:	8.18.2

Document for:	Discussion and Decision

Introduction

The RACH partitioning is considered to enable early identification of the feature and/or feature combination. This Work Item is aim to design a common RACH procedure for RedCap, SDT, CovEnh and Slicing. The related RAN2 #116[1] meeting agreements are following:

Agreements: 1 No new feature and/ feature combination specific preambles are defined within the “not available” preambles defined at the end of a RO through the legacy totalNumberOfRA-Preambles 2 Specification allows for use of Separate time-frequency resources, not defined through legacy RRC signalling, within Contention free preamble defined through legacy RRC signaling and the combination of these (i.e. using the reserved preamble at the end of SSBs like 2-step RACH) 3 RAN2 baseline is that preambles for a particular feature combination shall be present in all SSBs (e.g., a feature combination cannot only have preambles in SSB0 but not SSB1) 4 As a baseline, a feature combination shall have the same number of preambles in all SSBs 5 Signaling should allow that a particular feature/feature combination can be mapped only to a subset of the RACH occasions of a RACH configuration. 6 The legacy masking index approach is reused in Rel-17 RA partitioning 7 RAN2 adopts Approach A as baseline (an IE contains one field for each of the features) for indicating which feature/feature combination a partition applies to. Details are FFS, e.g. details around slicing. FFS how to encode and design the signaling in a future compatible way (i.e. naming) 8 As a baseline, multiple "RA partitions" for one RA type which map to the same feature/feature combination is not supported on a given BWP. FFS if there is any special use case that requires multiple RA partition configuration. Agreements 1 RAN2 assumes that the network may not provide all possible permutation. FFS whether the selection in case of missing combination is specified or left to UE implementation 2 For slicing, unified partitioning framework should take priority

In this document, we intend to provide our suggestion on common RACH procedure.

Discussions

According to the description in clause 5.1 of TS38.321 [2], the overall Random Access procedures are specified. We can briefly analyze the overall RA procedure and the agreements of each R-17 feature. Then we discuss the impact of RACH partitioning on RACH procedures.

2.1 Carrier selection and BWP operation

In RA procedure, firstly, the MAC entity will flush the Msg3/MSGA buffer and initialize the related parameters. Then the MAC entity will select the NUL carrier or SUL carrier according to the signaling indication or the relationship between the RSRP of downlink path loss reference and rsrp-ThresholdSSB-SUL. On the selected carrier, MAC entity then performs the BWP operation.

Regarding the issue, it was agreed that in RAN2 #115e [5], carrier selection (between NUL/SUL) should happen ahead of the initial RACH resource selection (i.e. feature combination is not considered in carrier selection).

6. As a baseline, the RA procedure design for Rel-17 should adhere to the following general principles: a: Carrier selection (between NUL/SUL) should happen ahead of the initial RACH resource selection (i.e. feature combination is not considered in carrier selection).

And in the email discussion [Post116-e][515] [3], this issue was also discussed, i.e. whether UE should perform carrier selection and BWP operation based on the thresholds in the RACH configuration. And majority companies show their preference that carrier selection and BWP operation is performed before feature combination selection.

For CE, we have agreed that [1]:

1. Confirm Msg3 repetition is supported on both NUL and SUL, and network can configure different RSRP thresholds for requesting Msg3 repetition on NUL and SUL.

It can be seen that one separate RSRP threshold for requesting Msg3 will be configured for NUL and SUL. Hence, carrier selection should be performed firstly. Otherwise, the UE has confusion on whether Msg3 repetition will be performed or not since the carrier is not determined and definitely RSRP threshold for requesting Msg3 repetition is undecided.

Proposal 1: Carrier selection and BWP operation is performed before the feature combination selection for CE.

For RedCap and slice, there is no agreement on whether one separate RSRP threshold is defined. Hence, we can follow the procedure as CE. Then it is proposed that:

Proposal 2: Carrier selection and BWP operation is performed before the feature combination selection for RedCap and Slicing.

For SDT, it was agreed that one separate RSRP threshold is configured for SDT [4].

4 RSRP threshold for carrier selection is specific to SDT (i.e. separately configured for SDT). This is optional for the network.

So for SDT feature, if the UE does not know whether SDT is initiated or not, the UE can’t figure out the RSRP threshold for carrier selection. It was captured in the MAC running CR that carrier selection is performed after SDT is determined [6].

The MAC entity shall, if triggered by the upper layers for SDT transmission:1> if the data volume of the pending UL data across all RBs configured for SDT is less or equal to sdt-DataVolumeThreshold; and NOTE: For SDT procedure, the MAC entity also consideres the suspended RBs configured with SDT for data volume calculation and buffered packets in PDCP/RLC entities should be counted in SDT data volume calculation. It is up to the UE’s implementation how the UE calculates the data volume for the suspended RBs Editor’s NOTE: FFS whether the CCCH message is considered for data volume calculation. 1> if the RSRP of the downlink pathloss reference is higher than sdt-RSRP-Threshold: 2> if the Serving Cell for SDT is configured with supplementary uplink as specified in TS 38.331 [5]; and 2> if the RSRP of the downlink pathloss reference is less than sdt-RSRP-ThresholdSSB-SUL: 3> select the SUL carrier. 2> else: 3> select the NUL carrier.

And according to the running CR, it can also be seen that SDT is triggered by upper layer, i.e. RRC. Hence, we propose that:

Proposal 3: Carrier selection and BWP operation is performed before the feature combination selection for SDT based on the current SDT MAC running CR.

2.2 RA type selection

In accordance to the current RA procedure, the MAC entity will perform the RA type selection according to the signaling indication or the relationship between the RSRP of downlink pathloss reference and msgA-RSRP-Threshold. If the 2-step RA is selected, MAC entity will perform the RA resource selection procedure for 2-step RA. Else the MAC entity will perform the 4-step RA Resource selection procedure.

For CE, slice and RedCap, there is no agreement on whether one separate threshold is used for RA type selection. For SDT, according to the agreements of SDT specific parameters in RAN2 #113e [4], the RSRP threshold for RA type selection is specific to SDT (i.e. separately configured for SDT).

6 RSRP threshold for RA type selection is specific to SDT (i.e. separately configured for SDT)

When performing RA type, if the Random Access procedure is for SDT, UE should adopt the SDT specific RSRP threshold for RA type. So we propose that:

Proposal 4: For Random Access procedure for SDT, UE should perform RA type selection based on the SDT specific RSRP threshold for RA type.

2.3 RA resource selection

After the selection of RA type, the MAC entity will initialize the variables specific the RA type, including the prioritization parameters: PREAMBLE_POWER_RAMPING_STEP, SCALING_FACTOR_BI and so on. Then the MAC entity performs the RA resource selection for specific RA type.  In this procedure, MAC entity selects the SSB according to the relationship between RSRP of SSBs and rsrp-ThresholdSSB or msgA-RSRP-ThresholdSSB. Then the MAC entity performs preamble selection from RA preamble group A or group B according to the configuration and the relationship between transport block size and ra-Msg3SizeGroupA or ra-MsgA-SizeGroupA. Afterwards, the MAC entity selects a preamble to transmit. According to the PRACH occasion in which the RA preamble is transmitted, the RA-RNTI can be computed.

According to the agreements in RAN2 115e meeting, the slicing specific parameters include scalingFactorBI and powerRampingStepHighPriority and no additional parameters for this release.

5 For slice based RACH prioritization, RAN2 will stick to the current baseline parameters, i.e., scalingFactorBI and powerRampingStepHighPriority, and no additional parameters for this release

So when the MAC entity initialize the variables for the RA type, if the Random Access procedure is for slicing, UE should adopt the slicing specific prioritization parameter. Therefore, we propose that:

Proposal 5:  If the Random Access procedure is for Slicing, UE should adopt the Slicing specific prioritization parameter.

During the process of RA resource selection, the MAC entity will select the SSB according the relationship between RSRP of SSBs and rsrp-ThresholdSSB or msgA-RSRP-ThresholdSSB. According to the SSB related agreements of CovEnh and SDT, both CovEnh and SDT have the feature specific SSB selection parameters.

The related agreements of CovEnh specific parameters in RAN2 #115e [5] are shown below:

5. A separate rsrp-ThresholdSSB threshold is introduced for requesting Msg3 repetition.

The related agreements of SDT specific parameters in RAN2 #115e are shown below:

3. At least the following parameters can be RA-SDT specific. SSB selection related parameters, i.e., rsrp-ThresholdSSB, msgA-RSRP-ThresholdSSB.

Besides, for preamble group A and Group B, it was agreed in CovEnh in RAN2 #115e that:

2. Group B preambles with Msg3 repetition is supported, it is up to network to decide whether to configure Group B together with Msg3 repetition. 3. If Group B preambles with Msg3 repetition is configured, network can configure separate parameters for requesting Msg3 repetition, including ra-Msg3SizeGroupA, messagePowerOffsetGroupB and numberOfRA-PreamblesGroupA (ASN.1 details can be discussed in session on RACH partitioning)

And the related agreements of SDT specific parameters in RAN2 #115e are shown below:

3. At least the following parameters can be RA-SDT specific.  Preamble group related parameters, i.e., ra-Msg3SizeGroupA/ra-MsgA-SizeGroupA, messagePowerOffsetGroupB for 2-step RA-SDT and 4-step RA-SDT.

In our understanding, if the Random Access procedure is only for SDT or CovEnh, UE should use the SDT or CovEnh specific configurations, e.g. rsrp-ThresholdSSB, msgA-RSRP-ThresholdSSB, parameters in Group A/B. But if RA Procedure is for the feature combination including SDT and CovEnh, which parameter is used should be discussed. The solutions can be:

- Alternative 1: the network configures the priority for SDT, CovEnh or other features;

- Alternative 2: it is specified in MAC the priority for SDT, CovEnh or other features;

- Alternative 3: it is up to the UE implementation to select the proper value for rsrp-ThresholdSSB and msgA-RSRP-ThresholdSSB

And we propose that:

Proposal 6:  If the Random Access procedure is only for SDT or CovEnh, UE should adopt the SDT or CovEnh specific rsrp-ThresholdSSB or msgA-RSRP-ThresholdSSB.

Proposal 7:  If the Random Access procedure is for the feature combination including CovEnh and SDT, RAN2 to discuss the following solutions:

- Alternative 1: the network configures the priority for SDT, CovEnh or other features;

- Alternative 2: it is specified in MAC the priority for SDT, CovEnh or other features;

- Alternative 3: it is up to the UE implementation to select the proper value for rsrp-ThresholdSSB and msgA-RSRP-ThresholdSSB.

2.4 RA Preamble Transmission or MSGA transmission

After the above procedure, the MAC entity will instruct the physical layer to transmit the RA Preamble using the selected PRACH occasion, corresponding RA-RNTI, MSGB-RNTI, PREAMBLE_INDEX and PREAMBLE_RECEIVED_TARGET_POWER.

In this RACH procedure, the RA-RNTI/ MSGB-RNRI collision may happen. Since the new feature and/or feature combination specific preambles can be defined in separate time-frequency resources. RA-RNTI collision may happen when UEs use the different ROs configured in different RACH configurations but with the same starting symbol index, slot index and frequency index. According to agreements in RAN Slicing WI, the RA-RNTI collision can be left to network implementation to resolve. We can use the same principle for RA partitioning. So we propose that:

Proposal 8 The RA-RNTI/MSGB-RNTI collision in RACH partitioning can be left to network implementation to resolve it.

2.5 Random Access Response reception and Contention Resolution

Random Access Response reception

Once the Random Access preamble is transmitted, the MAC entity shall start the ra-ResponseWindow or msgB-ResponseWindow and monitor the PDCCH for Random Access Response identified by the RA-RNTI. If the RAR contains a MAC subPDU with Backoff Indicator, MAC entity will set the PREAMBLE_BACKOFF value of the BI field of the MAC subPDU. If the MAC entity receives the UL grant then it will indicate it to the lower layers. If the ra-ResponseWindow expires and if the Random Access Response containing Random Access Preamble identifiers that matches the transmitted PREAMBLE_INDEX has not been received, MAC entity shall consider the Random Access Response reception not successful and increment PREAMBLE_TRANSMISSION_COUNTER by 1. When the PREAMBLE_TRANSMISSION_COUNTER come to the preambleTransMax+1, the MAC entity will consider the RA procedure unsuccessfully completed.

Contention Resolution

Once Msg 3 is transmitted the MAC entity shall start the ra-ContentionResolutionTimer and monitor the PDCCH. If the received Contention ResolutionIdentity in MAC CE matches the CCCH SDU transmitted in Msg3, the MAC entity shall consider this Contention Resolution successful. And the MAC entity shall set the C-RNTI to the value of the TEMPORARY_C-RNTI and consider this Random Access procedure successfully completed. If the Random Access procedure is not completed and the RA type is selected to 4-step RA, the MAC entity shall select a random backoff time according to a uniform distribution between 0 and the PREAMBLE_BACKOFF and perform the Random Access Resource selection procedure after the backoff time. If the Random Access procedure is not completed and the RA type is set to 2-step RA, the MAC entity shall set the RA type to 4-step RA when the PREAMBLE_TRANSMISSION_COUNTER come to msgA-TransMax + 1. And the MAC entity shall perform initialization of variables specific to RA type and perform the Random Access Resource selection procedure. Else the MAC entity shall select a random backoff time according to a uniform distribution between 0 and the PREAMBLE_BACKOFF and perform Random Access Resource selection procedure for 2-step RA after the backoff time.

During the procedure of RAR reception and contention resolution, when the 2-step Random Access procedure is not completed after some times, the UE is allowed to fallback to 4-step Random Access procedure. The fallback is also discussed in some feature WI. According to the agreements in RAN2 #115e meeting, in SDT WI, UE can be configured to switch from 2-step RA-SDT to 4-step RA-SDT after N times of MsgA transmission.

6. The fallbackRAR reception as legacy 2-step RACH is supported in 2-step RA-SDT, i.e., fallback from 2-step RA-SDT to 4-step RA-SDT when fallbackRAR is received 7. As legacy, UE can be configured to switch from 2-step RA-SDT to 4-step RA-SDT after N times of MsgA transmission

According to the agreements in RAN2 #115e meeting, in Slicing WI, some proposals on fallback are provided and these will aligned to the common RACH partitioning discussion decisions.

6 For RACH type selection, UE first selects between slice-specific and common RACH, then selects between 2-step and 4-step. 9 The following fallback case is supported: – Fallback case 2: Fallback from 2-step slice specific RACH to 4-step common RACH, if 4-step slice specific RACH is not configured. 10 The following fallback cases are not supported in this release: – Fallback case 1: Fallback from 4-step slice specific RACH to 4-step common RACH – Fallback case 3: Fallback from 2-step slice specific RACH to 2-step common RACH, if neither 4-step slice specific RACH nor 4-step common RACH is configured.

In our understanding, in RACH partitioning, if the both 2-step and 4-step feature/feature combination specific RACH is configured, it is reasonable to allow the fallback from 2-step feature/feature combination specific RACH to 4-step feature/ feature combination specific RACH. So we propose that:

Proposal 9: As legacy, the UE can be configured the fallback from 2-step feature/feature combination specific RACH to 4-step feature/ feature combination specific RACH.

Conclusion

Base on the above discussion, we make the following proposals:

Proposal 1: Carrier selection and BWP operation is performed before the feature combination selection for CE.

Proposal 2: Carrier selection and BWP operation is performed before the feature combination selection for RedCap and Slicing.

Proposal 3: Carrier selection and BWP operation is performed before the feature combination selection for SDT based on the current SDT MAC running CR.

Proposal 4: For Random Access procedure for SDT, UE should perform RA type selection based on the SDT specific RSRP threshold for RA type.

Proposal 5:  If the Random Access procedure is for Slicing, UE should adopt the Slicing specific prioritization parameter.

Proposal 6:  If the Random Access procedure is only for SDT or CovEnh, UE should adopt the SDT or CovEnh specific rsrp-ThresholdSSB or msgA-RSRP-ThresholdSSB.

Proposal 7:  If the Random Access procedure is for the feature combination including CovEnh and SDT, RAN2 to discuss the following solutions:

- Alternative 1: the network configures the priority for SDT, CovEnh or other features;

- Alternative 2: it is specified in MAC the priority for SDT, CovEnh or other features;

- Alternative 3: it is up to the UE implementation to select the proper value for rsrp-ThresholdSSB and msgA-RSRP-ThresholdSSB.

Proposal 8 The RA-RNTI/MSGB-RNTI collision in RACH partitioning can be left to network implementation to resolve it.

Proposal 9: As legacy, the UE can be configured the fallback from 2-step feature/feature combination specific RACH to 4-step feature/ feature combination specific RACH.

Reference

- RAN2#116-e meeting report.

- 3GPP TS 38.321 V16.4.0, Medium Access Control (MAC) protocol specification.

- [Post116-e][515] [RACH partitioning] MAC Procedure aspects (ZTE)

- RAN2#113-e meeting report.

- RAN2#115-e meeting report.

- R2-2200031, Running MAC CR for Small Data, Huawei, HiSilicon