3GPP TSG-RAN WG2 Meeting #116bis-e                                                	 R2-2200443

E-Meeting: January 17-25, 2022

Agenda item:	9.2.5

Source:	Qualcomm Incorporated

Title:	Discussion on UE capabilities

Document for:	Discussion and Decision

# Introduction

In NR NTN, the UE capability was discussed in [1], followings are left for further discussion.

- Whether to define additional UE capability (or IOT bit) for the existing TN features as they are not tested in NTN environment, e.g., a NTN capable dish-type UE does not support TN;

- Whether/how to indicate a UE only supports NGSO or a UE only supports GSO;

- Whether to use nonTerrestrialNetwork-r17 as the Prerequisite for other optional NR NTN UE capabilities;

- Whether to have separate UE capability bit if one essential NTN feature can also be used in TN.

For per UE capabilities, the UE may need to signal them separately in TN and NTN. This document provides the discussion focusing on the “per UE capability” structure, for example, how network knows whether the existing eMTC or NB-IoT UE capabilities are supported in the NTN network.

# Discussion

TN vs NTN

A single container for both NTN RAT and TN RAT can be used as many features such as EDT, data inactivity monitoring and release assistance indication (earlyData-UP-r15, dataInactMon-r14, rai-SupportEnh-r16) can be commonly supported by both RATs. But one issue is that how to indicate to network some existing per UE capabilities may not be supported in NTN or some per UE feature may not be used in NTN due to lack of testing.

For example, a NE UE may support features such as PUR (pur-Parameters-r16) and WUS (wakeUpSignal-r15) in TN access but may not support them in NTN access due to large RTT delay or requirement for time and frequency compensation. These UE capabilities may need to be signalled differently for TN and NTN access such that connected mode and idle mode mobility between TN and NTN works efficiently. Other such example includes SON/MDT feature which have not been discussed yet in the context of NTN and support of 5GC which may happen in future releases.

Therefore, in our understanding a simple solution is to use same EUTRAN container to report TN and NTN UE capabilities but the existing per UE capabilities, by default, should be applicable to both TN and NTN and additionally NTN differentiation (i.e., UE-EUTRA-CapabilityNTN-Mode-r17 whether a TN feature is implemented/tested in NTN) can be introduced such that UE capabilities can be signalled differently. An example is shown below for eMTC UEs where, for example, MAC-Parameters-NTN-r17 can group all the versions of the MAC-Parameters IEs and include only the IEs that are implemented/tested in NTN.

UE-EUTRA-CapabilityNTN-Mode-r17 ::=      SEQUENCE {

pdcp-Parameters-NTN-r17         PDCP-Parameters-NTN-r17        	OPTIONAL,

rlc-Parameters-NTN-r17          RLC-Parameters-NTN-r17        	OPTIONAL,

mac-Parameters-NTN-r17          MAC-Parameters-NTN-r17 			OPTIONAL,

phy-Parameters-NTN-r17          Phy-Parameters-NTN-r17 			OPTIONAL,

measAndMobParameters-NTN-r17    MeasAndMobParameters-NTN-r17    OPTIONAL

}

Similarly for WUS, the radio paging information can be extended where wakeUpSignalMinGap-eDRX-r15 and groupWakeUpSignalAlternation-r16 can be used commonly for TN and NTN.

-- ASN1START

UE-RadioPagingInfo-r12 ::=				SEQUENCE {

ue-Category-v1250						INTEGER (0)			OPTIONAL,

...,

[[	ue-CategoryDL-v1310					ENUMERATED {m1}		OPTIONAL,

ce-ModeA-r13						ENUMERATED {true}	OPTIONAL,

ce-ModeB-r13						ENUMERATED {true}	OPTIONAL

]],

[[	wakeUpSignal-r15					ENUMERATED {true}	OPTIONAL,

wakeUpSignal-TDD-r15				ENUMERATED {true}	OPTIONAL,

wakeUpSignalMinGap-eDRX-r15			ENUMERATED {ms40, ms240, ms1000, ms2000}		OPTIONAL,

wakeUpSignalMinGap-eDRX-TDD-r15		ENUMERATED {ms40, ms240, ms1000, ms2000}		OPTIONAL

]],

[[	ue-CategoryDL-v1610					ENUMERATED {m2}		OPTIONAL,

groupWakeUpSignal-r16				ENUMERATED {true}	OPTIONAL,

groupWakeUpSignalTDD-r16			ENUMERATED {true}	OPTIONAL,

groupWakeUpSignalAlternation-r16	ENUMERATED {true}	OPTIONAL,

groupWakeUpSignalAlternationTDD-r16	ENUMERATED {true}	OPTIONAL

]],

[[	wakeUpSignal-NTN-r17				ENUMERATED {true}	OPTIONAL,

groupWakeUpSignal-NTN-r17			ENUMERATED {true}	OPTIONAL

]]

}

-- ASN1STOP

Reporting TN vs NTN capabilities

As described above, if non-critical extension is used to include “UE-EUTRA-CapabilityNTN-Mode-r17”, we can further clarify how this works. the “UE-EUTRA-CapabilityNTN-Mode-r17” should include all the per-UE EUTRAN TN capabilities up to Rel-17 to be able to indicate support/not support in NTN because those features may be supported in TN but not implemented/tested in NTN.

Table 1 How to interpret the new extension UE-EUTRA-CapabilityNTN-Mode-r17.

| UE capability reporting | UE-EUTRA-CapabilityNTN-Mode-r17 | Remarks |
| --- | --- | --- |
| Does not include NTN | Not included | Existing per UE capabilities apply to TN (legacy behavior) |
| Includes NTN | Not Included | Existing per UE capabilities apply to both TN and NTN. |
|  | Included | Existing UE capability indication applies to TN (if supported). UE-EUTRA-CapabilityNTN-Mode-r17 applies to NTN. |

NTN-specific capabilities extension (UE-EUTRA-CapabilityNTN-Mode-r17) is introduced to signal the per-UE features (TN capabilities up to Rel-17) differently for NTN access.

UE-EUTRA-CapabilityNTN-Mode-r17 includes all the per-UE TN capabilities up to Rel-17.

Add clarification on reporting TN-only or both TN and NTN UE capabilities as shown in the Table 1.

# Conclusion

Following proposals are made.

Proposal 1	NTN-specific capabilities extension (UE-EUTRA-CapabilityNTN-Mode-r17) is introduced to signal the per-UE features (TN capabilities up to Rel-17) differently for NTN access.

Proposal 2	UE-EUTRA-CapabilityNTN-Mode-r17 includes all the per-UE TN capabilities up to Rel-17.

Proposal 3	Add clarification on reporting TN-only or both TN and NTN UE capabilities as shown in the Table 1.

# Reference

[1] R2-2200040, “Report of email discussion [Post116-e][111][NTN] UE capabilities (Intel)”, RAN2#116bis-e, January 17-25, 2022