3GPP TSG RAN WG2 Meeting #116bis-e                                                        	   R2-2109627

E-Meeting, 17th – 25th January, 2022

Agenda Item:	8.8.4

Source: 	Intel Corporation

Title:	UE capability for Slicing enhancement

Document for: 	Discussion/Decision

# Introduction

In the last meeting, it is used the following as baseline for further discussion.

- As baseline, consider the following capabilities. FFS on details, can consider changes in the next meeting.

- #1: UE indicates its support of slice based cell reselection in the UE capability signalling with the following TS38.306 description.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| sliceInfoforCellReselection-r17 Indicates whether the UE supports sliceInformation on RRC release for slice based cell reselection in RRC _IDLE and RRC INACTIVE as defined in TS 38.304 [21]. | UE | No | No | No |

- #2: Since slice based RACH is only applicable for UE in RRC IDLE and RRC INACTIVE, there is no need for explicit capability to inform network and should just be “Optional without UE capability” as follow under Section 5.4 Other features:

Definitions for feature

Slice based random access It is optional for UE to support slice based random access as specified in TS 38.321 [8].

In this contribution, we discuss the need of optional UE capability without capability signalling for slice based cell reselection and slice based RACH.

# Discussion

## Need of UE capability for slice based cell reselection

In the last meeting, RAN2 confirmed the agreement that was agreed in previous meeting in the SI phase to provide slice info not only via SIB but also signalled via dedicated signalling as follow:

- Agreements

- 2	Following is taken as the baseline for Solution Option 4:

- The “slice info” (for a single slice or slice group) agreed to be provided to the UE in the last RAN2 meeting using both broadcast and dedicated signaling are provided for the serving as well as neighboring frequencies. The following steps are used for slice based cell (re)selection in AS:

- Step 0: NAS layer at UE provides slice information to AS layer at UE, including slice priorities.

- Step 1: AS sorts slices in priority order starting with highest priority slice.

- Step 2: Select slices in priority order starting with the highest priority slice.

- Step 3: For the selected slice assign priority to frequencies received from network.

- Step 4: Starting with the highest priority frequency, perform measurements (same as legacy).

- Step 5: If the highest ranked cell is suitable (as defined in 38.304) and supports the selected slice in step 2 then camp on the cell and exit this sequence of operation; FFS: How the UE determines whether the highest ranked cell supports the selected slice.

- Step 6: If there are remaining frequencies then go back to step 4.

- Step 7: FFS: If the end of the slice list has not been reached go back to step 2.

- Step 8: Perform legacy cell reselection.

- 1: Solution Option 4 is selected for further work i.e., resolve the FFSs, send any required LSs and consequently start to draft specification CRs.

In the SI phase, the following is agreed which indicated that the dedicated signalling is the RRC Release message before the UE enters idle or inactive state:

- 1	For cell reselection scenario, RAN2 to agree the following:

- To assist cell reselection, RAN can broadcast the supported slice info of the current cell and neighbour cells, and cell reselection priority per slice. The slice info may be: providing only SST, on-demand SIB, SIB segmentation, slice grouping (if any), or slice associated UAC information where other solutions are not precluded. Details can be discussed in WI phase.

- 2	Agree on adding the slice info (with similar information as agreed slice info in SI message) in RRC release message. Details can be discussed in WI phase.

- 3	Not pursue the solution of adding the intended slice for MT access in slice specific cell (re)selection.

- 4	The following solutions are recommended for normative work:

- -	To assist cell reselection, RAN can broadcast the supported slice info of the current cell and neighbour cells, and cell reselection priority per slice

- -	adding the slice info (with similar information as agreed slice info in SI message) in RRC release message

As captured above, RAN2 had previously agreed that network can provide slice info in RRC Release message that is UE specific. There is also a general principle that gNB signalling to UE should not provide dedicated configuration that is not supported by the UE.  In order for the gNB to decide whether it can provide the slice info in the RRC release message, UE needs to inform the gNB whether it support slice based cell reselection and the usual place for such support indication is in the UE capability.

Proposal#1: RAN2 confirms that UE indicates its support of slice based cell reselection in the UE capability signalling with the following TS38.306 description for the case slice information is provided in the RRC Release.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| sliceInfoforCellReselection-r17 Indicates whether the UE supports sliceInformation on RRCRelease for slice based cell reselection in RRC _IDLE and RRC INACTIVE as defined in TS 38.304 [21]. | UE | No | No | No |

For the case slice information is provided in the SIB, it is optional for the UE to support and since it is for UE in RRC IDLE and RRC INACTIVE, there is no need for UE capability signalling.

Proposal#2: For the case slice information is provided in the SIB, it is optional for the UE to support and there is no need for UE capability signalling since it is for UE in RRC IDLE and RRC INACTIVE. It should just be specified as “Optional without UE capability” as follow under Section 5.4 Other features:

Definitions for feature

Slice based cell reselection using SIB It is optional for UE to support slice based cell reselection using sliceInformation in the SIB as specified in TS 38.304 [21].

## Need of UE capability for slice based RACH

RAN2 agreed that the slice based RACH is only applicable to idle and inactive state as follow during SI phase:

- Slice based RACH configuration can be applied to idle/inactive UE.

In RAN2#113bis, it is agreed to down-prioritize RRC Connected mode:

- 2: RAN2 will prioritize the discussion for slice specific RACH for IDLE and INACTIVE mode. And CONNECTED mode is down prioritized and can be considered if time allows.

Based on the above and in view there is only 2 more meetings left for Rel-17, it seems that Slice based RACH for Connected mode may not be introduced into Rel-17.  In view of this with slice based RACH applicable only to idle and inactive state for the UE, slice based RACH for Rel-17 is Optional without UE capability.

In slice based RACH, 2 sub-features are introduced:

- slice based RACH resource partitioning

- slice based RACH parameters prioritization

We do not see a need to separate the 2 sub-features.

Proposal#3: RAN2 confirm that there is no need for explicit capability to inform network since slice based RACH is only applicable for UE in RRC IDLE and RRC INACTIVE and should just be specified as “Optional without UE capability” as follow under Section 5.4 Other features:

Definitions for feature

Slice based random access It is optional for UE to support slice based random access as specified in TS 38.321 [8]. UE that supporting slice based random access supports the following features: - slice based RACH resource partitioning - slice based RACH parameters prioritization

# Conclusion

It is requested that RAN2 discussed the following observations and proposals:

Proposal#1: RAN2 confirms that UE indicates its support of slice based cell reselection in the UE capability signalling with the following TS38.306 description for the case slice information is provided in the RRC Release.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| sliceInfoforCellReselection-r17 Indicates whether the UE supports sliceInformation on RRCRelease for slice based cell reselection in RRC _IDLE and RRC INACTIVE as defined in TS 38.304 [21]. | UE | No | No | No |

Proposal#2: For the case slice information is provided in the SIB, it is optional for the UE to support and there is no need for UE capability signalling since it is for UE in RRC IDLE and RRC INACTIVE. It should just be specified as “Optional without UE capability” as follow under Section 5.4 Other features:

Definitions for feature

Slice based cell reselection using SIB It is optional for UE to support slice based cell reselection using sliceInformation in the SIB as specified in TS 38.304 [21].

Proposal#3: RAN2 confirm that there is no need for explicit capability to inform network since slice based RACH is only applicable for UE in RRC IDLE and RRC INACTIVE and should just be specified as “Optional without UE capability” as follow under Section 5.4 Other features:

Definitions for feature

Slice based random access It is optional for UE to support slice based random access as specified in TS 38.321 [8]. UE that supporting slice based random access supports the following features: - slice based RACH resource partitioning - slice based RACH parameters prioritization