3GPP TSG RAN WG2 Meeting #116bis-e	R2-2200579

Electronic meeting, January 17th – January 25th, 2022

Agenda item:	8.1.3.3

Source:	TD Tech, Chengdu TD Tech

Title:	UE capabilities for NR MBS

Document for:	Discussion and Decision

- Introduction

The work item “NR Multicast and Broadcast Services” [1] was agreed in RAN#88-e to provide MBS in a cell. In the contribution, the UE capabilities for NR MBS are discussed and the related proposals are suggested.

2 UE capabilities for NR MBS

At present, a separate multicast capability and a separate broadcast capability are agreed to respectively indicate that UE support multicast sessions and broadcast sessions. Based on the agreement, the following UE capabilities are suggested.

- The maximum value of the G-RNTIs/G-CS-RNTIs supported by UE for multicast sessions

- The maximum value of the G-RNTIs/G-CS-RNTIs supported by UE for broadcast sessions

- The maximum value of the G-RNTIs/G-CS-RNTIs supported by UE for MBS sessions if multicast sessions and broadcast sessions can share the G-RNTIs/G-CS-RNTIs

- The maximum number of MRBs supported by RRC_IDLE/RRC_INACTIVE UE for broadcast sessions

- The maximum number of DRBs and MRBs supported by RRC_CONNECTED UE for unicast sessions, multicast sessions and broadcast sessions

- The maximum number of the FDMed PDSCHs in a timeslot supported by RRC_CONNECTED UE for unicast sessions, multicast sessions and broadcast sessions

- The maximum number of the FDMed PDSCHs in a timeslot by RRC_IDLE/RRC_INACTIVE UE for broadcast sessions

Based on the above discussion, the following proposal is suggested.

Proposal 1: Discuss the following UE capabilities for NR MBS.

- The maximum value of the G-RNTIs/G-CS-RNTIs supported by UE for multicast sessions

- The maximum value of the G-RNTIs/G-CS-RNTIs supported by UE for broadcast sessions

- The maximum value of the G-RNTIs/G-CS-RNTIs supported by UE for MBS sessions if multicast sessions and broadcast sessions can share the G-RNTIs/G-CS-RNTIs

- The maximum number of MRBs supported by RRC_IDLE/RRC_INACTIVE UE for broadcast sessions

- The maximum number of DRBs and MRBs supported by RRC_CONNECTED UE for unicast sessions, multicast sessions and broadcast sessions

- The maximum number of the FDMed PDSCHs in a timeslot supported by RRC_CONNECTED UE for unicast sessions, multicast sessions and broadcast sessions

- The maximum number of the FDMed PDSCHs in a timeslot by RRC_IDLE/RRC_INACTIVE UE for broadcast sessions

3 Conclusion

Based on the discussion in the above section, the following proposal is suggested.

Proposal 1: Discuss the following UE capabilities for NR MBS.

- The maximum value of the G-RNTIs/G-CS-RNTIs supported by UE for multicast sessions

- The maximum value of the G-RNTIs/G-CS-RNTIs supported by UE for broadcast sessions

- The maximum value of the G-RNTIs/G-CS-RNTIs supported by UE for MBS sessions if multicast sessions and broadcast sessions can share the G-RNTIs/G-CS-RNTIs

- The maximum number of MRBs supported by RRC_IDLE/RRC_INACTIVE UE for broadcast sessions

- The maximum number of DRBs and MRBs supported by RRC_CONNECTED UE for unicast sessions, multicast sessions and broadcast sessions

- The maximum number of the FDMed PDSCHs in a timeslot supported by RRC_CONNECTED UE for unicast sessions, multicast sessions and broadcast sessions

- The maximum number of the FDMed PDSCHs in a timeslot by RRC_IDLE/RRC_INACTIVE UE for broadcast sessions

4 References

- RP-201038, Revised Work Item on NR Multicast and Broadcast Services, RAN Meeting #88-e.

- R2-2008929, Discussion on the protocol stack for NR MBS, RAN2#112-e, Chengdu TD Tech, TD Tech

- R2-2108035, Discussion on notifications for NR MBS, RAN2#115-e, Chengdu TD Tech, TD Tech