3GPP TSG-RAN WG2 Meeting #116bis-e	R2-2200700

Electronic, 17 – 25 January 2022

Agenda item:	8.17.2

Source: 	Qualcomm Incorporated

Title:  	Configuration and procedures for ICBM and mTRP

Document for:	Discussion and decision

# Introduction

RAN2#114e has discussed inter-cell multi-TRP and L1/L2 mobility as part of the FeMIMO WI [1]. In regards to the RAN1 LS on L1/L2 mobility [2], RAN2 has considered and agreed on two models, called Scenario 1 and 2. The reply LS to RAN1 [3] included these agreements and also noted that the necessary work on these will require significant RAN2 work.

The scope of the WI [3] was re-evaluated at RAN#92 and it was agreed to restrict Rel-17 work to only Scenario 1.

RAN2#115-e has discussed inter-cell Beam Management (BM) and mTRP. There were many questions raised regarding the details of these two topics and their commonality and differences. In the end, an LS was sent to RAN1 asking questions for clarifications [4]. A reply LS from RAN1 [5] has provided responses to RAN2 questions.

RAN2#116e has made several agreements, while considering RAN1 responses.

In this document, we will discuss further details of inter-cell BM and mTRP.

# Discussion

RAN2#114e agreements related to Scenario 1 for the inter-cell mTRP is as follows:

RAN2 confirm the simplified procedures on the inter-cell multi-TRP-like model as a baseline RAN2 understanding:

- Scenario 1: Inter-cell multi-TRP-like model

- 1. UE receives from serving cell, configuration of SSBs of the TRP with different PCI for beam measurement, and configurations needed to use radio resources for data transmission/reception incl resources for differet PCI.

- 2. UE performs beam measurement for the TRP with different PCI and report it to serving cell.

- 3. Based on the above reports, TCI state(s) associated to the TRP with different PCI is activated from the serving cell (by L1/L2 signaling).

- 4. UE receives and transmits using UE-dedicated channel on TRP with different PCI.

- 5. UE should be in coverage of a serving cell always, also for multi-TRP case, e.g. UE should use common channels BCCH PCH etc. from the serving cell (as in legacy).

R2 assumes that for both multi-TRP and mobility scenarios, single protocol stack can be assumed (intra-DU)

R2 didn’t see a problem with using different C-RNTIs for different cells. Different C-RNTI seems more natural in a mobility scenario. No conclusion in R2 for mTRP scenario.

RRC configurations of the cells for L1/L2 centric mobility, including C-RNTI, are configured by RRC.

RAN2 prefer to restrict the scope of the deployment only for intra-DU case in Rel-17.

RAN2 assumes to prioritize intra-frequency case in Rel-17, but RAN2 follows the RAN4 decision to support inter-frequency case.

RAN2#115-e had further discussion and made the following agreements:

  - FFS whether common framework is feasible to support both “inter-cell beam management” and “inter-cell multi-TRP” considering differences/similarities between two operations.

  - R2 assumes at least TCI state information is required for TRP with different PCI.

  - R2 further discuss RRC parameters based on RAN1 RRC parameters andor R1 reply LS.

  - At R2 115-e the following RRC models is/were on the table: Option 1: Cell, Option 2: BWP, Option 3: beam resource (e.g. TCI state, QCL-info), Option 4: new structure (on high level similar to either of the other options)

RAN2#116-e has made good progress with these agreements:

1a: RAN2 to use the terminology "primary TRP (pTRP)" and "additional TRP (aTRP)" for RAN2 discussion purposes. FFS whether these will really be needed in Stage-2/3 specifications.

1b: RAN2 does not consider RLM for aTRP in Rel-17 work

2a: No RRM enhancements are done in Rel-17 (unless later found critical to the functionality).

2b: Add SSB/PCI information for ICBM as cell-level information and link unified TCI state information to that. FFS on exact Stage-3 details.

2c: RAN2 starts the RRC CR work based on latest RAN1 input before sending general RRC LS to RAN1.

3: The RAN1 parameters for "MultiBeam" are only applicable to ICBM with unified TCI framework (i.e. not to mTRP). Discuss further in Stage-3 phase how the UL PC configuration parameters are defined.

4: Rel-17 MPE configuration can be included in PHR-Config. Will ask R1 whether MPE information can apply to both ICBM and mTRP

6: RAN2 assumes "mTRP" parameters are not for ICBM and starts Stage-3 work based on that assumption. If ambiguities are found, LS can be sent to RAN1 to ask for clarification from next meeting.

7: RAN2 will use one RRC CR for the FeMIMO WI and start the work in post-meeting email discussion. Can discuss RRC structure during the discussion before going for final Stage-3 details.

The first difference between inter-cell BM and mTRP is stated in the RAN1 reply LS [5] as follows:

Answer 1: Rel17 Inter-cell BM and inter-cell mTRP have common points but they are not entirely the same. The common and different points are as follows: they both use the same beam measurement/reporting mechanisms but they have different TCI signaling framework (beam indication) as inter-cell BM is based on Rel17 unified TCI while inter-cell mTRP is based on Rel15/16 TCI framework. For inter-cell BM, UE assumes that the UE-dedicated channels/RSs can be switched to a TRP with different PCI according to DCI/MAC-CE based unified TCI update; for inter-cell mTRP, UE assumes mDCI-mTRPbased multi-PDSCH reception.

Based on this, we can conclude:

Proposal 1: RAN2 design on MAC CE for unified TCI applies to only inter-cell BM.

A second question to RAN1 was regarding simultaneous tx and rx are supported. RAN1 respond as follows:

Answer 2.h: It is correct understanding that simultaneous Rx in DL is not supported for inter-cell BM but supported for inter-cell mTRP, while simultaneous Tx in UL is not supported for both. From configuration perspective, regarding the last question, inter-cell BM will be supported based on the unified TCI framework to be introduced in Rel-17 so relevant Rel-17 TCI configuration parameters will be required to enable this feature. Meanwhile inter-cell mTRP feature is to extend Rel-16 multi-DCI mTRP functionality to TRPs with different PCI so that its configuration parameters will be same or similar to those defined for Rel-16 multi-DCI mTRP operation.

This response implies that the UE will not be receiving simultaneously on more than one DL beams for inter-cell BM. However, since the UE will be monitoring both sets of beam, it seems logical that the ongoing RAN2 design on TRP specific BFR can also be applicable to inter-cell mTRP.

Proposal 2: RAN2 should confirm that TRP specific BFR is also applicable to inter-cell BM.

The main issue from RAN2 perspective is how the “TRP with different PCI” should be configured and whether it is more similar to a serving cell or not.

RAN1 did not provide a list of configuration parameters which may be different for the TRP with different PCI as they stated that “RAN1 has not discussed or concluded to provide configuration parameter(s) for TRP with different PCI”. However, they do provide a response for the configuration of PxCH where they state the following:

Answer 5: There is only one physical layer configuration and that is applied to all the PUSCH/PUCCH/PDSCH/PDCCH associated with TCI state that is associated with either serving cell PCI or another different PCI. Regarding the PRACH transmission, RAN1 has not discussed configuration of PRACH for a TRP with different PCI.

Proposal 3: Based on RAN1 input, RAN2 should assume that both the serving cell and the TRP with different PCI will share a common PxCH configuration for both inter-cell BM and mTRP.

In RAN2#115-e, when modeling of the TRP with a different PCI was discussed, there were two main approaches:

1-) Broadening the SCell to include inter-cell TRP

2-) Defining the inter-cell TRP as a different cell with some different properties than a legacy SCell.

Both options can work solely for the Control Plane aspects, i.e. the configuration, measurement reporting and beam management. However, a critical difference lies in the User Plane.

RAN1 has the following responses for the HARQ entity and processes:

Question 4: How does the HARQ operation work with the multi-beam operation? In particular:

a) HARQ entity: Is there a single HARQ entity handling both the serving cell TRP and TRP with different PCI?

Answer 4.a: RAN1 assumes a single HARQ entity is used for both the serving cell TRP and TRP with different PCI.

b) HARQ retransmissions: Can retransmission occur from different TRP than initial transmission for the same HARQ process? E.g. can initial transmission be done from serving cell TRP and retransmission from TRP with different PCI?

Answer 4.b: Due to assumption on the same HARQ entity, it is possible to have initial transmission and re-transmission originating from TRPs with different PCIDs.

Based on these responses, RAN2 should also assume:

Proposal 4: Based on RAN1 input, RAN2 should assume that a single HARQ entity is used for the serving cell and the associated TRP with different PCI and HARQ re-transmissions across the TRPs are supported.

This condition makes it very difficult to model the TRP with different PCI as a different cell. In MAC, HARQ entity is per cell. Therefore, significant changes will be needed to share a HARQ entity between two different cells, even if they operate on the same frequency.

Observation 1: Sharing a single HARQ entity between two separate cells will require signification changes to MAC specification.

RAN2#116-e has agreed that the SSB/PCI information for ICBM will be added to the serving cell information as follows:

2b: Add SSB/PCI information for ICBM as cell-level information and link unified TCI state information to that. FFS on exact Stage-3 details.

For mTRP, a similar conclusion can be made and TRP with different PCI can be modeled as part of the serving cell. This implies that a grant for this serving cell can be used for either TRP.

Proposal 5: As a baseline, TRP with different PCI is modeled as part of the associated serving cell.

There was also a question in RAN2 LS on the number of TRPs with different PCI associated with a serving cell. RAN1 response states that:

RAN1 is still discussing the maximum number of RRC configured PCIs different from the serving cell for TCI beam indication, measurement and reporting and has made the following agreements

However, from the related RAN1 agreements, we can see that the UE will be using only one TRP with different PCI at any time. Therefore, this maximum number only affects the size of the RRC configuration and the associated indices, e.g. used for MAC CEs.

Observation 2: The maximum number of RRC configured TRPs with different PCIs only impacts the stage-3 details of configuration since there is at most one active TRP with different PCI.

RAN1 also responded that “inter-cell beam management and inter-cell mTRP can be applicable to any serving cell (i.e. PCell/PSCell/SCell).”. Therefore, RAN2 should take this into account in the design.

Proposal 6: RAN2 should assume that any legacy serving cell (SpCell or SCell) can have an associated TRP with different PCI.

# Conclusion

In this document, we have discussed inter-cell BM and mTRP and propose the following:

Proposal 1: RAN2 design on MAC CE for unified TCI applies to only inter-cell BM.

Proposal 2: RAN2 should confirm that TRP specific BFR is also applicable to inter-cell BM.

Proposal 3: Based on RAN1 input, RAN2 should assume that both the serving cell and the TRP with different PCI will share a common PxCH configuration for both inter-cell BM and mTRP.

Proposal 4: Based on RAN1 input, RAN2 should assume that a single HARQ entity is used for the serving cell and the associated TRP with different PCI and HARQ re-transmissions across the TRPs are supported.

Observation 1: Sharing a single HARQ entity between two separate cells will require signification changes to MAC specification.

Proposal 5: As a baseline, TRP with different PCI is modeled as part of the associated serving cell.

Observation 2: The maximum number of RRC configured TRPs with different PCIs only impacts the stage-3 details of configuration since there is at most one active TRP with different PCI.

Proposal 6: RAN2 should assume that any legacy serving cell (SpCell or SCell) can have an associated TRP with different PCI.

# References

[1] RP-211586, Revised WID: Further enhancements on MIMO for NR, Samsung

[2] R1-2102248, LS on TCI State Update for L1/L2-Centric Inter-Cell Mobility

[3] R2-2106787, Reply LS on TCI State Update and L1/L2-Centric Mobility, Samsung

[4] R2-2108925, LS on inter-cell beam management and multi-TRP in Rel-17, Nokia

[5] R1-2110631, LS Reply on inter-cell beam management and multi-TRP in Rel-17, Nokia