3GPP TSG-RAN WG2 Meeting #116bis electronic	R2-2200764

Online, January 17 - 25, 2022

Agenda item:		8.10.2.1

Source:		Lenovo, Motorola Mobility

Title:		Further discussion on TA reporting in NTN

Document for:		Discussion

- Introduction

The last RAN2#116e meeting made the following agreements regarding TA reporting in NTN:

- Do not mandate Msg3/MsgA or Msg5 to include TA report MAC CE, and whether it can be included depends on the TB size of Msg3/MsgA or Msg5. No spec change is needed for this - Reserved LCID is used for the TA report MAC CE. - Postpone the discussion on the size of the TA report MAC CE until RAN2 concludes on the content of TA report. - RAN2 do not pursue any enhancements to allow inclusion of TA information without extending Msg3 size. - Logical channel priority of the TA report MAC CE should be lower than that of “C-RNTI MAC CE or data from UL-CCCH” and higher than that of “data from any Logical Channel, except data from UL-CCCH”. - RAN2 further discuss the exact priority of the TA report MAC CE between “C-RNTI MAC CE or data from UL-CCCH” and “MAC CE for BSR, with exception of BSR included for padding - If the reported content of information about UE specific TA is TA pre-compensation value in connected mode, MAC CE is used to report. - In case UE location information can be reported to network, dedicated signaling is used to configure UE to report the UE location and/or the UE specific TA information for the purpose of TA reporting in connected mode. FFS if both mechanisms are needed in parallel.

This contribution further discusses the contents of TA reporting during RACH and in connected mode.

- Discussion

TA reporting during RACH

In the email discussion for RACH aspects at RAN2#116e, the contents of TA reporting were restricted to the following 2 options according to most companies’ opinions:

- Option 1: Full TA (i.e., T_TA as defined in the UE’s TA formula)

- Option 2: UE’s service link TA (i.e., NTA, UE-specific as defined in the UE’s TA formula)

For Option 1 the supporting companies consider that it is simple and straight-forward. However the full TA includes the common TA part that UE calculates from the parameters (common TA at an epoch time, common TA drift rate and common TA drift rate variation), which is already known at network side. Besides, the common TA at network side is more accurate than the common TA calculated at UE due to the large propagation delay. It is inefficient to include the common TA part in the TA reporting during RACH.

Another advantage of Option 2 is in reducing the required bit of TA reporting, or refining the granularity of TA reporting with the same size limit compared to Option 1, especially considering that it has been agreed that TA reporting is via MAC CE in Msg3 or Msg5.

Observation 1: Reporting UE’s service link TA instead of the full TA is more efficient in reducing overhead or increasing accuracy.

Besides, as the service link TA is twice of the propagation delay from UE to the serving satellite, reporting the propagation delay could be of less size or higher accuracy.

Proposal 1: The TA reporting in Msg3 or Msg5 via MAC CE is the UE’s service link TA or UE’s service link propagation delay.

TA reporting in connected mode

For TA reporting in connected mode, as SA3 has replied in S3-214103 that “SA3 recommends that RAN2 defines a solution that avoids sending unprotected UE location information to the gNB”, we focus on sending the TA or delay to gNB instead of location information in the following discussion.

Similarly to the TA reporting during RACH, as the common TA is already known and could be more accurate at network, reporting the UE calculated common TA in connected mode is unnecessary and reporting the UE’s service link TA is sufficient. From the perspective of reducing overhead or increasing accuracy for TA reporting using MAC CE, the following enhanced options can be considered:

- Option A: UE’s service link propagation delay, which is half of the service link TA

- Option B: The difference value of UE’s service link TA or propagation delay compared to the last reporting (e.g. TA reporting during RACH)

Proposal 2: The TA reporting in connected mode via MAC CE is the UE’s service link TA, or UE’s service link propagation delay, or the difference value compared to the last reporting.

As the UE assistance information for measurement window/gap configuration in NTN (e.g. propagation delay from UE to a neighbour satellite or propagation delay difference from UE to a serving and a neighbour satellite) is also being discussed, the TA reporting in connected can be part of the assistance information towards a unified solution for TA or delay reporting.

Proposal 3: The TA reporting in connected mode can be included in the UE assistance information for measurement window/gap configuration.

- Conclusion

In this contribution, we further discuss the contents of TA reporting during RACH and in connected mode. The following observation is given:

Observation 1: Reporting UE’s service link TA instead of the full TA is more efficient in reducing overhead or increasing accuracy.

And we propose:

Proposal 1: The TA reporting in Msg3 or Msg5 via MAC CE is the UE’s service link TA or UE’s service link propagation delay.

Proposal 2: The TA reporting in connected mode via MAC CE is the UE’s service link TA, or UE’s service link propagation delay, or the difference value compared to the last reporting.

Proposal 3: The TA reporting in connected mode can be included in the UE assistance information for measurement window/gap configuration.