3GPP TSG-RAN WG2 Meeting #116bis-e	R2-2200976

Electronic meeting, 17th – 25th January 2022

Source: 	Huawei, HiSilicon

Title: 	Discussion on UE capabilities for RAN slicing

Agenda Item:	8.8.4	UE capabilities

Document for:	Discussion and decision

- Introduction

During the last RAN2#116-e meeting [1] , agreements were achieved for UE capabilities related to RAN2-defined features for RAN slicing.

As baseline, consider the following capabilities. FFS on details, can consider changes in the next meeting.

#1: UE indicates its support of slice based cell reselection in the UE capability signalling with the following TS38.306 description.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| sliceInfoforCellReselection-r17 Indicates whether the UE supports sliceInformation on RRC release for slice based cell reselection in RRC _IDLE and RRC INACTIVE as defined in TS 38.304 [21]. | UE | No | No | No |

#2: Since slice based RACH is only applicable for UE in RRC IDLE and RRC INACTIVE, there is no need for explicit capability to inform network and should just be “Optional without UE capability” as follow under Section 5.4 Other features:

Definitions for feature

Slice based random access It is optional for UE to support slice based random access as specified in TS 38.321 [8].

In this contribution, the discussion focuses primarily on the details of UE capabilities required for slice based cell reselection.

- Discussion

For the UE capability bit sliceInfoforCellReselection-r17, currently it refers to RRC release case. So there may be two understandings:

- It only includes RRC release case, and a separate UE capability bit for supporting slice info on SIB would be needed

- It includes both RRC release and SIB cases, i.e. the UE supporting this capability bit shall also support slice info on SIB

Between (1) and (2), we slightly prefer (1) because it is flexible for UEs, i.e. the UE can decide to implement slice info for cell reselection for RRC Release and/or SIB. Following (1), some clarifications are needed, so we propose:

Proposal 1: It is proposed to introduce a new UE capability bit for supporting slice info on SIB, and it is optional without UE capability parameter, e.g. sliceInfoforCellReselectionOnSIB-r17.

Proposal 2: The name of the previous UE capability bit sliceInfoforCellReselection-r17 is changed to focus on RRC release case, e.g. sliceInfoforCellReselectionOnRRCRel-r17.

For slice related UE capability bits, it is also required to discuss the following aspects:

- Need of FDD/TDD differentiation

- Need of FR1/FR2 differentiation

We think slice related UE capability bits are mainly about cell reselection and RACH procedure, and it seems no strong needs to have FDD/TDD and FR1/FR2 differentiations.

Proposal 3: For new UE capabilities related to slice, it is proposed RAN2 to discuss whether to have FDD/TDD differentiation and FR1/FR2 differentiation.

- Conclusion

In this contribution, we mainly discuss the UE capabilities for RAN slicing, and it is proposed:

Proposal 1: It is proposed to introduce a new UE capability bit for supporting slice info on SIB, and it is optional without UE capability parameter, e.g. sliceInfoforCellReselectionOnSIB-r17.

Proposal 2: The name of the previous UE capability bit sliceInfoforCellReselection-r17 is changed to focus on RRC release case, e.g. sliceInfoforCellReselectionOnRRCRel-r17.

Proposal 3: For new UE capabilities related to slice, it is proposed RAN2 to discuss whether to have FDD/TDD differentiation and FR1/FR2 differentiation.

- References

- Draft_R2-116e_Meeting_Report_v3

- R2-2110649, Discussion on slice related UE capabilities, Huawei, HiSilicon

- 3GPP TS 38.306 V16.7.0, User Equipment (UE) radio access capabilities (Release 16).