3GPP TSG-RAN WG2 Meeting #116-bis                                          R2- 22 01217

Electronic, Juanuary 17 – 25, 2022

Agenda item:		8.6.3

Source:		Sharp

Title:	 RNA Update during SDT

WID/SID:	       NR_SmallData_INACTIVE

Document for:		Discussion and Decision

- Introduction

In this contribution, we discussed the issue how UE to handle RANU during the SDT.

- Discussion

Currently, the UE released to RRC_INACTIVE may be configured with t380 which will trigger the UE to perform RNA update procedure when T380 expires as showed below[1]:

TS38.331 #### 5.3.13.8 RNA update In RRC_INACTIVE state, the UE shall: 1> if T380 expires; or 1> if RNA Update is triggered at reception of SIB1, as specified in 5.2.2.4.2: 2> initiate RRC connection resume procedure in 5.3.13.2 with resumeCause set to rna-Update; 1> ……

Observation 1 RRC connection resume procedure is initiated when T380 expires.

After UE “Shaking hands” with gNB, e.g., by sending RRCResumerequest/RRCConectionReqest and receiving RRCSetup/RRCResume/RRCRelease, stops the timer T380. During the procedure, gNB identifies UE and knows that UE is in its RNA.

In SDT, there is a period of time used for SDT after RRCResumerequest is sent but before RRCRelease message is received. In this situation, there may be a case that the SDT is on-going but T380 expires which will trigger UE to initiate a RRC connection resume procedure to notify the network that the UE is in the RAN area according the current specification. Considering that UE has already identified by the network during the SDT procedure even if RRCRelease message has not been received. So, in this case RNA update should be avoided.

Proposal 1 RNA update triggered by expires of T380 during SDT procedure should be avoided when UE has already identified by the network.

- Conclusion

In this section, we summarized our observation and proposal as showed below:

Observation 1 RRC connection resume procedure is initiated when T380 expires.

Proposal 1 RNA update triggered by expires of T380 during SDT procedure should be avoided when UE has already identified by the network.

- References

TS38.331-g70.